3GPP TSG-RAN WG1 Meeting #92	R1-1801502

Athens, Greece 26th Feb – 2nd Mar 2018

Agenda Item:	6.2.6.6.3

Source: 	Ericsson

Title:  	DL/UL common aspects of TDD for NB-IoT

Document for:	Discussion and Decision

# Introduction

According with the Work Item Description (WID) on further NB-IoT enhancements [1], revised in [2] and recently in [3] (text in a different font colour), one of the objectives refers to work on the support of the TDD operation into NB-IoT, which commenced from RAN #76.

B. Work on the following objective to commence from RAN#76

Support for TDD [RAN1, RAN2, RAN4]

Specify TDD support for in-band, guard-band, and standalone operation modes of NB-IoT. The design shall assume no UL compensation gaps are needed by UE, and strive towards a common design among the deployment modes.

- Relaxations of MCL and/or latency and/or capacity targets to be considered by RAN1.

- Baseline is to support the same features as Rel-13 NB-IoT, additionally considering small-cells scenarios.

- In addition to the baseline, support the following:

  - Based on Rel-14 FDD designs:

    - OTDOA positioning using Rel-14 NPRS RE patterns and sequences. Subframe configurations Part A and Part B shall be used with necessary amendments, if any.

    - Non-anchor carrier operation for paging and random access

    - UE category NB2, with the same TBS table as FDD, and support for 1 and 2 UL/DL HARQ processes. The support of 2 UL/DL HARQ processes by UE is an optional capability available to Cat NB2, i.e. same way as FDD.

  - Non-anchor carrier operation for system information (MIB-NB and any SIB-NB) can be considered.

- Specify band specific requirements for band 41.

This contribution deals with common aspects for the support of TDD NB-IoT, continuing the discussions on the better suitability that some LTE TDD configurations the physical channels and signals use by NB-IoT, as well as the potential usage of UpPTS, and DwPTS in TDD NB-IoT, bitmaps, and common aspects associated with HARQ and multi-carrier support. The document analyzing downlink (DL) and uplink (UL) aspects can be found in [4-5].

# Background

In RAN1 #90bis, the TSG RAN WG1 focused their discussions in downlink and uplink aspects for which several agreements were reached [6].

One of the topics treated in RAN1 #90bis, which touches upon common aspects of TDD NB-IoT, refers to the support only certain LTE TDD configurations. In relation to it, from the discussions held on downlink aspects the following agreements were reached:

Agreements:

- TDD UL:DL configuration 0 is not supported in TDD NB-IoT in Rel-15

Working assumption:

- TDD UL:DL configuration 6 is not supported in TDD NB-IoT in Rel-15

In this contribution, we continue the analysis on the TDD configurations that seem to be more suitable for supporting the signals and physical channels as they were designed for NB-IoT.

Other topics that have been preliminary discussed in RAN1 without reaching any agreement yet, refer to the potential usage of the special subframes for TDD NB-IoT, as well as the potential need of defining bitmaps, which are topics also addressed in this contribution.

Finally, HARQ and multicarrier aspects are discussed, for example on the possibility for a UE to be allowed to perform an UL transmission while there is a DL reception still ongoing (A/N), and the ability to perform cross carrier scheduling, respectively.

# Common aspects on the TDD support into NB-IoT

This section analyses first the LTE TDD configurations within the scope of NB-IoT TDD. Then, other common aspects such as the usage of the UpPTS, and DwPTS, bitmaps, HARQ aspects, and the support of multi-carrier are addressed.

## TDD configurations

### UL & DL in the TDD configurations

In a TDD operation, the downlink and uplink radio resources have been made to coexist within the same radio frame, being the switching between downlink and uplink performed during a guard period (GP) contained within a special subframe [3]. Table 1 shows the existing LTE TDD configurations as described by the LTE standard [7].

Table 1 Uplink-downlink TDD configurations

| Uplink-downlink configuration | Downlink-to-Uplink Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  | Number of subframes / frame |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | DL | UL | S |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U | 2 | 6 | 2 |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D | 4 | 4 | 2 |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D | 6 | 2 | 2 |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D | 6 | 3 | 1 |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D | 7 | 2 | 1 |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D | 8 | 1 | 1 |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D | 3 | 5 | 2 |

D: Downlink; U: Uplink; S: Special Subframe

The existing LTE TDD configurations count with a well-defined and commercially used framework. Therefore, in our view the existing LTE TDD configurations should be used as a baseline for introducing the TDD support into NB-IoT. This is especially important for “in-band” and “guard-band” deployments, but relevant as well for “stand-alone” operation mode if we consider that there might be two variants: 1.- In the same band and 2.- far away from the mobile broadband.

Proposal 1: Use the supported LTE TDD configurations in all deployment modes (i.e., in-band, guard-band, and stand-alone) of TDD NB-IoT.

In terms of resource utilization, one important aspect to be taken into consideration is that the physical channels and signals as they were designed for NB-IoT make a significant use of the allocated resources in the time domain, which makes that some TDD configurations result to be too limiting or restrictive for the support of TDD into NB-IoT. In RAN1 #90bis, it was agreed that the “TDD UL:DL configuration 0 is not supported in TDD NB-IoT in Rel-15,” while the following statement “TDD UL:DL configuration 6 is not supported in TDD NB-IoT in Rel-15” remains as a Working assumption [6].

For the TDD configuration #6 there are only three DL subframes per radio frame, meaning that the resources are hardly sufficient for carrying NPSS, NSSS, and NPBCH with no room or almost no room (subframe #0 is also used by SIB1-NB ) for other channels, if only one DL carrier is configured. Thus, if the Working assumption is not confirmed, meaning that TDD configuration #6 will indeed be supported for TDD NB-IoT, one of the consequences/implications is that there are no DL subframes left for NPDCCH/NPDSCH or for other signals such as NPRS, MBSFN, or eIMTA. In RAN1#90bis, the substantial number of UL subframes in TDD configuration #6 was highlighted, which was mentioned to be beneficial for IoT systems that are UL heavy. However, the UL traffic needs to be supported by DL scheduling (even with semi-persistent scheduling (SPS)), and as mentioned earlier for TDD configuration #6 (depicted at the bottom of Figure 1) there are no DL subframes left to be used for NPDCCH or NPDSCH.

Figure 1 Resource mapping for the TDD configurations on the anchor carrier based on the agreements that have been reached until RAN1 #90bis

Moreover, in terms of uplink the TDD configuration #6 also imposes some complications because it counts with an asymmetrical number of UL subframes in each half of a radio frame, which may for NPRACH require either using two different NPRACH formats within a radio frame, or in some cases adding rules for using one NPRACH format at a time within a radio. Based on the difficulties that the TDD configuration #6 implies for both, UL and DL, it seems to be better to confirm the working assumption saying “TDD UL:DL configuration 6 is not supported in TDD NB-IoT in Rel-15”.

Proposal 2: In Rel-15 prioritize LTE TDD configurations having a more adequate number of UL and DL subframes (e.g., TDD configuration #1, #2, #3, #4).

### Special Subframes in the TDD configurations

As shown Table 1 there are seven possible ways in which the TDD operation can be configured, three of them (i.e., configuration #3, #4, and #5) have a Downlink-to-Uplink switching periodicity equal to 10ms, meaning that there is only one “special subframe” per every radio frame. While, all the other TDD configurations (i.e., configuration #0, #1, #2, and #6) use a Downlink-to-Uplink switching periodicity equal to 5ms, where there are two “special subframes” per every radio frame.

A special subframe includes three fields DwPTS (Downlink Pilot Time Slot), GP (Guard Period), and UpPTS (Uplink Pilot Time Slot) of variable length, which are subject to a total length equal to[7]. Figure 2 depicts the structure of a special subframe.

Figure 2 Fields carried over the special subframe

Table 2 shows the available special subframe configurations for normal cyclic prefix in both uplink and downlink.

Table 2 Configuration of special subframe (lengths of DwPTS/GP/UpPTS)

| Special subframe configuration | Normal cyclic prefix in downlink |  | Number of OFDM symbols/subframe |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DwPTS | UpPTS (Normal cyclic prefix in uplink) | DwPTS | GP | UpPTS |
| 0 |  | 2192∙Ts | 3 | 10 | 1 |
| 1 |  |  | 9 | 4 | 1 |
| 2 |  |  | 10 | 3 | 1 |
| 3 |  |  | 11 | 2 | 1 |
| 4 |  |  | 12 | 1 | 1 |
| 5 |  | 4384∙Ts | 3 | 9 | 2 |
| 6 |  |  | 9 | 3 | 2 |
| 7 |  |  | 10 | 2 | 2 |
| 8 |  |  | 11 | 1 | 2 |
| 9 |  |  | 6 | 6 | 2 |
| 10 |  | 13152∙Ts | 6 | 2 | 6 |

In LS reply from RAN4 (R1-1719322) , it states that

“Considering NB-IoT UEs should be low complexity, RAN4 further discussed the minimum time for DL-to-UL and UL-to-DL switching on one NB-IoT carrier for TDD NB-IoT UEs and evaluated it to be equal to 40 s for both direction.”

Notice that in LTE, the useful symbol time is ~ 66.7 s. For normal mode, the first symbol in a slot has a cyclic prefix (CP) of ~5.2 s  and the remaining six symbols have a CP of  ~4.7 s. The reply from RAN4 implies that the LTE special subframe configurations with more than 1 guard symbol can be supported in NB-IoT. However, how to use the special subframes needs to be further discussed.

Proposal 3:  Special subframe configurations with more than 1 guard symbol is supported in NB-IoT.

In [8], it was shown how the UL and DL capacity can slightly be increased when the available resources in UpPTS and DwPTS are utilized. Nonetheless, and contrarily to a conventional LTE system, NB-IoT transmissions can make use of a large number of repetitions and variable Resource Unit (RU) lengths, reason why any transmission to be performed over UpPTS and DwPTS cannot be applied in the same way it is done for NB-IoT.

For example, the usage of DwPTS in NB-IoT TDD can be envisioned to consider the following approaches:

- Repeating OFDM symbols transmitted in the DL subframes over the OFDM symbols of DwPTS: A base station can use OFDM symbols in the DwPTS to repeat in a predetermined manner some of the OFDM symbols transmitted in either an immediately preceding downlink subframe, or in a succeeding downlink subframe.  The symbols repeated in the special subframe can be coherently combined at the receiver with corresponding symbols transmitted in a downlink subframe to improve decoding performance and reduce the Block Error Rate (BLER), which will increase system capacity.

- Using the OFDM symbols available for the DwPTS as part of the RU, according the following cases:

  - When the number of OFDM symbols available for the DwPTS is sufficiently large: At the moment a TBS is going to be mapped over a number of RUs (one or more), and the number of OFDM symbols available for the DwPTS is larger than a given threshold, the special subframe can be counted as part of a resource unit and rate matching is performed using the available OFDM symbols in the DwPTS. This is illustrated in Figure 3 when a TB is mapped to two resource units (RU).

Figure 3 Usage of special subframe (S) for DL, when the number of symbols in a DwPTS is larger than a given threshold

  - When the number of OFDM symbols available for the DwPTS is small: At the moment a TBS is going to be mapped over a number of RUs (one or more), and the number of OFDM symbols available for the DwPTS is smaller than a given threshold, the special subframe may be counted as part of a resource unit (i.e., being DwPTS a supplementary part of the subframes over which the TBS is usually mapped), but the OFDM symbols in the special subframe are not used for rate matching, i.e., special subframe punctures the allocated resource. This is illustrated in Figure 4 when a TB is mapped to two resource units (RU).

Figure 4 Usage of special subframe (S) for DL, when the number of symbols in a DwPTS is smaller than a given threshold

A similar approach as the one described above for DL can also be considered to be applied for UL, by making use of the available symbols in UpPTS when possible for NPUSCH transmissions.

From the set of options described above, and being aware that the DwPTS count with more symbols available than UpPTS, the following concrete proposals are made:

Proposal 4: Utilize the OFDM symbols in the DwPTS as part of the RU along with a threshold to determine whether the special subframe can be counted as part of a resource unit and rate matching is performed using the available OFDM symbols in the DwPTS.

Proposal 5: Utilize the SC-FDM symbols in the UpPTS to repeat in a predetermined manner some of the SC-FDM symbols transmitted in either an immediately succeeding uplink subframe, or in a preceding uplink subframe.

## DL and UL Bitmaps for TDD NB-IoT

Preliminary discussions on whether UL, DL, or symbol level bitmaps are needed for TDD NB-IoT took place in RAN1 #90bis without reaching any agreement. Below we perform an analysis on the potential needs of having bitmaps  in TDD NB-IoT.

In NB-IoT FDD, valid DL subframe configuration is over 10ms or 40ms for inband and 10ms for standalone/guardband. This is to provide a good co-existence with the LTE MBSFN service. Therefore, for the NB-IoT TDD DL, the same valid downlink subframe bitmap concept should also be introduced.

For the UL, according to [8], the motivation for having UL bitmaps is related with the potential co-existence of TDD NB-IoT with LTE TDD cells configured with eIMTA (Enhanced Interference Management and Traffic Adaptation), most commonly known as “Dynamic TDD”. The background on eIMTA is given in the appendix, and later we provide an analysis on whether an UL bitmap for TDD NB-IoT would be needed or not.

Given that a valid downlink subframe bitmap is 10 ms or 40 ms long, it can include indications of both DL and UL valid subframes in a radio frame. In addition, given the number of DL and UL subframes are different for each of the TDD configurations, it is convenient to use a single bitmap that covers both DL and UL. Notice that in eIMTA, the largest period is 80 ms. Therefore, if supporting eIMTA is one of the reason, then an 80 ms bit map should also considered, and this applies for all operation modes, since there can be eIMTA small cells near an NB-IoT standalone cell. Also, it is beneficial to have a unified design for all three operation modes, i.e., the same length of the bitmap can be applied to all three operation modes.

Proposal 6: For NB-IoT TDD, introduce only one bitmap that indicates both valid DL and UL subframes. The length of the bitmap is at least 40 ms for all operation modes.

Proposal 7: RAN1 discuss whether to introduce an 80 ms bitmap to indicate valid DL and UL subframes.

### Symbol level bitmaps

The motivation for potentially having Symbol level bitmaps is associated with the potential usage of the “special subframes” in TDD NB-IoT. In section 3.1.2, the structure of the special subframe has been depicted in Figure 1, while the figure below shows a symbol level picture of the special subframe.

Figure 5 signals carried over the special subframe

From the figure above, the following points can be highlighted:

- DwPTS: In LTE the first two symbols are used for carrying RS and control information, the third one is used for PSS mapping, while the rest of the symbols can be used for transmitting data.

- GAP: It is used for Downlink-to-Uplink switching.

- UpPTS: The SC-FDMA symbols can be used by the devices for transmitting SRS.

Based on the above analysis, the following consideration on the usage of DwPTS, GAP, and UpPTS for TDD NB-IoT should be taken into account:

- DwPTS: TDD NB-IoT would require do not interfere with the first two symbols used in legacy for transmitting RS and control information. Hence, the DwPTS symbols usable for TDD NB-IoT could be those starting from symbol #2 onwards, without requiring any symbol level bitmap.

- GAP: According with the LS response from RAN4, all the configurable gap lengths larger than 1 symbol can be supported for TDD NB-IoT

- UpPTS: An LTE device may be configured to transmit SRS on a PRB that is used as a NB-IoT carrier, in which case collisions can occur. Optimization have been made in NB-IoT for avoiding collisions, but collisions can still occur, which are handled by network using for example puncturing. Similarly, and given that for TDD NB-IoT it would be valuable to use resources in UpPTS, it can be let up to network handling the potential collisions with SRS.

Observation 1: For DwPTS, TDD NB-IoT would require do not interfere with the first two symbols used in legacy for transmitting RS and control information. Hence, the DwPTS symbols usable for TDD NB-IoT could be those starting from symbol #2 onwards, without requiring any symbol level bitmap.

Observation 2: For UpPTS, the SC-FDMA symbols can be used by the devices for transmitting SRS. In today’s NB-IoT systems, collisions with SRS can occur, which is sorted out by the network (e.g., via puncturing). Similarly, and given that for TDD NB-IoT it would be valuable to use the resources in UpPTS, it can be let up to network to handle the potential collisions with SRS as it does today.

Proposal 8: Symbol level bitmaps on special subframes are not needed.

## MCL relaxations and capacity targets

The coexistence of DL and UL transmissions within the same radio frame will inherently impose significant limitations in terms of resource mapping, which in the end will lead to obtain a different performance as compared to FDD even if the same NB-IoT setup were used. This is the case of the resulting MCL and throughput, which will be impacted by the transmission gaps introduced by the TDD configurations.

In RAN1 #90, the following statements associated to MCL targets were agreed:

Agreements:

- MCL target of 164 dB at an ‘application layer’ data rate of 160 bps is targeted for at least one UL:DL configuration (FFS which one or more than one).

  - NOTE: The at least one UL:DL configuration may or may not be different for UL MCL target than DL MCL target

According to the WID that

“Relaxations of MCL and/or latency and/or capacity targets to be considered by RAN1.”

Since the coverage enhancement of NB-IoT is done through repetitions, it is expected that the number of repetitions of NB-IoT TDD design is not reduced comparing to NB-IoT FDD. Therefore, the MCL target of 164 dB can be met without any problem. However, the latency requirement may need to be relaxed, at least for some of the TDD configurations.

Following the same methodology used in NB-IoT FDD [9], we evaluate the exceptional report latency for the TDD configurations. In Figure 6, the different steps of delivering an exception report are depicted. We only show the latency evaluation at the 164 dB MCL standalone case, and we show the 90% confidence level.

Figure 6 Illustration of steps required during an exception reporting event

As it is expected the synchronization design of NB-IoT TDD should have the same performance as the NB-IoT FDD, we can directly use the synchronization time from the NB-IoT FDD design. At 164 dB MCL, they synchronization time for 90% of the UEs is 364 ms for the standalone. The MIB acquisition time depends on the TDD configurations. As discussed in RAN#90, for some TDD configuration#0, it is not possible to have the same NPBCH density as it is in FDD. Therefore, for TDD configuration#0, we as an estimation, we assume the NPBCH acquisition time is doubled. For MIB reception time it is assumed that 1/1/4 code sub-blocks are needed for reception at 144/154/164 dB coupling loss. Each code sub-block is repeated 8 times and spread over an 80 ms interval (one repetition in each subframe 0). For terminals with 164 dB coupling loss, four code sub-blocks are required to decode MIB.  However, since MIB is updated in every TTI (640 ms), the four code sub-blocks must be in the same TTI in order for decoding to succeed. In the worst case, waiting until this condition is fulfilled causes an extra delay of four code-sub blocks, or 4*80=320 ms.  Reading the four code sub-block then takes 3*80+71=311 ms. Thus the total required time is 320+311=631 ms.

After acquiring the MIB, the UE needs to perform random access. In NB-IoT, the same four-step random access procedure as in LTE is followed:

  - Msg 1: UL: Random access preamble on PRACH

  - Msg 2: DL: Random access response (C-RNTI, TA, uplink grant)

  - Msg 3: UL: “Connection request” (TLLI, Access cause, BSR)

  - Msg 4: DL: Contention resolution message (copy of TLLI)

The NPRACH preamble design in NB-IoT TDD has not started yet. Hence, in this evaluation, we use the single tone design in NB-IoT FDD as a reference to estimate the time required for NB-IoT TDD NPRACH, and fits the required number of repetitions to the UL resources available in each of the TDD configurations.  We assume the UE sends NPRACH in the next available NPRACH slot. Therefore, for the worst case, it takes at most one NPRACH period as the waiting time before the UE can send NPRACH. In NB-IoT FDD, for UEs with 164 dB coupling loss, the preamble transmission takes 32 repetitions. We assume 4 ms waiting time before the random-access response (RA) window starts. To fit the NPRACH preambles to the available consecutive UL subframes, we proposed some designs in our companion paper [4].  The NPRACH transmission time is estimated based on the NB-IoT FDD performance, not based on simulations.

The time required for RA mgs2-4 transmission can be calculated directly from the NB-IoT FDD design. We assume the UE gets the NPDCCH assignment immediately in the beginning of the USS. In addition, we assume T = 1.5 and Rmax = 128, for the USS configuration only counting the DL subframes. For the worst case, the UE needs to wait for 128*1.5 = 192 ms for the NPDCCH. We assume the other timing relationships are the same as used in NB-IoT FDD. The size of the messages and the required transmission time in NB-IoT FDD are given in Table 3.

Table 3 Transmission time for RA msg2-4 in NB-IoT FDD at 164 dB MCL, standalone

|  | Bytes | Required time in NB-IoT FDD [ms] |
| --- | --- | --- |
| DCI for RAR (DL) |  | 128 |
| RAR, including UL grant (DL) | 8 | 160 |
| RA msg 3 (UL) | 11 | 288 |
| DCI for DL assignment (DL) |  | 128 |
| RA msg 4 (DL) | 8 | 160 |
| ACK/NACK for msg4 (UL) |  | 64 |

Once random access is completed the terminal waits for an uplink grant on NPDCCH. Based on the buffer status report included in RA msg3 the eNB selects a suitable grant size and transmits the NPDCCH. Once the UE receives the UL assignment, it sends the exception report on the scheduled UL resources. For the exception report, according to [8], in total 100 bytes are assumed for the overhead of the upper layer protocols. At 164 dB MCL, it takes 2560 ms to send the exception report. The UL transmission gap is not considered in NB-IoT TDD.

If the transmission is received in error or lost, the network will schedule the terminal to do a re-transmission. A re-transmission is requested by sending an uplink assignment in the next available NPDCCH with the NDI (“new data indicator”) flag not toggled. For the 90% confidence case, we assume no re-transmission.

The total time to send exception report for each of the TDD configurations is summarized in Table 4.  Notice that the NPRACH transmission time is estimated based on the NB-IoT FDD design. The proposed NPRACH designs are given in our companion paper [4].  The special subframes are not considered in the calculations.

Table 4 Exception report delivery time with 90% confidence for standalone case at 164 dB MCL

| Activity TDD configuration | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- |
| Tsync(ms) | 364 | 364 | 364 | 364 | 364 | 364 |
| TMIB(ms) | 631 | 631 | 631 | 631 | 631 | 631 |
| Waiting for NPRACH (ms) | 480 | 960 | 640 | 960 | 1920 | 640/960* |
| TPRACH(ms) | 480 | 960 | 640 | 960 | 1920 | 640/960* |
| TRAmsg2-4 (ms), including waiting time for NPDCCH and ACK/NACK for msg4 | 2640 | 2934 | 2347 | 1886 | 4400 | 3051 |
| TULgrant(ms), including waiting time for NPDCCH | 800 | 534 | 534 | 457 | 400 | 1067 |
| TULdata(ms) | 6400 | 12800 | 8533 | 12800 | 25600 | 5120 |
| Total time (ms) | 11795 | 19183 | 13689 | 18058 | 35235 | 11352/11672 |

*Note: TDD configuration #6 has an unbalanced number of UL within a radio frame. Thus, the latency is 640ms when NPRACH Format 2 is used and two symbol groups fit in 3 UL subframes (i.e., two ULs per radio frame wouldn’t be used by NPRACH) / 960ms when NPRACH Format 1 is used and two symbol groups fit in 2 UL subframes  (i.e., three ULs per radio frame wouldn’t be used by NPRACH).

As we can see that there is none of the TDD configurations can meet the 10 seconds latency requirements even for the standalone case at 164 dB MCL. Hence, we can expect a larger latency for the inband/guardband cases. This indicates that the application layer data rate of 160 bps cannot be guaranteed at the 164 dB MCL. Therefore, the above MCL target seems to be too aggressive to be met, since not all the subframes in a radio frame are usable for DL or UL respectively. Thus, we will have to find a way to compensate for the lack of available resources in both directions, which may easily take us to consider (i.e., among other things) the following:

  - Ending up with an excessive relaxation of the latency for NB-IoT TDD.

  - The need of using a much more aggressive channel estimator.

  - Make use of the DwPTS, and UpPTS in the special subframes.

The above considerations may help us to get closer to fulfill the MCL target of 164dB, but still is difficult to guarantee that such a MCL goal is going to be achieved.

Observation 3: A MCL target of 164dB seems to be too aggressive to be met in a TDD operation, and we will need to find a way to compensate for lack of available resources in DL and UL through an excessive relaxation of the latency, a much more aggressive channel estimator, the usage of DwPTS/UpPTS in the special subframes, among other things.

## HARQ aspects

In the FDD system asynchronized HARQ is supported. Two HARQ processes were introduced in Rel-14 as an optional UE capability. Therefore, considering it may potentially be the case that some of the chipset that are dimensioned for one HARQ process can be software updated to support TDD, it is beneficial to have similar arrangement for NB-IoT FDD, i.e., two HARQ processes should be an optional UE capability.

Furthermore, for some TDD configurations, the use of two HARQ processes does not increase the throughput significantly either. For example, as we see from Table 5, for TDD configuration#0, in the DL if one HARQ process is used, considering the best NPDCCH configurations and scheduling delays, the highest throughput is 33 kbps, and if two HARQ process are configured, at most the highest throughput is 42 kbps. If considering the scheduling for SI, and paging, the difference in achievable highest throughput between one and two HARQ process may be even smaller for some of the TDD configurations. By using 2 HARQ process, the most significant increase of DL throughput for NPDSCH is observed in the TDD configurations that have more DL subframes than UL subframes. However, in these configurations, the UL throughput increase by using 2 HARQ processes is foreseen to be limited.

Proposal 9: Supporting two HARQ processes should be an optional UE capability in NB-IoT TDD system.

Table 5 Comparison of Maximum achievable NPDSCH throughput of one and two HARQ processes

| Uplink-downlink configuration | One HARQ process | Two HARQ processes |
| --- | --- | --- |
| 1 | 50.72 kbps | 67.63 kbps |
| 2 | 61.85 kbps | 78.03 kbps |
| 3 | 61.85 kbps | 88.98 kbps |
| 4 | 61.85 kbps | 88.98 kbps |
| 5 | 51.75 kbps | 88.98 kbps |
| 6 | 42.26 kbps | 56.99 kbps |

Notice that in the current FDD system, per HARQ process, after receiving the NPDSCH, the UE needs 12 ms to decode the NPDSCH and send ACK/NACK. Since in the FDD system, the UE operates in a half-duplex fashion, the scheduler can optimize the scheduling in such a way that once the NPDSCH from both HARQ processes had been received, then the UL sends the ACK/NACK, as shown in Figure 7.

Figure 7 Example of two HARQ processes in an FDD system

However, in a TDD system, as the UL and DL subframes are interlaced, it is likely that for the DL transmission of the 2nd HARQ process to start before the ACK/NACK of the 1st HARQ process, as shown in Figure 8 (configuration#2 is used as an example).

Figure 8 Example of two HARQ processes in a TDD system

Therefore, from a resource utilization point of view, in a TDD system, if two HARQ process are configured, we should allow a UE to perform an UL transmission while there is a DL reception still in progress, and vice versa. This can further improve the throughput.

Proposal 10: To further improve the throughput, A UE should be allowed to have UL transmission while there is an DL reception, and vice versa.

## Multi-carrier support

In the current NB-IoT FDD design, multiple carriers (anchor and non-anchor) are supported. However, cross-carrier scheduling is not supported. Notice that in an FDD system, the number of UL and DL carriers can be independently configured (they do not have to be in pairs). Therefore, a simple support of multiple carriers can offer enough flexibility to cope with different load in the UL and DL, and it is not necessary to support cross-carrier scheduling.

However, this is not the case in a TDD system, where the UL and DL are separated in time. Therefore, when multiple carriers (anchor and non-anchor) are configured, the same TDD configurations are required for both anchor and non-anchor carriers. Therefore, the number of carriers can be dimensioned based on the direction (i.e., UL or DL) having most of the traffic. In this case, cross-carrier scheduling can help to achieve a better load balancing among carriers, and therefore potentially reduce the number of carriers that are needed in a cell.

As mentioned in our companion paper [5], DL gap should be supported in NB-IoT TDD due to the same reason as it is in FDD. In the TDD case, we should also take the DL and UL configurations from different TDD configurations into consideration.  When a DL gap is configured, we can notice that in some of the TDD configurations the number of available DL subframes are limited, which means there may not be enough DL subframes to finish sending the intended NPDSCH together with NPDCCH during a DL gap. However, since the DL transmission of NPDCCH is usually short, it can be beneficial to send the NPDCCH in the DL gap and redirect the UE to another carrier for the scheduled NPDSCH. In this case, the DL gap can be better utilized to solve the blocking problem.

In RAN1 #91, a Way Forward (WF) entitled “WF on Cross-carrier scheduling in TDD” was presented, which contained the following proposals [10]:

- In TDD NB-IoT, cross-carrier scheduling should be supported

  - A UE’s NPDCCH monitoring carrier, NPDSCH scheduled carrier, NPUSCH format 1 and NPUSCH format 2 scheduled carrier can be configured to be different

- When cross carrier scheduling is supported, the maximum number of configurable carriers for NPDSCH and NPUSCH format 1 should be downselected from the followings

  - Single carrier per channel (i.e., no CIF is required in DCI)

  - Up to [2] carriers per channel (i.e., [1] bit of CIF is required in DCI)

- When cross carrier scheduling is supported, configuration of the carrier for NPUSCH format 2 is FFS

  - (Opt.1) carrier for NPUSCH format 2 is configured by RRC

  - (Opt.2) carrier for NPUSCH format 2 is indicated by DL grant

  - (Opt.3) use the same carrier as NPDSCH or NPDCCH

  - Other options are not precluded including the combination of above options

Recognizing the importance of supporting cross-carrier scheduling for TDD NB-IoT, the three points above can be simplified aiming at making more feasible its support.

- In TDD NB-IoT, cross-carrier scheduling should be supported

  - A UE’s NPDCCH monitoring carrier, and NPDSCH (and/or NPUSCH Format 1) scheduled carrier can be configured to be different.

- When cross carrier scheduling is supported, the maximum number of configurable carriers for NPDSCH and NPUSCH format 1 should be:

  - One set of up to [2] configurable carriers is shared by both NPDSCH and NPUSCH Format 1.

- When cross carrier scheduling is supported, configuration of the carrier for NPUSCH format 2 is FFS

  - NPUSCH format 2 uses the same carrier as NPDSCH, since it will result in less impact.

Proposal 11: Cross carrier scheduling should be supported in NB-IoT TDD.

Proposal 12: For the support of Cross carrier scheduling in NB-IoT TDD.

- A UE’s NPDCCH monitoring carrier, and NPDSCH (and/or NPUSCH Format 1) scheduled carrier can be configured to be different.

- One set of up to [2] configurable carriers is shared by both NPDSCH and NPUSCH Format 1.

- NPUSCH format 2 uses the same carrier as NPDSCH, since it will result in less impact.

# Conclusions

This contribution provided an analysis on the better suitability that some LTE TDD configurations have as to be considered for supporting TDD into NB-IoT, the potential usage of UpPTS, and DwPTS in TDD NB-IoT, and common aspects associated with HARQ and multi-carrier support. From the analysis performed the following observations and proposals have been enunciated:

Observation 1: For DwPTS, TDD NB-IoT would require do not interfere with the first two symbols used in legacy for transmitting RS and control information. Hence, the DwPTS symbols usable for TDD NB-IoT could be those starting from symbol #2 onwards, without requiring any symbol level bitmap.

Observation 2: For UpPTS, the SC-FDMA symbols can be used by the devices for transmitting SRS. In today’s NB-IoT systems, collisions with SRS can occur, which is sorted out by the network (e.g., via puncturing). Similarly, and given that for TDD NB-IoT it would be valuable to use the resources in UpPTS, it can be let up to network to handle the potential collisions with SRS as it does today.

Observation 3: A MCL target of 164dB seems to be too aggressive to be met in a TDD operation, and we will need to find a way to compensate for lack of available resources in DL and UL through an excessive relaxation of the latency, a much more aggressive channel estimator, the usage of DwPTS/UpPTS in the special subframes, among other things.

Proposal 1: Use the supported LTE TDD configurations in all deployment modes (i.e., in-band, guard-band, and stand-alone) of TDD NB-IoT.

Proposal 2: In Rel-15 prioritize LTE TDD configurations having a more adequate number of UL and DL subframes (e.g., TDD configuration #1, #2, #3, #4).

Proposal 3:  Special subframe configurations with more than 1 guard symbol is supported in NB-IoT.

Proposal 4: Utilize the OFDM symbols in the DwPTS as part of the RU along with a threshold to determine whether the special subframe can be counted as part of a resource unit and rate matching is performed using the available OFDM symbols in the DwPTS.

Proposal 5: Utilize the SC-FDM symbols in the UpPTS to repeat in a predetermined manner some of the SC-FDM symbols transmitted in either an immediately succeeding uplink subframe, or in a preceding uplink subframe.

Proposal 6: For NB-IoT TDD, introduce only one bitmap that indicates both valid DL and UL subframes. The length of the bitmap is at least 40 ms for all operation modes.

Proposal 7: RAN1 discuss whether to introduce an 80 ms bitmap to indicate valid DL and UL subframes.

Proposal 8: Symbol level bitmaps on special subframes are not needed.

Proposal 9: Supporting two HARQ processes should be an optional UE capability in NB-IoT TDD system.

Proposal 10: To further improve the throughput, A UE should be allowed to have UL transmission while there is an DL reception, and vice versa.

Proposal 11: Cross carrier scheduling should be supported in NB-IoT TDD.

Proposal 12: For the support of Cross carrier scheduling in NB-IoT TDD.

- A UE’s NPDCCH monitoring carrier, and NPDSCH (and/or NPUSCH Format 1) scheduled carrier can be configured to be different.

- One set of up to [2] configurable carriers is shared by both NPDSCH and NPUSCH Format 1.

- NPUSCH format 2 uses the same carrier as NPDSCH, since it will result in less impact.

# References

RP-170732, “New WI on Further NB-IoT enhancements”, RAN #75.

RP-171440, “Way Forward on Prioritization of NB-IoT”, RAN #76.

RP-172063, “Revised WID on Further NB-IoT enhancements”, RAN 77.

R1-1719362, “UL aspects of TDD for NB-IoT,” RAN1 #91, Ericsson.

R1-1719361, “DL aspects of TDD for NB-IoT,” RAN1 #91, Ericsson.

Final Chairman’s Notes RAN1 #90, 3GPP.

3GPP TS 36.211, “Physical channels and modulation”, v14.2.0.

R1-1719101, “WF on TDD configurations,” LG Electronics, RAN1 #90bis.

3GPP TR 45.820, “Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT),” version 13.1.0.

R1-1721221, “WF on Cross-carrier scheduling in TDD”, source LG Electronics, Samsung, RAN1#91, Reno, USA, November 27th – December 1st, 2017

# Appendix

#### Background on eIMTA

With eIMTA, the uplink-downlink configuration can vary on a frame-by-frame basis. The switching of TDD configurations can only occur among certain TDD configurations, and for that purpose eIMTA makes use of three different types of uplink-downlink configurations:

- uplink reference configuration: The uplink reference configuration is obtained from SIB1. It is also the configuration used by non-eIMTA-capable devices. Downlink subframes are guaranteed to be downlink subframes despite any dynamic reconfiguration.

- downlink reference configuration: The downlink reference configuration is obtained from dedicated RRC signaling, specific to eIMTA-capable devices. Uplink subframes are guaranteed to be uplink subframes despite any dynamic reconfiguration.

- current uplink-downlink configuration: From the two reference configurations the so-called “flexible subframes” are identified, which are those subframes that result to be different when the UL and DL reference configurations are compared one-on-one. Thus, a flexible subframe can be used in either transmission direction (i.e., UL or DL). So, the current uplink-downlink configuration is the configuration that is broadcasted regularly and can be dynamically changeds

A device not detecting the current uplink-downlink configuration will assume the current allocation being equal to the uplink reference configuration for the coming frame(s). In other words, the device behaves in the same way as a non-eIMTA-enabled device in such a situation. The signaling periodicity for the current configuration can be set to once per 10, 20, 40, or 80 ms.