3GPP TSG RAN WG1 Meeting #92		R1-1801544

Athens, Greece, February 26th – March 2nd, 2018

Source:	vivo

Title:	Remaining issues on BWP operation

Agenda Item:	7.1.3.4.1

Document for:	Discussion and Decision

- Introduction

In RAN1 NR AH#1801, the alternatives for DCI based BWP switching were discussed extensively, mainly among the following options [1]:

- Option 1

  - DCI information relates to current BWP. DCI size determined by current BWP. PDSCH is received in current slot (or later slot in case of cross-slot scheduling)

  - If BWP index differs from current BWP, then a new BWP is activated X us later (X depends on the BWP transition latency and possibly K0)

    - A future DCI in the new BWP can schedule data using the new BWP

- Option 3b

  - Sizes of all DCI bitfields determined by current BWP. Data transmitted on the BWP indicated by the BWP index. If the BWP index activates another BWP, transform as follows:

    - Zero-pad too small bitfields to match the new BWP

    - Truncate too large bitfields to match the new BWP

- Option 3c

  - Sizes of all DCI bitfields determined by current BWP. Data transmitted on the BWP indicated by the BWP index. If the BWP index activates another BWP, transform as follows:

    - Zero-pad too small bitfields to match the new BWP

    - Truncate too large bitfields to match the new BWP

  - Note: It is assumed that it is possible to switch BWP without scheduling data, e.g.  by scheduling using a null assignment

- Option 4

  - If the UE is supposed to switch BWP no payload is scheduled

Basically in option 3 (3b/3c) the UE is cross BWP scheduled during the BWP switch, however, the DCI bitfield size are determined by the current BWP. After reading the BWP index field which indicates a different BWP index than the current one, UE regards the current DCI is for BWP switch and shall interpret the DCI fields according to the configuration of the target BWP. Since the DCI field size is determined by the current BWP, UE shall transform the DCI field by UE zero-padding (UE to add some zeros to the decoded DCI field and interpret it afterwards) or truncating (UE to delete some DCI bits from the decoded DCI filed and interpret it afterwards) behavior.

During the discussion, it was realized that at least the RA field is BWP dependent thus zero-padding or truncating behavior shall be performed if either option 3b or 3c is to be adopted. It was also commented that other DCI field may also be BWP dependent but the impact to be checked further. Note that option 1 or option 4 does not require such zero-padding or truncating behavior. Finally option 3b was agreed as working assumption which may be reconsidered if significant problem regarding zero-padding and truncating operation is identified.

In this contribution, we analyzed the DCI fields in DL and UL non-fallback DCI format and observed that lots of DCI fields are actually BWP dependent which means the zero-padding or truncating operation shall be applied to all these DCI fields, causing significant problems in specification and implementation, in order to handle the size mismatch between current and target BWP for each relevant DCI field. Therefore we propose that RAN1 to reconsider the working assumption made for DCI based BWP switching mechanism.

- Discussion

  - DCI field with BWP dependency

Based on the latest endoresed version of  TS38.212 [2] and updated RRC parameters, the analysis on BWP dependency for UL and DL non-fallback DCI is shown in the following table 1 and 2, respectively

Table 1- BWP dependency for DCI format 0_1

| DCI field | Field size | BWP dependency | Comment |
| --- | --- | --- | --- |
| Carrier indicator | 0 or 3 bits | No | Same size across BWPs |
| UL/SUL indicator | 0 or 1bit | No | Same size across BWPs |
| Identifier for DCI formats | [1] bit | No | Same size across BWPs |
| Bandwidth part indicator | 0, 1 or 2 bits | No | Same size across BWPs |
| Frequency domain resource assignment | X bits | Yes | Size determined by the active DL BWP, size can be different across BWPs |
| Time domain resource assignment | 0, 1, 2, 3, or 4 bits | Yes | Configured per BWP, size can be different across BWPs |
| VRB-to-PRB mapping | 0 or 1 bit | Yes | Has dependency on the resource allocation type which is configured per BWP, size can be different across BWPs |
| Frequency hopping flag | 0 or 1 bit | Yes | Has dependency on the resource allocation type which is configured per BWP, size can be different across BWPs |
| Modulation and coding scheme | 5 bits | No | Fixed size across BWPs |
| New data indicator | 1 bit | No | Fixed size across BWPs |
| Redundancy version | 2 bits | No | Fixed size across BWPs |
| HARQ process number | 4 bits | No | Fixed size across BWPs |
| 1st downlink assignment index | 1 or 2 bits | No | Has dependency on the HARQ-ACK codebook type (semi-static or dynamic) which is configured per UE, size is the same across BWPs |
| 2nd downlink assignment index | 0 or 2 bits | No | Has dependency on the HARQ-ACK codebook type (semi-static or dynamic) which is configured per UE, size is the same across BWPs |
| TPC command for scheduled PUSCH | 2 bits | No | Same size across BWPs |
| SRS resource indicator | [0 or] or bits | Yes | Size determined by the number of SRS resource, which is configured per BWP, size can be different across BWPs, up to 4 bits difference |
| Precoding information and number of layers | 0, 1, 2, 4, 5, or 6bits | Yes | Size determined by the following configurations, which are all per BWP configuration, size can be size can be different across BWPs, up to 6 bits difference - ulTxConfig -PUSCH-tp - ULmaxRank |
| Antenna ports | 2, 3, 4 or 5 bits | Yes | Size determined by the following configurations, which are all per BWP configuration, size can be size can be different across BWPs, up to 3 bits difference - PUSCH-tp - dmrs-Uplink |
| SRS request | 2 or 3bits | No | 2bits if UE is not configured with SUL, 3bits otherwise. |
| CSI request | 0, 1, 2, 3, 4, 5, or 6 bits | Yes | Size determined by the configuration of ReportTriggerSize, which is per BWP configured. Size can be different across BWPs, up to 6 bits difference |
| CBG transmission information (CBGTI) | 0, 2, 4, 6, or 8 bits | FFS | It is under discussion whether CBG is configured per BWP or per cell |
| PTRS-DMRS association | 0 or 2bits | Yes | Size determined by the following configurations, which are all per BWP configuration, size can be size can be different across BWPs, up to 2bits difference - UL-PTRS-present -PUSCH-tp |
| beta_offset indicator | 0 or 2bits | No | Configured per serving cell, same size across BWPs |
| DMRS sequence initialization | 0 or 1bit | Yes | Size determined by the configuration of PUSCH-tp, which is per BWP configured. Size can be different across BWPs, up to 1 bit difference |

Table 2- BWP dependency for DCI format 1_1

| DCI field | Field size | BWP dependency | Comment |
| --- | --- | --- | --- |
| Carrier indicator | 0 or 3 bits | No | Configured per serving cell, same size across BWPs |
| Identifier for DCI formats | [1] bits | No | Same size across BWPs |
| Bandwidth part indicator | 0, 1 or 2 bits | No | Configured per serving cell, same size across BWPs |
| Frequency domain resource assignment | Y bits | Yes | Size determined by the active DL BWP , size can be different across BWPs |
| Time domain resource assignment | 0, 1, 2, 3, or 4 bits | Yes | Configured per BWP, size can be different across BWPs |
| VRB-to-PRB mapping | 0 or 1 bit | Yes | Has dependency on the resource allocation type which is configured per BWP, size can be different across BWPs |
| PRB bundling size indicator | 0 or 1 bit | Yes | Configured per BWP, size can be different across BWPs |
| Rate matching indicator | 0, 1, or 2bits | Yes | Configured per BWP, size can be different across BWPs |
| ZP CSI-RS trigger | 0, 1, or 2 bits | Yes | Configured per BWP, size can be different across BWPs |
| Modulation and coding scheme for TB1 | 5bits | No | Fixed size across BWPs |
| New data indicator for TB1 | 1bit | No | Fixed size across BWPs |
| Redundancy version for TB1 | 2bits | No | Fixed size across BWPs |
| Modulation and coding scheme for TB2 | 5bits | No | Fixed size across BWPs |
| New data indicator for TB2 | 1bit | No | Fixed size across BWPs |
| Redundancy version for TB2 | 2bits | No | Fixed size across BWPs |
| HARQ process number | 4bits | No | Fixed size across BWPs |
| Downlink assignment index | 0, 2,or 4bits | No | Configured per UE, Fixed size across BWPs |
| TPC command for scheduled PUCCH | 2bits | No | Fixed size across BWPs |
| PDSCH-to-HARQ_feedback timing indicator | 3 bits | No | Fixed size across BWPs 0bit under discussion |
| Antenna port(s) | 4, 5, or 6 bits | Yes | Configured per BWP, size can be different across BWPs |
| Transmission configuration indication | 0 or 3 bits | Yes | Configured per BWP, size can be different across BWPs |
| SRS request | 2 or 3bits | No | 2bits if UE is not configured with SUL, 3bits otherwise. |
| CBG transmission information (CBGTI) | 0, 2, 4, 6, or 8 bits | FFS | It is under discussion whether CBG is configured per BWP or per cell |
| CBG flushing out information (CBGFI) | 0 or 1 bit | FFS | It is under discussion whether CBG is configured per BWP or per cell |
| DMRS sequence initialization | 1 bit | No | Fixed size across BWPs |

Table 3 Summary of BWP dependent DCI fields

| DCI format | Number of DCI field which size has BWP dependency | Number of DCI field which size may have BWP dependency |
| --- | --- | --- |
| 0_1 | 10 | 1 |
| 1_1 | 8 | 2 |

Table 3 provides a summary of BWP dependent DCI fields according to the analysis in table 1 and 2. It can be seen that up to 10 and 8 DCI fields in DCI format 0_1 and 1_1 has field size determined by the per BWP configuration. And 1 or 2 more fields in DCI formant 0_1 and 1_1 may also be determined by the per BWP configuration, depending on the further decisions. This means if option 3b (as current working assumption) is adopted, special rules of zero-padding and truncating of DCI field for BWP switching should be discussed and specified in a per DCI field basis which requires significant efforts and results in unnecessary specification complexity. In addition, UE implementation complexity is also increased undesirably.

It is therefore suggested that RAN1 to reconsider the working assumption on option 3b and select an option which does not require cross-BWP scheduling during BWP switch (thus does not suffer from DCI field size mis-match issue across BWPs). It is proposed to adopt either option 1 or option 4 to conclude the issue.

- Conclusion

In this contribution, we discuss the mechanisms for DCI based BWP switching and showed the problems of current agreed working assumption (option 3b). As a conclusion, it is suggested that RAN1 to reconsider the working assumption on option 3b and select an option which does not require cross-BWP scheduling during BWP switch (thus does not suffer from DCI field size mis-match issue across BWPs). It is proposed to adopt either option 1 or option 4 to conclude the issue.

Proposal :

Adopt either option 1 or option 4 as the solution for DCI based BWP switching.

- Option 1

  - DCI information except the “Bandwidth part indicator” relates to current BWP. DCI size determined by current BWP. If the “Bandwidth part indicator” indicates a different BWP, it triggers a BWP switch. PDSCH scheduled by a DCI triggers BWP switch is received in current slot (or later slot in case of cross-slot scheduling)

- Option 4

  - If the UE is supposed to switch BWP no payload is scheduled, i.e. the BWP switching is triggered by a DCI with “Null assignment”.

- References

- R1-1801102, Outcome of offline discussion on 7.3.3.1 (resource allocation) – part II, Ericsson

- R1-1801292,  3GPP TS 38.212 V15.0.1 (2018-2)

- R1-1801281, LS on correction to RRC parameters for NR, RAN1