3GPP TSG RAN WG1 Meeting #92															R1-1801547

Athens, Greece, February 26th – March 2nd, 2018

Source:	     vivo

Title:		Discussion on Rel-15 NR UE features

Agenda Item:    7.1.7

Document for:	 Discussion and Decision

# Introduction

In this contribution we provide our views on UE feature list from based on the endorsed version from RAN1 NR AH#1801[1], including our recommendation on mandatory/optional support of UE features and FR1/FR2 differentiation. The recommendations are shown in the tables with track change.

References

[1] R1-1801289, NR UE feature list, NTT DOCOMO, AT&T

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) | Consequences if the feature is not supported by the UE | Type (see R2-1712078) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL | 1) CP-OFDM for DL 2) CP -OFDM for UL |  | Yes |  | N.A. | N.A. | N.A. |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 0-2 | DFT-S-OFDM waveform for UL | Transform precoding for single-layer PUSCH |  | Yes |  | N.A. | N.A. | N.A. |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 0-3 | DL modulation scheme | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |  | Yes |  | N.A. | N.A. | N.A. |  | RAN4 will check 64QAM modulation for FR2 |  | Mandatory without capability signaling |  |
|  | 0-4 | UL modulation scheme | 1) QPSK modulation 2) 16QAM modulation |  | Yes |  | N.A. | N.A. | N.A. |  |  |  | Mandatory without capability signaling |  |
|  | 0-5 | pi/2-BPSK for PUSCH | pi/2-BPSK for PUSCH | 0-2 | Yes | pi/2-BPSK for PUSCH is not possible | [Type 3] |  | Yes |  | RAN4 will discuss if it is per band, common for all bands or FR1/2 | RAN4 | optional |  |
|  | 0-6 | 64QAM for PUSCH | 64QAM for PUSCH |  | Yes | 64QAM for PUSCH is not possible | [Type 3] |  | Yes |  | RAN4 will discuss if it is per band or common for all bands | RAN4 | Mandatory with capability signaling for FR1 Optional for FR2 |  |
|  | 0-7 | 256QAM for PDSCH | 256QAM for PDSCH |  | Yes | 256QAM for PDSCH is not possible | [Type 3] |  | Yes |  | RAN4 will discuss if it is per band or common for all bands | RAN4 | Optional |  |
|  | 0-8 | 256QAM for PUSCH | 256QAM for PUSCH |  | Yes | 256QAM for PUSCH is not possible | [Type 3] |  | Yes |  | RAN4 will discuss if it is per band or common for all bands | RAN4 | Optional |  |
|  | 0-9 | Subcarrier spacings and FFT size in conjunction with supportable BW with normal CP | 1) 15kHz 2) 30 kHz 3) 60 kHz 4) 120 kHz |  | Yes |  | Type 4 [Type 3] |  | Yes |  | It is up to RAN4 decision Baseband processing (memory) in CA combination related as well as RF (SCS support is per band between sub6 and mmWave) | RAN4 | FR1: 15KHz mandatory with capability signaling 30KHz mandatory with capability signaling 60KHz optional FR2: 60KHz mandatory with capability signaling 120KHz mandatory with capability signaling |  |
|  | 0-10 | Extended CP | Extended CP | 0-10 (component 3; SCS60) | Yes |  | Type 4 | [No need] | [No need] |  | RAN4 to check | RAN1 | Optional with capability signaling |  |
|  | 0-11 | pi/2-BPSK for PUCCH format 3/4 | pi/2-BPSK for PUCCH format 3/4 | 0-2 | Yes | pi/2-BPSK for PUCCH format 3/4 is not possible | Type 3 |  | Yes |  | RAN4 will discuss if it is per band, common for all bands, or FR1/2 | RAN4 | optional |  |
|  | 0-12 | Non-contiguous UL CP-OFDM |  | 0-1 | Yes |  |  | No |  |  | It is up to RAN4 to decide | RAN4 | Optional |  |
|  | 0-13 | [Phase coherence across non-contiguous UL symbols in slot] |  |  | Yes |  |  | Yes |  |  | RAN1 needs further check | RAN1 |  |  |
|  | 0-14 | [1-symbol GP for 120KHz SCS in unpaired spectrum] | 1) Slot formats with 1-symbol GP(s) for 120KHz SCS in unpaired spectrum |  | Yes |  | Type 4 | Applicable only to TDD | Applicable only to FR2 |  | RAN4 to check whether this feature is included in their list |  |  |  |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1) RACH preamble format other than what is listed in 1-2 2) SS block based RRM measurement [3) RMSI/broadcast OSI reception] [4] Paging] |  | [Yes, or No] |  | N.A. | No need | No need |  | RAN2 to check components 3 and 4 for SA and NSA applicability | RAN1 | Mandatory without capability signaling |  |
|  | 1-1a | [On demand based system information] |  |  | [Yes, or No] |  |  | [Yes] | [No need] |  | RAN2 to check | RAN2 |  |  |
|  | 1-2 | [PRACH configuration] | 1) RACH preamble format A1, A2, A3, C0 for SCS120 |  | Yes |  | Type 1 | [Yes] | Applicable only to FR2 |  | Component 1) RACH preamble format A1, A2, A3, C0 having one or two OFDM symbols need to get confirmation by RAN4 on impact for ON-OFF / OFF-ON mask. Optional PRACH foramt if any can only be used for non-initial access case | RAN1 | [Mandatory without capability signaling or not supported for initial access / optional for non-initial access] |  |
|  | 1-3 | SS block based RS-SINR measurement (SS-SINR) | 1) SS-SINR measurement | 1-1 | Yes | Not support SS-SINR measurement | Type 4 | No need | [No need] |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 1-4 | SS block based RLM | 1) SS block based RLM | 1-1 | Yes | Not support SS block based RLM | Type 4 | No need | No need |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 1-5 | CSI-RS based RRM measurement | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement | 1-1, CSI-RS | Yes | Not support CSI-RSRP and CSI-RSRQ measurement | Type 4 | No need | No need |  |  | RAN1 | Optional |  |
|  | 1-6 | CSI-RS based RS-SINR measurement | 1) CSI-SINR measurement | 1-1 1-5 | Yes | Not support CSI-SINR measurement | Type 4 | No need | [No need] |  |  | RAN1 | Optional |  |
|  | 1-7 | CSI-RS based RLM | 1) CSI-RS based RLM | 1-1, CSI-RS | Yes | Not support CSI-RS based RLM | Type 4 | No need | No need |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 1-8 | RLM based on a mix of SS block and CSI-RS signals |  | 1-4 and 1-7 | Yes | UE does not support RLM based on a mix of SS block and CSI-RS signals | Type 4 | No need | No need |  |  | RAN1 | Optional |  |
|  | 1-9 | CSI-RS based contention free RA for HO |  | 1-1 1-2, CSI-RS | Yes | UE does not support CSI-RS based contention free RA for HO | Type 4 | No need | No need |  |  | RAN1 | Optional |  |
|  | 1-10 | Support of SCell without SS/PBCH block | 1) Support SCell without SS/PBCH block | 1-1 | Yes | Not support SCell without SS/PBCH | Type 3 | No need | No need |  | Component 1) Whether or not UE is able to use SS/PBCH block from other Cells for time/frequency synchronization of SCell without SS/PBCH block | RAN1 | Optional |  |
| 2. MIMO | 2 -1 | Basic PDSCH reception note: for both slot based and non-slot based | 1. Data RE mapping 2. Single layer transmission |  |  |  |  |  | No need |  |  |  | Mandatory without capability signaling |  |
|  | 2-2 | PDSCH beam switching | 1. Time duration (definition follows section 5.1.5 in TS 38.214) to determine and apply spatial QCL information for corresponding PDSCH reception Note: candidate value will be decided after feature is completed. (note: this may not needed) | 2-1 |  |  | Type-3 |  | Yes |  |  |  | mandatory with capability Only apply to FR2 Possible candidate values: {14, 28} or {12, 26} depending on how to count the starting symbols Note: if only one value is selected, then no capability is needed. |  |
|  | 2-3 | PDSCH MIMO layers | 1. Maximal number of MIMO layers, candidate values: {1,2,4,8} | 2-1 |  |  | Type 3 |  | Yes |  |  |  | Up to 4 layers mandatory with capability signaling 8 layers optional |  |
|  | 2-4 | TCI states for PDSCH | 1. Support number of active TCI states per CC 2. maximum number of configured TCI states, | 2-1 |  |  | Type 1 |  | Yes |  |  |  | Candidate values for Component-1: {1, 2, 4, 8 } For component 1: Values above 2 is optional; candidate values for Component-2: {4, 8, 16, 32, 64} For component 2: Values above 16 are optional |  |
|  | 2-5 | Basic downlink DMRS for scheduling type A | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. | 2-1 |  |  |  |  | No need |  | conditioned to whether PDSCH scheduling type A is supported |  | Mandatory without UE capability(condition to scheduling capability) |  |
|  | 2-6 | Basic downlink DMRS for scheduling type B | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  | No need |  | conditioned to whether PDSCH scheduling type B is supported |  | Mandatory without UE capability (condition to scheduling capability) |  |
|  | 2-6 | Support 1+2 DMRS | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | 2-5 |  |  | Type 4 |  | No need |  |  |  | Mandatory with UE capability signaling |  |
|  | 2-7 | Supported 2 symbols front-loaded DMRS(downlink) | 1. Support 2 symbols FL-DMRS | 2-5 |  |  | Type 4 |  | Yes |  |  |  | FR1, mandatory; FR2, Optional;, Further decide whether 2-7 and 2-8 can be jointly signaled. |  |
|  | 2-8 | Supported 2 symbols front-loaded +2 symbols additional DMRS(downlink) | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-5 |  |  | Type 4 |  | Yes |  |  |  | FR1, mandatory with capability signaling; FR2, Optional; |  |
|  | 2-9 | Support 1+3 DMRS symbols(downlink) | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-5 |  |  | Type 4 |  | No need |  |  |  | Mandatory with UE capability signalling |  |
|  | 2-10 | Support DMRS type (downlink) | Support DMRS {type 1, type 2} |  |  |  | Type 4 |  | No need |  |  |  | Type1 mandatory; Type 2 optional; At least one of the type is mandatory |  |
|  | 2-11 | Downlink PRB bundling (downlink) | 1. Support dynamic PRB bundling indication via DCI 2. Support of semi-static PRB bundling--mandatory | 2-1 |  |  | Type 4 |  | No need |  |  |  | Semi-static mandatory; Dynamic, optional; |  |
|  | 2-12 | Basic PUSCH transmission | Data RE mapping Single layer (single Tx) transmission |  |  |  |  |  | No need |  |  |  | Mandatory without UE capability |  |
|  | 2-13 | PUSCH transmission coherence | 1. Transmission coherence | 2-12 |  |  | Type 1 |  | Yes. Depending on number of SRS ports. FR2 may only have one Tx as mandatory, and thus may not need to report this capability |  |  |  | FR1:Non-coherent mandatory. Candidate values: {non-coherent, partial-non-coherent, full-coherent} |  |
|  | 2-14 | Codebook based PUSCH MIMO transmission | 1. Supported codebook based PUSCH MIMO with maximal number of supported layers | 2-13 |  |  | Type 3 |  | Yes. |  |  |  | FR1: 2 layer mandatory with capability signaling FR2: 1 layer mandatory At least 1 layer is mandatory. Candidate value: {2, 4} |  |
|  | 2-15 | non-codebook based PUSCH transmission | 1. Maximal number of supported layers (non-codebook transmission scheme): [FFS: whether the above apply for “reciprocity based”] | 2-12 |  |  | Type 3 |  | No need |  |  |  | Mandatory with 1 layer transmission Component-1 candidate values: {1, 2, 4} |  |
|  | 2-15a | Reciprocity based PUSCH transmission | Support reciprocity based PUSCH precoding | 2-12 |  |  | Type 1 |  | No need |  |  |  | Optional |  |
|  | 2-16 | Basic DMRS (uplink) Note: for both slot based and non-slot based | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbols 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols |  |  |  |  |  | No need |  |  |  | Mandatory without UE capability |  |
|  | 2-17 | Support DMRS type (uplink) | Support DMRS {type 1, type 2 } | 2-16 |  |  | Type 4 |  | No need |  |  |  | Type 1 is mandatory. At least one of the type is mandatory. RAN1 will further discuss which type or both types are mandatory |  |
|  | 2-18 | Supported 2 symbols front-loaded DMRS (uplink) | 1. Support 2 symbols FL-DMRS | 2-16 |  |  | Type 4 |  | Yes |  |  |  | FR1: mandatory FR2: optional Further decide whether component-1 and 2 will be separate FG or not. |  |
|  | 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-16 |  |  |  |  | Yes |  |  |  | Prefer jointly signaled Further decide whether 2-18 and 2-18a can be jointly signaled |  |
|  | 2-19 | Support 1+3 uplink DMRS symbols(uplink) | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-16 |  |  | Type 4 |  | No need |  |  |  | Mandatory with UE capability signaling |  |
|  | 2-20 | Beam correspondence | 1. Support Beam correspondence |  |  |  | Type 1 |  | Yes |  |  |  | Mandatory at least for FR2 |  |
|  | 2-21 | Periodic beam management report | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  |  |  | Type 1 |  | Yes |  |  |  | Mandatory at least for FR2 |  |
|  | 2-22 | Aperiodic beam management report | 1. Support report on PUSCH |  |  |  | Type 1 |  | Yes |  |  |  | Mandatory at least for FR2 |  |
|  | 2-23 | Semi-persistent beam management report | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  |  |  | Type 1 |  | Yes |  |  |  | Optional |  |
|  | 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) to measure L1-RSRP within a slot shall not exceed MB_1 2. The max number of SSB/CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS {1 only, 3 only, both 1 and 3} | 2-21, 2-22 or 2-23 |  |  | Type 1 |  | Yes |  |  |  | Support MB_1 =8 SSB/CSI-RS resources is mandatory for at least for >6Ghz bands Support MB_2 =8 SSB/CSI-RS resources is mandatory for at least for >6Ghz bands At least density of CSI-RS =3 is mandatory at least for FR2 |  |
|  | 2-25 | Beam reporting timing | 1. The number of symbols between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RB Further discussion on the unit of RB (in absolute time or number of symbols using data numerology) | 2-24 |  |  |  |  | Yes |  |  |  | Prefer Alt2. Alt 1: {14,28,42 (symbols) } --- 28 is mandatory at least for > 6GHz Alt2: Same as minimum CSI acquisition time? (Z1,Z2,Z3,Z4) for different SCS. |  |
|  | 2-26 | Receiving beam selection | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Support number of Rx beams: {1, 2, 3, 4, 5, 6, 7, 8} |  |  |  | Type 1 |  | Yes |  |  |  | Mandatory number of Rx beams 2; Optional for beam switching within one CSI-RS set; Support Rx beam switching is mandatory for bands at least > 6GHz |  |
|  | 2-27 | Beam switching | 1. Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC BB_Total. | 2-24 |  |  | Type 1 |  | Yes |  |  |  | Fix the value as 4. 2 for Tx beam, 2 for Rx beam. No need of this FG. Several possible candidate values {4, 7, 14} for further discussion If only one value is selected, this FG is not needed |  |
|  | 2-28 | A-CSI-RS beam switching timing | 1. Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KB symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS). Further discussion on the unit of KB (in absolute time or number of symbols using data numerology) | 2-27 |  |  | Type1 |  | Yes |  |  |  | 28 mandatory with capability signaling Candidate values {14, 26, 28, 42} |  |
|  | 2-29 | Beam group reporting | 1 support of beam group RSRP reporting for group of 2 beams | 2-24 |  |  |  |  | Yes |  |  |  | Optional |  |
|  | 2-30 | Uplink beam management | 1 Support of SRS based beam management and SRI indication for uplink |  |  |  |  |  | Yes |  |  |  | Mandatory |  |
|  | 2-31 | Beam failure recovery | 1. Maximal number of CSI-RS resources configured for monitoring PDCCH quality 2. Maximal number of different SSBs for monitoring PDCCH quality |  |  |  | Type 1 |  | Yes |  |  |  | Optional Component-1 candidate: {from 1 to 8} Component-2 candidate: {from 1 to 8} At least 4 CSI-RS and 4 SSBs are mandatory for FR2 |  |
|  | 2-32 | Basic CSI feedback | 1. Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2. 2Tx codebook for FR1 and FR2 3. 4Tx codebook for FR1 4. 8Tx codebook for FR1 when configured as wideband CSI report 5. p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot 6. p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot 7. a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) |  |  |  |  |  | Yes |  |  |  | Mandatory without UE capability |  |
|  | 2-33 | CSI-RS and CSI-IM reception for CSI feedback | 1. Supported max # of configured NZP-CSI-RS resources per CC, 2. Supported max # of ports across all configured NZP-CSI-RS resources per CC Note: all the candidate values are the range of capability signaling which doesn’t determine whether UE is mandatory to support all the signaling values. 3. Supported max # of CSI-IM resources 4. Supported max # of resource, including NZP-CSI-RS, ZP-CSI-RS and CSI-IM, per OFDM symbol | 2-32 |  |  | Type 3 |  | Yes. For FR1 and FR2, mandatory number of resources may be different. |  |  |  | Component-1 candidate values: {from 1 to 32} Component-2 candidate values: , further down-select between: Alt.1: {from 4 to 64}, Alt.2: {from 4 to 256}. |  |
|  | 2-34 | NZP-CSI-RS based interference measurement | 1. Support NZP-CSI-RS based interference measurement | 2-33 |  |  | Type 4 |  | No need |  |  |  | Optional |  |
|  | 2-35 | CSI report framework | 1. Maximum number of periodic CSI report setting per CC 2. Maximum number of aperiodic CSI report setting per CC 3. Maximum number of semi-persistent CSI report setting per CC 4. Minimum duration Zk,l (in symbols)for processing a CSI, k is level of CSI latency class, l is the index of SCS, l=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. 5. UE can process X CSI report(s) simultaneously. (FFS if X needs to be defined as per latency class or per codebook type or neither) FFS: candidate values: {from 5 to 32} FFS: whether X applies to A-CSI only or all CSI types | 2-34 |  |  | Type 1 |  | Yes. Mandatory number of reporting settings may be different for different FR. Reported Z may also depend on FR. X should be defined per latency class and applied to all CSI types. |  |  |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate value Zk,l: FFS |  |
|  | 2-36 | Type I single panel codebook | 1. Max # of Tx ports, FFS on whether this ports number needs to be jointly defined with # of CSI-RS resources/CSI Reports 2. Supported Codebook Mode(s) | 2-35 |  |  | Type 3 |  | Yes. Mandatory requirement would be different for different FR. |  |  |  | Component-1 candidate values: candidate values {4, 8, 12, 16, 24, 32} Component-2 candidate values: down-select between: Alt.1 Mode-1 as mandatory Alt.2: Both Mode-1 and Mode-2 are mandatory |  |
|  | 2-37 | Support Semi-open loop CSI | 1. Support Semi-open loop CSI report | 2-35 |  |  |  |  | No need |  |  |  | Optional |  |
|  | 2-38 | CSI report without PMI | CSI report without PMI | 2-35 |  |  |  |  | No need |  |  |  | Optional |  |
|  | 2-39 | CSI report with CRI | CSI report with CRI | 2-35 |  |  |  |  | No need |  |  |  | Optional |  |
|  | 2-40 | Type I multi-panel codebook | 1. Max # of Tx ports FFS on whether this ports number needs to be jointly defined with # of CSI-RS resources/CSI Reports 2. Supported Codebook Mode(s): 3. FFS whether parameter Ng need to be signaled as capability | 2-35 |  |  | Type 3 |  | No need |  |  |  | Optional Component-1 candidate values {4, 8, 12, 16, 24, 32} Component-2 candidate values: down-select between: Alt.1 Mode-1 only as mandatory Alt.2: Both Mode-1 and Mode-2 are mandatory |  |
|  | 2-41 | Type II codebook | 1. Max # of Tx ports FFS on whether this ports number needs to be jointly defined with # of CSI-RS resources/CSI Reports 2. Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type 4. Support amplitude subset restriction level, candidate value set: {no restriction, subset restriction} 5. Support bit allocation for amplitude scaling and phase, candidate value set: {equal, unequal, both} FFS to have one FG or multiple FG on above components. | 2-35 |  |  | Type 4 |  | No need |  |  |  | Optional Component-1 , candidate values {4, 8, 12, 16, 24, 32} Component-2, candidate values FFS Component-3, candidate values set: {wideband, wideband/subband} |  |
|  | 2-42 | Support Type II periodic feedback on long PUCCH | 1.Support type II periodic feedback part-1 on PUCCH formats over 4 – 14 OFDM symbols once per slot | 2-41 |  |  | Type 4 |  | No need |  |  |  | Optional |  |
|  | 2-43 | Type II codebook with port selection | 1. Number of Tx (4, 8, 12, 16, 24, 32) with parameters (L) 2. Support amplitude scaling type {wideband, subband} |  |  |  | Type 4 |  | No need |  |  |  | Optional |  |
|  | 2-44 | Basic DL PTRS | Support 1 port of PTRS is mandatory at least on FR2 |  |  |  |  |  | Yes |  |  |  | Mandatory without UE capability signaling |  |
|  | 2-45 | Downlink PTRS | 1. Supported 2 ports of PTRS | 2-44 |  |  | Type 1 |  | Yes |  |  |  | Optional |  |
|  | 2-46 | Downlink PTRS density recommendation | 2. Preferred threshold set for determine PTRS density, candidate value range is the same as that of downlink PTRS RRC configuration. | 2-44 |  |  | Type 1 |  | Yes |  |  |  | Mandatory to support the default values |  |
|  | 2-47 | Basic UL PTRS | Support 1 port of PTRS is mandatory at least on FR2 |  |  |  |  |  | Yes |  |  |  | Mandatory for FR2. For FR1, mandatory with capability, but may only support part of the patterns. |  |
|  | 2-48 | Uplink PTRS | 1. Supported 2 ports of PTRS | 2-47 |  |  | Type 1 |  | Yes |  |  |  | optional |  |
|  | 2-49 | Uplink PTRS density recommendation | 2. Preferred threshold set for determine PTRS density, candidate value range is the same as that of uplink PTRS RRC configuration. | 2-47 |  |  | Type 1 |  | Yes |  |  |  | Mandatory to support the default values |  |
|  | 2-50 | Basic TRS | 1. Support of TRS (mandatory) 2. All the periodicity are supported. |  |  |  |  |  |  |  |  |  | Mandatory without UE capability signaling |  |
|  | 2-51 | TRS (CSI-RS for tracking) | 1. Support TRS BW, candidate values set: {BWP, min(50,BWP), both} 2. TRS burst length (X), candidate values {1,2} 3. Max # of TRS resource UE is able to track simultaneously 4. Max # of TRS resources configured to UE (subject to spec support) | 2-50 |  |  | Type 1 |  | Yes |  |  |  | Max # of TRS, 1 is mandatory. |  |
|  | 2-52 | Basic SRS | 1. Support 1 port SRS transmission 2. Support periodic/aperiodic SRS transmission 3. Support SRS Frequency intra/inter-slot hopping within BWP 4. At least one SRS resource |  |  |  |  |  | No need |  |  |  | Mandatory without UE capability |  |
|  | 2-53 | SRS resources | 1. Maximum number of aperiodic SRS resources (configured to UE) per CC 2. Maximum number of aperiodic SRS resources (configured to UE) per CC per slot 3. Maximum number of periodic SRS resources (configured to UE) per CC 4. Maximum number of periodic SRS resources (configured to UE) per CC per slot 5. Maximum number of semi-persistent SRS resources (configured to UE) per CC 6. Maximum number of semi-persistent SRS resources (configured to UE) per CC per slot 7. Maximum number of SRS port per resource | 2-52 |  |  | Type 3 |  | Yes |  |  |  | Component-1: candidate value: {from 1 to 16} Component-2 candidate value: {1,2,3,4,6} Component-3: candidate value: {from 1 to 16} Component-4 candidate value: {1,2,3,4,6} Component-5: candidate value: {from 1 to 16} Component-6 candidate value: {1,2,3,4,6} Component-7 candidate values: {2, 4} |  |
|  | 2-54 | SRS transmission | 1. Minimum time interval, N in unit of symbols, between DCI triggering and A-SRS transmission, candidate value range is the same as that of N2 Note: there is a minimal timing (42 symbols) between A-CSI-RS reception and updating of A-SRS precoding 2. Maximum number of simultaneous transmitted SRS resources at one symbol, | 2-53 |  |  | Type 4 |  | Yes |  |  |  | Typically, this value should be the same as DCI triggered A-CSI-RS + RxTx switching time. Note that the 42 symbols restriction is only for non-codebook based transmission. |  |
|  | 2-55 | SRS Tx switch | 1. Support SRS Tx port switch, 2. Report whether the uplink TX switching impact to downlink receiving in a band, candidate value set {yes, no} | 2-53 |  |  | Type 3 |  | Yes |  |  |  | 2T4R, mandatory with capability Component-1 is a list of TRx pairs, candidates are {1T2R, 1T4R, 2T4R} |  |
|  | 2-56 | SRS carrier switch | 1. Report inter-cell switching time capability | 2-53 |  |  | Type 1 |  | Yes |  |  | RAN1/4 | Optional candidate values set FFS |  |
| 3.DL control channel and procedure | 3-1 | Basic DL control channel | 1) One UE-specific configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size [or CORESET size] - PDCCH DMRS scrambling determination - Single TCI state for a CORESET configuration 2) CSS and USS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - For unicast PDCCH, monitoring occasion is within the first 3 OFDM symbols of a slot - For PDCCH scheduling RMSI/OSI/RAR/paging, monitoring occasion is any OFDM symbol(s) of a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table |  | Yes |  | N.A. | N.A. | N.A |  |  |  | Mandatory without capability signaling |  |
|  | 3-2 | Unicast PDCCH monitoring following Case 1-2 | - For unicast PDCCH, monitoring occasion is within a single span of 3 OFDM symbols within a slot - Number of PDCCH blind decodes per slot with a given SCS follows Case 1-2 table for 15kHz SCS |  | Yes |  | Type 4 | No need | Applicable only to FR1 |  |  |  | optional |  |
|  | 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 |  |  | Yes |  | Type 4 | No need | No need |  |  |  | optional |  |
|  | 3-4 | More than one TCI state configurations per CORESET |  |  | Yes |  | Type 4 | No need | No need |  |  |  | optional |  |
|  | 3-5 | For unicast PDCCH, monitoring occasion is any OFDM symbol(s) of a slot |  |  | Yes |  | Type 4 | No need | [Yes] |  | RAN1 will discuss the definition of this feature |  | Need further discussion. Maybe split into two sub-cases. |  |
|  | 3-6 | Dynamic SFI monitoring and dynamic UL/DL determination |  |  | Yes |  | Type 4 | No need | No need |  |  |  | optional |  |
| 4.UL control channel and procedure | 4-1 | Basic UL control channel | 1) PUCCH format 0 over [1]-2 OFDM symbols once per slot with FH for FR1 2) PUCCH format 0 over [1]-2 OFDM symbols once per slot without FH for FR2 3) PUCCH format 1 over [4] – 14 OFDM symbols once per slot with frequency-hopping for FR1 4) PUCCH format 1 over [4] – 14 OFDM symbols once per slot without frequency-hopping for FR2 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) Multiplexing of SR and HARQ-ACK on a PUCCH 8) HARQ-ACK piggyback on PUSCH 9) Semi-static beta-offset configuration for HARQ-ACK |  | Yes |  | N.A. | N.A. | N.A. |  |  |  | Mandatory without capability signaling |  |
|  | 4-2 | [PUCCH format 0/2 once per slot for HARQ-ACK, and/or PUCCH format 0 once per slot for SR and/or PUCCH format 2 once per slot for CSI over two consecutive OFDM symbols] |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  | RAN1 | [Mandatory with capability signaling] |  |
|  | 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with FH for FR1 and without FH for FR2 |  |  | Yes |  | [Type 4] | No need | Yes |  |  | RAN1 | [Mandatory with capability signaling] |  |
|  | 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with FH for FR1 and without FH for FR2 |  |  | Yes |  | [Type 4] | No need | Yes |  |  | RAN1 | [Mandatory with capability signaling] |  |
|  | 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with FH for FR1 and without FH for FR2 |  |  | Yes |  | [Type 4] | No need | Yes |  |  | RAN1 | [Mandatory with capability signaling] |  |
|  | 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 for FR1 |  |  | Yes |  | [Type 4] | No need | Yes |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 for FR1 |  |  | Yes |  | [Type 4] | No need | Yes |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 4-8 | Frequency hopping for PUCCH formats 0 and 2 for FR2 |  |  | Yes |  | [Type 4] | No need | Yes |  |  | RAN1 | Mandatory with capability signaling |  |
|  | 4-9 | Frequency hopping for PUCCH format 1, 3, and 4 for FR2 |  |  | Yes |  | [Type 4] | No need | Yes |  |  | RAN1 | Mandatory with capability signaling |  |
|  | 4-10 | Dynamic HARQ-ACK codebook |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | [Mandatory with capability signaling] |  |
|  | 4-11 | Semi-static HARQ-ACK codebook |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | [Mandatory with capability signaling] |  |
|  | 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  | Maximum number of DL layer >=4 | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 4-13 | More than one SR configurations per PUCCH group |  |  | Yes |  | [Type 4] | No need | No need |  | RAN2 to check | RAN1 and RAN2 | optional |  |
|  | 4-14 | PUCCH transmission carrying P-CSI reporting (or piggybacked on a PUSCH) |  | PUCCH format 2, 3, or 4 | Yes |  | [Type 4] | [No need] | [No need] |  |  | RAN1 | Mandatory with capability signaling |  |
|  | 4-15 | PUCCH transmission carrying SP-CSI reporting (or piggybacked on a PUSCH) |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  | RAN1 | optional |  |
|  | 4-16 | PUSCH transmission carrying SP-CSI reporting |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  | RAN1 | optional |  |
|  | 4-17 | PUSCH transmission carrying A-CSI reporting |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  | RAN1 | Mandatory without capability signaling |  |
|  | 4-18 | More than one CSI reporting on one channel once per slot |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  | RAN1 | optional |  |
|  | 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | Mandatory with capability signaling |  |
|  | 4-20 | UCI code-block segmentation |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | Mandatory with capability signaling |  |
|  | 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | optional |  |
|  | 4-22 | More than one PUCCH transmissions per slot |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | Mandatory with capability signaling |  |
|  | 4-23 | Repetitions for PUCCH format 1, 3,and 4 over multiple slots with K = 1, 2, 4, 8 |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | [Mandatory with capability signaling] |  |
|  | 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  |  | Yes |  | [Type 4] | No need | No need |  |  | RAN1 | optional |  |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - [2 – 14] OFDM symbols for PDSCH [1-14] OFDM symbols for PUSCH once per slot - Starting symbol, and duration are determined by using the DCI - PDSCH/PUSCH mapping [type A and type B] 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL/UL HARQ processes of up to 16 [6) Semi-static UL/DL assignment by UE-common higher-layer configuration] 7) Intra-slot frequency-hopping for PUSCH [for Msg. 3] |  | Yes |  | N.A. | N.A. | N.A. |  | Note: If UE is configured with more than 8 HARQ processes, RAN4 continue to discuss the impact of 16 HARQ processes | RAN1 | Mandatory without capability signaling |  |
|  | 5-2 | RA Type 0 for PUSCH |  | Non-contiguous UL CP-OFDM | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Mandatory with capability signaling |  |
|  | 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  | Non-contiguous UL CP-OFDM | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-5 | Advanced UE processing time for N1 and N2 for PDSCH mapping type A (Capability #2) |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-6 | UE processing time for N1 and N2 for PDSCH mapping type B |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Should be included in 5-1 |  |
|  | 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Mandatory with capability signaling |  |
|  | 5-8 | Interleaving for VRB-to-PRB mapping for PUSCH |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Mandatory with capability signaling |  |
|  | 5-9 | Intra-slot frequency-hopping for PUSCH [except for Msg. 3] |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | [Mandatory with capability signaling] |  |
|  | 5-10 | Inter-slot frequency hopping for PUSCH |  |  |  |  | [Type 4] | [No need] | [No need] |  |  |  | Mandatory with capability signaling |  |
|  | 5-11 | More than one PDSCHs per slot for different TBs [in TDM manner FDM?] |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Need more discussion |  |
|  | 5-12 | More than one PUSCHs per slot for different TBs [in TDM manner FDM?] |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | More than one FDMed PUSCHs is not supported within one CC TDM need more discussion |  |
|  | 5-13 | Type 1 configured PUSCH repetitions within a slot | 1) K = 2, 4, 8 times repetitions with RV sequences | 5-19 | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-14 | Type 1 configured PUSCH repetitions over multiple slots | 1) K = 2, 4, 8 times repetitions with RV sequences | 5-19 | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-15 | Type 2 configured PUSCH repetitions within a slot | 1) K = 2, 4, 8 times repetitions with RV sequences | 5-20 | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-16 | Type 2 configured PUSCH repetitions over multiple slots | 1) K = 2, 4, 8 times repetitions with RV sequences | 5-20 | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-17 | PUSCH repetitions over multiple slots | 1) K = 2, 4, 8 times repetitions |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Mandatory with capability signaling |  |
|  |  | [PUSCH repetitions within a slot] |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5-18 | DL SPS |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Mandatory with capability signaling |  |
|  | 5-19 | Type 1 Configured UL grant | 1) K = 1 |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-20 | Type 2 Configured UL grant | 1) K = 1 |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | Mandatory with capability signaling |  |
|  | 5-21 | Pre-emption indication for DL |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-22 | CBG-based re-transmission for DL using CBGTI |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-23 | CBGFI for CBG-based re-transmission for DL |  | 5-22 | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-25 | CBG-based re-transmission for UL using CBGTI |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-26 | Semi-static rate-matching resource set configuration for DL | 1) Bitmap 1/2/3 |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  |  |  |
|  | 5-27 | Dynamic rate-matching resource set configuration for DL | 1) Bitmap 1/2/3 |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-28 | Rate-matching around LTE CRS |  |  | Yes |  | [Type 4] | [No need] | [No need] |  |  |  | optional |  |
|  | 5-29 | [DM-RS bundling across slots (1, 2, 4) slot repetition for DL/UL?)] |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 2) RRC reconfiguration of any parameters related to BWP |  | Yes |  | Type 4 |  | No need |  | This feature should be mandatory for at least BWPs which is the same as the set of specified channel BW RAN4 may discuss other BW requirements. UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP |  | Mandatory without capability signaling |  |
|  | 6-2 | [Type A BWP adaptation with same numerology] | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier | 6-1 | Yes | Type A BWP adaptation with same numerology is not possible | Type 4 |  | No need |  |  |  | Mandatory with capability signaling |  |
|  | 6-3 | [Type B] BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier | 6-1 | Yes | Type B BWP adaptation with same numerology is not possible | Type 4 |  | No need |  |  |  | Optional |  |
|  | 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time | 6-1 | Yes | BWP adaptation with different numerologies is not possible | Type 4 |  | Yes |  |  |  | Optional with capability signaling |  |
|  | 6-5 | Basic DL NR-NR CA operation | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel [at a given time] |  | Yes |  | N.A. |  | No need |  | This is conditioned on the support of DL CA band combination(s). The band combination definition is up to RAN4. |  | optional |  |
|  | 6-6 | Basic UL NR-NR CA operation | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel [at a given time] 3) One PUCCH group 4) Single TAG | [6-5] | Yes |  | N.A. |  | No need |  | This is conditioned on the support of UL CA band combination(s). The band combination definition is up to RAN4. |  | optional |  |
|  | 6-7 | Two PUCCH group | 1) Same numerology across carriers for data/control channel [at a given time] | 6-5, 6-6 |  | PUCCH on SCell is not possible | Type 2 |  | No need |  |  |  | optional |  |
|  | 6-8 | Different numerology across PUCCH groups |  | 6-5, 6-7 |  |  | Type 2 | Yes | [No need] |  |  |  | optional |  |
|  | 6-9 | Different numerologies across carriers within the same PUCCH group | 1) Same numerology between DL and UL per carrier for data/control channel [at a given time] | 6-5 |  |  | Type 2 |  | [No need] |  |  |  | optional |  |
|  | 6-10 | Cross carrier scheduling | 1) Cross carrier scheduling with CIF | 6-5, 6-6 | Yes | Cross carrier scheduling is not possible | Type 2 |  | No need |  |  |  | optional |  |
|  | 6-11 | Number of supported TAGs | Need of multiple capability question about the resolution here |  | Yes |  | Type 2 |  | No need |  |  |  | Optional for more than one TAGs |  |
|  | 6-12 | Support 2 simultaneous UL transmissions for problematic cases | Delete [1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell] [2) Case 2: Release 15 LTE-FDD HARQ timing] [3) UL offset for Case 1 based HARQ feedback] |  | Yes | 2 simultaneous UL transmissions are not supported for problematic cases | Type 2 |  | Yes |  | RAN2/4 to decide This is a UE feature for LTE for a LTE/NR dual connectivity UE | RAN2/4 | optional |  |
|  | 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) HARQ subframe offset |  | Yes |  | Type 2 | Yes | Yes |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  |  |
|  | [6-14] | [Case 2 Single Tx UL LTE-NR DC] | [Case 2: Release 15 LTE-FDD HARQ timing] | [6-xx] | [Yes] |  |  | [No] | Yes |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  |  |
|  | 6-15 | 7.5kHz UL raster shift | 7.5kHz UL raster shift |  | Yes |  | Type 2 |  | Yes |  |  | RAN4 |  |  |
|  | 6-16 | Supplemental uplink | Initial access and RRC connected operation on SUL carrier (incl 7.5kHz configurable shift) 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL | 6-15 | Yes | The UE will not be able to access or operate on a SUL carrier UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL | Type 2 or 4 |  |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | Mandatory without capability signaling for UEs supporting SUL band combination |  |
|  | 6-17 | [Supplemental uplink with different numerology] | Numerology other than that of associated DL | 6-16? | Yes | The UE will not be able to access or operate on a SUL carrier | Type 2 or 4 | Need Support not required for an FDD-only UE |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | optional |  |
|  | 6-18 | [Supplemental uplink with dynamic switch] | DCI based selection of PUSCH carrier | 6-16, 6-17? | Yes |  | Type 2 or 4 | Need Support not required for an FDD-only UE |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  | optional |  |
|  | 6-19 | [Simultaneous transmission of SRS on an UL carrier and PUSCH/PUCCH/SRS on the other UL carrier] |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6-20 | [Simultaneous reception and transmission on different carriers for each band combination] |  |  |  |  | [Type 2] | [Yes] |  |  |  |  |  |  |
|  | 6-21 | [DL search space sharing for CA] |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6-22 | [UL search space sharing for CA] |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. Channel coding | 7-1 | Channel coding | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |  | Yes | UE will not be able to transmit or receive data or control | N.A. | No need | No need |  |  | RAN1 | Mandatory without capability signaling |  |
| 8. UL TPC | 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. | EN-DC | Yes |  | Type 2 | [Yes] | Yes |  |  | RAN | optional |  |
|  | 8-2 | Operation A with single UL Tx case 1 |  | EN-DC | Yes |  | Type 2 | [Yes] | [Yes] |  |  |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, optional for UEs supporting dynamic power sharing |  |
|  | 8-2 | Basic power control operation | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 10) PHR |  | Yes |  | Type 4 | No need | No need |  |  |  | Mandatory without capability signaling |  |
|  | 8-3 | TPC-PUSCH-RNTI | Specific group DCI message for TPC commands for PUSCH |  | Yes |  | Type 4 | No need | No need |  |  |  | Mandatory without capability signaling |  |
|  | 8-4 | TPC-PUCCH-RNTI | Specific group DCI message for TPC commands for PUCCH |  | Yes |  | Type 4 | No need | No need |  |  |  | Mandatory without capability signaling |  |
|  | 8-5 | TPC-SRS-RNTI | Specific group DCI message for TPC commands for SRS |  | Yes |  | Type 4 | No need | No need |  |  |  | Mandatory without capability signaling |  |
|  | 8-6 | Absolute TPC command mode | Absolute TPC command mode |  | Yes |  | Type 4 | No need | No need |  |  |  | Mandatory without capability signaling |  |
|  | 8-7 | UL power control with 2 PUSCH closed loops | Two different TPC loops |  | Yes |  | [Type 3 or Type 4] | No need | No need |  |  |  | Mandatory with capability signaling |  |
|  | 8-8 | UL power control with 2 PUCCH closed loops | Two different TPC loops |  | Yes |  | [Type 3 or Type 4] | No need | No need |  |  |  | Mandatory with capability signaling |  |
|  | 8-9 | [Power headroom report with actual UL allocation with K2<X] |  |  | Yes |  |  | No need | No need |  |  | RAN1 | Optional |  |