3GPP TSG RAN WG1 Meeting #92                                              R1-1801658

Athens, Greece, February 26nd – March 2nd, 2018

Agenda item:	7.6.4

Source: 	MediaTek Inc.

Title: 	Considerations on initial access in NR unlicensed bands

Document for:	Discussion

# Introduction

The study item of NR-based access to unlicensed spectrum was approved in RAN #75 meeting. Besides NR based license assisted access (LAA), standalone (SA) operation of unlicensed spectrum is also in the scope of this study item. In this contribution, we discuss some potential technical problems during initial access to an unlicensed band operation. Some potential solutions are shared as well.

# Discussion

In order to support standalone operation on unlicensed spectrum, initial access designs need to be addressed. In our view, a unified design for both NR licensed and unlicensed operations should be pursued whenever it is possible. However, due to some regulation requirements in unlicensed spectrum, such as listen-before-talk (LBT) and occupied channel bandwidth (OCB), some designs from NR licensed operation may not be adopted to NR unlicensed operation directly without any modification.

## SSB/RMSI transmission

In NR licensed operation, there are three patterns to multiplex SSB and RMSI transmissions as shown in Figure 1. For unlicensed band operation, how to multiplex SSB and RMSI should be discussed since there is requirement on the occupied channel bandwidth (OCB). In order to meet OCB, multiplexing SSB and RMSI in the frequency dimension, such as Pattern 3 in Figure 1, can be considered. However, the practical limitation and UE capability requirement should be discussed when this pattern is employed.

Figure 1: Multiplexing patterns of SS/PBCH and RMSI transmission

As to the transmission occasion of SSB, due to the channel uncertainty caused by LBT, periodic transmission of SSBs cannot be guaranteed. Therefore, whether opportunistic transmission occasions following periodic ones are allowed should be discussed. It is beneficial to arrange such additional opportunities for SSB/RMSI transmissions especially for cell search and RRM measurement purposes. But how to design them and what disadvantages they may cause should be carefully addressed.

Observation 1: In order to meet the OCB requirement, multiplexing SSB and RMSI transmission in the frequency dimension can be considered with NR unlicensed operation.

Observation 2: If network cannot transmit SSB and RMSI at pre-defined periodic time slots due to LBT failure, it is beneficial to have additional opportunities for SSB/RMSI transmission immediately following the pre-defined time slots.

Proposal 1:  Design for SSB and RMSI transmission in NR unlicensed bands should take OCB and LBT regulations into consideration.

## Random access procedure

It is essential to support RACH procedure in unlicensed spectrum in order to support standalone unlicensed operation. Unlike LTE, there are many different numerologies supported in NR. Consequently, for PRACH waveform design, the OCB requirement may be easily met by the scalable numerology in NR without employing the block interleaved FDMA (B-IFDMA) design.

To reduce the channel uncertainty due to LBT requirement, it is desirable to exploit the possibility to allocate pre-defined RACH occasions to the same maximum channel occupancy time (MCOT) as SSB/RMSI transmission. In this way, it can not only reduce UE’s LBT overhead but also RACH access latency. However, it may be challenging for UE to complete downlink synchronization, system information reading and PRACH transmission within such a tight duration. Whether this design is advantageous and practical deserves further discussion.

Regarding the reduction of RACH access latency, a straightforward solution would be to reduce the number of steps required in the RACH procedure. Therefore, the simplified two-step RACH procedure may be more suitable for unlicensed band operation than the conventional four-step RACH. Furthermore, if the two-step RACH is employed, the OCB requirement for RACH can be probably easily met since UE transmits PRACH and data simultaneously in the first step.

In summary, we think NR-unlicensed operation should seek to reuse designs for NR-licensed operation when it is possible to meet the regulation requirements. In addition, NR should strive to reduce UE’s LBT overhead and RACH access latency in NR unlicensed spectrum.

Observation 3: For PRACH design, the OCB requirement may be easily met by the scalable numerology in NR.

Observation 4: It can reduce UE’s LBT overhead and RACH access latency if pre-configured RACH resources immediately following SSB and RMSI transmission within MCOT. However, it may be challenging for UE to complete downlink synchronization, system information reading and PRACH transmission within such a tight time window.

Observation 5: Since the availability of channel access becomes unpredictable with LBT, the two-step RACH may be more appealing than the 4-step RACH.

Observation 6: If the two-step RACH is employed, the OCB requirement can be easily met since UE transmits PRACH and data simultaneously in the first step.

Proposal 2: NR strives to reduce UE’s LBT overhead and RACH access latency in a NR unlicensed operation.

Proposal 3: Regarding the PRACH waveform design, NR seeks for a unified design for NR licensed and unlicensed operations as much as possible.

## Paging and measurement

For paging and measurement, pre-defined occasions are given in licensed spectrum operation to ensure reliable services. However, with unlicensed operation, the availability of medium becomes unpredictable due to LBT. For paging, this implies UE may not able to receive paging signal at pre-defined time slots when LBT fails. Similarly, periodic measurement samples may not be available since the network cannot guarantee the periodic downlink reference signal transmission is always possible. Thus, it is a design issue about how to take the LBT regulation into account when designing paging, measurement and mobility for unlicensed operation.

Proposal 4: NR takes the LBT regulation into consideration when designing paging, measurement and mobility for unlicensed operation.

# Conclusion

In this paper, we discuss design issues with during the initial access procedure in NR unlicensed spectrum. Our observations and proposals are summarized as follows.

Observation 1: In order to meet the OCB requirement, multiplexing SSB and RMSI transmission in the frequency dimension can be considered with NR unlicensed operation.

Observation 2: If network cannot transmit SSB and RMSI at pre-defined periodic time slots due to LBT failure, it is beneficial to have additional opportunities for SSB/RMSI transmission immediately following the pre-defined time slots.

Observation 3: For PRACH design, the OCB requirement may be easily met by the scalable numerology in NR.

Observation 4: It can reduce UE’s LBT overhead and RACH access latency if pre-configured RACH resources immediately following SSB and RMSI transmission within MCOT. However, it may be challenging for UE to complete downlink synchronization, system information reading and PRACH transmission within such a tight time window.

Observation 5: Since the availability of channel access becomes unpredictable with LBT, the two-step RACH may be more appealing than the 4-step RACH.

Observation 6: If the two-step RACH is employed, the OCB requirement can be easily met since UE transmits PRACH and data simultaneously in the first step.

Proposal 1:  Design for SSB and RMSI transmission in NR unlicensed bands should take OCB and LBT regulations into consideration.

Proposal 2: NR strives to reduce UE’s LBT overhead and RACH access latency in a NR unlicensed operation.

Proposal 3: Regarding the PRACH waveform design, NR seeks for a unified design for NR licensed and unlicensed operations as much as possible.

Proposal 4: NR takes the LBT regulation into consideration when designing paging, measurement and mobility for unlicensed operation.