3GPP TSG RAN WG1 RAN1 Meeting #92                                                   R1-1801664

Athens, Greece, Feb. 26th – Mar. 2nd, 2018

Agenda Item: 7.1.6.1

Source: MediaTek Inc.

Title: Remaining issues of UL power control

Document for: Discussion and Decision

# Overview

UL power control is an important factor for network coverage. In NR, owing to the rich scheduling flexibility, careful speciation on the timing of applying a UL power control command is required. From UE perspective, the timing requirement will define the practicality of the UL power control. In this contribution, we will discuss the values for KPUSCH, KPUCCH and KSRS, regarding the consistency with the timing requirement of N2 for UL transmissions.

# Values of KPUSCH and KPUCCH

According to the following agreement, the values of KPUSCH and KPUCCH should follow the specification in DCI, i.e., K2 if available:

Agreement For PUSCH transmission triggered by uplink grant, K_PUSCH is the time duration between the uplink grant and the start of the PUSCH transmission For PUCCH transmission triggered by PDSCH corresponding to downlink assignement, K_PUCCH is the time duration between the downlink assignment and the start of the PUCCH

On the other hand, for the case with group-common DCI where there is no K2 indication, additional specification is required. Since UE can receive other UL grant and CSI request from the same control occasion,  KPUSCH and KPUCCH should be regarded as part of the UL processing and apply a timing requirement no shorter than N2. Based on the summarized proposals in [1], the following are suggested:

Proposal 1: For UE-specific DCI, i.e., DCI format 0_0 or DCI format 0_1 scrambled by the UE’s RNTI, the following option in [1] for KPUSCH is adopted:

Alt1: KPUSCH=K2, where K2 denotes the delay between UL grant in DL and UL data (PUSCH) transmission.

Proposal 2: For group-common DCI, i.e., DCI format 2_2 scrambled with TPC-PUSCH-RNTI, either of the following options in [1] for KPUSCH is adopted so that no stricter timing requirement than N2 is imposed:

Alt1: KPUSCH is defined as a fixed value.

      - e.g. implicitly derive KPUSCH value from its own K2 set as configured by RRC.

Alt2: KPUSCH is to be specified as the latest slot which guarantees UE’s minimum PUSCH processing time.

      - e.g. as the UE’s minimum processing time for PUSCH, that is, as the latest slot which guarantees UE’s minimum PUSCH processing time,  as defined in section 6.4 of TS 38.214

For KPUCCH, analogous proposals are suggested:

Proposal 3: For UE-specific DCI, i.e., DCI format 1_0 or DCI format 1_1 scrambled by the UE’s RNTI, the following option in [1] for KPUCCH is adopted:

Alt1: KPUCCH=K2, where K2: delay between UL grant in DL and UL PUSCH transmission.

Proposal 4: For group-common DCI, i.e., DCI format 2_2 scrambled with TPC-PUCCH-RNTI, either of the following options in [1] for KPUCCH is adopted so that no stricter timing requirement than N2 is imposed:

Alt1: KPUCCH is defined as a fixed value.

Alt2: KPUCCH is the same as KPUSCH for group-common DCI.

# Value of KSRS

SRS transmission can share similar consideration as for PUCCH. It is also useful to follow N2 requirement as for other UL transmissions and UL related requests can be jointly processed. In this regard, the following option for KSRS in [1] is suggested:

Proposal 5: For group-common DCI, i.e., DCI format 2_3 scrambled with TPC-SRS-RNTI, either of the following options in [1] for KSRS is adopted so that no stricter timing requirement than N2 is imposed:

Alt1: KSRS is the same as KPUSCH for group-common DCI.

Alt3: KSRS is RRC configured as part of the SRS resource set.

# Summary

In this contribution, the following suggestions for completing the timing requirement for UL power control are provided:

Proposal 1: For UE-specific DCI, i.e., DCI format 0_0 or DCI format 0_1 scrambled by the UE’s RNTI, the following option in [1] for KPUSCH is adopted:

Alt1: KPUSCH=K2, where K2 denotes the delay between UL grant in DL and UL data (PUSCH) transmission.

Proposal 2: For group-common DCI, i.e., DCI format 2_2 scrambled with TPC-PUSCH-RNTI, either of the following options in [1] for KPUSCH is adopted so that no stricter timing requirement than N2 is imposed:

Alt1: KPUSCH is defined as a fixed value.

      - e.g. implicitly derive KPUSCH value from its own K2 set as configured by RRC.

Alt2: KPUSCH is to be specified as the latest slot which guarantees UE’s minimum PUSCH processing time.

      - e.g. as the UE’s minimum processing time for PUSCH, that is, as the latest slot which guarantees UE’s minimum PUSCH processing time,  as defined in section 6.4 of TS 38.214

For KPUCCH, analogous proposals are suggested:

Proposal 3: For UE-specific DCI, i.e., DCI format 1_0 or DCI format 1_1 scrambled by the UE’s RNTI, the following option in [1] for KPUCCH is adopted:

Alt1: KPUCCH=K2, where K2: delay between UL grant in DL and UL PUSCH transmission.

Proposal 4: For group-common DCI, i.e., DCI format 2_2 scrambled with TPC-PUCCH-RNTI, either of the following options in [1] for KPUCCH is adopted so that no stricter timing requirement than N2 is imposed:

Alt1: KPUCCH is defined as a fixed value.

Alt2: KPUCCH is the same as KPUSCH for group-common DCI.

Proposal 5: For group-common DCI, i.e., DCI format 2_3 scrambled with TPC-SRS-RNTI, either of the following options in [1] for KSRS is adopted so that no stricter timing requirement than N2 is imposed:

Alt1: KSRS is the same as KPUSCH for group-common DCI.

Alt3: KSRS is RRC configured as part of the SRS resource set.

References

- R1-1801047, “Offline summary for Al 7.6.1 NR UL power control in non-CA aspects”, ZTE, Sanechips, Vancouver, Canada, Jan. 22nd -26th, 2018.