3GPP TSG RAN WG1 Meeting #92 	R1-1801678

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item: 7.2.5

Source: MediaTek Inc.

Title: ACK/NACK feedback design and reliability for NR URLLC

Document for: Discussion and Decision

# Introduction

One of the NR work item [1] objectives for URLLC is:

- Support of ultra-reliable part of URLLC [RAN1, RAN2, RAN4]

  - Identify techniques to meet the URLLC requirements set forth by [TR38.913] starting after RAN#76

  - Conduct corresponding URLLC specific normative work after RAN#78 for the selected techniques

A general URLLC reliability requirement for one transmission of a packet is 1-10-5 for 32 bytes with a user plane latency of 1ms [2]. For URLLC the target for user plane latency should be 0.5ms for UL and 0.5ms for DL.

The physical uplink control channel in NR supports multiple formats as shown in Table 1 [3].

Table 1: PUCCH formats.

| PUCCH format | Length in OFDM symbols | Number of bits |
| --- | --- | --- |
| 0 | 1 – 2 | ≤2 |
| 1 | 4 – 14 | ≤2 |
| 2 | 1 – 2 | >2 |
| 3 | 4 – 14 | >2 |
| 4 | 4 – 14 | >2 |

In this contribution, we analysis the reliability aspects for PUCCH format_0, and provide a mechanism to enhance the PUCCH performance in terms of NACK-to-ACK errors. This is a re-submission of R1-1716621.

# Discussion

HARQ based transmission is essential to achieve the strict reliability requirements for URLLC with efficient use of radio resources. For HARQ based DL transmission, the probability for successful DL transmission will depends on the reliability of the uplink control channel (PUCCH) that carries ACK/NACK feedback. Considering an HARQ based DL transmission with one retransmission, the probability (P) of successfully delivering a packet is given by

P=PPDCCHPPDSCH1Successful receptionfrom 1st trans.+1-PPDCCH1-PDTX→ACKPPDCCH,Re-TxPPDSCH,  Re-Tx1Successful reception after 1st PDCCH failure+PPDCCH1-PPDSCH11-PNACK→ACKPPDCCH,Re-TxPPDSCH,Re-Tx2Successful reception after 1st PDSCH failure,                   (1)

where PPDCCH is the probability of successfully decoding the PDCCH, PPDSCH1 is the probability of successfully decoding the PDSCH transmission without soft combining, PPDSCH2 is the probability of successfully decoding the PDSCH transmission with soft combining. PDTX→ACK (resp. PNACK→ACK) are the probabilities of falsely detecting DTX (resp. NACK) as ACK at the gNB. As it can be seen in (1), the successful detection of uplink DTX and NACK at the gNB is essential for the reliability and latency of HARQ based DL transmission. Thus, design of PUCCH should be ensure very low impact of DTX-to-ACK and NACK-to-ACK errors. Missed ACK error (i.e. ACK-to-DTX) results in unnecessary retransmission, and it does not affect the reliability of HARQ based DL transmission. However, missed ACK errors need to be kept low to preserve the spectral efficiency. To avoid unnecessary retransmissions, missed ACK target similar to LTE should be considered, which is 10-2.

Comparing the contributions of the DTX-to-ACK and NACK-to-ACK errors, we can see that DTX-to-ACK errors can only occur when PDCCH detection fails, while the NACK-to-ACK errors occur when the PDSCH decoding fails. For LTE PDCCH BLER target was ~ 10-2 and PDSCH BLER target was ~ 10-1. For URLLC similar asymmetry between PDCCH BLER and PDSCH BBLER can be expected with a PDCCH BLER target between 10-6 and 10-3 and a larger PDSCH BLER target potentially between 10-1 and 10-3, we can therefore expect that for URLLC the NACK-to-ACK to be more critical than the DTX-to-ACK errors.

In this contribution, we focus on methods to enhance the PUCCH performance in terms of missed ACK and NACK-to-ACK errors. For the analysis we consider a fixed DTX-to-ACK error probability of 10-2, a missed ACK target of 10-2 and a NACK-to-ACK target ≤10-4.

Given the low latency requirements for URLLC, PUCCH format_0 can be considered the most relevant for URLLC scenario. The main approaches to enhance the reliability of PUCCH are receive diversity, time repetition and frequency hopping.

## Missed ACK and NACK-to-ACK errors

Here we show the performance of PUCCH format_0 with different number of receive antennas. Simulation parameters are provided in the Table 3 in the appendix. Figure 1 shows the missed ACK and NACK-to-ACK errors with number of receive antennas 1, 2 and 4. Increasing the number of receive antennas enhances the PUCCH reliability. Further enhancements can be achieved with repetition and frequency hopping.

Figure 1: Missed ACK and NACK-to-ACK error rates for PUCCH format_0, 2PRBs

Table 2 lists the required SNR for achieving a given PUCCH reliability targets. It can be seen that one receive antenna at the gNB is not enough to achieve the required performance targets when PUSCH is used. Furthermore, the required SNR for achieving the target NACK-to-ACK error rate is generally higher than the required SNR for achieving the target missed ACK rate. This gap gets even larger with lower NACK-to-ACK error rate target. In addition, the gap between the required SNRs for missed ACK and NACK-to-ACK is changing based on the system settings (here it is the number of PRBs and number of receive antennas).

Table 2: Required SNR (dB) for PUCCH format_0 reliability targets

|  | 1PRB |  |  | 2PRBs |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | nRX = 1 | nRX = 2 | nRX = 4 | nRX = 1 | nRX = 2 | nRX = 4 |
| Missed ACK (1%) | 16 | 5.8 | 0.02 | 12.9 | 3 | -2.82 |
| NACK-to-ACK (10-4) | 19.33 | 6.8 | 0.7 | 15.24 | 3.65 | -2 |
| NACK-to-ACK (10-5) | Too high | 12.2 | 3.73 | 26 | 10 | 0.2 |

Observation 1: receive diversity is essential for enhancing the reliability of PUCCH.

Observation 2: the required SNR for achieving the target NACK-to-ACK error rate is generally higher than the required SNR for achieving the target missed ACK rate.

Observation 3: the difference between the required SNR for achieving the target missed ACK and NACK-to-ACK error rates depends on the system setting (e.g. number of PRBs and number of receive antennas).

# Asymmetric NACK and ACK transmissions

One option to achieve the reliability targets (missed ACK and NACK-to-ACK errors) for PUCCH is by transmitting the PUCCH with higher power (PPUCCH) to achieve the stricter of the two targets. In addition to other system parameters, the PUCCH power can be specified as follows

PPUCCH=f(maxRequired_PMissed ACK, Required_PNACK-to-ACK),

where Required_PMissed ACK is the required transmission power to achieve the target missed ACK rate, and Required_PNACK-to-ACK is the required transmission power to achieve the target NACK-to-ACK error rate. Considering that ACK transmission are expected to happen more often compared to NACK transmission, this approach will lead to inefficient use of the transmission power. Another approach is to adjust the transmission power of PUCCH based on the UCI content, i.e. depending on whether the ACK or NACK is being transmitted. The PUCCH transmission power PPUCCH will be based on Required_PMissed ACK if there is an ACK is to be transmitted, and based on Required_PNACK-to-ACK if there is a NACK to be transmitted. From the ACK/NACK perspectives, two PUCCH transmission powers can be defined as: PPUCCHACK is the PUCCH power when there is an ACK to be transmitted, PPUCCHNACK is the PUCCH power when there is a NACK to be transmitted. For two bits UCI, the PPUCCHACK can be used when both bits are ACK, and PPUCCHNACK when at least one of the bits is NACK. Figure 2 shows the PUCCH performance with different transmission powers for ACK and NACK. The NACK-to-ACK target of 10-4 can be achieved with about 1 dB and 2 dB increase (for nRX= 2 and 1, respectively) in the PUCCH power in the case on NACK transmission. Higher power increase for NACK transmission will be need to achieve the 10-5 NACK-to-ACK target.

Figure 2: Missed ACK and NACK-to-ACK error rates for PUCCH format_0 with asymmetrical transmission power (2PRBs)

The main advantage of using different power levels for ACK and NACK transmissions is that average consumed power by the UE will be less compared to using the same power to achieve the targeted missed ACK and NACK-to-ACK performance. For example, assuming 10% NACK probability (as a worst case scenario) then using a PPUCCHNACK with 3dB boost compared to the PPUCCHACK results in 0.4dB average power increase only, this is to be compared with the 3dB average power increase if both the PPUCCHNACK and PPUCCHACK are boosted by 3dB. Adopting this solution will reduce the power consumption and the inter-cell interference as well.

Proposal 1: Support different PUCCH transmission power level depending on whether ACK or NACK is transmitted.

# Conclusion

In this contribution, we mainly discuss the reliability of PUCCH format_0 for ACK/NACK feedback transmission, we have the following observations and proposal:

Observation 1: receive diversity is essential for enhancing the reliability of PUCCH.

Observation 2: the required SNR for achieving the target NACK-to-ACK error rate is generally higher than the required SNR for achieving the target missed ACK rate.

Observation 3: the difference between the required SNR for achieving the target missed ACK and NACK-to-ACK error rates depends on the system setting (e.g. number of PRBs and number of receive antennas).

Proposal 1: Support different PUCCH transmission power level depending on whether ACK or NACK is transmitted.

# References

- RP-170847, “New WID on New Radio Access Technology”, NTT DOCOMO.

- Rel-14 TR 38.913, "Study on Scenarios and Requirements for Next Generation Access Technologies", RAN#74, December 2016.

- 3GPP TS 38.211: “NR; Physical channels and modulation”, V15.0.0 (2017-12)

# Appendix

Table 3: Simulation parameters for link-level simulation

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Bandwidth | 20 MHz |
| Subcarrier Spacing | 15 kHz |
| Antenna Configuration | 1Tx, and (1, 2, 4) Rx antennas |
| Number of PRBs | 1 and 2 |
| Number of Symbols | 1 Symbol |
| Channel | TDL-C with 300ns RMS delay, @ 3 km/h |
| Noise estimation | Ideal |
| Performance metrics | DTX-to-ACK probability of 0.01 |