3GPP TSG RAN WG1 Meeting #92		R1-1801697

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.3.1.4

Source:	NEC

Title:	SPS Activation and Deactivations in NR

Document for:	Discussion and decision

# Introduction

In NR, SPS activation and deactivation (i.e. Type 2 UL transmission without UL grant/DL SPS) is supported dynamically via DCI format. The following agreement was reached in the last meeting:

Agreements (RAN1#90):

- Support using MAC CE as an acknowledgement for L1 signalling for activation/deactivation of Type 2 UL transmission without grant (similar/same behaviour as in LTE SPS).

Agreements (RAN1#91):

- Activation and deactivation signaling for Type 2 UL transmission without UL grant/DL SPS is differentiated by different values of two fields in the DCI.  FFS details.

In this contribution, we discuss how to achieve SPS activation and deactivation dynamically via DCI format in NR.

# SPS Activation and Deactivation in NR

In LTE, Layer 1 achieves the following functionalities or procedures via the existing DCI format: to activate SPS transmission, to enable adaptive retransmissions and to deactivate/release the SPS transmissions. The procedure is that the SPS is activated or deactivated by scrambling the DCI with SPS C-RNTI and by setting some special fields in the DCI format to a specific bit pattern, for example as shown on Table 1 and 2 below for activation and releasing respectively [3]. In addition, for LTE, some bit patterns were used for extending the CRC (i.e. virtual CRC) to reduce the DCI false detection, and for NR, it is acceptable to keep the CRC extension as in LTE SPS.

Table 1. Special fields for Semi-Persistent Scheduling Activation PDCCH/EPDCCH Validation

|  | DCI format 0 | DCI format 1A | DCI format 2/2A/2B/2C/2D |
| --- | --- | --- | --- |
| TPC command for scheduled PUSCH | set to '00' | N/A | N/A |
| Cyclic shift DM RS | set to '000' | N/A | N/A |
| Modulation and coding scheme and redundancy version | MSB is set to '0' | N/A | N/A |
| HARQ process number | N/A | FDD: set to '000' TDD: set to '0000' | FDD: set to '000' TDD: set to '0000' |
| Modulation and coding scheme | N/A | MSB is set to '0' | For the enabled transport block: MSB is set to '0' |
| Redundancy version | N/A | set to '00' | For the enabled transport block: set to '00' |
| New data indicator | set to '0' | set to '0' | For the enabled transport block: set to '0' |

Table 2. Special fields for Semi-Persistent Scheduling Release PDCCH/EPDCCH Validation

|  | DCI format 0 | DCI format 1A |
| --- | --- | --- |
| TPC command for scheduled PUSCH | set to '00' | N/A |
| Cyclic shift DM RS | set to '000' | N/A |
| Modulation and coding scheme and redundancy version | set to '11111' | N/A |
| Resource block assignment and hopping resource allocation | Set to all '1's | N/A |
| HARQ process number | N/A | FDD: set to '000' TDD: set to '0000' |
| Modulation and coding scheme | N/A | set to '11111' |
| Redundancy version | N/A | set to '00' |
| Resource block assignment | N/A | Set to all '1's |
| New data indicator | set to '0' | set to '0' |

For NR, most of the fields of the DCI formats are already agreed, but not completely stable. However, some fields can be already identified to validate the SPS activation and deactivations like LTE. For example, some or all of the following special fields can be set to a specific bit pattern for UL DCI formats 0_0/0_1 and DL DCI formats 1_0/1_1:

Table 3. Special fields for Semi-Persistent Scheduling Activation PDCCH Validation

|  | DCI format 0_0 | DCI format 0_1 | DCI format 1_0/1_1 |
| --- | --- | --- | --- |
| Modulation and coding scheme | MSB is set to '0' | MSB is set to '0' | For the enabled transport block: MSB is set to '0' |
| New data indicator | set to '0' | set to '0' | For the enabled transport block: set to '0' |
| Redundancy version | set to '00' | set to '00' | For the enabled transport block: set to '00' |
| HARQ process number | set to '0000' | set to '0000' | set to '0000' |
| TPC command for scheduled PUSCH | set to '00' | set to '00' | N/A |

Table 4. Special fields for Semi-Persistent Scheduling Release PDCCH Validation

|  | DCI format 0_0 | DCI format 1_0 |
| --- | --- | --- |
| Frequency domain resource assignment | set to all '1's | set to all '1's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| New data indicator | set to '0' | set to '0' |
| Redundancy version | set to '00' | set to '00' |
| HARQ process number | set to '0000' | set to '0000' |
| TPC command for scheduled PUSCH | set to '00' | N/A |

Furthermore, for NR, it is also important to support HARQ-ACK feedback transmission confirming the DL SPS release similar to LTE to avoid unwanted PUCCH transmission from the UE if it missed the DL SPS release. Therefore, PUCCH resource indicator field as well as PDSCH-to-HARQ feedback timing indicator field in DCI format 1_0 should be used to identify the PUCCH resource and the transmission timing of the HARQ feedback.

Proposal 1: Similar to LTE, the SPS in NR should be activated or deactivated by scrambling the DCI with SPS C-RNTI and by setting some special fields in the DCI format to a specific bit pattern, for example some or all of the fields shown on Table 3 and 4.

Proposal 2: Support the HARQ-ACK feedback transmission confirming the DL SPS release for NR where PUCCH resource indicator field as well as PDSCH-to-HARQ feedback timing indicator field in DCI format 1_0 are used to identify the PUCCH resource and the transmission timing of the HARQ feedback.

# Conclusion

In this contribution, we discussed how to achieve SPS activation and deactivation dynamically via DCI format in NR and we have the following proposals:

Proposal 1: Similar to LTE, the SPS in NR should be activated or deactivated by scrambling the DCI with SPS C-RNTI and by setting some special fields in the DCI format to a specific bit pattern, for example some or all of the fields shown on Table 3 and 4.

Proposal 2: Support the HARQ-ACK feedback transmission confirming the DL SPS release for NR where PUCCH resource indicator field as well as PDSCH-to-HARQ feedback timing indicator field in DCI format 1_0 are used to identify the PUCCH resource and the transmission timing of the HARQ feedback.

# References

- Chairman's Notes from RAN1#90bis (final version).

- Chairman's Notes from RAN1#91 (final version).

- LTE TS 36.213