3GPP TSG RAN WG1 Meeting #92		                                                      R1-1801704

Athens, Greece, February 26th – March 2nd, 2018

Source:	CATT

Title:	Discussion on Tx diversity scheme in PC5

Agenda Item:	6.2.3.3.1

Document for:	Discussion and Decision

Introduction

In RAN1 #90 meeting, the potential Tx diversity schemes for PC5 are discussed and evaluated, particularly with following achieved working assumption [1].

Working Assumption (may be revisited based on RAN4 response):

- For designing PSSCH, RAN1 assumes the use of two-port non-transparent transmit diversity

  - The use of non-transparent transmit diversity is configured.

  - Details, including diversity scheme, are FFS

  - Support of transmission and/or reception up to UE capability

    - Note: It is RAN1 understanding that requirements on capabilities can be set at regional level and are outside 3GPP scope

  - Send LS to RAN4 to ask their opinion about when non-transparent scheme for transmit diversity is used by Rel-15 UEs:

    - Impact on Rel-14 UEs of PSSCH-RSRP measurement accuracy

    - MPR for Rel-15 UEs

- Non-transparent Transmit diversity is not used in the following cases:

  - When communicating with Rel-14 UEs

  - When there is a high probability of resource collision with Rel-14 UEs

- Note: Some companies observe that the performance of MMSE-IRC receiver degrades when a non-transparent Transmit diversity scheme is used in interference limited scenarios with a dominant interferer

Furthermore, RAN1 has the following list of candidate two-port non-transparent diversity schemes [2]:

- STBC (including half symbol STBC proposal in R1-1705002)

- SFBC

- PVS in time domain

Note: other schemes are not precluded

In RAN1 #91 meeting, the following agreement is achieved:

Agreement:

- Assuming the previous WA of introducing non-transparent transmit diversity is confirmed, for two-port non-transparent transmit diversity for PSSCH, downselect option 1 as WA among the following candidate schemes

  - Working assumption: Option 1: SFBC-based scheme (including PAPR preserving)

    - FFS whether to apply slot-level PVS

  - Option 2: STBC-based (including half symbol)

Note: Companies are encouraged to perform evaluations for the above options

This is a revision of R1-1720161 [3], some Tx diversity schemes were added in this contribution.

Two-port DMRS design

The following paragraphs are mainly focused on DMRS designs on two antenna ports. There are four potential DMRS design options which are raised in RAN1 #89 meeting:

- Option 1: CDM manner [4], as shown in Figure 4-a. Legacy DMRS sequence is used in one antenna port, the DMRS sequence in another antenna port can be generated by different DMRS cyclic shift.

- Option 2: CDM manner [5], as shown in Figure 4-b. This option is similar as option 1, while the difference is that legacy DMRS sequence is alternated between two antenna ports.

- Option 3: FDM manner [6], as shown in Figure 4-c. Legacy DMRS sequence is transmitted through two antenna ports in a comb manner.

- Option 4: Partial CDM manner [6], as shown in Figure 4-d. The first DMRS sequence is the same as a legacy Rel-14 DMRS, while the second DMRS sequence with half the length of the first sequence is specifically designed based on the first DMRS sequence.

Since option 1 and option 2 are very similar, the only difference is that a DMRS sequence will alternate between two antenna port or not. In option 4, only half sequence will be employed to channel estimation in antenna port #2, it will degrade the channel estimation performance. In our evaluation, only option 1 and option 3 are evaluated and the evaluation results are shown in [7]. Based on the evaluation results, it can be observed that the link performance of option 1 is slightly better than option 3, and option 1 has better CM performance than option 3. Therefore, option 1 is preferred in DMRS design for two antenna port Tx diversity scheme.

Figure 1: DMRS design options

Proposal 1: In two-port DMRS design, the DMRS sequence in one of the antenna ports shall be same as that of Rel-14, and the DMRS sequence in another antenna port could be differentiated by different cyclic shift.

Tx diversity schemes in PSSCH

The details about the candidate Tx diversity schemes are provided as following:

- STBC, as shown in Figure 2.

The symbols sent from two antennas over two paired SC-FDM symbols form an orthogonal Alamouti code. STBC requires even number of OFDM symbols, which can obtain full diversity gain.

The details of the orphan and paired symbols are illustrated in Figure 2. With the consideration of AGC and GP symbols, two orphan symbols are assigned to AGC and GP symbols, others symbols can construct paired symbols. The orphan symbols in AGC and GP symbol can be transmitted by repetition manner in two antennas.

Figure 2: STBC

- SFBC, as shown in Figure 3.

Similar as 2 antenna Tx diversity in DL, the adjacent REs in frequency domain will be constructed as a paired RE. This scheme can obtain full diversity gain. However, it breaks the single carrier property, and leads to higher CM.

Figure 3: SFBC Tx diversity scheme

- SC-SFBC (PAPR preserving) [8], as shown in Figure 4.

SC-SFBC is a variation of SFBC. Alamouti precoding is applied at subcarrier level, after DFT precoding. Alamouti precoded symbols can be mapped onto non-adjacent subcarrier k1 and k2=p-1-k1modM in such a way that PAPR is preserved, M is the DFT precoder size and P is an even integer (as closed as possible to M/2). But this scheme will also lead to the degradation of link performance gain.

Figure 4: SC-SFBC Tx diversity scheme

- Virtual Half-symbol STBC [9], as shown in Figure 5.

This scheme is a variation of STBC, which employs Alamouti coding before DFT processing. Based on these operations, the half-symbol STBC scheme is not constrained by the paired symbols, it can be employed by any number of symbols. Furthermore, it can maintain the single carrier property and provide full diversity gain

However, since the paired virtual half-symbols don’t have a cyclic shift interval between them, therefore this scheme is sensitive to ISI (ICI) due to loss of orthogonality between the paired virtual half-symbols, especially,  in multipath frequency selective channels.

Figure 5: Half-symbol STBC

- PVS+STBC/SFBC [10], as shown in Figure 6.

In this scheme, the combination of time-domain pre-coder vector switching and two antenna port STBC (or SFBC) is shown as Figure 5. Two-antenna port STBC (or SFBC) is applied on each data symbol, and the pre-coder vector switching across slots is applied to two antenna ports. The scheme requires at least 4 antennas to provide better link performance [10]. Currently, the work assumption of antenna configuration is 2x2 in the study of Tx diversity schemes. Whether supporting of 4 transmission antenna needs further discussion.

Figure 6: PVS+STBC/SFBC

Based on the above analysis, we will mainly focus on STBC and SFBC schemes, the related evaluation results on both schemes are provided in company’s contribution [7][11]. We can obtain the following observations based on the evaluation results:

- The CM performance of SFBC in antenna port 2 is about 0.5dB higher than that of Rel-14 signal in 16QAM modulation, and about 0.7 dB higher in QPSK modulation. STBC and SC-SFBC keep the same CM performance as that of Rel-14 signal.

- In noise scenario, SFBC provides the best link performance gain over STBC and SC-SFBC in all the scenarios, but SC-SFBC is still better than STBC, STBC will degrade the link performance gain in high moving speed scenario

- In interference scenario, with the working assumption that MMSE-MRC receiver is used in Rel-14 UE, Rel-14 UE receiver is not sensitive to the interference type, whether the interferer is Rel-14 UE or Rel-15 UE with different Tx diversity schemes.

- In interference scenario, if MMSE-IRC receiver is used in Rel-14 UE, only in the scenario with low speed, higher SNR and only one dominant interfering UE, Rel-14 UE receiver is sensitive to the interference type. In other scenarios, Rel-14 is not sensitive to the interference type.

Based on the above analysis and observations in the link level evaluation results, we prefer SFBC or SC-SFBC shall be applied for Rel-15 PSSCH.

Proposal 2: SFBC or SC-SFBC shall be applied for Rel-15 PSSCH.

Conclusion

In this contribution, the details of antenna ports and Tx diversity schemes in PSSCH are discussed. Particularly, we have following proposals:

Proposal 1: In two-port DMRS design, the DMRS sequence in one of the antenna ports shall be same as that of Rel-14, and the DMRS sequence in another antenna port could be differentiated by different cyclic shift.

Proposal 2: SFBC or SC-SFBC shall be applied for Rel-15 PSSCH.

References

  - RAN1 #91 chairman notes

  - R1-1715308, “LS on transmit diversity for PC5”, RAN WG1

  - R1-1720161, “Discussion on Tx diversity scheme in PC5”, CATT

  - R1-1707451, “Discussion on Tx diversity scheme in PC5”, CATT

  - R1-1707003, “Transmit diversity solutions for R15 sidelink”, Huawei, HiSilicon

  - R1-1708966, “DMRS design for Rel-15 V2X transmissions with transmit diversity”, Ericsson LM

  - R1-1801705, “Evaluations for Tx diversity schemes in PC5”, CATT

  - R1-1720147, “Low PAPR SFBC for V2X transmit diversity”,  Mitsubishi Electric

  - R1-1702601, “On UL diversity transmission scheme”, Qualcomm

  - R1-1713532, “Evaluation results for Tx diversity for PSSCH”, Samsung

  - R1-1712343, “Evaluations for Tx diversity schemes in PC5”, CATT