3GPP TSG RAN WG1 Meeting #92     	                                                R1-1801710

Athens, Greece, February 26th – March 2nd, 2018

Source:	CATT

Title:	Summary of Offline Discussion on RMSI

Agenda Item:	7.1.1.2.2

Document for:	Discussion and Decision

# Introduction

This purpose of the document is to provide a summary on the remaining key issues and the proposed solutions on AI 7.1.1.2.2 “Remaining minimum system information” for facilitating the continue discussion in RAN1#92 based on the contributions submitted under this AI [1-14].

In this document, the discussion of the remaining issues is organized as follows:

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 1

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 2

RMSI CORESET Configurations for 10 MHz min CH BW with 15 kHz SS

Other Proposals on RMSI CORESETs

RMSI PDCCH

RMSI PDSCH

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 1

Background

For the SS/PBCH block and CORESET multiplexing pattern 1, a UE monitors PDCCH in the Type0-PDCCH common search space over two consecutive slots. The starting from slot  , the index for the first symbol of the control resource sets, and the SFNs of the  common search space are determined based on the SS/PBCH block with index , the parameters  and  provided by Tables 13-11 and 13-12 in TS 38.213. It has been identified that for some RMSI CORESET configurations, it may happen that

RMSI CORESET OFDM symbols could be overlapped with the OFDM symbols of a candidate SS/PBCH block;

RMSI CORESET OFDM symbols associated with two candidate SS/PBCH blocks may be overlapped with each other;

Submitted Proposals

Different approaches were proposed to how to deal with the collision issue.

- Proposal from R1-1801508:

For collision between SS block and RMSI CORESET, following alternatives can be taken into account and alt.1 is preferred:

Alt.1: UE assumes resources within RMSI CORESET that partially or fully overlapped with candidate SS block resources are not used for RMSI PDCCH transmission.

Alt.2: gNB’s indication of transmitted SS block that partially or fully overlap with RMSI CORESET is considered an error in configuration and no UE behavior needs to be specified.

- Proposal from R1-1801642:

Any collision between RMSI CORESET and the SSB in pattern 1 will be avoided by the network and deemed as the error case if happened.

- Proposal from R1-1801709:

For the SS/PBCH block and CORESET multiplexing pattern 1, existing configurations with potential overlapping of the RMSI PDCCH search spaces associated with different SSBs can be kept. It will be up to the network to decide which configurations are used and whether the overlapping is allowed in the implementation. From UE’s perspective, UE assumes there is no overlapping.

- Proposal from R1-1801947:

The collision issue can be fully handled by network implementation

- Proposal from R1-1802191:

Observation 1:

In the major of cases at FR2, the collision between the SS/PBCH block and PDSCH for RMSI could be prevented by a good combination of configuration parameter (e.g. selection of SS/PBCH block index, subcarrier spacing of PDSCH, value of monitoring window offset, etc.).

In some case (i.e. M=2 at FR2), it is hard to avoid collision between SS/PBCH block and PDSCH for RMSI. One possible solution is to limit the number of SS/PBCH block.

- Proposal from R1-1802462:

Regarding the collision issue between SS/PBCH block location and RMSI CORESET monitoring window, there is no need to modify the current configuration of RMSI CORESET monitoring window.

- Proposal from R1-1802891:

When attempting to detect the PDCCH scheduling [SystemInformationBlockType1], a UE may assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDCCH.

From the submitted proposals, it seems the majority view is that the collision issue can be handled by the network implementation and there is no need to change the current RMSI CORESET configurations associated with Pattern 1. From UE’s perspective, UE assumes there is no overlapping.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | There is another discussion in control session on how to handle the collision b/w SSB and PDCCH (email discussion thread: [RAN1#92] [7.1.3.1.1] PDCCH structure]) after a UE obtains the SSB transmission information from RMSI (SSB-transmitted-SIB1) or RRC signaling (SSB-transmitted). The collision issue discussed in RMSI session will only cover the case before the UE obtains the SSB transmission information from RMSI or RRC signaling. |

Suggested offline agreement

For potential collision of SS/PBCH block and RMSI PDCCH with the multiplexing pattern 1,

No need to modify the current RMSI PDCCH monitoring requirements;

UE assumes no overlapping when monitoring Type0-PDCCH common search space.

========== START of Text Proposal (TS 38.213) =========

# 10	UE procedure for receiving control information

== UNCHANGED TEXT OMITTED ==

A UE can be configured by higher layer parameter SSB-periodicity-serving-cell a periodicity of half frames for reception of SS/PBCH blocks in a serving cell. If the UE has received SSB-transmitted-SIB1 and has not received SSB-transmitted and if REs for a PDCCH reception overlap with REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted-SIB1, the UE receives the PDCCH by excluding REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted-SIB1. If a UE has received SSB-transmitted and if REs for a PDCCH reception overlap with REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted, the UE receives the PDCCH by excluding REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted. When a UE follows the procedure in Subclause 13 to monitor Type0-PDCCH common search space before receiving SSB-transmitted-SIB1 and SSB-transmitted, a UE may assume that no SS/PBCH block is transmitted in REs used for the reception of the Type0-PDCCH.

== UNCHANGED TEXT OMITTED ==

Collision Issue: SS/PBCH block and CORESET multiplexing pattern 2

Background

For the SS/PBCH block and control resource set multiplexing pattern 2, a UE monitors PDCCH in the Type0-PDCCH common search space over one slot with Type0-PDCCH common search space periodicity equal to the periodicity of SS/PBCH block. For a SS/PBCH block with index , the UE determines the slot index  and  based on parameter provided by Tables 13-13 through 13-14 in TS 38.213.

| SSB | 120 kHz | 0 | 1 | 2 | 3 | SSB#0 |  | SSB#1 |  | 12 | 13 | 0 | 1 | SSB#2 |  | SSB#3 |  | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMSI | 60 kHz | 0 |  | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 |  | 8 | 9 | 10 | 11 | 12 |  | 13 |  |

- RMSI PDCCH has only 1 OFDM symbol

| SSB | 120 kHz | 0 | 1 | 2 | 3 | SSB#0 |  |  | SSB#1 |  |  |  | 12 | 13 | 0 | 1 | SSB#2 |  |  | SSB#3 |  |  |  | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMSI | 60 kHz | 0 |  | 1 |  | 2 | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  |
|  |  |  |  | 1 |  | 2 |  |  |  |  |  |  |  |  | 7 |  | 8 |  |  |  |  |  |  |  |  |  |  |

- RMSI PDCCH has 2 OFDM symbols

Figure 3-1 Illustration of the RMSI CORESET monitoring window for Pattern 2

When RMSI CORESET configuration has the duration of 1 OFDM symbols for Patter 2, there will be no overlapping of the RMSI CORESET with each other or with SSBs. However, when RMSI CORESET configuration uses the duration of 2 OFDM symbols for Patter 2, there will be overlapping of the RMSI CORESET with each other or with SSBs as shown in above Figure 3-1.

In addition, in order to guarantee UL control transmission at every slot, RAN1 agreed that SS/PBCH block(s) are not allocated at the OFDM symbols of the index of 12 and 13. However, in PDCCH monitoring occasion of pattern 2 for {SS/PBCH block, PDCCH} subcarrier spacing {240, 120} kHz, the configuration to set the monitoring occasion of PDCCH at the OFDM symbols of the index of 12 and 13 is defined, as shown in Figure 3-2.

Submitted Proposals:

- Proposal from R1-1801329:

To allow overlapping RMSI CORESETs in Pattern 2, UE Rx spatial operations need to be addressed. Adding new first symbol index entries into Table 13-13 and Table 13-14 of TS 38.213 (for Option 2) or specifying UE’s assumption on RMSI CORESET monitoring (for Option 3) can be for further considerations..

Figure 3-3. New configuration of RMSI CORESET and associated SSB (R1-1801329)

- Proposal from R1-1801406:

NR should support RMSI CORESETs for two continuous SS/PBCH blocks overlapping with each other.

- Proposal from R1-1801508

For collision between SS block and RMSI CORESET, following alternatives can be taken into account and alt.1 is preferred:

Alt.1: UE assumes resources within RMSI CORESET that partially or fully overlapped with candidate SS block resources are not used for RMSI PDCCH transmission.

Alt.2: gNB’s indication of transmitted SS block that partially or fully overlap with RMSI CORESET is considered an error in configuration and no UE behavior needs to be specified.

- Proposal from R1-1801642:

Option 1 by removing the configurations with =2 in Table 13-5 and Table 13-8 of TS 38.213 is adopted:

- Proposal from R1-1801709:

For the SS/PBCH block and CORESET multiplexing pattern 2, down-select one of the following options for dealing with the potential collision between SSBs and the RMSI CORESET:

Option 1: Remove the configurations with =2 for Pattern 2 in Table 13-7 and Table 13-10 of TS 38.213;

Option 2: Modify RMSI CORESET configurations in Tables 13-13 and 13-14 of TS 38.213 in order to support =2 for Pattern 2 as shown in Figure 3-2.

| SSB | 120 kHz | 0 | 1 | 2 | 3 | SSB#0 |  |  |  | SSB#1 |  |  |  | 12 | 13 | 0 | 1 | SSB#2 |  |  |  | SSB#3 |  |  |  | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RMSI | 60 kHz | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  |
|  |  | 0 |  | 1 |  |  |  |  |  |  |  |  |  | 6 |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 3-2: Illustration of the supporting RMSI CORESET monitoring windows with 1 and 2 OFDM symbols for Pattern 2 (R1-1801709)

- Proposal from R1-1802142:

For multiplexing pattern 2, it is proposed to support the collision cases and left gNB’s choice.

- Proposal from R1-1802191:

RAN1 allows the additional entries RMSI PDCCH monitoring window occasion for pattern 2 as follows:

Limit the number of SS/PBCH block

Entry #1: 0, 1, 2, 3, 0, 1 in i=8k, i=8k+1, i=8k+2, i=8k+3, i=8k+4, i=8k+5 (nc = nSSB,i)

Allow the mapping of the multiple SS/PBCH index to the same OFDM symbol

Entry #2: 0, 2, 0 in i={8k, 8k+1}, i={8k+2, 8k+3}, i={8k+4, 8k+5} (nc = nSSB,i)

Entry #3: 0, 1, 0, 1 in i={8k, 8k+1}, i={8k+2, 8k+3}, i={8k+4, 8k+5}, i={8k+6, 8k+7} (nc = nSSB,i)

- Text from R1-1802383:

“In case of SSB and RMSI SCS of {240kHz, 120kHz}, it would not be possible to support 2 OFDM symbol duration CORESET configuration. Therefore, entries need to be removed.”

- Proposal from R1-1802891:

Support =2 for pattern 2 with SS/PBCH block, PDCCH} subcarrier spacing {240, 120} kHz by allowing the search spaces corresponding to consecutive SS/PBCH blocks to overlap. For SS/PBCH block, PDCCH} subcarrier spacing {120, 60} kHz omit the =2 and support only =1 for pattern 2.

- Proposal from  R1-1802462:

Regarding the collision issue between SS/PBCH block location and RMSI CORESET monitoring window, there is no need to modify the current configuration of RMSI CORESET monitoring window.

The above proposals may be summarised as follows:

- Option 1:

Remove the configurations with =2 in Table 13-7 for {SSB SCS, PDCCH SCS} = {120, 60} kHz

Supported by: MediaTek, Intel, Nokia

Remove the configurations with =2 in Table 13-10 for {SSB SCS, PDCCH SCS} = {120, 60} kHz

Supported by: MediaTek, Intel

- Option 2:

Modify Table 13-13 to support =2 for {SSB SCS, PDCCH SCS} = {120, 60} kHz

      - Supported by: ZTE, CATT

  - Modify Table 13-14 to support =2 for {SSB SCS, PDCCH SCS} = {240, 120} kHz

    - Supported by: ZTE, Nokia, CATT, LGE, Huawei

      - Note: Different ways of the modifications are proposed by different companies

- Option 3: No change in the current RMSI CORESET configurations for Pattern 2. It will be up to the network on how to deal with the overlapping issue.

  - Supported by: ETRI, NTT DOCOMO

Comments

| Company Name |  | Comments |
| --- | --- | --- |

Suggested offline agreements

For {SSB SCS, PDCCH SCS} = {120, 60} kHz, Remove the configurations with =2 in Table 13-7.

For {SSB SCS, PDCCH SCS} = {240, 120} kHz, down-select one of the following options:

Remove the configurations with =2 in Table 13-10

  - Modify Table 13-14 to support =2 for {SSB SCS, PDCCH SCS} = {240, 120} kHz

  - No change in the current RMSI CORESET configurations.

RMSI CORESET configurations for SSB SCS 15kHz with minimum channel bandwidth 10MHz

## Support of RMSI CORESET configurations for “SSB SCS 15kHz + min CH BW 10MHz”

Background

RAN1 has defined RMSI CORESET configurations supporting for all combinations of {SSB SCS, RMSI SCS, minimum channel BW} defined in RAN4, except the following:

- {15kHz, 15kHz, 10MHz}

- {15kHz, 30kHz, 10MHz}

In RAN1#AH18_01, it was discussed on whether there was a need to define the RMSI CORESET configuration tables for 15kHz SCS for SS/PBCH block with minimum channel bandwidth of 10MHz w/o conclusion. It was decided to send an LS (R1-1801246) to RAN4, informing RAN4 all of the RMSI CORESET configuration tables so far developed in RAN1, and then waiting for RAN4’s response on whether to add the new tables to support the above two cases.

Submitted Proposals:

- Proposal from R1-1802191:

RAN1 defines a new RMSI CORESET configuration table for the following cases.

{SS/PBCH block, PDCCH} subcarrier spacing is {15,15}kHz with minimum channel bandwidth 10MHz

{SS/PBCH block, PDCCH} subcarrier spacing is {15,30}kHz with minimum channel bandwidth 10MHz

- Proposal from R1-1801947:

If RAN4 supports 15 kHz SS SCS for band n41, add two new configuration tables for the following cases:

{SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for with minimum channel bandwidth 10MHz

{SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz for with minimum channel bandwidth 10MHz

According to TS 38.101-1, RAN4 has actually defined at least one band (i.e., n41) that supports the minimum channel BW of 10MHz with SS SCS of 15 kHz.

Suggested offline agreements

- If RAN4 decides to support 15 kHz SS SCS with minimum channel bandwidth 10MHz, RAN1 will define new RMSI CORESET configuration tables for supporting the following cases:

{SS/PBCH block, PDCCH} subcarrier spacing is {15,15}kHz with minimum channel bandwidth 10MHz

{SS/PBCH block, PDCCH} subcarrier spacing is {15,30}kHz with minimum channel bandwidth 10MHz

## RMSI CORESET configuration tables for “SSB SCS 15kHz + min CH BW 10MHz”

Background

- If it reaches the consensus to define new RMSI CORESET configuration tables for the following cases SSB SCS=15kHz with minimum channel bandwidth 10MHz, then the next issue is how to define the RMSI CORESET configuration tables for above two cases.

Proposal from R1-1802191:

Alt.1: Two tables with 4bits configuration

Alt.2: Single table with 5bits configuration

Proposal from R1-1801947:

Details of the tables should depend on RAN4’s decision on the step size of sync raster for band n41, and examples in R1-1801071 can be used as a starting point.

Suggested offline agreements

- TBD

# Other Proposals on RMSI CORESETs

## Modification of RMSI PDCCH monitoring occasions for Pattern 1

Background

In R1-1802135 it is proposed to optimize the DL/UL resource configuration in NR system. The starting slot should locate on the same half frame index with SSBs.

Submitted Proposals

The determination of the index of slot should be changed in TS 38.213 should be changed.

- satisfying  if  or satisfying  if .

where, HRF is half frame bit indicated in PBCH.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | It is unclear to us why the starting slot should be located on the same half frame index with SSBs. In addition, with the proposed n0: it has n0O=n0O+5, when O=0,2,2.5, since 5∙2μ=12Nslotframe,μ. For example, the configuration index 0 (with O=0) and index 4 (with O=5) in Table 13-11 are actually the same configuration. |
| OPPO | For CATT’s question above, the intention is that trying to allocate both the SSBs and RMSIs in either the 1st half frame or within the 2nd half frame so that slots in another half frame can be configured as UL slots. This is to solve the problem that if the offset of SSBs and RMSI is 5 or 7, there would be no or few UL slots within one frame based on the current formulation. Please note that the SSBs and RMSIs may be in the same frame or in different frame based on the SSB index . For the determination of , if index of is in a span of half frame, is even. If index of is beyond the span of half frame, is odd. satisfying if or in a frame with SFN satisfying if . |

Suggested offline agreements

TBD

## Change the definition of the initial active DL BWP for RMSI multiplexing pattern 3 case

Background

Initial active DL BWP is defined to the bandwidth of the RMSI CORESET. UE does not perform RLM nor measurements outside the active DL BWP for the serving cell. In R1-1802383 it was discussed that this poses problem to measurement and RLM, and thus it is proposed to “change the definition of the initial active DL BWP for RMSI multiplexing pattern 3 case, such that the initial active DL BWP is the total bandwidth encompassing the RMSI CORESET and SSB bandwidth”.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | The definition of initial active DL BWP applies only during the stage of initial access. RLM is, however, carried out after Initial Access is completed. UE is not required to perform RLM during Initial Access. |

Suggested offline agreements

TBD

RMSI PDCCH

A number of issues associated with the details on RMSI PDCCH format design are brought up in a number of contributions submitted to AI 7.1.1.2.2.

Comments

| Company Name |  | Comments |
| --- | --- | --- |
| CATT | It was agreed in RAN1#AH18_01 that the details of RMSI PDCCH format design would be covered under control channel session after the coordination between the feature leads. Thus, I assume the proposals submitted to AI 7.1.1.2.2 on the details of RMSI PDCCH format design are also submitted to the control session, and thus would not be discussed on RMSI offline session unless there is a special need to do so. |

## Compact DCI format

Background

It was proposed to support compact RMSI PDCCH DCI format in R1-1801947.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## CE to REG mapping mode and precoding granularity for RMSI CORESET

Background

It was proposed in R1-1801508 that

Interleaved CCE-to-REG mapping mode is adopted for RMSI CORESET,

the same precoding in the frequency domain is applied within a REG bundle, the size of which is 6 REG.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## Dependency of RMSI PDCCH on SS/PBCH block index

Background

For multiplexing pattern 1, the monitoring window duration for RMSI CORESET is 2 slots. In many cases, the two monitoring windows corresponding to two different SS/PBCH blocks are partially (M = 1) or fully (M = 1/2) overlapped. In these cases, if a UE detects a PDCCH for RMSI scheduling in the overlapped slot(s), the UE cannot be convinced whether the detected PDCCH is for the corresponding SS/PBCH block or not. It was proposed in R1-1802142 that “A part of PDCCH for RMSI scheduling should be dependent on an SS/PBCH block index.”

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## Parameter Configuration for RMSI CORESET signaling

Background

The list of parameters needed for reception of RMSI PDCCH is not completed. It was proposed in R1-1802383 to set the following default values for PDCCH related parameters for RMSI.

| Parameter | Proposed values for RMSI PDCCH | Notes |
| --- | --- | --- |
| CORESET-ID | Set to 0 |  |
| PDCCH-DMRS-Scrambling-ID | Set to nIDcell (i.e. cell ID) |  |
| CORESET-time-duration | Signaled in PBCH | Already captured in spec |
| CORESET-freq-dom | Signaled in PBCH | Already captured in spec |
| CORESET-CCE-to-REG-mapping-type | Set to interleaved |  |
| CORESET-REG-bundle-size | Set to 6 | Already captured in spec |
| CORESET-shift-index | Set to nIDcell | Already captured in spec |
| TCI-StatesPDCCH | Set to not indicated (empty list) |  |
| TCI-PresentInDCI | Set to not-present |  |
| CORESET-precoder-granularity | Set to CORESET-REG-bundle-size |  |
| Common-search-space-flag | Set to common-search-space |  |
| Aggregation-level-1 | Set to 0 | Already captured in spec |
| Aggregation-level-2 | Set to 0 | Already captured in spec |
| Aggregation-level-4 | Set to 4 | Already captured in spec |
| Aggregation-level-8 | Set to 2 | Already captured in spec |
| Aggregation-level-16 | Set to 1 |  |
| Monitoring-periodicity-PDCCH-slot | Defined in 38.213 | Already captured in spec |
| Monitoring-offset-PDCCH-slot | Signaled in PBCH | Already captured in spec |
| Monitoring-symbols-PDCCH-within-slot | Signaled in PBCH | Already captured in spec |

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## DMRS sequence for PDCCH from the CORESET configured by PBCH

Background

It was proposed in R1-1802383 that

- For PDCCH DM-RS sequence generation, include the following text in [Section 7.4.1.3.1, 3]

- nID = nIDcell for CORESET configured by PBCH

Comments

5-1

| Company Name |  | Comments |
| --- | --- | --- |

## Relative time position between the PDCCH scheduling RMSI and the PDSCH carrying RMSI

Background

The time domain allocation of the PDSCH carrying messages scheduled by the PDCCH in CORESET configured by PBCH are highly depending on the CORESET configuration, and this time domain allocation has not been explicitly indicated in the specification. For example, if the SSB periodicity is greater than half-Frame duration, and for limited bandwidth in initial DL BWP, the gNB for scheduling flexibility may choose to schedule RMSI PDSCH in a manner TDM with the SSB burst. It was proposed in to “Consider the RMSI CORESET configuration when specifying the time domain allocation of PDSCH carrying messages scheduled by PDCCH in CORESET configured by PBCH” in R1-1802942, and “RMSI PDCCH Grant should support the slot offset between PDCCH Grant and PDSCH equal to or larger than half-Frame duration” in R1-1802811.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

## RMSI PDCCH format supporting rate matching of RMSI PDSCH around SS/PBCH blocks

Background

In RAN1#AH1801, it was agreed that “When receiving the PDSCH conveying [SystemInformationBlockType1], a UE may assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.” In R1-1802811, it is discussed that the above agreement is very restrictive in the sense that this disallows the RMSI PDSCH rate-matched around the SSB, and preferable to rate-match RMSI PDSCH around it’s associated SS/PBCH for some cases.  It is proposed that

RAN1 use the following table for RMSI PDSCH SLIV indication. Furthermore, some bits in Format 1_0 such as TPC command [2], PUCCH resource indicator [2] and PDSCH-to-HARQ feedback timing indicator [3] can be repurposed for RMSI PDSCH SLIV.

| K0 Values, Half-Frame Duration & slot offset | Start and Length of PDSCH Symbol | SSB Bitmap | Entire OFDM Symbol vs. Only SSB REs |
| --- | --- | --- | --- |
| 2 bits | 3 bits | 2 bits | 1 bit |

Comments

| Company Name |  | Comments |
| --- | --- | --- |

RMSI PDSCH

## RMSI PDSCH modulation scheme

Background

In LTE, only QPSK is used for the common downlink message in PDSCH, e.g. RMSI, broadcast OSI, paging, RACH response, based on the consideration of the cell edge coverage requirement. It was proposed in a number of contributions (R1-1801329, R1-1801642, R1-1802383) that only QPSK is used for RMSI PDSCH.

Comments

| Company Name |  | Comments |
| --- | --- | --- |

Suggested offline agreement

- RMSI PDSCH only uses QPSK for modulation.

## Maximum TBS for RMSI PDSCH

Background

In LTE, the maximum TBS for SI, RAR, and Paging were limited to 2216 bits when using DCI format 1A, and 1732 bits when using DCI format 1C. For NR, the maximum TBS for SI, RAR, and Paging is undecided. It was proposed in R1-1802383 that the “maximum TBS for PDSCH containing RMSI is limited to [3000] bits.”

Comments

| Company Name |  | Comments |
| --- | --- | --- |

Suggested offline agreement

TBD

References

- R1-1801329	Remaining  on RMSI configurations	Huawei, HiSilicon

- R1-1801406	Remaining details of RMSI	ZTE, Sanechips

- R1-1801508	Discussion on Remaining Minimum System Information	vivo

- R1-1801642	Remaining issues for RMSI	MediaTek Inc.

- R1-1801709	Remaining Issues on RMSI	CATT

- R1-1801947	Corrections on Remaining Minimum System Information	Samsung

- R1-1802135	RMSI PDCCH monitoring window configuration	Guangdong OPPO Mobile Telecom

- R1-1802142	Remaining issues on RMSI delivery	ETRI

- R1-1802191	RMSI delivery and CORESET configuration	LG Electronics

- R1-1802383	Remaining details of RMSI	Intel Corporation

- R1-1802462	Remaining issues on RMSI	NTT DOCOMO, INC.

- R1-1802811	Remaining system information delivery consideration	Qualcomm Incorporated

- R1-1802891	Remaining open issues on RMSI CORESET configuration	Nokia, Nokia Shanghai Bell

- R1-1802942	Remaining details of remaining minimum system information	Ericsson

- Draft Report of 3GPP TSG RAN WG1 #AH_1801 v0.1.0, ETSI

- 3GPP TS 38.213, V15.0.0 (2018-02)

- R1-1721643	Reply LS on Minimum Bandwidth		RAN4

- R1-1800008	LS on channel raster and SS raster 		RAN4