3GPP TSG RAN WG1 Meeting #92                                                                       R1-1801769

Athens, Greece, February 26th – March 2nd, 2018

Source:	CATT

Title:	Initial self evaluation for Indoor Hotspot-eMBB

Agenda Item:	7.7

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performances towards all the ITU-R IMT-2020 requirements. The key minimum technical performance requirements are provided in Report ITU-R M.2410 [2] for the test environment of Indoor Hotspot-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.3 | 0.21 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |

According to the evaluation methodology of Report ITU-R M.2412 [3], the average spectral efficiency and the 5th percentile user spectral efficiency need system-level simulation evaluation to demonstrate the achievement for the ITU-R technical performance requirements, while area traffic capacity can be calculated by average spectral efficiency SEavg through equation in [2] (this KPI is only applied to Indoor Hotspot-eMBB).

Carea = ρ × W × SEavg

In our company contribution [4], some initial evaluation results including the DL average spectral efficiency, the DL 5th percentile user spectral efficiency and the area traffic capacity are provided. In this document, we focus on the Indoor Hotspot-eMBB and more evaluation results with updated overhead assumptions while more antenna configurations are provided to meet the technical performance requirements.

Evaluation methodology

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

For the area traffic capacity, the TRxP density ρ(TRxP/m2) can be calculated according to the network layout defined in 8.3.1 of Report ITU-R M.2412 [3], such as 0.002 for the 12 TRxPs topology and 0.006 for the 36 TRxPs topology. The area traffic capacity can be calculated based on proper bandwidth to demonstrate the achievement for the requirement of the ITU-R.

Evaluation assumption

In [3], a number of evaluation parameters are defined for the detailed evaluations. Some of them need further decision from IMT-2020 technology proponent, e.g. the exact number of antenna elements applied and it is also necessary to discuss the parameters which were not detailed (“reported by proponent”) in ITU report, e.g. TxRU to antenna element mapping. In this contribution, different antenna configurations are evaluated to observe the proper antenna configurations to fulfil the ITU minimum performance requirements, including different numbers of antenna elements and different TxRU mapping. The evaluation assumptions used in this document refer to our company contribution [5] and are detailed summarized in Annex, which will be updated when 3GPP has further discussions and/or agreements.

Initial downlink evaluation results

Based on the above information, the initial evaluation results with 4GHz are provided for average spectral efficiency and 5th percentile spectral efficiency under Indoor Hotspot-eMBB. Evaluation assumptions can be found in Annex.

Initial downlink evaluation results

The initial evaluation results for DL average spectral efficiency and DL 5th percentile spectral efficiency under Indoor Hotspot-eMBB are provided in Table 1~ 4. The area traffic capacity results are provided in Table 6 and Table 7 which are calculated based on initial results of the average spectral efficiency.

Spectral efficiency:

Note: Impacts from HARQ, sub-band SRS and overhead are updated. Further non-ideal evaluation assumptions would be updated in future. Evaluation results are provided by both channel model A and channel model B.

Table 1 Initial DL average spectral efficiency evaluation for 12 TRxPs (Indoor Hotspot-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 12 TRxPs Config. A, 4GHz InH-A | TDD | 16T/4R, (2,4,2,1,1) | 16Tx/4Rx, (2,4,2,1,1) | 9 | 10.63 | 18.11% |
|  |  | 32T/2R , (4,4,2,1,1) | 32Tx/2Rx, (4,4,2,1,1) |  | 10.00 | 11.11% |
|  |  | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 14.34 | 59.33% |
|  | FDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 11.23 | 24.78% |
| 12 TRxPs Config. A, 4GHz InH-B | TDD | 16T/4R, (2,4,2,1,1) | 16Tx/4Rx, (2,4,2,1,1) |  | 10.96 | 21.78% |
|  |  | 32T/2R , (4,4,2,1,1) | 32Tx/2Rx, (4,4,2,1,1) |  | 10.17 | 13.00% |
|  |  | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 14.60 | 62.22% |
|  | FDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 11.28 | 25.33% |

Table 2 Initial DL 5th percentile spectral efficiency evaluation for 12 TRxPs (Indoor Hotspot-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 12 TRxPs Config. A, 4GHz InH-A | TDD | 16T/4R, (2,4,2,1,1) | 16Tx/4Rx, (2,4,2,1,1) | 0.3 | 0.35 | 16.67% |
|  |  | 32T/2R , (4,4,2,1,1) | 32Tx/2Rx, (4,4,2,1,1) |  | 0.31 | 3.33% |
|  |  | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 0.53 | 76.67% |
|  | FDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 0.42 | 40.00% |
| 12 TRxPs Config. A, 4GHz InH-B | TDD | 16T/4R, (2,4,2,1,1) | 16Tx/4Rx, (2,4,2,1,1) |  | 0.37 | 23.33% |
|  |  | 32T/2R , (4,4,2,1,1) | 32Tx/2Rx, (4,4,2,1,1) |  | 0.31 | 3.33% |
|  |  | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 0.56 | 86.67% |
|  | FDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |  | 0.43 | 43.33% |

Table 3 Initial DL average spectral efficiency evaluation for 36 TRxPs (Indoor Hotspot-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 36 TRxPs Config. A, 4GHz InH-A | TDD | 32T/4R, (2,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) | 9 | 10.22 | 13.56% |
|  |  | 64T/4R, (4,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) |  | 11.33 | 25.89% |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 10.06 | 11.78% |
| 36 TRxPs Config. A, 4GHz, InH-B | TDD | 32T/4R, (2,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) |  | 10.31 | 14.56% |
|  |  | 64T/4R, (4,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) |  | 11.43 | 27.00% |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 10.13 | 12.56% |

Table 4 Initial DL 5th percentile spectral efficiency evaluation for 36 TRxPs (Indoor Hotspot-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 36 TRxPs Config. A, 4GHz InH-A | TDD | 32T/4R, (2,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) | 0.3 | 0.36 | 20.00% |
|  |  | 64T/4R, (4,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) |  | 0.39 | 30.00% |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.36 | 20.00% |
| 36 TRxPs Config. A, 4GHz, InH-B | TDD | 32T/4R, (2,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) |  | 0.36 | 20.00% |
|  |  | 64T/4R, (4,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) |  | 0.39 | 30.00% |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.36 | 20.00% |

Observation 1: For both 12 TRxPs and 36TRxPs topology with sub 6 GHz, NR can fulfil the DL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in both TDD and FDD mode.

Based on the assumptions in this contribution and [5], according to the above initial evaluation results, the sufficient antenna configurations to meet the minimum requirement are summarized in the following Table 5. Considering the potential more overhead or non-ideal aspects, more TxRU and antenna elements size may be needed and the possible further evaluation antenna configurations are also listed in the following Table 5.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 5 when the same assumptions in this contribution are applied.

Table 5 Sufficient antenna configurations for Indoor Hotspot-eMBB at 4GHz

| Test environment | Duplex scheme | Sufficient antenna configuration |  | Possible further evaluation antenna configuration |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) |
| 12TRxPs, Config.A 4GHz | TDD | 16T/4R, (2,4,2,1,1) or 32T/2R , (4,4,2,1,1) | 16Tx/4Rx, (2,4,2,1,1) or 32Tx/2Rx, (4,4,2,1,1) | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) |
|  | FDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) | 32T/4R , (4,4,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) |
| 36TRxPs, Config.A 4GHz | TDD | 32T/4R, (2,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 32T/8R, (2,8,2,1,1) | 128Tx/8Rx, (8,8,2,1,1) |

Area traffic capacity:

Considering various bandwidth at 4GHz, initial area traffic capacities are provided in Table 6 and Table 7.

Table 6 Initial area traffic capacity evaluation for 12 TRxPs at 4GHz

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Bandwidth W (MHz) | Carea (Mbit/s/m2) | Requirement (Mbit/s/m2) |
| --- | --- | --- | --- | --- | --- | --- |
| 12 TRxPs Config. A, 4GHz InH-A | TDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) | 300 | 8.60 | 10 |
|  |  |  |  | 350 | 10.04 |  |
|  |  |  |  | 400 | 11.47 |  |
|  | FDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) | 400 | 8.98 |  |
|  |  |  |  | 450 | 10.11 |  |
|  |  |  |  | 500 | 11.23 |  |
| 12 TRxPs Config. A, 4GHz InH-B | TDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) | 300 | 8.76 |  |
|  |  |  |  | 350 | 10.22 |  |
|  |  |  |  | 400 | 11.68 |  |
|  | FDD | 32T/4R , (4,4,2,1,1) | 32Tx/4Rx, (4,4,2,1,1) | 400 | 9.02 |  |
|  |  |  |  | 450 | 10.15 |  |
|  |  |  |  | 500 | 11.28 |  |

Table 7 Initial area traffic capacity evaluation for 36 TRxPs at 4GHz

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Bandwidth W (MHz) | Carea (Mbit/s/m2) | Requirement (Mbit/s/m2) |
| --- | --- | --- | --- | --- | --- | --- |
| 36 TRxPs Config. A, 4GHz InH-A | TDD | 64T/4R, (4,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) | 100 | 6.80 | 10 |
|  |  |  |  | 150 | 10.20 |  |
|  |  |  |  | 200 | 13.60 |  |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 150 | 9.05 |  |
|  |  |  |  | 200 | 12.07 |  |
|  |  |  |  | 250 | 15.09 |  |
| 36 TRxPs Config. A, 4GHz InH-B | TDD | 64T/4R, (4,8,2,1,1) | 64Tx/4Rx, (4,8,2,1,1) | 100 | 6.86 |  |
|  |  |  |  | 150 | 10.29 |  |
|  |  |  |  | 200 | 13.72 |  |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 150 | 9.12 |  |
|  |  |  |  | 200 | 12.16 |  |
|  |  |  |  | 250 | 15.20 |  |

Observation 2: The ITU requirement on area traffic capacity can be met by NR.

Observation 3: More TRxPs can increase the area traffic capacity even when spectral efficiency is lower than that of the case of less TRxPs.

Note that the evaluation results of area traffic capacity and the requirement of aggregated bandwidth to achieve the ITU minimum requirement are closely related to the evaluation results of DL average spectral efficiency, and the above results will be updated when evaluation assumptions and spectral efficiencies are further updated in future. It would be good to list both results from minimum required antenna configuration and a reasonable configuration which can provide required bandwidth respectively to meet area traffic capacity requirement.

Proposal 2: Further discuss which antenna configuration(s) are applied to area traffic capacity evaluation, which leads to different presentation of required bandwidth.

Initial uplink evaluation results

Spectral efficiency:

The initial FDD evaluation results for UL average spectral efficiency and UL 5th percentile spectral efficiency under Indoor Hotspot-eMBB are provided in Table 8 and Table 9. As initial evaluation, SU-MIMO can be as a baseline MIMO transmission technology. From initial results, it shows that SU-MIMO can fulfil the ITU requirement very well. Moreover, advanced MIMO transmission technology can be considered in future to enhance spectral efficiency performance, e.g., MU-MIMO.

Note: Non-ideal evaluation assumptions and overhead may be updated in future.

Table 8 Initial UL average spectral efficiency evaluation for 12 TRxPs (Indoor Hotspot-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 12 TRxPs Config. A, 4GHz InH-A | FDD | 4T/32R, (4,4,2,1,1) | 4Tx/32Rx, (4,4,2,1,1) | 6.75 | 8.43 | 24.87% |
| 12 TRxPs Config. A, 4GHz InH-B | FDD | 4T/32R, (4,4,2,1,1) | 4Tx/32Rx, (4,4,2,1,1) |  | 8.27 | 22.50% |

Table 9 Initial UL 5th percentile spectral efficiency evaluation for 12 TRxPs (Indoor Hotspot-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| 12 TRxPs Config. A, 4GHz InH-A | FDD | 4T/32R, (4,4,2,1,1) | 4Tx/32Rx, (4,4,2,1,1) | 0.21 | 0.42 | 100.00% |
| 12 TRxPs Config. A, 4GHz InH-B | FDD | 4T/32R, (4,4,2,1,1) | 4Tx/32Rx, (4,4,2,1,1) |  | 0.41 | 92.70% |

Observation 4: For 12 TRxPs topology with sub 6 GHz, NR can fulfil the UL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in FDD mode.

Observation 5: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Proposal 3: Further investigation is needed to find proper antenna configuration for indoor uplink considering antenna size, complexity and performance.

Observation 6: Evaluation results on spectral efficiency by channel model A or B at 4GHz are similar (including DL and UL) in Indoor Hotspot-eMBB.

Conclusion

In this contribution, we provide our preliminary evaluation results for Indoor Hotspot-eMBB, we have the following observations and proposals:

Observation 1: For both 12 TRxPs and 36TRxPs topology with sub 6 GHz, NR can fulfil the DL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in both TDD and FDD mode.

Observation 2: The ITU requirement on area traffic capacity can be met by NR.

Observation 3: More TRxPs can increase the area traffic capacity even when spectral efficiency is lower than that of the case of less TRxPs.

Observation 4: For 12 TRxPs topology with sub 6 GHz, NR can fulfil the UL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in FDD mode.

Observation 5: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Observation 6: Evaluation results on spectral efficiency by channel model A or B at 4GHz are similar (including DL and UL) in Indoor Hotspot-eMBB.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 5 when the same assumptions in this contribution are applied.

Proposal 2: Further discuss which antenna configuration(s) are applied to area traffic capacity evaluation, which leads to different presentation of required bandwidth.

Proposal 3: Further investigation is needed to find proper antenna configuration for indoor uplink considering antenna size, complexity and performance.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - R1-1800268, “Initial self evaluation for Indoor Hotspot-eMBB”, CATT

  - R1-1801767, “Discussions on system level simulation configurations and technical features for self evaluation”, CATT

- Annex: Evaluation assumptions

In this contribution, the downlink and uplink evaluation assumptions for Indoor Hotspot-eMBB are provided in the following table.

Table A-1 Evaluation assumption for DL and UL

| Indoor Hotspot - eMBB | Config. A |  |
| --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz |  |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M |  |
| Slots structure | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD |  |
| Subcarrier spacing | 15KHz |  |
| Symbols number per slot | 14 |  |
| BS antenna height | 3 m |  |
| Total transmit power per TRxP | FDD: 21 dBm TDD: 24 dBm |  |
| UE power class | 23 dBm |  |
| Inter-site distance | 20m |  |
| TRxP number per site | 1 | 3 |
| Mechanic tilt | 180° in GCS | 110°in GCS |
| Electronic tilt | 90° in LCS | 90° in LCS |
| Number of antenna elements per TRxP | 16Tx/Rx, (M,N,P,Mg,Ng) = (2,4,2,1,1), 32Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | 64Tx/Rx, (M,N,P,Mg,Ng) = (4,8,2,1,1), 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 16TXRU, (M,N,P,Mg,Ng) = (2,4,2,1,1) 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) | 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) 64TXRU, (Mp,Np,P,Mg,Ng) = (4,8,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |  |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) (1-to-1 mapping) |  |
| UE speeds of interest | 3 km/h |  |
| BS noise figure | 5 dB |  |
| UE noise figure | 7 dB |  |
| BS antenna element gain | 5dBi |  |
| BS antenna element pattern | See Table 10 in Report ITU-R M.2412 |  |
| UE antenna element gain | 0 dBi |  |
| UE antenna element pattern | Omni-directional |  |
| Thermal noise level | -174 dBm/Hz |  |
| Traffic model | Full buffer |  |
| UE density | 10 UEs per TRxP |  |
| UE antenna height | 1.5m |  |
| Channel model variant | Channel model A and B |  |
| UT attachment | Based on RSRP from port 0 |  |
| Wrapping around method | No wrapping around |  |
| Polarized antenna model | Model-2 in TR36.873 |  |
| TX mode | SU/MU-MIMO |  |
| Receiver | MMSE-IRC |  |
| DL CSI feedback | 10ms period with delay |  |
| DL CSI method | FDD: CSI Type II codebook based TDD: full reciprocity based |  |
| Channel estimation | ideal |  |
| Interference estimation | ideal |  |
| UE precoder scheme | Codebook based |  |
| UL CSI derivation | SRS based with delay |  |
| UL waveform | OFDM |  |
| DMRS configuration | Type 2 |  |