3GPP TSG RAN WG1 Meeting #92                                                                       R1-1801770

Athens, Greece, February 26th – March 2nd, 2018

Source:	CATT

Title:	Initial self evaluation for Dense Urban-eMBB

Agenda Item:	7.7

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performance towards all the ITU-R IMT-2020 requirements. The key minimum technical performance requirements are provided in Report ITU-R M.2410 [2] for the test environment of Dense Urban-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Dense Urban – eMBB | 0.225 | 0.15 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Dense Urban – eMBB (Note 1) | 7.8 | 5.4 |

NOTE 1 – This requirement applies to Macro TRxP layer of the Dense Urban – eMBB test environment as described in Report ITU-R M.2412-0.

According to the evaluation methodology of Report ITU-R M.2412 [3], the average spectral efficiency and the 5th percentile user spectral efficiency need system-level simulation evaluation to demonstrate the achievement for the ITU-R technical performance requirements, while user experienced data rate can be calculated by 5th percentile user spectral efficiency SEuser through equation in [2] (this KPI is only applied to Dense Urban-eMBB).

Ruser = W × SEuser

In our company contribution [4], some initial evaluation results including the DL average spectral efficiency, the DL 5th percentile user spectral efficiency and the DL user experienced data rate are provided. In this document, we focus on the Dense Urban-eMBB and more evaluation results with updated overhead assumptions while more antenna configurations are provided to meet the technical performance requirements.

Evaluation methodology

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

The user experienced data rate can be calculated based on proper bandwidth to demonstrate the achievement for the requirement of the ITU-R.

Evaluation assumption

In [3], a number of evaluation parameters are defined for the detailed evaluations. Some of them need further decision from IMT-2020 technology proponent, e.g. the exact number of antenna elements applied and it is also necessary to discuss the parameters which were not detailed (“reported by proponent”) in ITU report, e.g. TxRU to antenna element mapping. In this contribution, different antenna configurations are evaluated to observe the proper antenna configurations to fulfil the ITU minimum performance requirements, including different number of antenna elements and different TxRU mapping. The evaluation assumptions used in this document refer to our company contribution [5] and are detailed summarized in Annex, which will be updated when 3GPP has further discussions and/or agreements.

Initial downlink evaluation results

Based on the above information, the initial evaluation results with 4GHz are provided for average spectral efficiency and 5th percentile spectral efficiency under Dense Urban-eMBB. Evaluation assumptions can be found in Annex.

Initial downlink evaluation results

The initial evaluation results for DL average spectral efficiency and DL 5th percentile spectral efficiency under Dense Urban-eMBB are provided in Table 1 and Table 2. The DL user experienced data rates results are provided in Table 4 which are calculated based on initial results of the DL 5th percentile spectral efficiency.

Spectral efficiency:

Note: Impacts from HARQ, sub-band SRS and overhead are updated. Further non-ideal evaluation assumptions would be updated in future.

Table 1 Initial DL average spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | TDD | 8T/4R, (1,4,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 7.8 | 8.82 | 13.08% |
|  |  | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 12.26 | 57.18% |
|  | FDD | 16T/4R , (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 9.53 | 22.18% |
|  |  | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 11.47 | 47.05% |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | TDD | 8T/4R, (1,4,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 8.65 | 10.90% |
|  |  | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 12.12 | 55.38% |
|  | FDD | 16T/4R , (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 9.42 | 20.77% |
|  |  | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 11.65 | 49.36% |

Table 2 Initial DL 5th percentile spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | TDD | 8T/4R, (1,4,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 0.225 | 0.266 | 18.22% |
|  |  | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.378 | 68.00% |
|  | FDD | 16T/4R , (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.271 | 20.44% |
|  |  | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.377 | 67.56% |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | TDD | 8T/4R, (1,4,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.253 | 12.44% |
|  |  | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.378 | 68.00% |
|  | FDD | 16T/4R , (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.281 | 24.89% |
|  |  | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |  | 0.374 | 66.22% |

Observation 1: For sub 6 GHz, NR can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

Based on the assumptions in this contribution and [5], according to the above initial evaluation results, the sufficient antenna configurations to meet the minimum requirement are summarized in the following Table 3. Considering the potential more overhead or non-ideal aspects, more TxRU and antenna elements size may be needed and the possible further evaluation antenna configurations are also listed in the following Table 3.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 3 when the same assumptions in this contribution are applied.

Table 3 Sufficient antenna configuration for Dense Urban-eMBB at 4GHz

| Test environment | Duplex scheme | Sufficient antenna configuration |  | Possible further evaluation antenna configuration |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) |
| Dense Urban-eMBB Config. A, 4GHz | TDD | 8T/4R, (1,4,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |
|  | FDD | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) |

User experienced data rate:

Considering various bandwidth at 4GHz, initial DL user experienced data rates are provided in Table 4.

Table 4 Initial DL user experienced data rate evaluation at 4GHz

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Bandwidth W (MHz) | Requirement (Mbit/s) | Ruser (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A,4GHz UMa-A | TDD | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 250 | 100 | 94.25 |
|  |  |  |  | 300 |  | 113.10 |
|  |  |  |  | 350 |  | 131.95 |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 250 |  | 92.50 |
|  |  |  |  | 300 |  | 111.00 |
|  |  |  |  | 350 |  | 129.50 |
| Dense Urban-eMBB Config. A,4GHz UMa-B | TDD | 16T/4R, (1,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 250 |  | 94.50 |
|  |  |  |  | 300 |  | 113.40 |
|  |  |  |  | 350 |  | 132.30 |
|  | FDD | 32T/4R, (2,8,2,1,1) | 128Tx/4Rx, (8,8,2,1,1) | 250 |  | 93.50 |
|  |  |  |  | 300 |  | 112.20 |
|  |  |  |  | 350 |  | 130.90 |

Observation 2: The ITU requirement on DL user experienced data rate can be met by NR.

Note that the evaluation results of DL user experienced data rate and the requirement of aggregated bandwidth to achieve the ITU minimum requirement are closely related to the evaluation results of DL 5th percentile spectral efficiency, and the above results will be updated when evaluation assumptions and spectral efficiencies are further updated in future. It would be good to list both results from minimum required antenna configuration and a reasonable configuration which can provide required bandwidth respectively to meet DL user experienced data rate requirement.

Proposal 2: Further discuss which antenna configuration(s) are applied to DL user experienced data rate evaluation, which leads to different presentation of required bandwidth.

Initial uplink evaluation results

The initial FDD evaluation results for UL average spectral efficiency and UL 5th percentile spectral efficiency under Dense Urban-eMBB are in Table 5 and Table 6. The UL user experienced data rates results are provided in Table 7 which are calculated based on initial results of the UL 5th percentile spectral efficiency. As initial evaluation, SU-MIMO can be as a baseline MIMO transmission technology. From initial results, it shows that SU-MIMO can fulfil the ITU requirement very well. Moreover, advanced MIMO transmission technology can be considered in future to enhance spectral efficiency performance, e.g., MU-MIMO.

Spectral efficiency:

Note: Non-ideal evaluation assumptions and overhead may be updated in future.

Table 5 Initial UL average spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz) | SEavg (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 5.4 | 7.13 | 31.94% |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) |  | 7.11 | 31.62% |

Table 6 Initial UL 5th percentile spectral efficiency evaluation (Dense Urban-eMBB at 4GHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz) | SEedge (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A, 4GHz UMa-A | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 0.15 | 0.158 | 5.11% |
| Dense Urban-eMBB Config. A, 4GHz UMa-B | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) |  | 0.161 | 7.15% |

Observation 3: For sub 6 GHz, NR can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in FDD mode.

Observation 4: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Proposal 3: Further investigation is needed to find proper antenna configuration for dense urban uplink considering antenna size, complexity and performance.

User experienced data rate:

Considering various bandwidth at 4GHz, initial UL user experienced data rates are provided in Table 7.

Table 7 Initial UL user experienced data rate evaluation at 4GHz

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Bandwidth W (MHz) | Requirement (Mbit/s) | Ruser (Mbit/s) |
| --- | --- | --- | --- | --- | --- | --- |
| Dense Urban-eMBB Config. A,4GHz UMa-A | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 300 | 50 | 47.29 |
|  |  |  |  | 350 |  | 55.18 |
|  |  |  |  | 400 |  | 63.07 |
| Dense Urban-eMBB Config. A,4GHz UMa-B | FDD | 4T/16R, (1,8,2,1,1) | 4Tx/128Rx, (8,8,2,1,1) | 300 |  | 48.21 |
|  |  |  |  | 350 |  | 56.26 |
|  |  |  |  | 400 |  | 64.29 |

Observation 5: The ITU requirement on UL user experienced data rate can be met by NR.

Note that the evaluation results of UL user experienced data rate and the requirement of aggregated bandwidth to achieve the ITU minimum requirement are closely related to the evaluation results of UL 5th percentile spectral efficiency, and the above results will be updated when evaluation assumptions and spectral efficiencies are further updated in future. It would be good to list both results from minimum required antenna configuration and a reasonable configuration which can provide required bandwidth respectively to meet UL user experienced data rate requirement.

Proposal 4: Further discuss which antenna configuration(s) are applied to UL user experienced data rate evaluation, which leads to different presentation of required bandwidth.

Observation 6: Evaluation results on spectral efficiency by channel model A or B at 4GHz are similar (including DL and UL) in Dense Urban-eMBB.

Conclusion

In this contribution, we provide our preliminary evaluation results of Dense Urban -eMBB, we have following observations and proposals:

Observation 1: For sub 6 GHz, NR can fulfill the DL spectral efficiency requirements for the Dense Urban-eMBB test environment in both TDD and FDD mode.

Observation 2: The ITU requirement on DL user experienced data rate can be met by NR.

Observation 3: For sub 6 GHz, NR can fulfill the UL spectral efficiency requirements for the Dense Urban-eMBB test environment in FDD mode.

Observation 4: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Observation 5: The ITU requirement on UL user experienced data rate can be met by NR.

Observation 6: Evaluation results on spectral efficiency by channel model A or B at 4GHz are similar (including DL and UL) in Dense Urban-eMBB.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 3 when the same assumptions in this contribution are applied.

Proposal 2: Further discuss which antenna configuration(s) are applied to DL user experienced data rate evaluation, which leads to different presentation of required bandwidth.

Proposal 3: Further investigation is needed to find proper antenna configuration for dense urban uplink considering antenna size, complexity and performance.

Proposal 4: Further discuss which antenna configuration(s) are applied to UL user experienced data rate evaluation, which leads to different presentation of required bandwidth.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - R1-1800269, “Initial self evaluation for Dense Urban-eMBB”, CATT

  - R1-1801767, “Discussions on system level simulation configurations and technical features for self evaluation”, CATT

- Annex: Evaluation assumptions

In this contribution, the evaluation assumptions for Dense Urban-eMBB are provided in the following table.

Table A-1 Evaluation assumptions for DL and UL

| Dense Urban - eMBB | Config. A |
| --- | --- |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M |
| Slots structure | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD |
| Subcarrier spacing | 15KHz |
| Symbols number per slot | 14 |
| BS antenna height | 25 m |
| Total transmit power per TRxP | FDD: 41 dBm TDD: 44 dBm |
| UE power class | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss (applies to Channel model B) |
| Inter-site distance | 200 m |
| Number of antenna elements per TRxP | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) 32TXRU, (Mp,Np,P,Mg,Ng) = (2,8,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |
| Device deployment | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| BS antenna element gain | 8 dBi |
| BS antenna element pattern | See Table 9 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi |
| UE antenna element pattern | Omni-directional |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer |
| UE density | 10 UEs per TRxP |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Channel model | Channel model A and B |
| TRxP number per site | 3 |
| Mechanic tilt | 90° in GCS |
| Electronic tilt | 102° in LCS |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping |
| Polarized antenna model | Model-2 in TR36.873 |
| TX mode | SU/MU-MIMO |
| Receiver | MMSE-IRC |
| DL CSI feedback | 10ms period with delay |
| DL CSI method | FDD: CSI TypeII codebook based TDD: full reciprocity based |
| Channel estimation | ideal |
| Interference estimation | ideal |
| UE precoder scheme | Codebook based |
| UL CSI derivation | SRS based with delay |
| UL Waveform | OFDM |
| DMRS configuration | Type 2 |