3GPP TSG RAN WG1 Meeting #92                                                                       R1-1801771

Athens, Greece, February 26th – March 2nd, 2018

Source:	CATT

Title:	Initial self evaluation for Rural-eMBB

Agenda Item:	7.7

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, new Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performances towards all the ITU-R IMT-2020 requirements. The key minimum technical performance requirements are provided in Report ITU-R M.2410 [2] for the test environment of Rural-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Rural – eMBB | 0.12 | 0.045 |

The performance requirement for Rural-eMBB is not applicable to Rural-eMBB LMLC (low mobility large cell) which is one of the evaluation configurations under the Rural- eMBB test environment.

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Rural – eMBB | 3.3 | 1.6 |

The performance requirement for Rural-eMBB is also applicable to Rural-eMBB LMLC which is one of the evaluation configurations under the Rural- eMBB test environment.

According to the evaluation methodology of Report ITU-R M.2412 [3], the average spectral efficiency and the 5th percentile user spectral efficiency need system-level simulation evaluation to demonstrate the achievement for the ITU-R technical performance requirements.

In our company contribution [4], some initial evaluation results including the DL average spectral efficiency and the DL 5th percentile user spectral efficiency. In this document, we focus on the Rural-eMBB and more evaluation results with updated overhead assumptions while more antenna configurations are provided to meet the technical performance requirements.

Evaluation methodology and assumption

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

In [3], a number of evaluation parameters are defined for the detailed evaluations. Some of them need further decision from IMT-2020 technology proponent, e.g. the exact number of antenna elements applied and it is also necessary to discuss the parameters which were not detailed (“reported by proponent”) in ITU report, e.g. TxRU to antenna element mapping. In this contribution, different antenna configurations are evaluated to observe the proper antenna configurations to fulfil the ITU minimum performance requirement, including different number of antenna elements and different TXRU mapping. The evaluation assumptions used in this document refer to our company contribution [5] and are detailed summarized in Annex, which will be updated when 3GPP has further discussions and/or agreements.

Initial downlink evaluation results

Based on the above information, the initial evaluation results for average spectral efficiency and 5th percentile spectral efficiency are provided under Rural-eMBB. Evaluation assumptions can be found in Annex.

Initial downlink evaluation results

The initial evaluation results for DL average spectral efficiency and DL 5th percentile spectral efficiency under Rural-eMBB are provided in Table 1 and Table 2.

Note: Impacts from HARQ, sub-band SRS and overhead are updated. Further non-ideal evaluation assumptions would be updated in future.

Table 1 Initial DL average spectral efficiency evaluation (Rural-eMBB)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-A | TDD | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) | 3.3 | 5.33 | 61.52% |
|  |  | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 8.14 | 146.67% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 6.03 | 82.73% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 9.09 | 175.45% |
| Rural-eMBB Config. A, 700 MHz, RMa-B | TDD | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) |  | 5.32 | 61.21% |
|  |  | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 8.17 | 147.58% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 6.12 | 85.45% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 8.78 | 166.06% |
| Rural-eMBB Config. B, 4GHz, RMa-A | TDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 6.95 | 110.61% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 5.43 | 64.55% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 8.13 | 146.36% |
| Rural-eMBB Config. B, 4GHz, RMa-B | TDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 6.88 | 108.48% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 5.52 | 67.27% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 8.24 | 149.70% |

Table 2 Initial DL 5th percentile spectral efficiency evaluation (Rural-eMBB)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-A | TDD | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) | 0.12 | 0.14 | 16.67% |
|  |  | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.22 | 83.33% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.19 | 58.33% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 0.28 | 133.33% |
| Rural-eMBB Config. A, 700 MHz, RMa-B | TDD | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) |  | 0.13 | 8.33% |
|  |  | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.23 | 91.67% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.19 | 58.33% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 0.27 | 125.00% |
| Rural-eMBB Config. B, 4GHz, RMa-A | TDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.19 | 58.33% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.15 | 25.00% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 0.22 | 83.33% |
| Rural-eMBB Config. B, 4GHz, RMa-B | TDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.19 | 58.33% |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |  | 0.15 | 25.00% |
|  |  | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |  | 0.23 | 91.67% |

Observation 1: For both 700MHz and 4GHz, NR can fulfill the DL spectral efficiency requirements for the Rural-eMBB test environment in both TDD and FDD mode.

Observation 2: In Rural-eMBB DL, configuration A at 700MHz overperformances the case of configuration B at 4GHz assuming the same antenna configuration, although more antenna elements are more feasible at 4GHz

Based on the assumptions in this contribution and [5], according to the above initial evaluation results, the sufficient antenna configurations to meet the minimum requirement are summarized in the following Table 3. Considering the potential more overhead or non-ideal aspects, more TXRU and antenna elements size may be needed and the possible further evaluation antenna configurations are also listed in the following Table 3.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 3 when the same assumptions in this contribution are applied.

Table 3 Sufficient antenna configurations for Rural-eMBB

| Test environment | Duplex scheme | Sufficient antenna configuration |  | Possible further evaluation antenna configuration |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) | TxRU configuration(No. at transmitter/receiver, configuration at transmitter) | Element configuration(No. at transmitter/receiver, configuration at transmitter) |
| Rural-eMBB, Config. A, 700 MHz | TDD | 4T/2R, (1,2,2,1,1) | 4Tx/2Rx, (1,2,2,1,1) | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |
| Rural-eMBB, Config. B, 4 GHz | TDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |
|  | FDD | 8T/2R, (1,4,2,1,1) | 8Tx/2Rx, (1,4,2,1,1) | 16T/2R, (1,8,2,1,1) | 16Tx/2Rx, (1,8,2,1,1) |

Initial uplink evaluation results

The initial FDD evaluation results for UL average spectral efficiency and UL 5th percentile spectral efficiency under Rural-eMBB are provided in Table 4 and Table 5. As initial evaluation, SU-MIMO can be as a baseline MIMO transmission technology. From initial results, it can be observed that spectrum efficiency is a relaxed requirement in Rural-eMBB. Coverage may be a more important evaluation metric for Rural-eMBB.

Note: Non-ideal evaluation assumptions and overhead may be updated in future.

Table 4 Initial UL average spectral efficiency evaluation (Rural-eMBB at 700MHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz/TRxP) | SEavg (bit/s/Hz/TRxP) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-A | FDD | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) | 1.6 | 6.81 | 325.63% |
| Rural-eMBB Config. A, 700 MHz, RMa-B | FDD | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) |  | 6.89 | 330.63% |

Table 5 Initial UL 5th percentile spectral efficiency evaluation (Rural-eMBB at 700MHz)

| Test environment | Duplex scheme | TxRU configuration(No. at transmitter/receiver, configuration at receiver) | Element configuration(No. at transmitter/receiver, configuration at receiver) | Requirement (bit/s/Hz) | SEuser (bit/s/Hz) | Gain |
| --- | --- | --- | --- | --- | --- | --- |
| Rural-eMBB Config. A, 700 MHz, RMa-A | FDD | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) | 0.045 | 0.177 | 293.33% |
| Rural-eMBB Config. A, 700 MHz, RMa-B | FDD | 2T/8R, (1,4,2,1,1) | 2Tx/64Rx, (8,4,2,1,1) |  | 0.169 | 275.56% |

Observation 3: For 700MHz, NR can fulfill the UL spectral efficiency requirements for the Rural-eMBB test environment in FDD mode.

Observation 4: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Proposal 2: Further investigation is needed to find proper antenna configuration for rural uplink considering antenna size, complexity and performance.

Observation 5: Evaluation results on spectral efficiency by channel model A or B are similar (including DL and UL) in Rural-eMBB.

Conclusion

In this contribution, we provide our preliminary evaluation results of Rural-eMBB, we have the following observation:

Observation 1: For both 700MHz and 4GHz, NR can fulfill the DL spectral efficiency requirements for the Rural-eMBB test environment in both TDD and FDD mode.

Observation 2: In Rural-eMBB DL, configuration A at 700MHz overperformances the case of configuration B at 4GHz assuming the same antenna configuration, although more antenna elements are more feasible at 4GHz

Observation 3: For 700MHz, NR can fulfill the UL spectral efficiency requirements for the Rural-eMBB test environment in FDD mode.

Observation 4: Even SU-MIMO can fulfil the requirement for uplink. It is possible that reduced number of TxRU at receiver (eNB) can be sufficient to meet the requirement considering antenna size and complexity at eNB.

Observation 5: Evaluation results on spectral efficiency by channel model A or B are similar (including DL and UL) in Rural-eMBB.

Proposal 1: For downlink spectral efficiency evaluation to meet the requirements, the sufficient antenna configurations can refer to the configurations in Table 3 when the same assumptions in this contribution are applied.

Proposal 2: Further investigation is needed to find proper antenna configuration for rural uplink considering antenna size, complexity and performance.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

  - R1-1800270, “Initial self evaluation for Rural-eMBB”, CATT.

  - R1-1801767, “Discussions on system level simulation configurations and technical features for self evaluation”, CATT

- Annex: Evaluation assumptions

In this contribution, the evaluation assumptions for Rural-eMBB are provided in the following table.

Table A-1 Evaluation assumptions for DL and UL

| Rural - eMBB | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 4 GHz |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M | FDD: 10M+10M TDD: 20M |
| Slots structure | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD |
| Subcarrier spacing | 15KHz | 15KHz |
| Symbols number per slot | 14 | 14 |
| BS antenna height | 35 m | 35 m |
| Total transmit power per TRxP | FDD: 46 dBm TDD: 49 dBm | FDD: 46 dBm TDD: 49 dBm |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 1732 m | 1732 m |
| Number of antenna elements per TRxP | 4 Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), 8 Tx/Rx, (M,N,P,Mg,Ng) = (1,4,2,1,1), 16 Tx/Rx, (M,N,P,Mg,Ng) = (1,8,2,1,1), 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 8 Tx/Rx, (M,N,P,Mg,Ng) = (1,4,2,1,1), 16 Tx/Rx, (M,N,P,Mg,Ng) = (1,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) |
| Number of UE antenna elements | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) UL: 2TX/RX 0°,90° polarization | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°,90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) | 2TXRU (1-to-1 mapping) |
| Device deployment | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 9 in Report ITU-R M.2412 | See Table 9 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m |
| Channel model variant | Channel model B | Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS | 90° in GCS |
| Electronic tilt | 100° in LCS | 100° in LCS |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP from port 0 | Based on RSRP from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |
| TX mode | SU/MU-MIMO | SU/MU MIMO |
| Receiver | MMSE-IRC | MMSE-IRC |
| DL CSI feedback | 10ms period with delay | 10ms period with delay |
| DL CSI method | FDD: CSI TypeII codebook based TDD: full reciprocity based | FDD: CSI TypeII codebook based TDD: full reciprocity based |
| Channel estimation | ideal | ideal |
| Interference estimation | ideal | ideal |
| UE precoder scheme | Codebook based | -- |
| UL CSI derivation | SRS based with delay | -- |
| UL Waveform | DFT-s-FDMA | -- |
| DMRS configuration | Type 2 | Type 2 |