3GPP TSG RAN WG1 Meeting #92	R1-1801790

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	7.7

Source:	Huawei, HiSilicon

Title:	Initial evaluation results of eMBB spectral efficiency

Document for:	Discussion

Introduction

In this contribution, the initial evaluation results for eMBB spectral efficiency (average and the 5th percentile user spectral efficiency) are provided. The initial evaluation is applied to NR for various duplexing schemes including conventional FDD, TDD, and a flexible duplex scheme.

Test environments for eMBB spectral efficiency

According to Report ITU-R M.2410, average spectral efficiency and 5th percentile user spectral efficiency are used to evaluate the achievement for three times spectral efficiency improvement in ITU-R vision (see Recommendation ITU-R M.2083). The eMBB test environments and their related evaluation configurations are employed in Report ITU-R M.2412 to test the fulfillment of the spectral efficiency requirements. In this contribution, a subset of the eMBB test environments and the related evaluation configurations as given in Table 1 is considered for initial spectral efficiency evaluation.

TABLE 1

Evaluation configuration for initial spectral efficiency evaluation

| Test env. | Evaluation configuration | Carrier frequency | ISD | Remark |
| --- | --- | --- | --- | --- |
| Indoor Hotspot – eMBB | Config. A | 4 GHz | 20 m |  |
| Dense Urban – eMBB | Config. A | 4 GHz | 200 m | Macro layer only |
| Rural - eMBB | Config. A | 700 MHz | 1 732 m |  |
|  | Config. C (LMLC) | 700 MHz | 6 000 m |  |

Evaluated technical features

The IMT-2020 eMBB spectral efficiency requirement is three times higher compared to IMT-Advanced. Therefore, it is a challenging requirement and the NR features that could contribute to the improvement of spectral efficiency should be selected.

On the other hand, it is recalled that for LTE-Advanced evaluation towards IMT-Advanced submission, the evaluation is basically applied based on duplexing schemes, i.e., to FDD and TDD, respectively. This is due to the fact that, duplexing scheme is one of the fundamental features among the other features that impact spectral efficiency performance.

Duplexing scheme

In NR design, the flexible duplexing scheme is available, e.g.,

- Different transmission directions in either part of a paired spectrum,

- TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing.

The new duplexing has the potential to improve the spectral efficiency. In the initial evaluation, the following duplexing schemes with related technical features are considered

- FDD (for paired spectrum): NR features that can be applied to FDD operation are invoked (e.g., for DL evaluation, NR codebook based CSI feedback with NR CSI-RS transmission is employed).

- Flexible duplex on DL (for paired spectrum): On DL part of the spectrum, a DL dominant slot with UL part SRS transmission is configured in every radio frame. By this means the DL channel reciprocity is enabled which could further improve the spectral efficiency over conventional FDD.

- TDD (for unpaired spectrum): NR features that can be applied to TDD operation are invoked (e.g., for DL evaluation, DL CSI is derived from SRS transmission using enhanced NR SRS capacity).

For “Flexible duplex on DL”, it is noted that the DL dominant slot includes an UL part to transmit UL SRS to enable the channel reciprocity on DL part of the paired spectrum. In the evaluation, both the location of the DL dominant slot in a radio frame, and the location and the length of the UL part within the DL dominant slot are configured to be identical across the BSs in the synchronized network. Therefore there is no cross-link interference in this configuration.

Spectral efficiency calculation for duplexing scheme

The spectral efficiency of different duplexing schemes can be calculated according to Report ITU-R M.2412.

For DL average spectral efficiency and 5th percentile spectral efficiency,

- In case of FDD, the simulation bandwidth is 10 MHz for DL and 10 MHz for UL. The DL average spectral efficiency is given by

(1)

where W is the DL bandwidth of 10 MHz; Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control and DL reference signals on the DL bandwidth of 10 MHz is taken into account when deriving Ri (T); and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

- For TDD, the simulation bandwidth is 20 MHz for DL and UL. The DL average spectral efficiency is given by (1), where W is the effective DL bandwidth that accounts for the time-frequency resource used for DL transmission (including GP symbols); Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control, DL reference signal on the DL effective bandwidth is taken into account; and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

- For flexible duplex on DL, the simulation bandwidth is 10 MHz for DL and 10 MHz for UL. The DL average spectral efficiency is given by (1), where W is the bandwidth of DL part of 10 MHz; Ri (T) denotes the number of correctly received bits of user i, and the overhead of DL control, DL reference signal and UL OFDM symbols for SRS transmission on the DL part the spectrum is taken into account when deriving Ri (T); and T is the simulation time. Similar notations are applied to 5th percentile user spectral efficiency.

For UL average spectral efficiency and 5th percentile spectral efficiency, similar way is employed to derive the evaluation results for these two metrics.

Initial evaluation results

Based on the above discussion, initial evaluation results for average spectral efficiency and 5th percentile spectral efficiency is evaluated. Evaluation assumptions can be found in Appendix.

DL spectral efficiency evaluation results

The initial DL spectral efficiency evaluation results for NR are given in Table 2 and Table 3.

TABLE 2

Initial DL spectral efficiency evaluation (for different duplexing schemes)

| Test env. | Evaluation configuration | Average spectral efficiency (bit/s/Hz/TRxP) |  |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDD | Flexible duplex on DL | TDD | Req. | FDD | Flexible duplex on DL | TDD | Req. |
| Indoor Hotspot | Config. A (4 GHz); 32T4R | 10.31 | 11.03 | 11.54 | 9 | 0.36 | 0.36 | 0.38 | 0.3 |
| Dense Urban | Config. A (4 GHz); 32T4R | 9.79 | 11.63 | 12.04 | 7.8 | 0.31 | 0.32 | 0.33 | 0.225 |
| Rural | Config. A (700 MHz; 8T2R | 5.44 | 5.99 | 6.53 | 3.3 | 0.12 | 0.12 | 0.12 | 0.12 |
|  | Config. C (700 MHz); 8T4R | 6.68 | 6.85 | 7.30 | 3.3 | 0.20 | 0.19 | 0.20 | 0.12 |

TABLE 3

Initial DL spectral efficiency evaluation (for different antenna configurations)

| Test env. | Evaluation configuration | Average spectral efficiency (bit/s/Hz/TRxP) |  |  | 5th percentile spectral efficiency (bit/s/Hz) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TDD |  |  | TDD |  |  |
|  |  | 32T2R | 32T4R | Requirement | 32T2R | 32T4R | Requirement |
| Dense Urban | Config. A (4 GHz) | 8.30 | 12.04 | 7.8 | 0.20 | 0.33 | 0.225 |

Observation 1: NR with sub 6 GHz has the potential to fulfill DL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

Observation 2: NR flexible duplex on DL can provide 10~20% spectral efficiency gain over NR FDD in most test environments.

Observation 3: Four receive antenna elements are desired at UE side to confidently achieve ITU-R DL spectral efficiency requirements.

UL spectral efficiency evaluation results

The initial UL spectral efficiency evaluation results for NR are given in Table 4. Observation is made based on these initial evaluation results

TABLE 4

Initial UL spectral efficiency evaluation

| Test env. | Evaluation configuration | Average spectral efficiency (bit/s/Hz/TRxP) |  | 5th percentile spectral efficiency (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | FDD | Requirement | FDD | Requirement |
| Indoor Hotspot | (Config. A 4 GHz; 2T16R) | 7.27 | 6.75 | 0.35 | 0.21 |
| Dense Urban | (Config. A 4 GHz; 2T16R) | 8.15 | 5.4 | 0.30 | 0.15 |
| Rural | (Config. A 700 MHz; 1T8R) | 3.88 | 1.6 | 0.10 | 0.045 |

Observation 4: NR with sub 6 GHz has the potential to fulfill UL spectral efficiency requirements in all test environments.

Conclusion

In this document, the initial evaluation of eMBB spectral efficiency for NR operating on frequency range 1 (FR1, i.e., below 6 GHz) is provided. Based on the evaluation, the following observations are made.

Observation 1: NR with sub 6 GHz has the potential to fulfill DL spectral efficiency requirements in all test environments, including Rural – eMBB configuration C (LMLC).

Observation 2: NR flexible duplex on DL can provide 10~20% spectral efficiency gain over NR FDD in most test environments.

Observation 3: Four receive antenna elements are desired at UE side to confidently achieve ITU-R DL spectral efficiency requirements.

Observation 4: NR with sub 6 GHz has the potential to fulfill UL spectral efficiency requirements in all test environments.

- Appendix: Evaluation assumptions

Table A-1 Evaluation assumptions for DL

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Test environment | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |
| Evaluation configuration | Configuration A | Configuration A | Configuration A, Configuration C |
| Channel model | InH_A | UMa_A | RMa_A |
| ISD | 20 m (12TRxP) | 200 m | Configuration A: 1 732 m Configuration C: 6 000 m |
| Carrier Frequency | 4 GHz | 4 GHz | 700 MHz |
| System bandwidth | TDD: 20MHz | TDD: 20MHz | TDD: 20MHz |
|  | FDD:10MHz | FDD:10MHz | FDD:10MHz |
|  | Flexible duplex: 10MHz | Flexible duplex: 10MHz | Flexible duplex: 10MHz |
| Subcarrier spacing | 15 kHz | 15 kHz | 15 kHz |
| Symbols number per slot | 14 | 14 | 14 |
| Number of antenna elements per TRxP | 32Tx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 128Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 64Tx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP | 32TXRU | For 32TXRU: Vertical 2-to-8 For 16TXRU: Vertical 1-to-8 | 8TXRU Vertical 1-to-8 |
| Number of antenna elements per UE | 4Rx with 0°,90° polarization | 4Rx or 2Rx with 0°,90° polarization | Configuration A: 2Rx Configuration C: 4Rx with 0°,90° polarization |
| Transmit power per TRxP | TDD: 24 dBm | TDD: 44 dBm | TDD: 49 dBm |
|  | FDD: 21 dBm | FDD: 41 dBm | FDD: 46 dBm |
|  | Flexible duplex: 21dBm | Flexible duplex: 41dBm | Flexible duplex: 46dBm |
| TRxP number per site | 1 | 3 | 3 |
| Mechanic tilt | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt | 90deg in LCS | 105deg in LCS | Configuration A: 100deg in LCS Configuration C: 92deg in LCS |
| UT attachment | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 |
| Scheduling | MU-PF | MU-PF | MU-PF |
| MIMO mode | MU-MIMO with rank 4 adaptation per user; Maximum MU layer = 12 | MU-MIMO with rank 4 adaptation per user Maximum MU layer = 12 | MU-MIMO with rank 2 adaptation per user Maximum MU layer = 8 |
| Guard band ratio | TDD: 4.6% (for 20 MHz) | TDD: 4.6% (for 20 MHz) | TDD: 4.6% (for 20 MHz) |
|  | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) | FDD: 6.4% (for 10 MHz) |
|  | Flexible duplex: 6.4% (for 10 MHz) | Flexible duplex: 6.4% (for 10 MHz) | Flexible duplex: 6.4% (for 10 MHz) |
| BS receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| CSI feedback | 5ms period based on CSI-RS (ideal) with delay | 5ms period based on CSI-RS (ideal) with delay | 5ms period based on CSI-RS (ideal) with delay |
| Precoder derivation | TDD: SRS (ideal) based | TDD: SRS (ideal) based | TDD: SRS (ideal) based |
|  | FDD: NR Type II codebook based | FDD: NR Type II codebook based | FDD: NR Type II codebook based |
|  | Flexible duplex: SRS (ideal) based | Flexible duplex: SRS (ideal) based | Flexible duplex: SRS (ideal) based |
| Waveform | OFDM | OFDM | OFDM |

Table A-2 Evaluation assumptions for UL

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Test environment | Indoor Hotspot – eMBB | Dense Urban – eMBB | Rural - eMBB |
| Evaluation configuration | Configuration A | Configuration A | Configuration A |
| Channel model | InH_A | UMa_A | RMa_A |
| Subcarrier spacing | 15 kHz | 15 kHz | 15 kHz |
| Symbols number per slot | 14 | 14 | 14 |
| Number of antenna elements per TRxP | 32Rx cross-polarized antenna (M,N,P,Mg,Ng) = (4,4,2,1,1) | 128Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,8,2,1,1) | 64Rx cross-polarized antenna (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP | 16TXRU Vertical 1-to-2 | 16TXRU Vertical 1-to-8 | 8TXRU Vertical 1-to-8 |
| Number of antenna elements per UE | 2Tx with 0°,90° polarization | 2Tx with 0°,90° polarization | 1Tx |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Mechanic tilt | 180deg in GCS (pointing to the ground) | 90deg in GCS (pointing to the horizontal direction) | 90deg in GCS (pointing to the horizontal direction) |
| Electronic tilt | 90deg in LCS | 105deg in LCS | 100deg in LCS |
| UT attachment | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 | Based on RSRP (formula) from CRS port 0 |
| Scheduling | MU-PF | MU-PF | SU-PF |
| MIMO mode | MU-MIMO with rank 2 adaptation per user Maximum paired number of user = 2 | MU-MIMO with rank 2 adaptation per user Maximum paired number of user = 2 | SIMO |
| BS receiver type | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| UE precoder scheme | Codebook based | Codebook based | N.A. |
| UL CSI derivation | SRS (ideal) based, with delay | SRS (ideal) based, with delay | SRS (ideal) based, with delay |
| Waveform | OFDM | OFDM | OFDM |