3GPP TSG RAN WG1 Meeting AH 1801	R1-18XXXX

3GPP TSG RAN WG1 Meeting #92	R1-1801794

Athens, Greece, February 26th – March 2nd 2018

Agenda Item:	7.7

Source:	Huawei, HiSilicon

Title:	Consideration on self evaluation of IMT-2020 for peak spectral efficiency and peak data rate

Document for:	Discussion

Introduction

In this contribution, the consideration on self evaluation for peak spectral efficiency and peak data rate is provided.

In previous contribution [1], the calculation method on peak spectral efficiency and peak data rate were provided. In RAN1#91 meeting, the data rate calculation method was adopted in NR UE category discussion, as shown in [2]. In this contribution, the peak spectral efficiency and peak data rate evaluation for NR are considered for FDD and TDD. In addition, the two frequency ranges, i.e., frequency range 1 (FR1) for sub 6 GHz, and frequency range 2 (FR2) for above 24 GHz, are taken into account.

Peak spectral efficiency

According to Report ITU-R M.2410 (see [3]), the peak spectral efficiency is defined as in subsection 3.1. Based on the definition, the DL and UL peak spectral efficiency of NR is analyzed.

Calculation of peak spectral efficiency

From the definition and [2] we can derive the generic formula for peak spectral efficiency for FDD and TDD for a specific component carrier (say j-th CC) as below.

(1)

wherein

- Rmax = 948/1024

- For the j-th CC,

  - is the maximum number of layers

  - is the maximum modulation order

  - is the scaling factor

      - The scaling factor can at least take the values 1 and 0.75.

      - is signalled per band and per band per band combination as per UE capability signalling

  - is the numerology (as defined in TS38.211)

  - is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

  - is the maximum RB allocation in bandwidth  with numerology , as given in TR 38.817-01 section 4.5.1, where  is the UE supported maximum bandwidth in the given band or band combination.

  - is the overhead  calculated as the average ratio of the number of REs occupied by L1/L2 control, Synchronization Signal, PBCH and reference signals etc. with respect to the total number of REs in effective bandwidth  time product.

 is the normalized scalar considering the downlink/uplink ratio; for FDD  for DL and UL; and for TDD and other duplexing  for DL and UL is calculated based on the frame structure.

DL peak spectral efficiency

Based on the formula provided in Eq. (1), the DL peak spectral efficiency for NR is given in this sub-section.

FDD DL peak spectral efficiency

The detailed parameters for FDD DL peak spectral efficiency are listed in Table 11 in the appendix. According to Eq. (1) and these parameters, the DL peak spectral efficiency for FDD for different bandwidth and SCS parameters are given in Table 2.

Table 2 FDD DL peak spectral efficiency for bandwidth and SCS parameters

| Bandwidth and SCS parameters |  |  | DL peak spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Frequency range | Bandwidth | Sub-carrier spacing |  |  |
| FR1 (Below 6 GHz) | 50MHz | 15 kHz | 48.38 | 30 |
|  | 100MHz | 30 kHz | 48.91 |  |
|  | 100MHz | 60 kHz | 48.38 |  |
| FR2 (Above 24 GHz) | 200MHz | 60 kHz | 43.10 |  |
|  | 400MHz | 120 kHz | 43.10 |  |

TDD DL peak spectral efficiency

Based on Eq. (1), DL peak spectral efficiency of TDD can also be derived with a straight forward extension. Most of the values of the key parameters are aligned with FDD that have been provided in Table 11, except the effective bandwidth parameter, , which would impact the calculation of overhead. In this evaluation, two cases, i.e., the DL dominant DL/UL configuration and UL dominant DL/UL configuration are selected, and the detailed overhead parameters are provided in Table 12 and Table 13 in the appendix.

Therefore, the TDD DL peak spectral efficiency for different bandwidth and SCS parameters are shown in Table 3 and in Table 4, respectively.

Table 3 TDD DL peak spectral efficiency for bandwidth and SCS parameters with DL:UL=4:1 and

| Bandwidth and SCS parameters |  |  | DL peak spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Frequency range | Bandwidth | Sub-carrier spacing |  |  |
| FR1 (Below 6 GHz) | 50MHz | 15 kHz | 47.30 | 30 |
|  | 100MHz | 30 kHz | 47.83 |  |
|  | 100MHz | 60 kHz | 47.30 |  |
| FR2 (Above 24 GHz) | 200MHz | 60 kHz | 42.05 |  |
|  | 400MHz | 120 kHz | 42.05 |  |

Table 4 TDD DL peak spectral efficiency for bandwidth and SCS parameters with DL:UL=1:4 and

| Bandwidth and SCS parameters |  |  | DL peak spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Frequency range | Bandwidth | Sub-carrier spacing |  |  |
| FR1 (Below 6 GHz) | 50MHz | 15 kHz | 43.54 | 30 |
|  | 100MHz | 30 kHz | 44.57 |  |
|  | 100MHz | 60 kHz | 44.08 |  |
| FR2 (Above 24 GHz) | 200MHz | 60 kHz | 38.37 |  |
|  | 400MHz | 120 kHz | 38.89 |  |

UL peak spectral efficiency

Similarly, based on the formula provided in Eq. (1), the UL peak spectral efficiency for NR is given as follows.

FDD UL peak spectral efficiency

The detailed parameters on FDD UL peak spectral efficiency are provided in Table 14 in the appendix. With Eq. (1) and these parameters, the UL peak spectral efficiency of NR are shown in Table 5.

Table 5 FDD UL peak spectral efficiency for bandwidth and SCS parameters

| Bandwidth and SCS parameters |  |  | UL peak spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Frequency range | Bandwidth | Sub-carrier spacing |  |  |
| FR1 (Below 6 GHz) | 50MHz | 15 kHz | 25.00 | 15 |
|  | 100MHz | 30 kHz | 25.27 |  |
|  | 100MHz | 60 kHz | 25.00 |  |
| FR2 (Above 24 GHz) | 200MHz | 60 kHz | 23.39 |  |
|  | 400MHz | 120 kHz | 23.39 |  |

TDD UL peak spectral efficiency

Similarly, with the overhead parameters in Table 15 and Table 16, the TDD UL peak spectral efficiency of NR are shown in Table 6 and Table 7, respectively.

Table 6 TDD UL peak spectral efficiency for bandwidth and SCS parameters for DL:UL=1:4 and

| Bandwidth and SCS parameters |  |  | UL peak spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Frequency range | Bandwidth | Sub-carrier spacing |  |  |
| FR1 (Below 6 GHz) | 50MHz | 15 kHz | 25.00 | 15 |
|  | 100MHz | 30 kHz | 25.27 |  |
|  | 100MHz | 60 kHz | 25.00 |  |
| FR2 (Above 24 GHz) | 200MHz | 60 kHz | 23.39 |  |
|  | 400MHz | 120 kHz | 23.39 |  |

Table 7 TDD UL peak spectral efficiency for bandwidth and SCS parameters for DL:UL=4:1 and

| Bandwidth and SCS parameters |  |  | UL peak spectral efficiency (bit/s/Hz) | Requirement (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Frequency range | Bandwidth | Sub-carrier spacing |  |  |
| FR1 (Below 6 GHz) | 50MHz | 15 kHz | 24.73 | 15 |
|  | 100MHz | 30 kHz | 25.27 |  |
|  | 100MHz | 60 kHz | 25.00 |  |
| FR2 (Above 24 GHz) | 200MHz | 60 kHz | 23.39 |  |
|  | 400MHz | 120 kHz | 23.39 |  |

Observations

Based on the above evaluation results, it is observed that,

Observation1: NR can fulfill DL and UL peak spectral efficiency requirements for both below and above 6GHz.

Observation2: The peak spectral efficiency of above 6GHz is lower than that of below 6GHz, considering the overhead needed.

Peak data rate

According to Report ITU-R M.2410, the peak data rate is defined as in subsection 4.1. Based on the definition, the DL and UL peak data rate of NR is analyzed.

Calculation of peak data rate

It is noted that Wj in (2) is the effective bandwidth that takes into account the uplink/downlink ratio. In this case, generic equation for DL/UL peak data rate for FDD and TDD is given as below

(3)

where the scaling factor (j) is the value for DL or UL on band j, respectively.

DL peak data rate

From RAN1 specification perspective, the maximum number of NR carriers for carrier aggregation and dual connectivity is 16. Considering the peak spectral efficiency, the required number of component carriers to achieve the target, and the corresponding DL peak data rate is shown in Table 9.

Table 9 DL peak data rate for bandwidth and SCS parameters

| Duplexing | Bandwidth and SCS parameters |  |  | DL peak data rate (Gbit/s) | Requirement (Gbit/s) |
| --- | --- | --- | --- | --- | --- |
|  | Frequency range | Total Bandwidth | Sub-carrier spacing |  |  |
| FDD | FR1 (Below 6 GHz) | 500MHz (10CC) | 15 kHz | 24.19 | 20 |
|  |  | 500MHz (5CC) | 30 kHz | 24.46 |  |
|  |  | 500MHz (5CC) | 60 kHz | 24.19 |  |
|  | FR2 (Above 24 GHz) | 600MHz (3CC) | 60 kHz | 25.86 |  |
|  |  | 800MHz (2CC) | 120 kHz | 34.48 |  |
| TDD DL:UL=4:1 () | FR1 (Below 6 GHz) | 550MHz (11CC) | 15 kHz | 20.81 |  |
|  |  | 600MHz (6CC) | 30 kHz | 22.96 |  |
|  |  | 600MHz (6CC) | 60 kHz | 22.70 |  |
|  | FR2 (Above 24 GHz) | 600MHz (3CC) | 60 kHz | 20.18 |  |
|  |  | 800MHz (2CC) | 120 kHz | 26.91 |  |

UL peak data rate

Similar to DL, the required number of component carriers to achieve the target, and the corresponding UL peak data rate is shown in Table 10.

Table 10 UL peak data rate for bandwidth and SCS parameters

| Duplexing | Bandwidth and SCS parameters |  |  | UL peak data rate (Gbit/s) | Requirement (Gbit/s) |
| --- | --- | --- | --- | --- | --- |
|  | Frequency range | Total Bandwidth | Sub-carrier spacing |  |  |
| FDD | FR1 (Below 6 GHz) | 500MHz (10CC) | 15 kHz | 12.50 | 10 |
|  |  | 500MHz (5CC) | 30 kHz | 12.63 |  |
|  |  | 500MHz (5CC) | 60 kHz | 12.50 |  |
|  | FR2 (Above 24 GHz) | 600MHz (3CC) | 60 kHz | 14.03 |  |
|  |  | 800MHz (2CC) | 120 kHz | 18.71 |  |
| TDD DL:UL=1:4 () | FR1 (Below 6 GHz) | 500MHz (10CC) | 15 kHz | 10.00 |  |
|  |  | 500MHz (5CC) | 30 kHz | 10.11 |  |
|  |  | 500MHz (5CC) | 60 kHz | 10.00 |  |
|  | FR2 (Above 24 GHz) | 600MHz (3CC) | 60 kHz | 11.22 |  |
|  |  | 800MHz (2CC) | 120 kHz | 14.97 |  |

Observations

Based on the above evaluation results, it is observed that,

Observation3: NR can fulfill DL and UL peak data rate requirements for both below and above 6GHz with carrier aggregation.

Observation4: The frequency range of below 6 GHz can achieve similar peak data rate performance to above 6GHz by using less bandwidth.

Conclusions

In this document, the peak spectral efficiency and peak data rate of NR for the two frequency ranges are analysed. The analysis shown in this document is based on the assumption that the component carrier bandwidth is assigned to one transmission direction for either paired spectrum or unpaired spectrum. The following observations are made.

Observation1: NR can fulfill DL and UL peak spectral efficiency requirements for both below and above 6GHz.

Observation2: The peak spectral efficiency of above 6GHz is lower than that of below 6GHz, considering the overhead needed.

Observation3: NR can fulfill DL and UL peak data rate requirements for both below and above 6GHz with carrier aggregation.

Observation4: The frequency range of below 6 GHz can achieve similar peak data rate performance to above 6GHz by using less bandwidth.

- References

R1-1719419, “Consideration on self evaluation of peak spectral efficiency and peak data rate for IMT-2020”, Huawei, HiSilicon, November 2017.

R1-1721732, “Reply to LS on NR UE Category”, RAN1, November 2017.

Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Appendix       Detailed evaluation assumptions

Table 11 The parameters and values of FDD DL peak spectral efficiency

| - Parameters | - Values | - Remarks |
| --- | --- | --- |
|  | 8 | NR supports up to 8 layers for a single user for DL |
|  | 8 | NR supports up to 256QAM for DL |
|  | 1 | The value of 1 is chosen as scaling factor for DL peak spectral efficiency evaluation. |
| Rmax | 948/1024 = 0.9258 | NR supports highest coding rate as Rmax=948/1024. |
|  | 0, 1, 2, 3 | SCS, i.e., - 15kHz SCS for - 30kHz SCS for - 60kHz SCS for - 120kHz SCS for |
|  | For FR1: - 270 for 50MHz with 15kHz SCS - 273 for 100MHz with 30kHz SCS - 135 for 100MHz with 60kHz SCS For FR2: - 264 for 200MHz with 60kHz SCS - 264 for 400MHz with 120kHz SCS | See Section 5.3.2 of TS38.104 v0.5.0 |
|  |  | SCS, i.e., - 15kHz SCS for - 30kHz SCS for - 60kHz SCS for - 120kHz SCS for |
|  | For FR1: - 0.10 for 50MHz with 15kHz SCS - 0.10 for 100MHz with 30kHz SCS - 0.10 for 100MHz with 60kHz SCS | For FR1: - 1 single symbol CORESET of 12 RB (2 CCE) in every slot - 12 RE/RB/slot - TRS burst of 2 slots with periodicity of 20ms and occupies 24RB - 6 RE/RB/slot - DMRS: 16 RE/RB/slot in every slot and RB - CSI-RS: 8 CSI-RS ports with 8 RE/RB/slot with periodicity of 20ms in every RB - 2 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 RB × 12 REs/RB |
|  | For FR2: - 0.18 for 200MHz with 60kHz SCS - 0.18 for 400MHz with 120kHz SCS | For FR2: - 1 single symbol CORESET of 24 RB (4 CCE) in every slot - 12 RE/RB/slot - TRS burst of 2 slots with periodicity of 10ms and occupies 24RB - 6 RE/RB/slot - DMRS: 24 RE/RB/slot in every slot and RB - PTRS: 5 RE/RB/slot (2 port PT-RS, with 25% frequency domain density) in every slot and RB - CSI-RS: 8 CSI-RS ports with 8 RE/RB/slot with periodicity of 10ms in every RB - CSI-RS for BM: 1 CSI-RS port with 2 RE/RB/slot with periodicity of 10ms in every RB - 8 SS/PBCH blocks per 20ms; one SS/PBCH block occupies 960REs = 4 OFDM symbols × 20 RB × 12 REs/RB |
|  | For FR1: - 50MHz with 15kHz SCS - 100MHz with 30kHz SCS - 100MHz with 60kHz SCS For FR2: - 200MHz with 60kHz SCS - 400MHz with 120kHz SCS | See Section 5.3.2 of TS38.104 v0.5.0 |

Table 12 The parameters and values of TDD DL peak spectral efficiency for DL dominant DL/UL configuration

| - Parameters | - Values | - Remarks |
| --- | --- | --- |
|  | 0.8 | This value corresponds to DL:UL=4:1, where 4 DL slots and 1 UL slot are configured in every 5 slots; one symbol for GP is assumed in DL slot when the slot switches from DL to UL. |
|  | For FR1: - 0.12 for 50MHz with 15kHz SCS - 0.12 for 100MHz with 30kHz SCS - 0.12 for 100MHz with 60kHz SCS For FR2: - 0.20 for 200MHz with 60kHz SCS - 0.20 for 400MHz with 120kHz SCS | The CORESET and RS configurations are the same as FDD DL in Table 2. GP overhead is included in DL slot. |

Table 13 The parameters and values of TDD DL peak spectral efficiency for UL dominant DL/UL configuration

| - Parameters | - Values | - Remarks |
| --- | --- | --- |
|  | 0.2 | This value corresponds to DL:UL=1:4, where 1 DL slot and 4 UL slots are configured in every 5 slots; one symbol for GP is assumed in DL slot when the slot switches from DL to UL. |
|  | For FR1: - 0.19 for 50MHz with 15kHz SCS - 0.18 for 100MHz with 30kHz SCS - 0.18 for 100MHz with 60kHz SCS For FR2: - 0.27 for 200MHz with 60kHz SCS - 0.26 for 400MHz with 120kHz SCS | The CORESET and RS configurations are the same as FDD DL in Table 2. GP overhead is included in DL slot; and the GP overhead becomes large due to the limited DL slot duration. |

Table 14 The parameters and values of FDD UL peak spectral efficiency

| - Parameters | - Values | - Remarks |
| --- | --- | --- |
|  | 4 | NR supports up to 4 layers for a single user for UL |
|  | 8 | NR supports up to 256QAM for UL |
|  | 1 | The value of 1 is chosen as scaling factor for UL peak spectral efficiency evaluation. |
| Rmax | 948/1024 = 0.9258 | NR supports highest coding rate as Rmax=948/1024. |
|  | 0, 1, 2, 3 | SCS, i.e., - 15kHz SCS for - 30kHz SCS for - 60kHz SCS for - 120kHz SCS for |
|  | For FR1: - 270 for 50MHz with 15kHz SCS - 273 for 100MHz with 30kHz SCS - 135 for 100MHz with 60kHz SCS For FR2: - 264 for 200MHz with 60kHz SCS - 264 for 400MHz with 120kHz SCS | See Section 5.3.2 of TS38.104 v0.5.0 |
|  |  | SCS, i.e., - 15kHz SCS for - 30kHz SCS for - 60kHz SCS for - 120kHz SCS for |
|  | For FR1: - 0.07 for 50MHz with 15kHz SCS - 0.07 for 100MHz with 30kHz SCS - 0.07 for 100MHz with 60kHz SCS For FR2: - 0.11 for 200MHz with 60kHz SCS - 0.11 for 400MHz with 120kHz SCS | For FR1: - DM-RS: 1 symbol per slot - SRS: 2 symbols per slot with periodicity of 5ms and SRS BW of 8 PRB For FR2: - DM-RS: 1 symbol per slot - SRS: 4 symbols per slot with periodicity of 5ms and SRS BW of 16 PRB - PTRS: 6 RE/RB/slot (2 port PT-RS, with 25% frequency domain density) in every slot and RB |
|  | For FR1: - 50MHz with 15kHz SCS - 100MHz with 30kHz SCS - 100MHz with 60kHz SCS For FR2: - 200MHz with 60kHz SCS - 400MHz with 120kHz SCS | See Section 5.3.2 of TS38.104 v0.5.0 |

Table 15 The parameters and values of TDD UL peak spectral efficiency for UL dominant DL/UL configuration

| - Parameters | - Values | - Remarks |
| --- | --- | --- |
|  | 0.8 | This value corresponds to DL:UL=1:4, where 1 DL slot and 4 UL slots are configured in every 5 slots; one symbol for GP is assumed in DL slot when the slot switches from DL to UL. |
|  | For FR1: - 0.07 for 50MHz with 15kHz SCS - 0.07 for 100MHz with 30kHz SCS - 0.07 for 100MHz with 60kHz SCS For FR2: - 0.11for 200MHz with 60kHz SCS - 0.11 for 400MHz with 120kHz SCS | The CORESET and RS configurations are the same as FDD UL in Table 8. GP overhead is included in DL. |

Table 16 The parameters and values of TDD UL peak spectral efficiency for DL dominant DL/UL configuration

| - Parameters | - Values | - Remarks |
| --- | --- | --- |
|  | 0.2 | This value corresponds to DL:UL=4:1, where 4 DL slots and 1 UL slot are configured in every 5 slots; one symbol for GP is assumed in DL slot when the slot switches from DL to UL. |
|  | For FR1: - 0.08 for 50MHz with 15kHz SCS - 0.07 for 100MHz with 30kHz SCS - 0.07 for 100MHz with 60kHz SCS For FR2: - 0.11 for 200MHz with 60kHz SCS - 0.11 for 400MHz with 120kHz SCS | The CORESET and RS configurations are the same as FDD UL in Table 8. GP overhead is included in DL. |