3GPP TSG RAN WG1 Meeting #92                                          R1-1801835

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.2.1.3

Source:	Spreadtrum Communications

Title:	Remaining issues on non-codebook based UL

Document for:	Discussion and decision

Introduction

The support for non-codebook based UL is almost complete for Rel-15 MIMO. However, there are still some minor issues remaining unsolved. One of them is the use of SRS-SpatialRelationInfo and SRS-AssocCSIRS, which was discussed in [1][2] during the last meeting.

In this contribution, we are going to provide our view on the use of those two values.

Discussion

Indeed, SRS-SpatialRelationInfo is introduced to aid the UE to determine its analog beam via DL-UL beam correspondence or UL-UL beam indication, while SRS-AssocCSIRS is mainly used for UE to derive the digital precoder. Usually the functions may overlap, and sometimes single SRS-AssocCSIRS is sufficient to derive both analog beam and digital precoder, rendering duplicated configuration. However, under some circumstances, it may not be true.

Aperiodic SRS associated with aperiodic CSI-RS

There are two mechanisms to trigger an AP CSI-RS. One is based on AP CSI report triggering, and the other is AP SRS triggering.

For AP CSI-RS reporting triggering, we agreed that the QCL information for an AP-CSI-RS is provided per triggering state [3]. It means that an AP CSI-RS resource (after RRC selection of a CSI-RS resource set per triggering state using a bitmap) can have different QCL assumption on different triggering states.

Agreement: - AP-CSI-RS – - Through DCI (AP-CSI-report-triggering state indication) - For each AP-CSI-RS resource associated with each triggering state, QCL configuration is provided through an association with one of the M candidate TCI states by RRC - FFS: Value of M - FFS: TCI association on NZP-CSI-RS/ZP-CSI-RS based IMR

Therefore, when the CSI-RS resource is triggered by an AP SRS, its QCL assumption is unclear, and more agreement is needed on how the QCL relation is assumed for the AP CSI-RS resource.

Reusing SRS-SpatialRelationInfo to convey the QCL assumption of AP-CSI-RS

One natural way is to reuse SRS-SpatialRelationInfo, since it can provide a DL RS for UE to determine the TX beam of SRS. If the DL RS is SS/PBCH block, periodic CSI-RS, or semi-persistent CSI-RS, whose large scale parameter is known to the UE, the AP CSI-RS can be assumed QCLed with the DL RS with respect to average delay, delay spread, Doppler shift, Doppler spread, and spatial Rx parameters.

When SRS-SpatialRelationInfo is another SRS, further reference is needed recursively, and sometimes it may end up with an SRS without configured SRS-SpatialRelationInfo, and we can leave it up to gNB to avoid such occurrence.

On the other hand, if SRS is associated with a periodic CSI-RS or semi-persistent CSI-RS, whose TCI state is already explicitly configured, gNB should ensure that QCL relationship derived from SRS-SpatialRelationInfo shall be consistent with the TCI configuration.

Proposal: Support reusing SRS-SpatialRelationInfo to derive the QCL assumption of AP CSI-RS associated with the AP SRS configured for non-codebook based UL transmission, where the AP CSI-RS can be assumed QCLed with the DL RS indicated in SRS-SpatialRelationInfo with respect to average delay, delay spread, Doppler shift, Doppler spread, and spatial Rx parameters.

To offer sufficient flexibility, explicit configuration of TCI states of AP CSI-RS associated with an AP SRS used for non-codebook based UL transmission per SRS triggering state can also be considered.

Conclusion

In this contribution, we discussed the QCL configuration of AP CSI-RS associated with an AP SRS used for non-codebook based UL transmission, which is unclear in the current specification. Based on the discussion, we have the following proposal.

Proposal: Support reusing SRS-SpatialRelationInfo to derive the QCL assumption of AP CSI-RS associated with the AP SRS configured for non-codebook based UL transmission, where the AP CSI-RS can be assumed QCLed with the DL RS indicated in SRS-SpatialRelationInfo with respect to average delay, delay spread, Doppler shift, Doppler spread, and spatial Rx parameters.

Reference

- 3GPP R1-1800181, “Remaining Issues and Text Proposals on Non-Codebook Based UL Transmission”, Vivo, RAN1 NR AH 1801, Vancouver, Canada, Jan. 2018.

- 3GPP R1-1800691, “Corrections for Non-Codebook Based UL MIMO”, Ericsson, RAN1 NR AH 1801, Vancouver, Canada, Jan. 2018.

- 3GPP RAN1, “Chairman's Notes RAN1 91 final after email approval”, RAN#91, Reno, US, Nov. 2017.