3GPP TSG RAN WG1 Meeting #92 	R1-1801853

Athens, Greece, Feb 26 - March 2, 2018

Agenda item:		6.2.1.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Changes to PDSCH /SPDSCH interaction procedures in 36.213

Document for:		Discussion and Decision

# 1	Introduction

This document summarizes encountered needed changes to SPDSCH and PDSCH interaction in 36.213.

# 2	Change NDI=0 to NDI not toggled

A retransmission is triggered by NDI not toggled and not NDI set to 0. This is to be corrected in 36.213 Sec. 7.1 as noted below:

## 7.1	UE procedure for receiving the physical downlink shared channel

< Unchanged parts are omitted >

For a UE configured with higher layer parameter ShortTTI-Length, the UE is not expected to receive

-	PDSCH data transmissions signalled via PDCCH/SPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI and DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G for a transport block corresponding to a HARQ process with NDI not toggled if the previous PDSCH transmission of the transport block was signalled via PDCCH/EPDCCH with CRC scrambled by the C-RNTI/SPS C-RNTI with DCI format other than DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G when the number of codewords for the previous PDSCH transmissions is two or the transport block size is more than the maximum transport block size supported for slot/subslot-based PDSCH transmission.

< Unchanged parts are omitted >