3GPP TSG RAN WG1 Meeting #92 	R1- 1801858

Athens, Greece, Feb 26 - March 2, 2018

Agenda item:		6.2.1.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Correction of DM-RS position indication in DCI Formats scheduling subslot PDSCH (36.212)

Document for:		Discussion and Decision

# 1	Introduction

In this document, we discuss the needed changes to the PDSCH DM-RS indication with respect to the applicable DCI formats.

# 2	Reason for Change/Motivation

| Reason for change: | - The DM-RS position indication should be included in DCI Formats 7-1F and 7-1G scheduling TM9 and TM10 DM-RS based subslot PDSCH (but not CRS based PDSCH) |  |  |  |
| --- | --- | --- | --- | --- |
| Summary of change: | - Remove the DM-RS position indication from Format 7-1A (which is used as baseline for all PDSCH DCI formats) and include it only in DCI formats 7-1F and 7-1G. |  |  |  |
| Consequences if not approved: | Unnecessary inclusion of DM-RS position indication in the DCI scheduling DL TM1, 2, 4, 6 for subslot TTI |  |  |  |
| Clauses affected: | 5.3.3.1.17, 5.3.3.1.22, 5.3.3.1.22 |  |  |  |
|  | Y | N |  |  |
| Other specs |  | X | Other core specifications | TS/TR ... CR ... |
| affected: |  | x | Test specifications | TS/TR ... CR ... |
| (show related CRs) |  | x | O&M Specifications | TS/TR ... CR ... |
| Other comments: |  |  |  |  |

| Clauses affected: | 36.212 Sec. 5.3.3.1.17, 5.3.3.1.22, 5.3.3.1.22 |
| --- | --- |

# 3	TP / Pseudo draft CR to 36.212

< Unchanged parts are omitted >

##### 5.3.3.1.17	Format 7-1A

DCI format 7-1A is used for the scheduling of one PDSCH codeword with slot or subslot duration in one cell.

The following information is transmitted by means of the DCI format 7-1A:

-	Flag for DL/UL differentiation – 1 bit, where value 0 indicates format 7-0A/B depending on the configured uplink transmission mode and value 1 indicates the DL DCI format

-	For resource allocation type 0 as defined in section 7.1.6.1 of [3]:

-	bits provide the resource allocation

where the value of P depends on the number of DL resource blocks as indicated in section 7.1.6.1 of [3]

-	For resource allocation type 2 as defined in section 7.1.6.3 of [3]:

-	bits provide the resource allocation

-	one additional bit if

where  is defined in section 7.1.6.3 of [3]

-	Modulation and coding scheme – 5 bits as defined in section 7.1.7 of [3]

-	HARQ process number – 4 bits

-	New data indicator – 1 bit

-	Redundancy version – 2 bits

-	TPC command for slot-SPUCCH or subslot-SPUCCH – 2 bits as defined in section 5.1.2.1 of [3]

-

-	Downlink Assignment Index – number of bits as specified in Table 5.3.3.1.2-2

-	SRS request – 0 or 1 bit as defined in section 8.2 of [3] (this field is present only for TDD operation and the UE is configured for SRS triggering)

-	Used/Unused SPDCCH resource indication – 2 bits (This field is present if the DCI is mapped to SPDCCH and L1-based reuse indication is configured by higher layers)

-	SPUCCH resource indication – 2 bit as defined in section 10.1.2 of [3].

If the UE is configured to decode SPDCCH CRC scrambled by the C-RNTI and the number of information bits in format 7-1A mapped onto a given search space is less than that of format 7-0A/B depending on the configured uplink transmission mode for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 7-1A until the payload size equals that of format 7-0A/B, except when format 7-1A assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

< Unchanged parts are omitted >

##### 5.3.3.1.22	Format 7-1F

DCI format 7-1F is used for the scheduling of one PDSCH codeword with slot or subslot duration in one cell.

The following information is transmitted by means of the DCI format 7-1F:

-	The fields of format 7-1A

-	DMRS position indicator – 1 bit as defined in section 6.10.3.2 in [2] (The field is present only if UE is configured with dl-TTI-Length=subslot)

-	Antenna port(s), scrambling identity and number of layers – 3 bits as specified in Table 5.3.3.1.22-1 where nSCID is the scrambling identity for antenna ports 7 and 8 defined in section 6.10.3.1 of [2], or 1 bit as specified in Table 5.3.3.1.5C-6 where nSCID is the scrambling identity for antenna ports 7 and 8 defined in section 6.10.3.1 of [2] when higher layer parameter semiOpenLoop is configured.

When higher layer parameter semiOpenLoop is configured, antenna ports 7 and 8 are used for transmit diversity.

Table 5.3.3.1.22-1: Antenna port(s), scrambling identity and number of layers indication

| Value | Message |
| --- | --- |
| 0 | 1 layer, port 7, nSCID=0 |
| 1 | 1 layer, port 7, nSCID=1 |
| 2 | 1 layer, port 8, nSCID=0 |
| 3 | 1 layer, port 8, nSCID=1 |
| 4 | 2 layers, ports 7-8, nSCID=0 |
| 5 | 2 layers, ports 7-8, nSCID=1 |
| 6 | 3 layers, ports 7-9 |
| 7 | 4 layers, ports 7-10 |

If the UE is configured to decode SPDCCH CRC scrambled by the C-RNTI and the number of information bits in format 7-1F mapped onto a given search space is less than that of format 7-0A/B depending on the configured uplink transmission mode for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 7-1F until the payload size equals that of format 7-0A/B, except when format 7-1F assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

##### 5.3.3.1.23	Format 7-1G

DCI format 7-1G is used for the scheduling of one PDSCH codeword with slot or subslot duration in one cell.

The following information is transmitted by means of the DCI format 7-1G:

-	The fields of format 7-1A

-	DMRS position indicator – 1 bit as defined in section 6.10.3.2 in [2] (The field is present only if UE is configured with dl-TTI-Length=subslot)

-	Antenna port(s), scrambling identity and number of layers – 3 bits as specified in Table 5.3.3.1.22-1 where nSCID is the scrambling identity for antenna ports 7 and 8 defined in section 6.10.3.1 of [2], or 1 bit as specified in Table 5.3.3.1.5C-6 where nSCID is the scrambling identity for antenna ports 7 and 8 defined in section 6.10.3.1 of [2] when higher layer parameter semiOpenLoop is configured.

-	PDSCH RE Mapping and Quasi-Co-Location Indicator – 2 bits as defined in sections 7.1.9 and 7.1.10 of [3]

When higher layer parameter semiOpenLoop is configured, antenna ports 7 and 8 are used for transmit diversity.

If the UE is configured to decode SPDCCH CRC scrambled by the C-RNTI and the number of information bits in format 7-1G mapped onto a given search space is less than that of format 7-0A/B depending on the configured uplink transmission mode for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 7-1G until the payload size equals that of format 7-0A/B, except when format 7-1G assigns downlink resource on a secondary cell without an uplink configuration associated with the secondary cell.

< Unchanged parts are omitted >