3GPP TSG RAN WG1 Meeting #92			R1-1801936

Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	6.2.6.5.2

Source:	Samsung

Title:	Range enhancment for NPRACH

Document for:	Discussion and Decision

# 1. Introduction

To support up to 100km cell range, NPRACH will be further enhanced in Rel-15 NB-IoT. The agreements have been made in 3GPP RAN1 Meeting #89 as follows,

Agreements:

- To support of cell range of at least 100 km, FFS between:

    - Cat 1: Rel-13 NPRACH

    - Cat 2: Sharing the same NPRACH resources as Rel-13 NPRACH formats, with symbol or symbol-group level scrambling; maintaining feasibility of FFT processing and orthogonality of preambles on different tones

    - CP length FFS between same as or longer than Rel-13 formats

    - Cat 3: New NPRACH numerology with CP length FFS between same as or longer than Rel-13 formats

Option A: 1.25 kHz subcarrier spacing with minimum hop distance of 1.25 kHz

Option B: 3.75 kHz subcarrier spacing with minimum hop distance 1.25 kHz

Option C: 3.75 kHz subcarrier spacing with minimum hop distance 3.75 kHz with new hopping pattern

    - Combinations of Category 2 and Category 3 solutions are not precluded

This contribution is to discuss the solutions on the table for the NPRACH enhancement.

# 3. Cell range enhancement on NPRACH

In Rel-13/Rel-14 NB-IoT, the NPRACH has two formats with two different lengths of CP, 66.7 us and 266.7 us, which support 10km cell radius and 40km radius, respectively. Rel-15 NB-IoT will extend the supported cell radius to 100 km [2]. In the latest #89 meeting, three candidate solutions have been proposed for this purpose.

The first solution is to keep Rel-13 NPRACH, as in Cat 1. Since NPRACH uses all ”1” sequence as preamble, the front symbol can serve as CP of the back symbol, which creates longer CP for larger cell. To use one unified NPRACH format for two cell radiuses, it needs eNB to determine the cell range by blind detections, according to different hypotheses with two different CP lengths. More repetitions will be needed to support larger cell, since some of NPRACH symbols are used as CP. However, it is impossible to configure different repetition numbers for two cell radiuses, which share the same NPRACH format with same configuration parameter. It comes to a dilemma: if eNB configures repetition number by assuming 100 km cell, it will cause unnecessary power consumption at UE for 40km cell; if eNB configures repetition number by assuming 40 km cell, it will lead to unsatisfied performance for 100 km cell.

Observation 1: With Cat 1, the required repetition numbers may be different when the same NPRACH format is used to support both 10 km/40 km and 100 km cell. The configuration of repetition number faces with a dilemma.

The second solution is to introduce scrambling on top of the Rel-13 NPRACH, as in Cat 2. In legacy NB-IoT, the preamble is all “1” sequence. The TA estimation is based on the phase difference between symbol groups, which is created by frequency hopping. The maximum value of estimated TA depends on the minimum hopping distance 3.75 kHz, which is no more than 266.7 us, corresponding to 40km cell radius. With scrambling on top of Rel-13 NPRACH, the sliding correlation with scrambling sequence helps to support the estimation of TA more than 266.7 us.

The scrambling sequence could be either symbol level or symbol group level. As long as the scrambling sequences are orthogonal with all “1” sequence, the NPRACH format for larger cell range can share the same resources with Rel-13 NPRACH formats. Saving physical resources is one of the advantages of this solution.

Note that the same scrambling method is also a candidate solution for false alarm enhancement. They can share the same design principle of scrambling sequence. It may prefer one unified solution for both the purposes of NPRACH enhancement, to reduce the specification impact. Cat 2 can be a decent choice.

Observation 2: Cat 2 can be a decent choice to use one same solution for both the purposes of NPRACH enhancement.

The third solution is to introduce new NPRACH numerology. Three detailed options are considered. As analyzed before, the maximum value of estimated TA, or saying the maximum cell range, is limited by the minimum hopping distance. Without scrambling, the minimum hopping distance should be no more than 1.25 kHz to support 100 km cell. It implies that option A and option B are valid solutions, while option C may not work if not combined with Cat 2. To compare option A with option B, the main difference is subcarrier spacing. The smaller subcarrier spacing of Option A can support larger coverage, which may suit for the extended cell range.

Observation 3: With option A of Cat 3, hopping distance of 1.25kHz is small enough to support TA estimation in a 100-km cell, and smaller subcarrier spacing can support larger coverage required by extended cell range.

Proposal: Cat 3 option A is preferred for cell range enhancement on NPRACH, and Cat 2 can be another choice when considering unified solution for both purposes of NPRACH enhancement.

# 4. Conclusion

Based analysis above, we have the following observations and proposal,

Observation 1: With Cat 1, the required repetition numbers may be different when the same NPRACH format is used to support both 10 km/40 km and 100 km cell. The configuration of repetition number faces with dilemma.

Observation 2: Cat 2 can be a decent choice to use one same solution for both the purposes of NPRACH enhancement.

Observation 3: With option A of Cat 3, hopping distance of 1.25kHz is small enough to support TA estimation in a 100-km cell, and smaller subcarrier spacing can support larger coverage required by extended cell range.

Proposal: Cat 3 option A is preferred for cell range enhancement on NPRACH, and Cat 2 can be another choice when considering unified solution for both purposes of NPRACH enhancement.

# Reference

[1] Chairman’s Notes RAN1 #89, May 2017.

[2] RP-170852,”New WID on Further NB-IoT enhancements,” 3GPP TSG RAN Meeting #75, Mar. 2017.