3GPP TSG RAN WG1 Meeting #92	R1-1801948

# Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	7.1.1.2.3

Source:	Samsung

Title:	Correction on other system information delivery

Document for:	Discussion and decision

- Introduction

This contribution discusses the remaining issues on OSI: The compact DCI is proposed which can be applicable for common control signaling in general. It is updated from R1-1800415.

- Compact DCI Format Design

During initial access, UE will monitor DCI format 1_0 having format size around 60 bits (range of 62 ~ 69 bits) including CRC in the CSS to receive SI, RA messages (i.e., RAR, msg.3 reTx and msg.4), or paging. The CORESET for broadcast OSI and paging reuses the same configuration for RMSI CORESET as indicated in PBCH. The CORESET for RAR is configured via RACH configuration where the default value is the same as RMSI CORESET. If USS is also configured in the RMSI CORESET, the chance to transmit UE-specific DCI at the same time with common DCI will be very limited. Therefore, introducing a compact DCI may resolve the problem caused by large size of the current DCI format 1_0. Most fields of DCI format 1_0 are not applicable during initial access, e.g., VRB-to-PRB mapping, HARQ process number, Downlink assignment index, PUCCH resource indicator, etc. Using compact DCI gives enhanced coverage, flexibility in RMSI CORESET configuration, and more efficient utilization of the RMSI CORESET resources. Our view on DCI fields for compact DCI is summarized in Table 1. The expected DCI size including CRC is around 40 bits including CRC. More details on compact DCI can be found in [1].

Table 1. Compact DCI

| Field for Format | Size (bits) |
| --- | --- |
| Identifier for DCI formats | 1 |
| Frequency domain resource assignment |  |
| Time domain resource assignment | ≤2 |
| Modulation and coding scheme | 5 |
| Total payload size | 17 ~ 21 |
| After CRC attachement | 41 ~ 45 |

Proposal: Support a compact DCI format for broadcast OSI with the contents in Table I as a baseline.

- Conclusion

Proposal: Support a compact DCI format for broadcast OSI with the contents in Table I as a baseline.

Table 1. Compact DCI

| Field for Format | Size (bits) |
| --- | --- |
| Identifier for DCI formats | 1 |
| Frequency domain resource assignment |  |
| Time domain resource assignment | ≤2 |
| Modulation and coding scheme | 5 |
| Total payload size | 17 ~ 21 |
| After CRC attachement | 41 ~ 45 |

Reference

[1] R1-1801976, DCI Contents and Formats, Samsung