3GPP TSG RAN WG1 Meeting #92			R1-1801955

Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	7.1.2.1.1

Source: 	Samsung

Title: 	Summary of issues on codeword mapping

Document for:	Discussion and Decision

# Issues on codeword mapping and proposed resolutions

In this contribution, a summary of issues on codeword mapping raised in the seven submitted contributions for RAN1 NR-AH 1801 is given ([1]-[7]). The issues are of three kinds and outlined in Table 1, 2, and 3:

- Specification refinement: e.g. clarification

- Resolving open issues: e.g. missing components in the specification

- New proposal: e.g. amending an agreement

Table 1 Specification refinement

|  | Issue | Summary of proposal | Companies |
| --- | --- | --- | --- |
| 1.1 | Scrambling sequence initialization | For nID, whether to use the configured value. It is proposed that this is applicable only for some RNTIs for DL and when the UE ID is known to the network for UL. | Ericsson |

The proposal for resolving issue 1.1 and 1.3 is intended to avoid ambiguities in nID. At this point, whether this is an issue or not needs to be discussed.

Therefore, the following resolution is possible.

Proposal:

- Issue 1.1: discuss further whether specification refinement is needed for nID

Table 2 Resolving open issues

|  | Issue | Summary of proposal | Companies |
| --- | --- | --- | --- |
| 2.1 | Ordering of DMRS ports | With one-CW transmission (DL), DMTS port ordering is performed as follows. - Alt 2.1.1. Simple ordering with increasing index (port number) - Alt 2.1.2. To accommodate multi-TRP/panel transmission with 1 CW, ordering follows DMRS port grouping. New table seems to be needed. - Proposals in this category exhibit some variation - Alt 2.1.3. To accommodate multi-TRP/panel transmission with 1 CW, ordering follows DMRS port grouping. Ordering principle can be pre-defined outside the DMRS table without changing current table (as in LTE). | Alt 2.1.1. vivo Alt 2.1.2. ZTE/Sanechips, CATT, LGE, AT&T Alt 2.1.3. Huawei/HiSi |
| 2.2 | TB to CW mapping when 1 TB is disabled | Two views: - Alt 2.2.1 Use the same approach as LTE in 7.3.1.2.2 of TS38.212 (always use CW0), reflect this in 5.1.3.2 of TS38.214 - Alt 2.2.2. Not needed | Alt 2.2.1 Ericsson, ZTE/Sanechips, Sharp, LGE Alt 2.2.2 vivo, AT&T |
| 2.3 | VRB-to-PRB interleaving | Two alternatives - Alt 2.3.1. Layer-specific VRB-to-PRB interleaving - Alt 2.3.2. Layer-common VRB-to-PRB interleaving | Alt 2.3.1 Intel, AT&T Alt 2.3.2 Samsung, Qualcomm, Huawei/HiSi, MediaTek, LGE, ZTE/Sanechips |

For issue 2.1, the proposal in Alt 2.1.2 is intended to follow up on a previous agreement on DMRS port grouping within one CW for rank <=4 transmission – primarily intended for multi-TRP/panel applications. On the other hand, since multi-TRP/panel optimization is not a high priority for Phase I, the urgency/necessity of this type of proposal is unclear. This view is also expressed in [6].

For issue 2.2, reusing the LTE scheme seems reasonable as there is no technical reason for adopting a more complex solution (e.g. switching TB ordering). TB-to-CW mapping is needed to map a logical channel entity to physical channel.

For issue 2.3, it should be noted that Alt 2.3.2 (layer-common VRB-to-PRB mapping) is more consistent with the agreed layer mapping scheme where layer-first CW-to-layer mapping is performed. Some link-level performance comparison demonstrating some link-level gain of Alt 2.3.1 over 2.3.2 is given in [4]. At the same time, Alt 2.3.1 evidently introduces frequency-first mapping operation (followed by time, then layer) which deviates from at least one of the original intents of the layer-first mapping. It is expected that Alt 2.3.1 requires additional UE complexity such as buffering.

Therefore, the following resolutions are possible.

Proposal:

- Issue 2.1: Discuss further, comparing Alt 2.1.1, 2.1.2 and 2.1.3.

  - The baseline should be Alt 2.1.1

- Issue 2.2: Agree on the proposal in [4] (LTE approach)

- Issue 2.3: Discuss further, comparing Alt 2.3.1 and 2.3.2

  - The baseline should be Alt 2.3.2

Table 3 New proposals

|  | Issue | Summary of proposal | Companies |
| --- | --- | --- | --- |
| 3.1 | Layer-specific bundle size | -- | AT&T |

Issue 3.1 argues that different layers may experience different precoding which can alter the frequency selective of the effective channel. However, since layer-common is already in place and working, this proposal is simply optimization.

Therefore, the following resolution is possible.

Proposal:

- Issues 3.1: Keep the specification on bundle size as they are

# Suggested TP

Section 7.3.1.2.2 of TS38.212

…

- DMRS sequence initialization – 1 bit if transform precoding is enabled

In case the higher layer parameter Number-MCS-HARQ-DL-DCI indicates that two codeword transmission is enabled and if both transport blocks are enabled, transport block 1 is mapped to codeword 0 and transport block 2 is mapped to codeword 1. In case one of the transport blocks is disabled as specified in section 5.1.3.2 of [TS38.214], the transport block to codeword mapping is specified according to Table X1

Table X1: Transport block to codeword mapping
(one transport block enabled).

| transport block 1 | transport block 2 | codeword 0 (enabled) | codeword 1 (disabled) |
| --- | --- | --- | --- |
| enabled | disabled | transport block 1 | - |
| disabled | enabled | transport block 2 | - |

Section 5.1.3.2 of TS38.214

In case the higher layer parameter Number-MCS-HARQ-DL-DCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if  and if rvid=1 for the corresponding transport block, otherwise the transport block is enabled.

For the PDSCH assigned by a PDCCH with DCI format 1_0/1_1 with CRC scrambled by C-RNTI,

if the higher layer parameter MCS-Table-PDSCH is set to '256QAM' is configured and  0 ≤ IMCS ≤27, or the higher layer parameter MCS-Table-PDSCH is not set to '256QAM' configured and  0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

…

…

…

# References

- R1-1801514, Remaining issues on codeword mapping, vivo

- R1-1801576, Remaining details on CW to layer mapping, ZTE, Sanechips

- R1-1801715, Remaining issues on codeword mapping, CATT

- R1-1802390, On VRB to PRB interleaving, Intel Corporation

- R1-1802590, On MIMO Symbol Level Interleaver for PDSCH/PUSCH with CP-OFDM, AT&T

- R1-1802662, Open issues on codeword mapping, Sharp

- R1-1802736, Correction on CW mapping and data scrambling, Ericsson

- 3GPP, TS38.211 V15.0.0

- 3GPP, TS38.212 V15.0.0

- 3GPP, TS38.213 V15.0.0

- 3GPP, TS38.214 V15.0.0

- 3GPP, TS38.331 V15.0.0