3GPP TSG RAN WG1 Meeting #92	R1-1802045

Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	7.1.3.3.4

Source: 	CMCC

Title: 	Discussion on remaining issue for SR and grant-free transmission

Document for:	Discussion and Decision

- Introduction

In the previous RAN1 meetings, the following agreements for NR TDD UL/DL configuration have been made:

Agreements:

- For the cell-specific RRC configuration of the semi-static DL/UL assignment,

  - Add additional periodicity 0.625ms (for 120KHz SCS only), 1.25ms (for >=60KHz SCS), and 2.5ms (for >=30KHz SCS)

  - Also support 2 concatenated DL-unknown-UL periodicity

    - Add 1 bit in semi-static DL/UL assignment to indicate if the second periodicity is included

    - The two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms

      - When two periodicities are included, the corresponding parameters are independently configured.

      - Note: it will be discussed to preclude some combinations (no additional higher-layer impact)

In addition, the SR configuration and the allowed periodicities for uplink transmission without grant are described in TS 38.213 and TS 38.214 respectively as follows:

Table 6.1.2.3-1: Allowed periodicities P for uplink transmission without grant

|  | CP | Possible values of periodicities P [symbols] |
| --- | --- | --- |
| 0 | Normal | 2, 7, n*14, where n={1, 2, 5, 10, 20, 32, 40, 64, 80, 128, 160, 320, 640} |
| 1 | Normal | 2, 7, n*14, where n={1, 2, 4, 10, 20, 40, 64, 80, 128, 160, 256, 320, 640, 1280} |
| 2 | Normal | 2, 7, n*14, where n={1, 2, 4, 8, 20, 40, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} |
| 2 | Extended | 2, 6, n*12, where n={1, 2, 4, 8, 20, 40, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} |
| 3 | Normal | 2, 7, n*14, where n={1, 2, 4, 8, 16, 40, 80, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} |

Table 9.2.4: UE-specific SR periodicity and offset configuration

| UE-specific SR periodicity and offset configuration (in symbols) |  |  |  |
| --- | --- | --- | --- |
|  | CP |  |  |
| 0 | Normal |  | 2, 7, n*14, where n={1, 2, 5, 10, 20, 32, 40, 64, 80} |
| 1 | Normal |  | 2, 7, n*14, where n={1, 2, 5, 10, 20, 32, 40, 64, 80, 160} |
| 2 | Normal |  | 2, 7, n*14, where n={1, 2, 5, 10, 20, 32, 40, 64, 80, 160, 320} |
| 2 | Extended |  | 2, 6, n*12, where n={1, 2, 5, 10, 20, 32, 40, 64, 80, 160, 320} |
| 3 | Normal |  | 2, 7, n*14, where n={1, 2, 5, 10, 20, 32, 40, 64, 80, 160, 320, 640} |

In this contribution, the remaining issues of SR and grant-free transmission are discussed.

- Discussion on SR and grant-free transmission configuration

Based on the agreement on semi-static DL/UL assignment, NR supports flexible UL/DL switching periodicity, such as 0.5ms, 0.625ms, 1ms, 2ms, 2.5ms, 5ms, 10ms. Therefore, two types of periodicity in terms of number of slots are supported: Type 1 is periodicity set with number of slots to be power of 2, i.e., {1,2,4,8,16…}; Type 2 is periodicity set with number of slots to be multiple of 5, i.e., {5,10,20,40…}. The semi-static transmission configurations, such as SR, grant-free transmission, etc, need to be able to fit into different periodicities sets, so as to allow efficient operation under different values of semi-static DL/UL periodicity. However, by going through the current specification, it is observed that some transmission configurations are missed for some semi-static DL/UL periodicities. For example, UE-specific SR periodicity misses most of type 1 periodicities i.e. {4,8,16}*14, meanwhile grant-free transmissions periodicity misses {4,8,16} for 15kHz SCS and {5}*14,{8,16,32}*14 for 30kHz SCS {5,16,32,64}*14 for 60kHz SCS, {5,10,20,32,64,128}*14 for 120kHz SCS. Therefore, it is recommended to have additional values {4,8,16}*14 to be included in the SR configuration table. Similar enhancements should be done to grant-free configuration.

Table I: supported DL/UL periodicity and reference SCSs for semi-static DL/UL assignment

|  | 0.5ms | 0.625ms | 1ms | 1.25ms | 2ms | 2.5ms | 5ms | 10ms |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | N/A | N/A | 1 | N/A | 2 | N/A | 5 | 10 |
| 30kHz | 1 | N/A | 2 | N/A | 4 | 5 | 10 | 20 |
| 60kHz | 2 | N/A | 4 | 5 | 8 | 10 | 20 | 40 |
| 120kHz | 4 | 5 | 8 | 10 | 16 | 20 | 40 | 80 |

Proposal 1: The SR configuration should additionally include the value set of {4,8,16}*14.

Proposal 2: Allowed periodicities P for uplink transmission without grant should additionally include the value set of {4,8,16}*14 for 15kHz SCS, {5, 8,16, 32}*14 for 30kHz SCS, {5,16,32,64}*14 for 60kHz SCS, {5,10,20,32,64,128}*14 for 120kHz SCS.

- Discussion on the alignment of 2 concatenated DL-unknown-UL periodicity

In RAN1 #91 meeting, it was agreed that two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10}ms. However, the 2 concatenated DL-unknown-UL periodicity may not align to the periodicities set with number of slots to be power of 2, i.e., {1,2,4,8,16} or the periodicities set with number of slots to be multiple of 5, i.e., {5,10,20,40}. As shown in Figure. 1, when concatenated DL-unknown-UL periodicity is configured as “2.5ms+1ms”, the 2.5ms periodicity includes 5 slots and the 1ms periodicity includes 2 slots (30kHz SCS). If SR transmits at slot #3 and the periodicity is 5 slots, then SR transmission opportunity will be missed at slot #8, because it is a DL slot, this may cause additional and instable SR delay. Therefore, the misalignment of 2 concatenated DL-unknown-UL periodicity should be solved.

Figure 1 The misalignment of 2 concatenated DL-unknown-UL periodicity

- SR configuration

If 2 concatenated DL-unknown-UL periodicities are configured, at most 2 SR periodicities as well as 2 SR offsets associated to either or both of 2 concatenated DL-unknown-UL periodicities can be configured to UE respectively as ( TSR-X ,Toffset-X )and/or (TSR-Y  Toffset-Y ). The UE behaviour towards receiving one or both of ( TSR-X ,Toffset-X )and (TSR-Y , Toffset-Y ) can be follows:

- If TSR-X or TSR-Y contains more slots than the associated DL-unknown-UL periodicity:

  - if ( TSR-X ,Toffset-X ) are configured, assuming that period X contains NX slots, period Y contains NY slots,TSR-X contains N slots, the equivalent periodicity can be calculated as TSR-equal=NNX(NX+NY), the equivalent offset can be calculated as Toffset-equal=NNX×NY+Toffset-X.

  - if (TSR-Y  Toffset-Y ) are configured, assuming that period X contains NX slots, Y contains NY slots, TSR-Y contains N slots, the equivalent periodicity can be calculated as TSR-equal=NNY(NX+NY), the equivalent offset can be calculated as Toffset-equal=(NNY+1)×NX+Toffset-Y.

- If TSR-X or TSR-Y contains less slots than the DL-unknown-UL periodicity, or the SR periodicity as well as SR offset are associated to neither of 2 concatenated DL-unknown-UL periodicities, the equivalent periodicity and equivalent offset are equal to the configured SR periodicity and SR offset respectively.

- Grant-free transmission

If 2 concatenated DL-unknown-UL periodicity is configured, at most 2 grant-free transmission periodicities associated to either or both of 2 concatenated DL-unknown-UL periodicities can be configured to UE respectively as PX and/or PY. The UE behaviour towards receiving one or both of PX and PY can be follows:

- If PX or PY contains more slots than the DL-unknown-UL periodicity, UE can judge the starting slot belongs to period X or period Y with parameter timeDomainOffset.

  - If belonging to X, assuming that period X contains NX slots, period Y contains NY slots, PX contains N slots, the equivalent periodicity can be calculated as Pequal=NNX(NX+NY);

  - If belonging to Y, assuming that period X contains NX slots, period Y contains NY slots, PY contains N slots, the equivalent periodicity can be calculated as Pequal=NNY(NX+NY);

- If PX and PY contains less slots than the DL-unknown-UL periodicity, or the grant-free transmission periodicity is associated to neither of 2 concatenated DL-unknown-UL periodicities, the equivalent periodicity is equal to the configured grant-free transmission periodicity.

Proposal 3: For SR transmission, if 2 concatenated DL-unknown-UL periodicities are configured, at most 2 SR periodicities as well as 2 SR offsets associated to either or both of 2 concatenated DL-unknown-UL periodicities can be configured to UE. An equivalent periodicity and offset can be calculated based on higher layer configured periodicities and offsets.

Proposal 4: For grant-free transmission, if 2 concatenated DL-unknown-UL periodicity is configured, at most 2 grant-free transmission periodicities associated to either or both of 2 concatenated DL-unknown-UL periodicities can be configured to UE, an equivalent periodicity can be calculated based on higher layer configured periodicities associated to either or both of 2 concatenated DL-unknown-UL periodicities.

- Conclusions

In this contribution, we discussed the remaining issues for SR and grant-free configurations and the following proposals are made:

Proposal 1: The SR configuration should additionally include the value set of {4,8,16}*14.

Proposal 2: Allowed periodicities P for uplink transmission without grant should additionally include the value set of {4,8,16}*14 for 15kHz SCS, {5, 8,16, 32}*14 for 30kHz SCS, {5,16,32,64}*14 for 60kHz SCS, {5,10,20,32,64,128}*14 for 120kHz SCS.

Proposal 3: For SR transmission, if 2 concatenated DL-unknown-UL periodicities are configured, at most 2 SR periodicities as well as 2 SR offsets associated to either or both of 2 concatenated DL-unknown-UL periodicities can be configured to UE. An equivalent periodicity and offset can be calculated based on higher layer configured periodicities and offsets.

Proposal 4: For grant-free transmission, if 2 concatenated DL-unknown-UL periodicity is configured, at most 2 grant-free transmission periodicities associated to either or both of 2 concatenated DL-unknown-UL periodicities can be configured to UE, an equivalent periodicity can be calculated based on higher layer configured periodicities associated to either or both of 2 concatenated DL-unknown-UL periodicities.

- References

- RAN1 Chairman’s notes, RAN1 meeting #91, December 2017.

- TS 38.213 v15.0.0 NR Physical layer procedures for control (Release 15).

- TS 38.214 v15.0.0 NR Physical layer procedures for data (Release 15).