3GPP TSG RAN WG1 Meeting #92                                            	R1-1802121

Athens, Greece, February 26th – March 2nd, 2018

Source:	OPPO

Title:	Collision randomization for grant free

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In grant free, resources are usually shared by UEs to improve resource utilization. However, due to traffic arrives randomly, UE collision is inevitable. Moreover, URLLC has ultra-high reliability requirement. Even collision with small possibility has strong impact. So collision problem needs to be studied in grant free. Collision randomization is one method, especially for repetition case. In this document, Frequency hopping and DMRS hopping are discussed.

Frequency hopping

In addition, current hopping pattern only depends on two parameters, and, available hopping pattern is very limited and randomness of collision among UE is very limited, even leads 100% collision, which is fatal for URLLC. So more flexible hopping pattern need to be studied. [1] Provides a simple enhancement on current hopping pattern and hopping resource per UE can be configured by Enb.

Proposal1: Hopping resource per UE can be configured by eNB to reduce collision probability.

DMRS hopping

Non-orthogonal resource allocation is usual for URLLC to support more grant free UE in limited resource, however DMRS collision leads UE misrecognition issue and channel estimation error. Therefore, it is necessary to avoid non-orthogonal DMRS interference, especially always-interference for a pair of UEs. UE-specific DMRS hopping is a good choice.

To coordinate hopping pattern among UEs easier, even the initial transmission occurs randomly in time domain, DMRS port for the initial transmission,Portstart ,should depend on time index.

In addition, there are three hopping schemes:

- Simple function with UE-specific parameters

Portn=(Portstart+(n mod N)* Portoffset)modNportsize), n is time index with unit of one transmission. N is available hopping resources. Portstart is starting DMRS port, Portoffset is hopping offset, Nportsize is total number of DMRS port. These parameters except n are all configured by RRC.

- Random function with UE-specific parameters

Random hopping can also solve always-collision issue, but it may lead unnecessary interference in case that orthogonal resource is enough. Therefore, if this scheme is applied, it’s better to configure on-off. When orthogonal resource is enough, fixed DMRS port or fixed hopping pattern is applied. Otherwise, Random hopping is applied.

- Definite UE-specific hopping pattern

Definite UE-specific hopping pattern is the most flexible hopping configuration, which can maximum system efficiency by reasonable configuration.

Proposal2: In order to avoid and reduce always-DMRS collision probability, DMRS hopping should be considered

Conclusions

In this contribution, we show our views on UL transmission without UL grant with following observation and proposals:

Proposal1: Hopping resource per UE can be configured by eNB to reduce collision probability.

Proposal2: In order to avoid and reduce always-DMRS collision probability, DMRS hopping should be considered

References

- R1-1802118 Remaining essential issues on grant free OPPO