3GPP TSG RAN WG1 Meeting #92		R1-1802127

Athens, Greece, February 26th – March 2nd, 2018

Source:	OPPO

Title:	Text Proposal for DL PRB Bundling

Agenda Item:	7.1.2.1.4

Document for:	Discussion and Decision

Motivation

Adapt new parameters for configuration

In RAN1 NR AdHoc meeting in Vancouver [1], an additional parameter PDSCH-bundle-size_2 is introduced to configure the second set of PRG value when dynamic PRB bundling is enabled [2]. In TS 38.331, the parameter pdsch-BundleSize2 is used to denote the above-mentioned parameter. Thus the specification should reflect the above parameter pdsch-BundleSize2 and the corresponding configurations.

Suggestion 1: In Section 5.1.2.3 of TS 38.214, specify that the second set of PRG value can be configured by the parameter pdsch-BundleSize2.

Alignment of RRC parameters and its values

In the list of RRC parameters [2], there are three candidate values {2,4,wideband} for the RRC parameter pdsch-BundleSize . In the current version of 38.214 [3], “scheduled bandwidth” is used as a candidate value of PRG. Thus there is misalignment between TS 38.214 and the RRC parameters in TS 38.331. It is better to align TS 38.214 with the RRC parameter list.

In TS 38.331 [4], the parameter prb-BundlingEnabled , rather than prbBundling, is used to configure whether or not to support dynamic PRB bundling indication.

Suggestion 2: In Section 5.1.2.3 of TS 38.214, change “scheduled bandwidth” to “wideband” to ensure the alignment with the list of RRC parameters.

Suggestion 3: In Section 5.1.2.3 of TS 38.214, change “prbBundling” to “prb-BundlingEnabled” to ensure the alignment with TS 38.331.

In the list of RRC parameters, only three candidate values are provided for pdsch-BundleSize, without any detailed information about configuration. In order to facilitate RAN2 to specify the associated configuration in TS 38.331, RAN1 should provide the corresponding information about how to configure pdsch-BundleSize. Thus we have the following suggestion:

Suggestion 4: RAN1 informs RAN2 that the RRC parameter pdsch-BundleSize can be configured to be one or two values out of {2, 4, wideband}

Text proposal

Based on the above discussion, we make the following text proposal to capture the suggestions and some minor corrections for Section 5.1.2.3 of TS 38.214

A UE may assume that precoding granularity is multiple resource blocks in the frequency domain. Precoding Resource Block Group (PRGs) of size partition the carrier bandwidth part i and each PRG consists of one or multiple consecutive PRBs, PRG value can be one of the values among {2, 4, wideband}. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI ,the PRG for each carrier bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingEnabled and pdsch-BundleSize. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that the PRG is equal to 2 PRBs. If a UE is configured with PRG of "wideband", the UE is not expected to be scheduled with non-contiguous physical resource blocks and the UE may assume that the same precoding is applied to the allocated resource blocks. When the bundling size is configured as one of the values among {2, 4}, the first PRG size is given by and the last PRG size given by if , and the last PRG size is if . The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above. If the higher layer parameter prb-BundlingEnabled is set to 'ON', the higher-layer parameter pdsch-BundleSize and pdsch-BundleSize2 configure two sets of PRG values respectively, where the first set includes one or two PRG values among {2, 4, wideband}, and the second set includes one PRG value. The UE is not expected to be configured with (2, 4) in the first set. If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] - is set to '0', the UE shall use the PRG value from the second set of PRG values when receiving PDSCH scheduled by the same DCI. - is set to '1' and one value is configured for the first set of PRG values, the UE shall use this PRG value when receiving PDSCH scheduled by the same DCI - is set to '1' and two values are configured for the first set of PRG values as (2,wideband) or (4,wideband), the UE shall use the PRG value when receiving PDSCH scheduled by the same DCI as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , the PRG is the same as the scheduled bandwidth, otherwise, the PRG is set to the remaining configured value of 2 or 4, respectively. If the higher layer parameter prb-BundlingEnabled is set to 'OFF', the PRG value is configured with the single value indicated by the higher-layer parameter pdsch-BundleSize. When a UE is configured with RBG = 2 according to Section 5.1.2.2.1, or when a UE is configured with resource block bundle size of 2 for VRB to PRB mapping provided by the higher layer parameter VRB-to-PRB-interleaver, the UE is not expected to be configured with = 4.

References

- RAN1 NR AdHoc 201801 Chairman’s notes

- R1-1801276 RRC Parameter list

- R1-1801294 TS 38.214, Physical Layer Procedures for Data

- TS 38.331 NR  Radio Resource Control (RRC)  v15.0.1