3GPP TSG RAN WG1 Meeting #92					R1-1802167

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	6.2.6.1.1.3

Source: 	LG Electronics

Title: 	Wake up signal design in NB-IoT

Document for:	Discussion and decision

- Introduction

In the last RAN1 meeting, working assumption and agreements were made as follows [1]:

Agreements: - Working assumption: WUS/DTX is adopted for the power saving signal for IDLE mode paging; - The UE is configured with a transmission duration of WUS by higher layers - The WUS signal may be decoded with or without relying on prior synchronization - Whether the UE needs to acquire (further) synchronization using NPSS/NSSS to decode the NPDCCH following the WUS is FFS - The power saving of using existing synchronization signal to achieve sync and using WUS for synchronization should be compared. - For RAN#78 timeline, RAN4 can assume the UE is synchronized prior to the WUS - After RAN#78, RAN1 and RAN4 will conduct additional work to allow the assumption that the UE is not synchronized prior to the WUS (with the same WUS signal design) for 144 dB MCL; and will study to allow the assumption that the UE is not synchronized prior to the WUS (with the same WUS signal design) for 154, 164 dB MCL - Status of work on ‘Relaxed monitoring for cell reselection’ in RAN2/4 should be considered Agreements: - RAN1 assumes that introduction of WUS does not alter PO/PF definition - At least in a UE’s DRX cycle: - WUS supports at least being applied to all the UEs monitoring WUS associated to a PO in a NB-IoT carrier; - FFS: eNB can configure WUS being applied to a group of more than one of the UEs associated to a PO in a NB-IoT carrier Working assumption: - At least in a UE’s DRX cycle, how the UE knows the WUS time location, is: - A WUS has a time location which is configurable with respect to the associated PO(s) location(s) Agreements: - At least in a UE’s DRX cycle, one WUS informs UE whether to monitor the PO in a single DRX cycle Agreements: - WUS signal is at least cell-specific; - FFS scrambling of WUS including time varying scrambling Long ZC sequence based signal is considered as the starting point for WUS signal: Agreement - The [maximum] duration of WUS is configured per NB-IoT carrier - FFS: WUS actual transmission duration can be shorter than the configured maximum duration of WUS. - Alt 1: The actual WUS duration is transmitted aligning to the start of the configured maximum duration of WUS. - Alt 2: The actual WUS duration is transmitted aligning to the end of the configured maximum duration of WUS. - There is a non-zero gap from the end of configured [maximum] WUS duration to the associated PO - FFS: exact value of non-zero-gap - FFS if it is fixed in spec or configurable explicitly, or known implicitly from other configured parameters Agreement - The network can enable or disable use of the WUS - How UE acquires information on WUS enabling/disabling is up to RAN2 decision Agreement - UE can assume all the REs for transmission of WUS in a given subframe use the same antenna port. - The UE shall not assume the transmission of WUS in more than X consecutive subframes use same antenna port. - FFS: value of X Agreement WUS sequence is based on ZC-sequence - When designing WUS sequence, negative impact on legacy NSSS detection should be avoided. Working assumption - WUS sequence is a sequence mapping within one subframe as a basic unit and repeated/extended for multiple subframes to support larger coverage. - Prioritizie to minimize impact on UE synchronization performance. - FFS: detailed design on time-varying of the WUS signal

For the wake up signal design, several aspects should be considered carefully such as power consumption efficiency, complexity of the UE, and information conveyed by the signal. In this contribution, we discuss the several consideration points for the wake up signal and propose the design of it.

- Wake up signal design

NSSS like sequence design

In general, NB-IoT UE should be a low cost and low complexity device. Thus, even if the new feature, the wake up signal, is introduced in Rel-15 NB-IoT, UE complexity and its cost should not be increased too much. However, if we introduce new sequence design for the wake up signal, increasing implementation complexity could not be ignored. Furthermore, if the wake up signal can be used to achieve the DL synchronization, detection complexity can be increased significantly depending on the design. In this perspective, reusing existing receiver algorithm which is already used for another purpose can be a good solution for the wake up signal design. Also, due to the limited time for the Rel-15 specification work, signal design that can minimize the specification work among the RAN groups is preferable.

NPSS and NSSS is a well-known signal in NB-IoT which performance is already verified. Thus reusing existing synchronization signal design can be the good starting point. Especially, NSSS could be a starting point of wake up signal design according to the agreement in a previous meeting; long ZC sequence which spans multiple OFDM symbols.

It is obvious that using same NSSS sequence for wake up signal should be avoided; reusing existing signal can make impact on the legacy DL synchronization performance. Therefore, modified signal design based on the NSSS structure should be considered carefully to avoid the negative impact on DL synchronization performance. In this point of view, we can consider two candidates to differentiate wake up signal from NSSS. One possible method is using cyclic shift value different from NSSS. It is well known that correlation of a ZC sequence with any cyclic shift of it is zero. In NB-IoT, only 4 different cyclic shift values is used for NSSS sequence generation; 0, 1/4, 2/4, and 3/4. Thus, cyclic shift value other than used for NSSS could be used for wake up signal design. Also, RE level differentiation could be considered as well. RE mapping for existing NSSS follows frequency first and time second order. For example, time first then frequency second RE mapping rule can be used for wake up signal generation to avoid miss understanding between NSSS and wake up signal. Figure 1-1 (a) and (b) shows an examples of RE mapping method for existing NSSS and wake up signal respectively. According to the evaluation results in section 3, impact on the DL synchronization performance due to the modified NSSS like signal is negligible both for different cyclic shift values θf={1/8, 3/8, 5/8, and 7/8} and different RE mapping rule. These kinds of adjustment of sequence designs do not change the major features of existing NSSS sequence. Thus it could be expected that properties and performance of the wake up signal are almost same with existing NSSS sequence which is verified well during the Rel-13 NB-IoT meetings. Moreover, according to our simulation results in next section, it can be seen that impact on existing NSSS is very negligible when wake up signal design follows NSSS sequence design with simple adjustments.

Proposal 1: Wake up signal design is based on the NSSS structure, where to avoid the impact on DL synchronization performance,

Alt. 1) wake up signal uses different cyclic shift values from existing NSSS.

(e.g. θf={1/8, 3/8, 5/8, 7/8})

Alt. 2) wake up signal uses different resource mapping rule from existing NSSS.

(e.g. time first then frequency second)

Figure 1-1 Examples of resource element mapping rule

Sequence hopping

In general, sequence hopping on signal could be used for inter cell interference randomization. Likewise, applying sequence hopping on wake up signal could be considered also. In addition to an inter cell interference randomization purpose, sequence hopping could be design in consideration of other aspects.

As discussed in our companion paper, wake up signal may be required conveying information of corresponding PO to avoid misunderstanding from other UE’s wake up signal which overlap with desired wake up signal location. To do so, initial sequence index for sequence hopping or sequence hopping pattern could be decided depends on the information of PO or starting subframe index of wake up signal transmission.

Also, estimating starting subframe number location of wake up signal could be performed with sequence hopping. If UE is not synchronized prior to wake up signal monitoring, timing drift value of the UE could be larger than 1ms. In this case, even the UE which detects the wake up signal may have misunderstanding of subframe index which causes significant impacts on subsequent procedures. Note that, in Rel-14 feNB-IoT, wake up signal may not be used for synchronization purpose, and it could be assumed that UE obtains DL synchronization prior to monitor the wake up signal. However, using wake up signal for synchronization purpose could be discussed further to enhance the power consumption efficiency of UE. Thus, consideration of conveying starting subframe information on wake up signal could be considered for forward compatible point of view.

Proposal 3: Sequence hopping is used for wake up signal.

- Evaluations

In this section, we analyze the performance of the NSSS like wake up signal. We consider evaluation assumptions which is described in [2]. The details of the evaluation assumptions can be found in Appendix. In our evaluation, NSSS like wake up signal designs are considered which are described in previous section. Specifically, we consider three alternatives as follows:

Alt. 1) Use different cyclic shift value from existing NSSS (θf={1/8, 3/8, 5/8, and 7/8})

Alt. 2) Use time first and frequency second RE mapping rule

Alt. 3) Use Gold sequence for cover code (LTE gold sequence)

First, we evaluate the impact on the wake up signal on existing NSSS performance. If wake up signal can be detected during the cell search process using existing NSSS sequence, cell search performance can be degraded due to the miss understanding. So, we evaluated the probability where a UE detects NSSS like wake up signal as existing NSSS during the cell search procedure. As shown in Table 1-1, impacts on existing NSSS procedure by NSSS like wake up signal design candidates are significantly low.

Table 1-1

| Candidates | Detection probability |
| --- | --- |
| Alt.1) Different cyclic shift values | < 10-3 |
| Alt.2) Different RE mapping | < 10-3 |
| Alt.3) Different cover code (Gold seq.) | < 10-3 |

Observation 1: Existing NSSS performance is not degraded due to the NSSS like wake up signal with modified cyclic shift value or RE mapping rule.

For performance comparison among the NSSS like wake up signal design, we evaluate the cross correlation of each sequences. We assume that 2016 sequence can be used for the wake up signal. Note that number of sequence could be smaller than our simulation assumption. For more detail evaluation, we subdivide the sequence candidates as follows:

ZC sequence + Cyclic shift(16)

ZC sequence + Gold sequence(16)

NSSS design with different cyclic shift values

As shown in Table 1-2 and Figure 1-2, sequence candidates have similar cross correlation properties. Strictly speaking, correlation property which reuses NSSS sequence design shows the best correlation performance among candidates.

Table 1-2

| Candidates | Min. Cross correlation | Max. cross correlation |
| --- | --- | --- |
| ZC seq. + cyclic shift | 0.1618 | 0.1770 |
| ZC seq. + Gold seq. | 0.2606 | 0.2727 |
| ZC seq. + Hadamard seq. +cyclic shift | 0.2180 | 0.2707 |

Figure 1-2

Observation 2: Cross correlation property of ZC sequence + Hadamard sequence + Cyclic shift is slightly better than ZC sequence + Gold sequence.

Next, we evaluate PAPR of wake up signal sequence candidates which could be a critical criterion in signal design. Although the wake up signal is DL signal, good PAPR property can be beneficial In Rel-13 NB-IoT, several candidates for NSSS sequence had discussed and PAPR performance of them are already discussed well. Based on the previous discussions [3], it can be seen that current NSSS sequence design, which uses combination of Hadamard sequence and cyclic shift values, shows better PAPR performance than the sequence design which uses random sequence cover code. In Figure 1-3, PAPR of ZC sequence with cyclic shift and ZC sequence with random sequence are compared. As shown in this figure PAPR performance with legacy NSSS design is much better than new sequence design with random sequence. Thus, it could be beneficial to use the combination of Hadamard sequence and cyclic shift values for the wake up signal, which is already verified well in existing NSSS sequence design.

Figure 1-3

To find a required wake up signal duration for NSSS like wake up signal design, we evaluate miss detection performance of the wake up signal itself for different MCL targets. Based on the agreement in a previous meeting, we assume that UE is synchronized prior to wake up signal. Moreover, we assume that a wake-up signal carries one bit information which indicates whether NPDCCH will be transmitted or not. However, it should be noted that wake up signal may require multiple distinguishable sequences if we want to carry more information than that via wake up signal.

To evaluate a false alarm and miss detection rate, we assume that the wake-up signal is transmitted on continuous subframes. The details of simulation assumptions are described in Appendix. Table 1-3 shows the evaluation results of wake-up signal miss detection rates with false alarm rates and the given wake-up signal repetition numbers. As shown in the table, required number of repetition of wake up signal depends on target MCL, target false alarm rate and miss detection rate.

Table 1-3

| 164dB | Repetition number | 32 | 64 | 128 | 256 |
| --- | --- | --- | --- | --- | --- |
|  | Miss detection rate (FAR 1%) | 12.96% | 5.30% | 1.40% | 0.04% |
|  | Miss detection rate (FAR 10%) | 3.08% | 0.66% | 0.08% | 0.02% |
| 154dB | Repetition number | 2 | 4 | 8 | 16 |
|  | Miss detection rate (FAR 1%) | 10.06% | 5.56% | 2.30% | 0.78% |
|  | Miss detection rate (FAR 10%) | 1.42% | 0.48% | 0.30% | 0.16% |
| 144dB | Repetition number | 1 | 2 | 4 | 8 |
|  | Miss detection rate (FAR 1%) | 0.30% | 0.16% | 0.06% | 0.04% |
|  | Miss detection rate (FAR 10%) | 0.04% | 0.00% | 0.00% | 0.00% |

Observation 3: To satisfy the false alarm 1% and miss detection rate under 1%,

256 repetition is required for 164dB MCL target

16 repetition is required for 154dB MCL target

Repetition is not required 144dB MCL target

Observation 4: Relaxing target false alarm rate can reduce the number of repetitions.

- Conclusion

In this contribution, we discussed wake-up signal design and shows evaluation results of it. The proposals of this contribution are summarized as follows.

Proposal 1: Wake up signal design is based on the NSSS structure, where to avoid the impact on DL synchronization performance,

Alt. 1) wake up signal uses different cyclic shift values from existing NSSS.

(e.g. θf={1/8, 3/8, 5/8, 7/8})

Alt. 2) wake up signal uses different resource mapping rule from existing NSSS.

(e.g. time first then frequency second)

Observation 1: Existing NSSS performance is not degraded due to the NSSS like wake up signal with modified cyclic shift value or RE mapping rule.

Observation 2: Cross correlation property of ZC sequence + Hadamard sequence + Cyclic shift is slightly better than ZC sequence + Gold sequence.

Observation 3: To satisfy the false alarm 1% and miss detection rate under 1%,

256 repetition is required for 164dB MCL target

16 repetition is required for 154dB MCL target

Repetition is not required 144dB MCL target

Observation 4: Relaxing target false alarm rate can reduce the number of repetitions.

- Reference

- Chairman’s note, RAN1Meetings

- R1-1714993, “Assumptions for NB-IoT power consumption for power saving signal/channel,” Ericsson

- R1-162460, “NB-SSS design for NB-IoT,” LG Electronics

- Appendix.

Table 5-1

| Parameter | Value |
| --- | --- |
| Operation Mode | In-band mode |
| Antenna Configuration | 2Tx in BS, 1Rx in UE |
| BS Power | 35 dBm |
| System BW | 180 kHz |
| Operating SNR | {-16.6, -6.6, 4.6} dB |
| Couple Loss | {164, 154, 144} dBm |
| Noise Figure | 9 dB |
| Carrier Frequency | 900 MHz |
| Channel Model | TU |
| Doppler Spread | 1 Hz |
| CRS | On |
| NRS | On |

Table 5-2

| Operating mode | Power [units/ms] | Latencies | Value (ms) |
| --- | --- | --- | --- |
| Receive | 100 | Synchronization | {100, 400, 1000} |
| Light sleep | 1 | NPDCCH | {4, 64, 512} |
| Idle, deep sleep | 0.015 | NPDSCH | {4, 64, 512} |