3GPP TSG RAN WG1 Meeting #92           	         R1-1802204

Athens, Greece, February 26th – March 2nd, 2018

Agenda item: 7.1.2.3.7

Source: LG Electronics

Title: Text proposals on QCL configurations for periodic CSI-RS

Document for: Discussion & Decision

- Introduction

In the current version of TS38.214 [1], the cases of aperiodic CSI-RS or semi-persistent CSI-RS as target RS are properly captured in sections 5.2.1.5.1 and 5.2.1.5.2, as highlighted below:

- In section 5.2.1.5.1, “For each aperiodic CSI-RS resource associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Subclause 5.1.5, through higher layer signaling of QCL-Info-AperiodicReportingTrigger which contains a list of references to TCI-RS-SetConfig's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a TCI-RS-SetConfig in the list is configured with a reference to an RS associated with QCL-TypeD, that RS may be an SS/PBCH block or a CSI-RS resource configured as periodic or semi-persistent.” - In section 5.2.1.5.2, “when a UE receives an activation command [10, TS 38.321] for CSI-RS resource(s) for channel measurement and CSI-IM/NZP CSI-RS resource(s) for interference measurement associated with configured CSI resource setting(s) in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions (including quasi co-location assumptions provided by a list of references to TCI-RS-SetConfig’s, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied no later than the minimum requirement defined in [10, TS 38.133].”

However, a similar description for periodic CSI-RS as target RS seems missing in the current version of TS38.214, so that we provide the text proposals in the following section.

- Text proposals on 38.214

5.1.6.1	CSI-RS reception procedure

The CSI-RS defined in Subclause 7.4.1.5 of [4, TS 38.211], may be used for time/frequency tracking, CSI computation and L1-RSRP computation. A UE configured with a CSI-RS resource, when configured with the higher layer parameter ResourceConfigType set to ‘periodic’, the quasi co-location configuration is indicated through higher layer signaling of QCL-InfoPeriodicCSI-RS, where the quasi co-location source(s) can be either SS/PBCH block or another periodic CSI-RS.

The UE may be configured to use the same OFDM symbols for the CSI-RS and CORESET when those are spatially quasi co-located and PRBs associated with CSI-RS are the outside of PRBs configured for CORESET.

---- Unchanged parts omitted ----

5.2.2.3.1	Non-zero power CSI-RS

The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameter NZP-CSI-RS-ResourceSetConfig Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s).

The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via higher layer parameter NZP-CSI-RS-ResourceConfig for each CSI-RS resource configuration:

-	NZP-CSI-RS-ResourceConfigId determines CSI-RS resource configuration identity.

-	NrofPorts defines the number of CSI-RS ports, where the allowable values are given inSubclause 7.4.1.5 of [4, TS 38.211].

-	CSI-RS-timeConfig defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS.

-	CSI-RS-ResourceMapping defines the OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	CSI-RS-Density defines CSI-RS frequency density, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211].

-	CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211].

-	Pc: which is the assumed ratio of PDSCH EPRE to NZPCSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size,

-	Pc_SS: which is the assumed ratio of SS/PBCH block EPRE to NZPCSI-RS EPRE.

-	ScramblingID: defines scrambling ID of CSI-RS with length of 10 bits

-	CC_Info defines which component carrier the configured CSI-RS is located in.

-	CSI-RS-ResourceRep parameter associated with a CSI-RS resource set defines whether a repetition in conjunction with spatial domain transmission filter is ON/OFF at gNB-side as described in Subclause 5.1.6.1.2.

-	QCL-InfoPeriodicCSI-RS parameter for periodic CSI-RS configures a reference to one TCI-RS-SetConfig in TCI-States for providing the QCL source and QCL type, where the source can be SS/PBCH block or another periodic CSI-RS.

The UE may be configured the CSI-RS in same OFDM symbols as SS/PBCH block, but not in the same PRBs, if the CSI-RS and SS/PBCH block are spatial quasi co-located and the higher layer parameter NrofPorts is configured as 1 or 2.

- Reference

[1] 3GPP TS 38.214 Release 15, Feb. 2018.