3GPP TSG RAN WG1 Meeting #92    				R1-1802241

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	6.1.4

Source: 	LG Electronics

Title: 	Discussion on CBSR for advanced CSI

Document for:	Discussion/Decision

- Introduction

In RAN1#90bis, following conclusion and observations regarding codebook subset restriction was captured as

Conclusion:

- Accept codebook subset restriction for Advanced CSI and Hybrid CSI for both eMIMO types under TEI-15

Observations:

- Alt 1: Beam restriction, restricting both strong and weak beams together

- Alt 2: Joint beam/power restriction as in R1-1718918

  - Different views on performance gain from including the per-beam power restriction

  - Some companies have concerns on:

    - complexity of joint beam/power search

    - restriction of flexibility of beam restriction for a given signalling overhead

- Alt 3: Two beam restriction bit maps, independently restricting the strong and weak beams.

  - Concerns on the signalling overhead

Next steps:

- Further evaluate the potential gains of Alt 2 and Alt 3 joint beam/power restriction under the assumption of the same signalling overhead as beam-only restriction.

- Aim to provide simulation results by 3rd November so that further email discussion can take place before RAN1#91.

In this contribution, we provide simulation results for codebook subset restriction for advance CSI for Rel-15 eFD-MIMO.

Simulation assumptions and results

In this section, we provide simulation assumptions to verify the CBSR in advanced CSI. Table I lists antenna setup and CBSR schemes, and other details are in Appendix A.

Table I. Simulation assumption

| Parameter | Value |
| --- | --- |
| Scenario | UMi (200m ISD), UMa(500m ISD) |
| Antenna setup and port layouts | - (M,N,P)=(8,1,2) antenna elements: (N1,N2) = (8,1), with 1x1 virtualization. - (M,N,P)=(8,2,2) antenna elements: (N1,N2) = (4,2), (8,2) with 2x1 and 1x1 virtualization respectively. 1st dimension corresponds to vertical dimension. |
| Schemes | Baseline: No CBSR Scheme 1: DFT beam-based CBSR where beam(s) around 90∘ is (are) removed. Opt 1-1. {vl,m :l=0} are removed, i.e. a single beam is removed from the 1st dim. Opt 1-2. {vl,m :l=0,1 and l=N1O1-1} are removed, i.e. 3 beams are removed from the 1st dim. Opt 2. Beam-based CBSR where beams around 90∘ are power restricted. i.e., {vl,m :l=0} are not allowed in W1. For {vl,m :l=1 and l=N1O1-1 } , the relative power larger than 0.5 is not allowed. |
| Cell association | Cell association pattern is approximated by one-TXRU pattern |

Figure 1. Performance comparison with various CBSR options for (8,1,2,16) with 3D-UMi

Figure 2. Performance comparison with various CBSR options for (4,2,2,16) with 3D-UMi

Figure 3. Performance comparison with various CBSR options for (8,2,2,32) with 3D-UMi

Figure 4. Performance comparison with various CBSR options for (8,1,2,16) with 3D-UMa

Figure 5. Performance comparison with various CBSR options for (4,2,2,16) with 3D-UMa

Figure 6. Performance comparison with various CBSR options for (8,2,2,32) with 3D-UMa

Figure 1-6 exhibit the performance comparison with various CBSR options and various antenna configurations. Based on the above results, following observations are given as:

Observation1. DFT-beam based CBSR applied for advanced CSI provides marginal performance loss or gain over non-CBSR case for both mean and 5% UE UPT.

Observation2. As the restricted beam increases, the corresponding performance is getting worse.

Observation3. By comparing Opt. 1-2 and Opt. 2, it is observed that applying both beam and power coefficient restriction for advanced CSI provides marginal performance gain.

Observation4. As shown in figure 4-6, similar performance trend can be observed in 3D-UMa environment.

Proposal1. If CBSR is supported for advanced CSI in eFD-MIMO, Class A-like CBSR is preferable.

Conclusion

This contribution presented simulation results of CBSR applied for advanced CSI in Rel-14 eFD-MIMO. Following observations and proposal are given, based on the discussion:

Observation1. DFT-beam based CBSR applied for advanced CSI provides marginal performance loss or gain over non-CBSR case for both mean and 5% UE UPT.

Observation2. As the restricted beam increases, the corresponding performance is getting worse.

Observation3. By comparing Opt. 1-2 and Opt. 2, it is observed that applying both beam and power coefficient restriction for advanced CSI provides marginal performance gain.

Observation4. As shown in figure 4-6, similar performance trend can be observed in 3D-UMa environment.

Proposal1. If CBSR is supported for advanced CSI in eFD-MIMO, Class A-like CBSR is preferable.

Annex A: Simulation Parameters and Assumptions

Table A-1. Simulation assumptions

| Scenarios | 3D-UMi with ISD = 200m in 2GHz, 3D-UMa with ISD = 500m in 2GHz |
| --- | --- |
| BS antenna configurations | Antenna elements config: (+/-45), 0.5λ horizontal / 0.8 λ vertical antenna spacing |
| MS antenna configurations | 2 Rx X-pol (0/+90) |
| System bandwidth | 10MHz (50RBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Duplex | FDD |
| Network synchronization | Synchronized |
| UE distribution | Follows TR36.873 |
| UE speed | 3km/h |
| Polarized antenna modeling | Model-2 from TR36.873 |
| UE array orientation | ΩUT,α uniformly distributed on [0,360] degree, ΩUT,β = 90 degree, ΩUT,γ = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (high ~70% RU) |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 [71-12] assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |
| CSI-RS, CRS | CSI-RS one-to-one mapping to TXRU, only CRS port 0 is modeled for UE attachment, CRS port 0 is associated with the first TXRU |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |
| Transmission scheme | TM10, single CSI process, MU-MIMO with rank adaptation (no CoMP) |
| Wrapping method | Geographical distance based |
| Handover margin | 3 dB |
| Metrics | Average UE throughput, 5% UE throughput. |
| Max MU layer | 4 |