3GPP TSG RAN1 Meeting #92                                                               R1-1802286

Athens, Greece, 26th February – 2nd March 2018

Source: Thales

Title:  NR-NTN update on Link budget analysis

TDOC Type: pCR

Agenda Item: 7.3.2 Other (under SID = sFS_NR_nonterr_nw)

Document for: Information

Release: Rel-15

Specification: 38.811

# Abstract

The objective of this document is to extend the link budget analysis to other deployment scenarios for NR operating via satellite.

# Proposed text for approval

It is proposed to add the following texts to TR 38.811 “Study on NR to support Non-Terrestrial Networks”.

* * * Start of changes * * * * (new text)

# Annex A: Link budget analysis for the different deployment scenarios

This annex provides examples of Link Budgets associated to the different deployment scenarios.

## A.1 Introduction

This section provides an short overview of the formula used in the satellite link budgets.

The general formula for link budgets in free space between a transmitter and a receiver is provided here below, expressed in dB. It is composed of two terms:

The first term is the received carrier power over noise density ratio in dBHz called C/No, and the second term is the C/Io ratio which the carrier power to interference nose density ratio. For convenience this second term includes all the potential sources of interference: other beams interference, other system interference, and intermodulation product interference.

The first term is the received power carrier to noise density ratio expressed in  dBHz

The second is C/Io is the carrier power over interference noise density, also in dBHz.

We have

- C/No : carrier power over noise density ratio in dBHz

- EIRP: useful transmitter EIRP

- G/T : figure of merit of the receiver, where G is the receiver antenna gain,a nd T the equivalent system temperature. The unit is dB/K

- Loss: free space loss (negative according to the definition)

- : propagation margins (here positive counted); these are provisioned for shadowing margins for instance when dealing with satellite systems that can suffer from shaded conditions (trees)

- Aloes:  atmospheric losses : due to gases and rain fades: in S or L bands atmospheric losses are very small typically between 0.5 and 1 dB, but they can be very high in Ka band. They depend on the location of the terminal with respect to the satellite, the elevation angle. In this document we provide only clear sky conditions Link Budgets, with a typical atmospheric attenuation of 1 dB.

- : Additional losses, like degradation due to feeder links in case of non-regenerative systems. Typically we provision 0.5 dB feeder link degradation

- k: Boltzmann Constant: -10*LOG(k) =-228.6 dBW/K/Hz

In fact there are interference noise coming from other spot beams, other system interference, and intermodulation product interference, and so we must considerer at the entrance of the receiver the C/Io ratio which the carrier power to interference nose density ratio

The relationship between C/No and C/(No+Io) in dBHz is the following:

CN0+I0=-10×Log(10-0.1×CN0+10-0.1×CI0)

And the relationship between C/N and C/No (and all equivalent parameters like C/I or C/(N+I) is

C/N =C/No -10xLOG (BW), where BWis the signal bandwidth expressed in Hz, and C/N is in dB.

Slant range : this is the distance between the satellite and the earth station.

We define

- RT: Earth Radius

- Hs: Satellite Altitude at nadir

- Α: elevation angle of the considered point with respect to the satellite.

D=-RT×sinα+(RT2×sinα2+HS2+2×RT×Hs)

Free space loss : if we have a waveform of wavelength λ, and R the distance between with transmitter and the receiver, the free space loss is the quantity:

, and in dB . The sign is minus.

In the link budgets presented all over this document, the required C/N is the nearest value of C/N compared to the obtained C/(N+I). Required C/N≤ Obtained C/(N+I)

Note 1: the relationship  between G/T and the minimum Rx level and receiver’s antenna is:

Min_Rx_Level_At_antenna = - (G/T )+10*LOG(k)+10*LOG (BW)+ (C/N)required

Where  BW is the signal bandwidth.

In satellite world G/T is commonly used.

Note 2 : in the link budgets  Symbol rate is defined as the ratio  Ncarrier/ Ts the Time Symbol is including CP length. It is expressed in Msymb/s.

Note 3 :in the Link Budgets in S band involving standard 3GPP UEs, some lines are now added to introduce the Noise Figure, and the equivalent system temperature according to the following equation

Where

- Ga is the antenna gain in dBi

- NF : Noise Figure in dB

- Teq is the equivalent terminal temperature computed by

- Teq =To+(Ta-To)*10-0.1*NF where

- To : ambient temperature (290 K)

- Ta : antenna temperature

In the case of Class 3 UE, antenna gain is 0 dBi and antenna temperature is equal to ambient temperature, and therefore Teq = Ta

In the case of vehicle mounted terminal, antenna temperature is in the order of 150 K.

Note 4: concerning the Ka band deployment scenarios, where terminals are based on dishes or phase-array antenna, the use of the figure of merit (G/T) is maintained in the link budgets, as most of the terminals data sheets provide the G/T as part of the terminal radio electrical performances.

Note 5 : the previous note concerns also satellite receiver, as it is always characterized by its figure of merit G/T instead of its noise figure, antenna gain and ambient temperature.

## A.2	Deployment scenario D1: GEO in Ka band

The following tables present the downlink and uplink link budgets, for fixed terminal featuring an antenna with 60 cm equivalent aperture (dish or phased array antenna) and maximum 2  W Transmit power.

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

### A.2.1 Downlink performance

An example of the downlink link budgets is presented here below, and the graph presents the spectral efficiency versus the EIRP density.

Table A.2.1-1: Example of GEO satellite Ka band Downlink link budgets for fixed terminal

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2 |
| Information data rate (after decoding) | Mb/s | 396 |
| Radio interface configuration | Unit | Value |
| Down-link Carrier Frequency | GHz | 20,0 |
| Channel bandwidth | MHz | 198 |
| SCS | kHz | 60 |
| Cyclic Prefix | µs | 1,2 |
| Total Symbol duration (µs) including CP | µs | 17,84 |
| Modulation Symbol rate | Msymb/s | 185,0 |
| Terminal RF characteristics |  |  |
| G/T ( figure of merit see Note above) | dB/K | 15,7 |
| Antenna polarization (Circular or Linear) |  | C |
| System assumptions |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30,0 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | Km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Satellite nominal altitude | Km | 35788,0 |
| Typical average C/I within a satellite beam | dB | 16,0 |
| Satellite beam polarization |  | C |
| Link budget |  |  |
| Free space propagation loss | dB | -210,2 |
| Atmospheric loss (in clear sky conditions) | dB | 1 |
| # Required C/(N+I) for the targeted data rate | # dB | # 6,700 |
| Required useful EIRP per channel | dBW | 57,7 |
| Required useful satellite EIRP density | dBW/MHz | 34,5 |

The figure below provides the required useful satellite EIRP density for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure A.2.1-1: Spectral Efficiency versus the required useful satellite EIRP density – GEO in Ka band

### A.2.2 Uplink performance

An example of the uplink link budgets is presented here below, and the graph  presents the spectral efficiency versus the satellite G/T.

The return link can use CP-OFDM or DFT-SOFDM.The first table is the results with DFT-SOFDM, the second with CP-OFDM.

Table A.2.2-1: Example of GEO satellite Ka band Uplink Link budgets with DFT-SOFDM

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 2,3750 |
| Information data rate (after decoding) | Mb/s | 21.54 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30,00000 |
| Channel Bandwidth | Mhz | 9,072 |
| Cyclic Prefix | µs | 2,3 |
| Total Symbol duration (µs) | µs | 35,68 |
| Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 35788,0 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 1,0 |
| Terminal useful EIRP per channel | dBW | 44,0 |
| Free space propagation loss | dB | -213,7 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| # Required C/(N +I) | # dB | # 8,2 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 22,0 |

Table A.2.2-2: Example of GEO satellite Ka band Uplink Link budget with CP-OFDM

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 2,0 |
| Information data rate (after decoding) | Mb/s | 18.44 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30 |
| Channel Bandwidth | Mhz | 9,072 |
| (Cyclic Prefix | µs | 2,3 |
| Total Symbol duration (µs) | µs | 35,68 |
| Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 35788,0 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 2,5 |
| Terminal useful EIRP per channel | dBW | 42,5 |
| Free space propagation loss | dB | -213,7 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| # Required C/(N+I) | # dB | # 6,7 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 22,0 |

The figure below provides the required satellite G/T for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure A.2.2-1: Spectral Efficiency versus the required satellite G/T – GEO in Ka band for DFTS-OFDM and CP-OFDM

## A.3	Deployment scenario D2 : GEO satellite in S band

The following tables present the downlink and uplink link budgets, for different kind of terminals

- a 3GPP class 3 UE with omnidirectional antenna and 23 dBm max transmit power, and with linear polarization (HH1)

- an improved 3 GPP Class UE with circular polarization(or any device like dual antenna allowing diversity gain of 3 dB) and 1 W RF power (30 dBm) (HH2)

- A vehicle mounted terminal with circular polarization, (VM) and 2 W ( 37 dBm EIRP) with antenna gain of 4 dBi

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

### A.3.1 Downlink performance

An example of the downlink link budgets is presented here below.

Table A.3.1-1: Example of GEO satellite S band Downlink link budgets for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 0.625 | 1.125 | 1,3750 |
| Information data rate (after decoding) | Mb/s | 3.15 | 5.67 | 6.93 |
| Radio interface configuration | Unit |  |  |  |
| Down-link Carrier Frequency | (GHz) | 2.170 | 2.170 | 2.170 |
| Channel bandwidth | MHz | 5.04 | 5.04 | 5.04 |
| SCS | kHz | 15 | 15 | 15 |
| Cyclic Prefix | µs | 4.7 | 4.7 | 4.7 |
| Total Symbol duration (µs) including CP | µs | 71.36 | 71.36 | 71.36 |
| Number of PRBs |  | 28 | 28 | 28 |
| Modulation Symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Antenna Gain | dBi | 0 | 0 | 4 |
| Noise Figure | dB | 6 | 6 | 2 |
| Antenna Temperature | K | 290 | 290 | 150 |
| Ambient Temperature | K | 290 | 290 | 290 |
| Antenna polarization (Circular or Linear) |  | L | C | C |
| System assumptions |  |  |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30 | 30 | 30 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 38611.5 | 38611.5 | 38611.5 |
| Satellite RF characteristics |  |  |  |  |
| Satellite nominal altitude | km | 35786 | 35786 | 35786 |
| Typical average C/I within a satellite beam | dB | 16 | 16 | 16 |
| Satellite beam polarization |  | C | C | C |
| Link budget |  |  |  |  |
| Free space propagation loss | dB | -190.9 | -190.9 | -190.9 |
| Atmospheric loss | dB | -0,1 | -0,1 | -0,1 |
| Shadowing margins | dB | 3 | 3 | 10 |
| # Required C/(N+I) for the targeted data rate | # dB | # -1.3 | # 2.2 | # 4.2 |
| Required useful EIRP per channel | dBW | 66 | 66 | 66 |
| Required useful satellite EIRP density | dBW/MHz | 59 | 59 | 59 |

Note : the link budgets include, when necessary, degradation due to polarization mismatch.

### A.3.2 Uplink performance

Table A.3.2-1: Example of GEO satellite S band Uplink Link budget for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 0,15 | # 1,25 | # 1,25 |
| Information data rate (after decoding) | kb/s | 27 | 225 | 225 |
| Radio interface configuration |  |  |  |  |
| Up-link carrier frequency | GHz | 1.98 | 1.98 | 1.98 |
| Channel Bandwidth | khz | 180 | 180 | 180 |
| Number of PRB |  | 1 | 1 | 1 |
|  | (µs | 66,6667 | 66,6667 | 66,6667 |
| (Cyclic Prefix | µs | 4,7 | 4,7 | 4,7 |
| Total Symbol duration (µs) | µs | 71,36 | 71,36 | 71,36 |
| Modulation symbol rate | ksymb/s | 3.4 | 13.6 | 13.6 |
| Terminal RF characteristics |  |  |  |  |
| Max EIRP (corresponding to 200 mW) | dBW | -10 | 0 | 7 |
| System assumptions |  |  |  |  |
| Elevation angle to satellite | ° | 30 | 30 | 30 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38611.6 | 38611.6 | 38611.6 |
| Satellite RF characteristics |  |  |  |  |
| Nominal altitude | km | 35786 | 35786 | 35786 |
| Typical average C/I in a beam | dB | 16 | 16 | 16 |
| Link budget |  |  |  |  |
| Output back off in the terminal | dB | 0 | 0 | 0 |
| Terminal useful EIRP per channel | dBW | -10 | 0 | 7 |
| Free space propagation loss | dB | -190.1 | -190.1 | -190.1 |
| Atmospheric propagation loss (in clear sky conditions) | dB | -0.1 | -0.1 | -0.1 |
| Shadowing margins | dB | 3 | 3 | 10 |
| # Required C/(N+I) | # dB | # -6.3 | # 3.1 | # 3.1 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 20.5 | 20.5 | 20.5 |

## A.4	Deployment scenario D3 : LEO satellite at 600 km in S band

The following tables present the downlink and uplink link budgets, for different kind of terminals

- a 3GPP class 3 UE with omnidirectional antenna and +23 dBm max transmit power, and with linear polarization (HH1)

- an improved 3 GPP Class UE with circular polarization(or any device like dual antenna allowing diversity gain of 3 dB) and 1 W RF power (30 dBm) (HH2)

- A vehicle mounted terminal with circular polarization, (VM) and 2 W ( 7dBW EIRP) (antenna gain is 4 dBi)

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

### A.4.1 Downlink performance

An example of the downlink link budgets is presented here below.

Table A.4.1-1: Example of LEO satellite S band Downlink link budgets for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 0.625 | 1.125 | 1,3750 |
| Information data rate (after decoding) | Mb/s | 3.15 | 5.67 | 6.93 |
| Radio interface configuration | Unit |  |  |  |
| Down-link Carrier Frequency | (GHz) | 2.170 | 2.170 | 2.170 |
| Channel bandwidth | MHz | 5.04 | 5.04 | 5.04 |
| SCS | kHz | 15 | 15 | 15 |
| Cyclic Prefix | µs | 4.7 | 4.7 | 4.7 |
| Total Symbol duration (µs) including CP | µs | 71.36 | 71.36 | 71.36 |
| Number of PRBs |  | 28 | 28 | 28 |
| Modulation Symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Antenna Gain | dBi | 0 | 0 | 4 |
| Noise Figure | dB | 6 | 6 | 2 |
| Antenna Temperature | K | 290 | 290 | 150 |
| Ambient Temperature | K | 290 | 290 | 290 |
| Antenna polarization (Circular or Linear) |  | L | C | C |
| System assumptions |  |  |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30 | 30 | 30 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075,2 | 1075,2 | 1075,2 |
| Satellite RF characteristics |  |  |  |  |
| Satellite nominal altitude | km | 600 | 600 | 600 |
| Typical average C/I within a satellite beam | dB | 16 | 16 | 16 |
| Satellite beam polarization |  | C | C | C |
| Link budget |  |  |  |  |
| Free space propagation loss | dB | -159,8 | -159,8 | -159,8 |
| Atmospheric loss | dB | -0,1 | -0,1 | -0,1 |
| Shadowing margins | dB | 3 | 3 | 10 |
| # Required C/(N+I) for the targeted data rate | # dB | # -1.3 | # 2.2 | # 4.2 |
| Required useful EIRP per channel | dBW | 35 | 35 | 35 |
| Required useful satellite EIRP density | dBW/MHz | 28 | 28 | 28 |

Note : the link budgets include, when necessary, degradation due to polarization mismatch.

### A.4.2 Uplink performance

Table A.4.2-1: Example of LEO satellite S band Uplink Link budgets for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 0,2500 | # 1,6250 | # 1,6250 |
| Information data rate (after decoding) | kb/s | 45, | 292,5 | 292,5 |
| Radio interface configuration |  |  |  |  |
| Up-link carrier frequency | GHz | 1.98 | 1.98 | 1.98 |
| Channel Bandwidth | khz | 180 | 180 | 180 |
| Number of PRB |  | 1 | 1 | 1 |
|  | (µs | 66,6667 | 66,6667 | 66,6667 |
| (Cyclic Prefix | µs | 4,7 | 4,7 | 4,7 |
| Total Symbol duration (µs) | µs | 71,36 | 71,36 | 71,36 |
| Modulation symbol rate | ksymb/s | 168.2 | 168.2 | 168.2 |
| Terminal RF characteristics |  |  |  |  |
| Max EIRP (corresponding | dBW | -10 | 0 | 7 |
| System assumptions |  |  |  |  |
| Elevation angle to satellite | ° | 30 | 30 | 30 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075,2 | 1075,2 | 1075,2 |
| Satellite RF characteristics |  |  |  |  |
| Nominal altitude | km | 600 | 600 | 600 |
| Typical average C/I in a beam | dB | 16 | 16 | 16 |
| Link budget |  |  |  |  |
| Output back off in the terminal | dB | 0 | 0 | 0 |
| Terminal useful EIRP per channel | dBW | -10 | 0 | 7 |
| Free space propagation loss | dB | -153,9 | -153,9 | -153,9 |
| Atmospheric propagation loss (in clear sky conditions) | dB | -0.1 | -0.1 | -0.1 |
| Shadowing margins | dB | 3 | 3 | 10,0 |
| # Required C/(N+I) | # dB | # -4,8 | # 5.4 | # 5.4 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | -13 | -13 | -13 |

## A.5  Scenario D4 : LEO at 600 km in Ka band

The following tables present the downlink and uplink link budgets, for fixed terminal featuring an antenna with 60 cm equivalent aperture (dish or phased array antenna) and xx W Transmit power.

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

### A.5.1 Downlink performance

An example of the downlink link budgets is presented here below, and the graph presents the spectral efficiency versus the EIRP density.

Table A.5.1-1: Example of LEO satellite Ka band Downlink link budgets for fixed terminal

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2.375 |
| Information data rate (after decoding) | Mb/s | 470 |
| Radio interface configuration | Unit | Value |
| Down-link Carrier Frequency | GHz | 20,0 |
| Channel bandwidth | MHz | 198 |
| SCS | kHz | 60 |
| Cyclic Prefix | µs | 1,2 |
| Total Symbol duration (µs) including CP | µs | 17,84 |
| Modulation Symbol rate | Msymb/s | 185,0 |
| Terminal RF characteristics |  |  |
| G/T ( figure of merit see Note above) | dB/K | 15,7 |
| Antenna polarization (Circular or Linear) |  | C |
| System assumptions |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30,0 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075.2 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Satellite nominal altitude | km | 600 |
| Typical average C/I within a satellite beam | dB | 16,0 |
| Satellite beam polarization |  | C |
| Link budget |  |  |
| Free space propagation loss | dB | -179.1 |
| Atmospheric loss (in clear sky conditions) | dB | 1 |
| # Required C/(N+I) for the targeted data rate | # dB | # 8.2 |
| Required useful EIRP per channel | dBW | 29.2 |
| Required useful satellite EIRP density | dBW/MHz | 6.3 |

The figure below provides the required useful satellite EIRP density for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure A.5.1-1: Spectral Efficiency versus the required useful satellite EIRP density – LEO in Ka band

### A.5.2 Uplink performance

An example of the uplink link budgets is presented here below, and the graph  presents the spectral efficiency versus the satellite G/T.

The return link can use CP-OFDM or DFT-SOFDM.The first table is the results with DFT-SOFDM, the second with CP-OFDM.

Table A.5.2-1: Example of LEO satellite Ka band Uplink Link budgets with DFT-SOFDM

| System performance target |  | Fixed |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 2,3750 |
| Information data rate (after decoding) | Mb/s | 21.5 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30,0 |
| Channel Bandwidth | Mhz | 9,072 |
| Cyclic Prefix | µs | 2,3 |
| Total Symbol duration (µs) | µs | 35,68 |
| Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075.2 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 600 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 1,0 |
| Terminal useful EIRP per channel | dBW | 44,0 |
| Free space propagation loss | dB | -182.6 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| # Required C/(N +I) | # dB | # 8,2 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | -9.05 |

Table A.5.2-2: Example of LEO satellite Ka band Uplink Link budget with CP-OFDM

| System performance target |  | Fixed |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 2,0 |
| Information data rate (after decoding) | Mb/s | 18.1 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30, |
| Channel Bandwidth | Mhz | 9,072 |
| (Cyclic Prefix | µs | 2,3 |
| Total Symbol duration (µs) | µs | 35,68 |
| Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 1075.2 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 600 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 2,5 |
| Terminal useful EIRP per channel | dBW | 42,5 |
| Free space propagation loss | dB | -182.6 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| # Required C/(N+I) | # dB | # 6,7 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | -6.8 |

The figure below provides the required satellite G/T for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure A.5.2-1: Spectral Efficiency versus the required satellite G/T – LEO in Ka band for DFTS-OFDM and CP-OFDM

## A.6 : Scenario D5: HAPS at 20 km in S band

The following tables present the downlink and uplink link budgets, for different kind of terminals

- a 3GPP class 3 UE with omnidirectional antenna and 23 dBm max transmit power, and with linear polarization (HH1)

- an improved 3 GPP Class UE with circular polarization(or any device like dual antenna allowing diversity gain of 3 dB) and 1 W RF power (30 dBm) (HH2)

- A vehicle mounted terminal with circular polarization, (VM) and 2 W ( 7dBW EIRP) (antenna gain is 4 dBi)

The radio interface configuration is provided in the different Link Budgets Tables.

We consider an On Board Processing Subsystem, which avoids feeder link degradation.

We consider  an elevation angle baseline of 30 ° with NR Interface.

We assume light indoor coverage conditions for hnadhelds.

### A.6.1 Downlink performance

Table A.6.1-1: Example of HAPS S band Downlink link budgets for different kinds of terminal and NR

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 1,625 | 2,375 | 3,750 |
| Information data rate (after decoding) | Mb/s | 8,19 | 11,97 | 18,9 |
| Radio interface configuration | Unit |  |  |  |
| Down-link Carrier Frequency | (GHz) | 2.140 | 2.140 | 2.140 |
| Channel bandwidth | MHz | 5.04 | 5.04 | 5.04 |
| SCS | kHz | 15 | 15 | 15 |
| Cyclic Prefix | µs | 4.7 | 4.7 | 4.7 |
| Total Symbol duration (µs) including CP | µs | 71.36 | 71.36 | 71.36 |
| Number of PRBs |  | 28 | 28 | 28 |
| Modulation Symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Antenna Gain | dBi | 0 | 0 | 4 |
| Noise Figure | dB | 6 | 6 | 2 |
| Antenna Temperature | K | 290 | 290 | 150 |
| Ambient Temperature | K | 290 | 290 | 290 |
| Antenna polarization (Circular or Linear) |  | L | C | C |
| System assumptions |  |  |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30 | 30 | 30 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 40 | 40 | 40 |
| HAPS RF characteristics |  |  |  |  |
| HAPS nominal altitude | km | 20 | 20 | 20 |
| Typical average C/I within a satellite beam | dB | 16 | 16 | 16 |
| HAPS beam polarization |  | C | C | C |
| Link budget |  |  |  |  |
| Free space propagation loss | dB | -131,1 | -131,1 | -131,1 |
| Atmospheric loss | dB | -0,1 | -0,1 | -0,1 |
| Shadowing margins | dB | 10 | 10 | 10 |
| # Required C/(N+I) for the targeted data rate | # dB | # 5,4 | # 8,2 | # 13,5 |
| Required useful EIRP per channel of 5 MHz | dBW | 19.5 | 19.5 | 19.5 |
| Required useful HAPS EIRP density | dBW/MHz | 12.5 | 12.5 | 12.5 |

### A.6.2 Uplink performance

Table A.6.2-1: Example of HAPS in  S band Uplink Link budget with NR DFT-SOFDM for different kinds of terminals

| System performance target |  | HH1 | HH2 | VM |
| --- | --- | --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 1,000 | # 2,875 | # 3,750 |
| Information data rate (after decoding) | Mb/s | # 5,04 | # 14,49 | # 18,9 |
| Radio interface configuration |  |  |  |  |
| Up-link carrier frequency | GHz | 1.94 | 1.94 | 1.94 |
| Channel Bandwidth | Mhz | 5.04 | 5.04 | 5.04 |
| Number of PRB |  | 28 | 28 | 28 |
|  | (µs | 66,6667 | 66,6667 | 66,6667 |
| (Cyclic Prefix | µs | 4,7 | 4,7 | 4,7 |
| Total Symbol duration (µs) | µs | 71,36 | 71,36 | 71,36 |
| Modulation symbol rate | Msymb/s | 4.709 | 4.709 | 4.709 |
| Terminal RF characteristics |  |  |  |  |
| Max EIRP | dBW | -10 | 0 | 7 |
| System assumptions |  |  |  |  |
| Elevation angle to satellite | ° | 30 | 30 | 30 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 40 | 40 | 40 |
| HAPS RF characteristics |  |  |  |  |
| Nominal altitude | km | 20 | 20 | 20 |
| Typical average C/I in a beam | dB | 16 | 16 | 16 |
| Link budget |  |  |  |  |
| Output back off in the terminal | dB | 0 | 0 | 0 |
| Terminal useful EIRP per channel | dBW | -10 | 0 | 7 |
| Free space propagation loss | dB | -130,2 | -130,2 | -130,2 |
| Atmospheric propagation loss (in clear sky conditions) | dB | -0.1 | -0.1 | -0.1 |
| Shadowing margins | dB | 10 | 10 | 10 |
| # Required C/(N+I) | # dB | # 1,7 | # 10,2 | # 13,5 |
| Required HAPS G/T (corresponding to Rx sensitivity) | dB/K | -9 | -9 | -9 |

## A.7 : Performances assessment summary

### A.7.1 Ka-band deployment scenarios: D1 and D4

For the Ka band deployment scenarios (GEO and LEO at 600 km), we summarise here under the minimum satellite payload performance requirements (EIRP density and figure of merit (G/T) for a spectral efficiency of 2.

It assumes for UE, a terminal equipped with 60 cm equivalent aperture antenna (dish or phased array antenna) and 45 dBW maximum EIRP corresponding to approximately 2 W transmit power.

Table A.7.1-1: Minimum Required Useful Satellite EIRP density (dBW/MHz) for a Spectral Efficiency of 2 bit/s/Hz

|  | GEO: 35786km | LEO: 600 km |
| --- | --- | --- |
| Minimum EIRP density (dBW/MHz) | 34.5 | 3.7 |

Table A.7.1-2: Minimum Required Useful Satellite G/T (dB/K) for a Spectral Efficiency of 2 bit/s/Hz

|  | GEO: 35786km DFT SOFDM | GEO: 35786km CP-OFDM | LEO: 600 km DFT SOFDM | LEO: 600 km CP-OFDM |
| --- | --- | --- | --- | --- |
| Minimum G/T (dB/K) | 20.5 | 22 | -10.5 | -9.0 |

### A.7.2 S-band deployment scenarios: D2, D3 and D5

For the S band deployment scenarios (GEO, LEO at 600 km and HAPS), we summarise here under,  the minimum space or airborne payload performance requirements (EIRP density and figure of merit, G/T) for various targeted data rates on respectively down and uplink and various UE types.

Table A.7.2-1: Minimum Required Useful space and airborne EIRP density (dBW/MHz) per 5 MHz channel

|  | Downlink data rate for HH1 UE | Downlink data rate for HH2 & VM UE | Minimum Required EIRP density (dBW/MHz) |
| --- | --- | --- | --- |
| GEO: 36000 km | 3.1 Mbps | 5.6 & 6.9 Mbps (respectively) | +59 dBW/MHz |
| LEO: 600 km | 3.1 Mbps | 5.6 & 6.9 Mbps (respectively) | +28 dBW/MHz |
| HAPS: 20 km | 8.2 Mbps | 12 & 18.9 Mbps (respectively) | +12.5 dBW/MHz |

Table A.7.2-2: Minimum Required space and airborne G/T (in dB/K)

|  | Channel bandwidth | Uplink data rate for HH1 UE | Uplink data rate for HH2 & VM UE | Min required G/T |
| --- | --- | --- | --- | --- |
| GEO: 36000 km | 180 KHz channel | 27 kbps | 225 kbps | +20.5 dB/K |
| LEO: 600 km | 180 KHz channel | 45 kbps | 290 kbps | -13 dB/K |
| HAPS: 20 km | 5 MHz channel | 5 Mbps | 14.5 & 18.9 Mbps (respectively) | -9 dB/K |

* * * End of Changes * * * *