3GPP TSG-RAN WG1 #92	R1-1802290

# Athens, Greece, February 26 – March, 2, 2018

Agenda Item:	7.1.3.1.2

Source:	Apple Inc.

Title:	Remaining Issues in PDCCH Search Space

Document for:	Discussion, Decision

# Introduction

In the discussion of search space design, we made following agreements from RAN1 AH in Jan 2018.

Agreements:

- For a search space configuration, monitoring periodicity of slot(s) is updated as follows:

  - For all SCS, {1, 2, 4, 5, 8, 10, 16, 20} slots

    - For INT-RNTI, a subset of {1,2,4} slots is applied

  - FFS: the case when concatenated semi-static DL/UL assignments is configured

This allows network to configure search space with periodicity from {1, 2, 4, 5, 8, 10, 16, 20} slots.

And, in GC-PDCCH discussion, we made following agreements from RAN1#91.

Agreements:

- For GC-PDCCH monitoring, the period is GC-PDCCH SCS dependent

  - For 15KHz SCS (slots based on 15kHz): 1,2,5,10,20

  - For 30KHz SCS (slots based on 30kHz): 1, 2, 4, 5, 10, 20

  - For 60KHz SCS(slots based on 60kHz): 1, 2, 4, 5, 8, 10, 20

  - For 120KHz SCS(slots based on 120kHz): 1, 2, 4, 5, 10, 20

This allows network to configure GC-PDCCH with certain periodicities depending on SCS. Note that the above two periodicities are configured in the units of slots.

In semi-static DL/UL assignment discussion, we made following agreement to support the concatenation of two periodicities from RAN1#91.

Agreements:

- For the cell-specific RRC configuration of the semi-static DL/UL assignment,

  - Add additional periodicity 0.625ms (for 120KHz SCS only), 1.25ms (for >=60KHz SCS), and 2.5ms (for >=30KHz SCS)

  - Also support 2 concatenated DL-unknown-UL periodicity

    - Add 1 bit in semi-static DL/UL assignment to indicate if the second periodicity is included

    - The two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms

      - When two perodicities are included, the corresponding parameters are independently configured.

      - Note: it will be discussed to preclude some combinations (no additional higher-layer impact)

This allows network to configure a concatenation of two periodicities for semi-static DL/UL assignments, where two periodicities X and Y could be chosen independently from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms. Note that the unit is in ms unlike from that for search space and GC-PDCCH.

# PDCCH Monitoring Periodicity

## Case of Concatenated Semi-static DL/UL Assignments

In semi-static UL/DL assignments, if two periodicities X and Y are chosen independently from the given set, then there could be 32 different combinations. The problem here is that for many such combinations with combined periodicity X+Y, we don’t have good PDCCH and GC-PDCCH monitoring periodicity which matches with the combined periodicity. By “good” periodicity we mean a periodicity which could serve two important purposes - monitoring search space and UE power saving – simultaneously.

However, due to limited number of periodicities available for PDCCH monitoring, some of them are not good for certain concatenated periodicities. For example, if X+Y=7ms (with e.g., X=5ms and Y=2ms) with SCS=15kHz, then, it is not easy to choose good PDCCH monitoring periodicity (and GC-PDCCH periodicity).

Figure 1 Concatenation of two semi-static UL/DL assignments

One simple solution is to use periodicity of one slot for both PDCCH monitoring and GC-PDCCH. This approach certainly works in every combination. However, then UE should monitor every DL and flexible symbols with search space configured, which is not desirable for UE power consumption perspective. Considering that motivation of introducing PDCCH monitoring period is for UE power saving, it becomes clear that the choice of periodicity of 1 slot is not desirable.

Configuring larger period is not always works well when it does not match well with concatenated period. Figure 2 shows the case when UE is configured to monitor PDCCH every 5 slots. Each box in the Figure denotes one slot in given SCS and each yellow box denotes the slots where UE monitors PDCCH. As seen in the figure, the location of slots with search space is irregular w.r.t. the concatenated period of X+Y, which means that this UE may not be fully utilize resources depending on the location of search space slot in each concatenated period. For example, 5th yellow slots falls in the uplink slots (symbols) so, in this case the distance between two search space slots is 10ms. To overcome this limitation, network should make UE monitors PDCCH more frequently, which again we want to avoid.

Figure 2 PDCCH monitoring period of 5 slots with X+Y=7ms in SCC=15kHz

One simple solution is to support more periodicities. To identify required periodicities, we made following table, where row and column denotes two periodicities for a given SCS. The sum of two periodicities and corresponding number of slots in the given SCS were denoted in each entry.

When SCS=15kHz as shown in Table 1, only {0.5, 1, 2, 5, 10}ms are available for X and Y. Thus, we have 15 different combinations of concatenated periods. In this case, concatenated periods of 1, 2, 4, 10, 20ms have matching PDCCH monitoring period. However, remaining concatenated periodicities such as 3, 6, 7, 11, 12, 15ms do not have matching PDCCH periodicity. In case of 15ms, PDCCH monitoring periodicity of 5 slots could be used. However, in general it is not easy to choose PDCCH monitoring periodicity for these cases. Adding additional set of periodicities {3, 6, 7, 11, 12, 15} slots could make it fully supports all combinations of periodicities with SCS=15kHz.

Table 1 Combination of two periods and corresponding number of slots when SCS=15kHz

For SCS=30kHz shown in Table 2, there is an additionally supported periodicity of 2.5ms for semi-static DL/UL assignment. Like above case, only a limited number of combinations have matching periodicity. For concatenated period of 15ms (30 slots), network can use a period of 10 slots. But, ideally we think it is better to support 30 slots as well to align with concatenated semi-static period X+Y. In Table 2, red numbers in each entry denotes the period translated to slots. Ideally, if these additional periods of {3, 6, 7, 9, 11, 12, 14, 15, 21, 22, 24, 25, 30, 40}slots are supported, then, any combinations of X and Y could be easily supported.

Table 2 Combination of two periods and corresponding number of slots when SCS=30kHz

Same analysis are done for SCS=60kHz in Table 3. Additionally required periodicity set for PDCCH monitoring is {6, 7, 9, 12, 13, 14, 15, 18, 24, 25, 28, 30, 40, 42, 44, 45, 48, 50, 60, 80}.

Table 3 Combination of two periods and corresponding number of slots when SCS=60kHz

Same analysis are done for SCS=120kHz in Table 4. Additionally required periodicity set for PDCCH monitoring is {9, 12, 13, 14, 15, 18, 21, 24, 25, 26, 28, 30, 32, 36, 40, 42, 44, 45, 48, 50, 56, 60, 80, 84, 85, 88, 90, 96, 100, 120, 160}.

Table 4 Combination of two periods and corresponding number of slots when SCS=120kHz

The union of additionally required periodicities across all SCS is given as a set P={3, 6, 7, 9, 11, 12, 13, 14, 15, 18, 21, 22, 24, 25, 26, 28, 30, 32, 36, 40, 42, 44, 45, 48, 50, 56, 60, 80, 84, 85, 88, 90, 96, 100, 120, 160}. Adding this additional periodicities would make the concatenation of two semi-static assignments more useful.

Another option is to add a new RRC parameter, e.g., Montoring-periodicity-PDCCH-slot2, for configuring PDCCH monitoring period in search space configuration without adding additional values to existing monitoring period. The new RRC parameter is added only when concatenation of two semi-static DL/UL assignments is used. With the second parameter, network can signal two different values for a single search space configuration. The PDCCH monitoring periodicity is determined as the sum of the two periodicities. This is also simple fix but requires RRC parameter change.

Another viable option is to limit the possible combination of concatenation, e.g., X and Y have the same value, or they have a certain ratio like 1:2 or 1:3, etc. We could easily limit the possible ratios since it is mostly be determined by UL to DL traffic ratio.

Proposal 1:

- Consider either to add additional periodicities of, e.g., {3, 6, 7, 9, 11, 12, 13, 14, 15, 18, 21, 22, 24, 25, 26, 28, 30, 32, 36, 40, 42, 44, 45, 48, 50, 56, 60, 80, 84, 85, 88, 90, 96, 100, 120, 160}, for PDCCH monitoring or to limit the possible combinations of concatenation with constraints (e.g., X:Y = 1:1, 1:2, or 1:3, etc).

## Case of Non-concatenated Semi-static DL/UL Assignments

Another problem we see is that the supported set of PDCCH monitoring periodicity is not large enough to support UE power saving in large SCS. Note that the power saving is one of strong use cases for non-contiguous PDCCH monitoring.

With a periodicity of 20 slots in SCS=120kHz, we have only 2.5ms between two PDCCH monitoring slots, which we think is not much helpful for UE power saving. So, here we see another reason for additional periodicities.

Following is the suggested text proposal for 38.331.

## Solution

The following includes text proposal for adding additional periodicities to PDCCH monitoring.

============================== text proposal for 38.331 ===================================
SearchSpace ::= SEQUENCE {

searchSpaceId 	SearchSpaceId,

-- The CORESET applicable for this SearchSpace.

-- FFS: Value 0 identifies the common CORESET configured in MIB?

-- FFS: Values 1..maxNrofControlResourceSets-1 identify CORESETs configured by dedicated signalling?

controlResourceSetId 	ControlResourceSetId,

-- Slots for PDCCH Monitoring configured as periodicity and offset. Corresponds to L1 parameters 'Montoring-periodicity-PDCCH-slot' and -- 'Montoring-offset-PDCCH-slot' (see 38.213, section 10)

-- sl15, sl10, sl20 FFS

monitoringSlotPeriodicityAndOffset CHOICE {

sl1	 	NULL,

sl2    		INTEGER (0..1),

sl3    		INTEGER (0..2),

sl5		INTEGER (0..4),

sl6		INTEGER (0..5),

sl7		INTEGER (0..6),

sl9		INTEGER (0..8),

sl10 	 	INTEGER (0..9),

sl11 	 	INTEGER (0..10),

sl12 	 	INTEGER (0..11),

sl13 	 	INTEGER (0..12),

sl14 	 	INTEGER (0..13),

sl15 	 	INTEGER (0..14),

sl18 	 	INTEGER (0..18),

sl20  		INTEGER (0..19),

sl21  		INTEGER (0..20),

sl22  		INTEGER (0..21),

sl24  		INTEGER (0..23),

sl25  		INTEGER (0..24),

sl26  		INTEGER (0..25),

sl28  		INTEGER (0..27),

sl30  		INTEGER (0..29),

sl32  		INTEGER (0..31),

sl36  		INTEGER (0..35),

sl40  		INTEGER (0..39),

sl42  		INTEGER (0..41),

sl44  		INTEGER (0..43),

sl45  		INTEGER (0..44),

sl48  		INTEGER (0..47),

sl50  		INTEGER (0..49),

sl56  		INTEGER (0..55),

sl60  		INTEGER (0..59),

sl80  		INTEGER (0..79),

sl84  		INTEGER (0..83),

sl85  		INTEGER (0..84),

sl88  		INTEGER (0..87),

sl90  		INTEGER (0..89),

sl96  		INTEGER (0..95),

sl100 		INTEGER (0..99),

sl120 		INTEGER (0..119),

sl160 		INTEGER (0..159)

} 													OPTIONAL, ============================================================================

# GC-PDCCH Monitoring Periodicity

The above problem applies to GC-PDCCH as well since the periodicity of GC-PDCCH is determined in slots with very limited number of periodicities. In GC-PDCCH case, the supported periodicities even depends on the SCS, which makes it even less useful for the case of concatenating two semi-static DL/UL assignments. Due to the same reason above, it is desirable to have additional periodicities for GC-PDCCH. To provide flexibility in configuration, it is desirable to have the same set of periodicities as PDCCH monitoring (i.e., search space).

Proposal 2:

- NR support the same set of periodicities for both PDCCH monitoring and GC-PDCCH.

# Conclusions

Proposal 1:

- Consider either to add additional periodicities of, e.g., {3, 6, 7, 9, 11, 12, 13, 14, 15, 18, 21, 22, 24, 25, 26, 28, 30, 32, 36, 40, 42, 44, 45, 48, 50, 56, 60, 80, 84, 85, 88, 90, 96, 100, 120, 160}, for PDCCH monitoring or to limit the possible combinations of concatenation with constraints (e.g., X:Y = 1:1, 1:2, or 1:3, etc).

Proposal 2:

- NR support the same set of periodicities for both PDCCH monitoring and GC-PDCCH.

# References