3GPP TSG RAN WG1 Meeting #92			R1-1802306

Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	6.1.7

Source: 	Qualcomm Incorporated

Title: 	Discussion on modulation enhancements

Document for:	Discussion/Decision

# Introduction

In RAN1#91, the following agreement was reached:

- RAN1 to consider one of the following solutions to this issue in Rel-15 under TEI-15:

  - Solution 1: Select modulation scheme based on TBS and number of REs used for rate matching.

  - Solution 2: Introduce “Modulation overriding” field in DCI to change the modulation scheme.

  - Solution 3: Introduce an alternative table for 256QAM by RRC configuration.

  - Solution 4: Extend the MCS field in DCI to 6 bits, and introduce overlapped entries with different modulation scheme.

    - Study the possibility of a single MCS table covering from QPSK to 1024QAM.

  - Solution 5: N_RB scaling for TBS selection (similar to TBS selection for special subframes)

- No changes to Rel-12/13/14 64QAM/256 QAM TBS/MCS entries

# Evaluation results

In Table 1, we simulated all the TBS of the 256QAM table in the following conditions:

- AWGN, 2 Receive antennas

- Configuration 1: 2 CRS ports, 1 control symbol

- Configuration 2: 4 CRS ports, 3 control symbols, 32 CSI-RS ports

Table 1 90% Tput SNR

|  |  |  | 1 Control - 2 CRS - No CSIRS - TM3 |  |  |  | 3 Control - 4 ports - 32 CSIRS - TM9 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mod | I_TBS | TBS | QPSK | 16QAM | 64QAM | 256QAM | QPSK | 16QAM | 64QAM | 256QAM |
| QPSK | 0 | 2792 | <-5 | -4.15 | NS | NS | <-5 | -3.55 | NS | NS |
| QPSK | 2 | 4584 | -4.15 | -2.45 | NS | NS | -3.5 | -1.75 | NS | NS |
| QPSK | 4 | 7224 | -2.1 | -0.6 | NS | NS | -0.9 | 0.25 | NS | NS |
| QPSK | 6 | 10296 | -0.6 | 0.9 | NS | NS | 1.7 | 1.75 | NS | NS |
| QPSK | 8 | 14112 | 1.35 | 2.4 | NS | NS | NA | 4.1 | NS | NS |
| 16QAM | 10 | 17568 | 2.85 | 3.4 | 4.9 | NS | NA | 6 | 6.55 | NS |
| 16QAM | 11 | 19848 | 3.9 | 3.9 | 5.85 | NS | NA | 7.25 | 7.65 | NS |
| 16QAM | 12 | 22920 | 5.25 | 4.9 | 6.4 | NS | NA | 8.85 | 9 | NS |
| 16QAM | 13 | 25456 | 6.6 | 5.85 | 6.9 | NS | NA | 10.45 | 10.35 | NS |
| 16QAM | 14 | 28336 | NA | 6.4 | 7.35 | NS | NA | NA | 11.55 | NS |
| 16QAM | 15 | 30576 | NA | 7.2 | 7.9 | NS | NA | NA | 12.55 | NS |
| 64QAM | 16 | 32856 | NS | 7.9 | 8.4 | 9.4 | NS | NA | 13.4 | 14.35 |
| 64QAM | 17 | 36696 | NS | 8.9 | 9.4 | 10.4 | NS | NA | 15.35 | 15.9 |
| 64QAM | 18 | 39232 | NS | 9.7 | 10.15 | 11.05 | NS | NA | 16.7 | 16.9 |
| 64QAM | 19 | 43816 | NS | 10.9 | 11.35 | 12.25 | NS | NA | NA | 18.55 |
| 64QAM | 20 | 46888 | NS | 11.95 | 11.9 | 12.9 | NS | NA | NA | 19.9 |
| 64QAM | 21 | 51024 | NS | 12.95 | 12.9 | 13.9 | NS | NA | NA | 21.75 |
| 64QAM | 22 | 55056 | NS | 15.25 | 13.9 | 14.85 | NS | NA | NA | 24.4 |
| 64QAM | 23 | 57336 | NS | NA | 14.4 | 15.35 | NS | NA | NA | NA |
| 64QAM | 24 | 61664 | NS | NA | 15.4 | 16.1 | NS | NA | NA | NA |
| 256QAM | 25 | 63776 | NS | NS | 15.9 | 16.4 | NS | NS | NA | NA |
| 256QAM | 27 | 66592 | NS | NS | 16.4 | 17.15 | NS | NS | NA | NA |
| 256QAM | 28 | 71112 | NS | NS | 17.4 | 18 | NS | NS | NA | NA |
| 256QAM | 29 | 73712 | NS | NS | 18 | 18.8 | NS | NS | NA | NA |
| 256QAM | 30 | 78704 | NS | NS | 19.55 | 19.7 | NS | NS | NA | NA |
| 256QAM | 31 | 81176 | NS | NS | 22.7 | 20.15 | NS | NS | NA | NA |
| 256QAM | 32 | 84760 | NS | NS | NA | 20.9 | NS | NS | NA | NA |
| 256QAM | 33 | 97896 | NS | NS | NA | 23.6 | NS | NS | NA | NA |
| 256QAM | 33A | 87936 | NS | NS | NA | 21.6 | NS | NS | NA | NA |

Underlined in each column are the values corresponding to the 256QAM MCS. Values highlighted in green are the ones for which the optimum modulation corresponds to the 256QAM modulation table. Values highlighted in yellow are those for which the optimum modulation scheme corresponds to a different modulation scheme than that in th e256QAM table. Entries labelled “NA” indicate that 90% throughput are not achieved, and entries labelled “NS” means that the corresponding entry was not simulated.

For the transition between 256QAM and 1024QAM we evaluated different TBS for 2x2 and 4x4 TM4, with the results shown in the following figure.

Figure 1 Performance of the 256QAM vs 1024QAM for 3 different 256QAM TBS sizes MCS (left – 2x2, right – corresponding 4x4 TBS). TM3/AWGN

We observe that, for high overhead cases, as expected, 1024QAM outperforms 256QAM.

# Evaluation of different solutions

In this section, and based on the result obtained in Table 1, we evaluate the different solutions that were agreed to consider in R1#91. The evaluation is made based on the following criteria:

- Completeness: whether the proposed method works for all cases.

- Overhead: whether the proposed method introduces additional overhead.

- Flexibility: whether the proposed method allows the eNB to dynamically choose the modulation scheme.

| Solution | Completeness | Overhead | Flexibility | Other |
| --- | --- | --- | --- | --- |
| Solution 1 | Yes | Additional RRC param | No | Main problem is need to agree in RAN1 to the thresholds for switching. |
| Solution 2 | Yes | 1 bit in DCI + RRC | Yes | This can be seen as a particular case of Solution 4 |
| Solution 3 | No | Additional RRC param | No | It only solves the problem for 256QAM, and in a semi-static way. |
| Solution 4 | Yes | 1 bit in DCI + RRC | Yes | As a generalization of Solution 2, this can be used to even include the “A/B” TBSs if there are spare entries. |
| Solution 5 | Unclear | Additional RRC param | No | Need to specify the scaling value for each of the overhead assumptions. Unclear effects of rounding. Proponents should give more details |

In view of the evaluation above, solution 3 can be seen not to be complete, and thus is not desirable. Solution 2 is a subset of Solution 4, and thus can be implemented within Solution 4. Thus, we make the following proposal:

Proposal: RAN1 to specify one of the following solutions in Rel-15:

  - Solution 1: Select modulation scheme based on TBS and number of REs used for rate matching.

  - Solution 4: Extend the MCS field in DCI to 6 bits, and introduce overlapped entries with different modulation scheme.

    - Study the possibility of a single MCS table covering from QPSK to 1024QAM, and potentially including both the normal and alternative TBS (e.g. 33/33A)