3GPP TSG RAN WG1 #92	R1-1802351

Feb. 26th – March. 2nd, 2018

Athens, Greece

Agenda item:	6.2.7.2.3

Source: 	Qualcomm Incorporated

Title: 	UL Control Related Techniques for URLLC

Document for:	Discussion and Decision

# 1	Introduction

One way to ensure the reliability of the UL control channel is to reduce the payload size. Besides assuming a single-CW for the LTE-URLLC as already considered in the sTTI design, prioritizing the UL control transmission corresponding to the LTE-URLLC over UCI of 1ms TTI operation can be considered. In particular, when the 1ms TTI needs to be dropped due to UL collision, its HARQ-ACK bits are not piggybacked on the URLLC channels.

Proposal 1: Prioritize the UL channels associated with the LTE-URLLC over those of the longer TTIs or different operations.

In general, the required reliability for the UL control channel should be specified in conjunction with the design/reliability of the DL control and data channels. Although at this stage, the required BLER is not determined, in the remainder of this paper, we show via link-level evaluations that using some entries of the SPUCCH format 1a/1b for channel estimation can considerably improve the NAK-to-ACK error probability.

# 2	SPUCCH FMT 1a/1b Performance Evaluation

We consider the performance of the sub-slot SPUCCH format 1a/1b under the following two detection schemes:

- Non-coherent detection, i.e., the correlation-based scheme is used.

- Coherent detection, where the identical entries of the UL sequences are used for channel estimation.

For the coherent detection, the cyclic shift indices are 0 and 6. Hence, assuming that the base sequence is b=1,1,1,1,1,1,1,1,1,1,1,1T, the ACK can be sent via ej2π0k/12b=1,1,1,1,1,1,1,1,1,1,1,1T, and NAK can be sent via ej2π6k/12b=1,-1,1,-1,1,-1,1,-1,1,-1,1,-1; the identical entries can therefore be used as pilots for channel estimation. The similar approach can be taken when 2bits of HARQ-ACK should be reported.

The link-level performance is compared in terms of (1) ACK error probability, and (2) NAK-to-ACK error probability. To obtain the results, a TDL-C channel with delay spread of 363ns, and UE speed of 3Kmph is considered. 2Rx antennas at the eNB, and 1Tx antenna at the UE is assumed.

Figure 1: ACK error and NAK->ACK error probability for SPUCCH format 1a.

Figure 2: ACK error and NAK->ACK error probability for SPUCCH format 1b.

Observation 1: For a 1-bit HARQ-ACK, the ACK error probability is almost identical with both coherent and non-coherent detection schemes. However, the coherent approach provides almost 5dB gain over the non-coherent approach at the NAK-to-ACK probability of 1e-4.

Observation 2: For a 2-bit HARQ-ACK, the coherent detection scheme provides almost 1dB gain at Ack error of 1e-2. Also, the coherent approach provides almost 4.5dB gain over the non-coherent approach at the NAK-to-ACK probability of 1e-4.

Observation 3: The coherent detection scheme significantly reduces the NAK-to-ACK error probability as compared to the non-coherent scheme funder both SPUCCH formats 1a and 1b.

# 3	Conclusions

Proposal 1: Prioritize the UL channels associated with the LTE-URLLC over those of the longer TTIs or different operations.

Observation 1: For a 1-bit HARQ-ACK, the ACK error probability is almost identical under both coherent and non-coherent detection schemes. However, the coherent approach provides almost 5dB gain over the non-coherent approach at the NAK-to-ACK probability of 1e-4.

Observation 2: For a 2-bit HARQ-ACK, the coherent detection scheme provides almost 1dB gain at ACK error probability of 1e-2. Also, the coherent approach provides almost 4.5dB gain over the non-coherent approach at the NAK-to-ACK probability of 1e-4.

Observation 3: The coherent detection scheme significantly reduces the NAK-to-ACK error probability as compared to the non-coherent scheme funder both SPUCCH formats 1a and 1b.