3GPP TSG RAN WG1 Meeting #92                                                              R1-1802437

Athens, Greece, February 26th – March 2nd, 2018

Source: 	Intel Corporation

Title:	Initial Calibration Results for IMT-2020 self-evaluation

Agenda Item:	7.7

Document for:	Discussion

# Introduction

This paper is related to system level simulator calibration for the “New Study Item on Self-Evaluation towards IMT-2020” [1]. In this paper, we present results for initial geometry calibration of the NR system level simulator for IMT-2020 self-evaluation study. The initial calibration assumptions were defined in [2] and the metrics under consideration are coupling loss and wideband SINR for the test scenarios defined by ITU-R evaluation guidelines in [3] for IMT-2020 candidate radio access technologies.

# Calibration Results for System Level Simulation

In preparation for the IMT-2020 self-evaluation study item, there has been an ongoing effort for system level simulation calibration within 3GPP. The initial geometry calibration is based on coupling loss and wideband SINR and the evaluation assumptions are summarized in [2] and also provided for reference in the Appendix. The test environments under consideration are Indoor Hotspot, Dense Urban and Rural macro for eMBB and Urban Macro for mMTC and URLLC. The detailed deployment scenarios as well as the channel models are outlined in [3]. In the following sections, we provide exhaustive calibration results for these scenarios with respect to coupling loss and wideband SINR for different channel models and carrier frequency ranges as specified in the Appendix and [2]. Note that for operation in FR1 (sub-6GHz) range [5] both IMT channel models A and B [3] are evaluated, while for operation in FR2 [6] and 70 GHz, IMT channel model B [3] is evaluated.

Indoor Hotspot - eMBB

    - Configuration A – 4GHz

    - Configuration B – 30GHz

    - Configuration C – 70GHz

## Dense Urban – eMBB

    - Configuration A – 1 Macro Layer 4 GHz

    - Configuration B – 1 Macro Layer 30 GHz

## Rural Macro – eMBB

    - Configuration A – 700 MHz

    - Configuration B – 4 GHz

    - Configuration C – 700 MHz LMLC (Low Mobility Large Cell)

## Urban Macro – mMTC

    - Configuration A – 700 MHz with 500m ISD

    - Configuration B – 700 MHz with 1732m ISD

## Urban Macro – URLLC

    - Configuration A – 4 GHz

    - Configuration B – 700 MHz

# Conclusions

In this paper, we have presented comprehensive geometry calibration results for all test scenarios outlined in IMT-2020 evaluation guidelines [3] and in accordance with evaluation assumptions outlined within 3GPP in [2] and also replicated in the Appendix.

# References

- RP-170832, New Study Item on Self-Evaluation towards IMT-2020, Huawei, Ericsson, 3GPP TSG RAN Meeting #75, Dubrovnik, Croatia, March 6-9, 2017.

- RT-170019, Summary of email discussion “[ITU-R AH 01] Calibration for self-evaluation, Huawei, 3GPP TSG RAN ITU-R Ad Hoc.

- ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020

- ITU-R M.2410-0, Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- 3GPP TS 38.101-1 v15.0.0, User Equipment radio transmission and reception: Part 1: Range 1 Standalone.

- 3GPP TS 38.101-2 v15.0.0, User Equipment radio transmission and reception: Part 2: Range 2 Standalone.

# Appendix

In this section, we provide the system level simulation assumptions set up in [3] for the purpose of initial system level geometry calibration for IMT-2020 self-evaluation.

## Indoor Hotspot - eMBB

The baseline parameters are provided in Table A1.

Table A1 Baseline parameter for Indoor Hotspot – eMBB

| Indoor Hotspot - eMBB | Config. A |  | Config. B |  | Config. C |  |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz |  | 30GHz |  | 70GHz |  |
| BS antenna height | 3 m |  | 3 m |  | 3 m |  |
| Total transmit power per TRxP | Baseline: 21 dBm for 10MHz bandwdith |  | Baseline: 20 dBm for 40 MHz bandwidth |  | Baseline: 18 dBm for 40 MHz bandwidth |  |
| UE power class | 23 dBm |  | 23 dBm |  | 21 dBm |  |
| Inter-site distance | 20m |  | 20 m |  | 20 m |  |
| Number of antenna elements per TRxP | 32Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |  | 64Tx/Rx, (M,N,P,Mg,Ng) = (4,8,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |  | 256Tx/Rx, (M,N,P,Mg,Ng) = (8,16,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |  |
| Number of TXRU per TRxP | 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) (1-to-1 mapping) |  | 8TXRU, (Mp,Np,P,Mg,Ng) =(2,2,2,1,1) |  | 8TXRU, (Mp,Np,P,Mg,Ng)=(2,2,2,1,1) |  |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |  | 32Tx/Rx, (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |  | 32Tx/Rx, (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |  |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |  | 4TXRU, (Mp,Np,P,Mg,Ng)=(1,1,2,1,2) |  | 4TXRU, (Mp,Np,P,Mg,Ng)=(1,1,2,1,2) |  |
| Device deployment | 100% indoor Randomly and uniformly distributed over the area |  | 100% indoor Randomly and uniformly distributed over the area |  | 100% indoor Randomly and uniformly distributed over the area |  |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |  | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |  | Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |  |
| UE speeds of interest | 3 km/h |  | 3 km/h |  | 3 km/h |  |
| Inter-site interference modeling | Explicitly modelled |  | Explicitly modelled |  | Explicitly modelled |  |
| BS noise figure | 5 dB |  | 7dB |  | 7dB |  |
| UE noise figure | 7 dB |  | 10dB |  | 10dB |  |
| BS antenna element gain | 5dBi |  | 5dBi |  | 5dBi |  |
| BS antenna element pattern | See Table 2 in Section 3.6 |  | See Table 2 in Section 3.6 |  | See Table 2 in Section 3.6 |  |
| UE antenna element gain | 0 dBi |  | 5dBi |  | 5dBi |  |
| UE antenna element pattern | Omni-directional |  | See Table 3 in Section 3.6 |  | See Table 3 in Section 3.6 |  |
| Thermal noise level | -174 dBm/Hz |  | -174 dBm/Hz |  | -174 dBm/Hz |  |
| Traffic model | Full buffer |  | Full buffer |  | Full buffer |  |
| Simulation bandwidth | 10MHz |  | 40MHz |  | 40MHz |  |
| UE density | 10 UEs per TRxP |  | 10 UEs per TRxP |  | 10 UEs per TRxP |  |
| UE antenna height | 1.5m |  | 1.5m |  | 1.5m |  |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B |  | (Channel model A or B is the same) |  | (Channel model A or B is the same) |  |
| TRxP number per site | 1 | 3 | 1 | 3 | 1 | 3 |
| Mechanic tilt | 180° in GCS (pointing to the ground) Top view: | [110°] in GCS | 180° in GCS (pointing to the ground) Top view: | [110°] in GCS | 180° in GCS (pointing to the ground) Top view: | [110°] in GCS |
| Electronic tilt | 90° in LCS | 90° in LCS | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | - | 30 / 150 / 270 degrees | - | 30 / 150 / 270 degrees | - | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| Wrapping around method | No wrapping around | No wrapping around | No wrapping around | No wrapping around | No wrapping around | No wrapping around |
| Minimum distance of TRxP and UE | d2D_min=0m | d2D_min=0m | d2D_min=0m | d2D_min=0m | d2D_min=0m | d2D_min=0m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | - | - | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -1*pi/8, 1*pi/8, 3*pi/8] Zenith angle θj = [pi/4 3*pi/4] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (phai_i, theta_j) is given by equation 1 in Appendix 1 (2D DFT beam) |  | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-7*pi/16, -5*pi/16, -3*pi/16, -1*pi/16, 1*pi/16, 3*pi/16, 5*pi/16, 7*pi/16] Zenith angle theta_j = [pi/8 3*pi/8 5*pi/8 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (phai_i, theta_j) is given by equation 1 in Appendix 1 (2D DFT beam) |  |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) |  | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) |  |
| Criteria for selection for serving TRxP | - | - | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |  | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |  |
| Criteria for analog beam selection for serving TRxP | - | - | Select the best beam pair among the set of DFT beams, based on the criteria of maximizing receive power after beamforming. |  | Select the best beam pair among the set of DFT beams, based on the criteria of maximizing receive power after beamforming. |  |
| Criteria for analog beam selection for interfering TRxP | - | - | Random selecting the random beams for non-serving TRxP |  | Random selecting the random beams for non-serving TRxP |  |

## Dense urban - eMBB

The baseline parameters are provided in Table A2.

Table A2 Baseline parameter for Dense Urban – eMBB

| Dense Urban - eMBB | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 1 layer (Macro) with 4 GHz | 1 layer (Macro) with 30 GHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 41 dBm for 10 MHz bandwidth | 37 dBm for 40 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss (applies to Channel model B) | 20% high loss, 80% low loss |
| Inter-site distance | 200 m | 200 m |
| Number of antenna elements per TRxP | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 256Tx/Rx, (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dg,H,dg,V) = (4.0, 2.0)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 4TXRU, (Mp,Np,P,Mg,Ng) = (2,1,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) =(1,1,2,2,2) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 32Tx/Rx, (M,N,P,Mg,Ng) = (2,4,2,1,2), (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; 0°,90° polarization |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) | 4TXRU, (Mp,Np,P,Mg,Ng)=(1,1,2,1,2) |
| Device deployment | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer | 80% indoor, 20% outdoor (in car) Randomly and uniformly distributed over the area under Macro layer |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 7 dB |
| UE noise figure | 7 dB | 10 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 1 in Section 3.6 | See Table 1 in Section 3.6 |
| UE antenna element gain | 0 dBi | 5 dBi |
| UE antenna element pattern | Omni-directional | See Table 3 in Section 3.6 |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 40 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | (Channel model A or B is the same) |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = [-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16] Zenith angle θj = [5*pi/8, 7*pi/8] NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = [-3*pi/8, -pi/8, pi/8, 3*pi/8]; Zenith angle θj = [pi/4, 3*pi/4]; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) |
| Criteria for selection for serving TRxP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |
| Criteria for analog beam selection for serving TRxP | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. |
| Criteria for analog beam selection for interfering TRxP | Random selecting the random beams for non-serving TRxP | Random selecting the random beams for non-serving TRxP |

## Rural – eMBB

The baseline parameters are provided in Table A3.

Table A3 Baseline parameter for Rural – eMBB

| Rural - eMBB | Config. A | Config. B | Config. C (LMLC) |
| --- | --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 4 GHz | 700 MHz |
| BS antenna height | 35 m | 35 m | 35 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 1732 m | 1732 m | 6000 m |
| Number of antenna elements per TRxP | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 128Tx/Rx, (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 16TXRU, (Mp,Np,P,Mg,Ng) = (1,8,2,1,1) | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) |
| Number of UE antenna elements | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°,90° polarization | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| Number of TXRU per UE | 2TXRU (1-to-1 mapping) | 4TXRU (1-to-1 mapping) | 4TXRU (1-to-1 mapping) |
| Device deployment | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 50% indoor, 50% outdoor (in car) Randomly and uniformly distributed over the area | 40% indoor, 40% outdoor (pedestrian), 20% outdoor (in-car) Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (in-car): 120 km/h; | Indoor users: 3 km/h; Outdoor users (pedestrian): 3 km/h; Outdoor users (in-car): 30 km/h |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 1 in Section 3.6 | See Table 1 in Section 3.6 | See Table 1 in Section 3.6 |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 10 MHz | 10 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | [100°] in LCS | [100°] in LCS | [96°] in LCS |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 | Model-2 in TR36.873 |

## Urban Macro - mMTC

The baseline parameters are provided in Table A4.

Table A4 Baseline parameter for Urban Macro – mMTC

| Urban Macro - mMTC | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 700 MHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 20% high loss, 80% low loss (applies to Channel model B) | 20% high loss, 80% low loss (applies to Channel model B) |
| Inter-site distance | 500 m | 1732 m |
| Number of antenna elements per TRxP | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of UE antenna elements | 1Tx/Rx 0° polarization | 1Tx/Rx 0° polarization |
| Number of TXRU per UE | 1TXRU | 1TXRU |
| Device deployment | 80% indoor, 20% outdoor Randomly and uniformly distributed over the area | 80% indoor, 20% outdoor Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |
| UE speeds of interest | 3 km/h for indoor and outdoor | 3 km/h for indoor and outdoor |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 1 in Section 3.6 | See Table 1 in Section 3.6 |
| UE antenna element gain | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 10 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | [99°] in LCS | [93°] in LCS |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |

## Urban Macro – URLLC

The baseline parameters are provided in Table A5.

Table A5 Baseline parameter for Urban Macro – URLLC

| Urban Macro - URLLC | Config. A | Config. B |
| --- | --- | --- |
| Carrier frequency for evaluation | 4 GHz | 700 MHz |
| BS antenna height | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 46 dBm for 10 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | 100% low loss (applies to Channel model B) |
| Inter-site distance | 500 m | 500 m |
| Number of antenna elements per TRxP | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | 16 Tx/Rx, (M,N,P,Mg,Ng) = (8,1,2,1,1), (dH,dV) = (N/A, 0.8)λ +45°, -45° polarization |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°, 90° polarization | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) 0°, 90° polarization |
| Number of TXRU per UE | 4TXRU (1-to-1 mapping) | 2TXRU (1-to-1 mapping) |
| Device deployment | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction |
| UE speeds of interest | 3 km/h for indoor and 30 km/h for outdoor | 3 km/h for indoor and 30 km/h for outdoor |
| Inter-site interference modeling | Explicitly modelled | Explicitly modelled |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 7 dB | 7 dB |
| BS antenna element gain | 8 dBi | 8 dBi |
| BS antenna element pattern | See Table 1 in Section 3.6 | See Table 1 in Section 3.6 |
| UE antenna element gain | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional |
| Thermal noise level | -174 dBm/Hz | -174 dBm/Hz |
| Traffic model | Full buffer | Full buffer |
| Simulation bandwidth | 10 MHz | 10 MHz |
| UE density | 10 UEs per TRxP | 10 UEs per TRxP |
| UE antenna height | 1.5 m | 1.5 m |
| Channel model variant | Alt. 1: Channel model A Alt. 2: Channel model B | Alt. 1: Channel model A Alt. 2: Channel model B |
| TRxP number per site | 3 | 3 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | [99°] in LCS | [99°] in LCS |
| Handover margin (dB) | 0 (i.e., the strongest cell is selected) | 0 (i.e., the strongest cell is selected) |
| TRxP boresight | 30 / 150 / 270 degrees | 30 / 150 / 270 degrees |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 |
| Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
| Minimum distance of TRxP and UE | d2D_min=10m | d2D_min=10m |
| Polarized antenna model | Model-2 in TR36.873 | Model-2 in TR36.873 |

## Antenna element pattern

The antenna element pattern is defined in [3]. For BS side, the TRxP antenna element pattern is defined in Table 8-6 in [3] for Dense Urban – eMBB, Rural – eMBB, Urban Macro – mMTC, and Urban macro – URLLC test environments. For Indoor Hotspot, the TRxP antenna element pattern is defined in Table 8-7 in [3]. They are copied in Table A6 and Table A7 for reference.

Table A6*

BS antenna element radiation pattern for 
Dense Urban – eMBB, Rural – eMBB, Urban Macro – mMTC, and Urban Macro - URLLC

| Parameters | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |

* Note: This is a copy of Table 8-6 in Report ITU-R M.[IMT-2020. EVAL]

Table A7*

BS antenna element radiation pattern for Indoor Hotspot - eMBB

| Parameters | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |

* Note: This is a copy of Table 8-7 in Report ITU-R M.[IMT-2020. EVAL]

For UE side, the UE antenna element pattern is Omni-directional for 4 GHz and 700 MHz; while for 30 GHz and 70 GHz, it is defined in Table 8-8 in [3], which is copied to Table A8 for reference.

Table A8*

UE antenna element radiation pattern for 30 GHz and 70 GHz

| Parameters | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |

* Note: This is a copy of Table 8-8 in Report ITU-R M.[IMT-2020. EVAL]

This parameter(s) is/are used for cell association