3GPP TSG RAN WG1 Meeting #92                                                              	R1-1802471

Athens, Greece, February 26th – March 2nd, 2018

Source:		NTT DOCOMO

Title:			Remaining issues on beam management

Agenda Item:		7.1.2.2.3

Document for: 	Discussion and Decision

- Introduction

NR beam management shall support the establishment and maintenance of beam pair links (BPLs) between TRP and UE and apply those beam pair links for control and data channels.

The main purpose of DL beam management is for the establishment and maintenance of beam pair links for DL channels. Therefore, it shall be composed of the following features:

- Transmission of beam management reference signals by gNB.

- Beam measurement and reporting by UE.

In addition, due to UE Rx analog beamforming, the UE shall understand the applied beam pair link before it receives signals from gNB, for that purpose, the following feature is needed:

- Beam indication by gNB.

In case of beam correspondence, the beam pair links applicable for UL channels can be derived from DL beam management, by appropriate design. Otherwise, the system may require an UL beam management procedure for the establishment and maintenance of beam pair links for UL channels.

In this contribution, we further discuss the remaining issues associated with beam indication.

- Beam indication for PDSCH

For beam indication for PDSCH, the following was agreed at the RAN1#91 meeting [1], which defined a general signaling framework.

Agreement: - When the scheduling offset is <=k, and the PDSCH uses QCL assumption that is based on a default TCI state - The default TCI state corresponds to the TCI state used for control channel QCL indication for the lowest CORESET ID in that slot Agreement: - A candidate set of DL RSs are configured using RRC mechanism - Each state of M TCI states is RRC configured with a downlink RS set used as a QCL reference, and MAC-CE is used to select up to 2^N TCI states out of M for PDSCH QCL indication - The same set of M TCI states are reused for CORESET - K TCI states are configured per CORESET - When K>1, MAC CE can indicate which one TCI state to use for control channel QCL indication - When K=1, no additional MAC CE signaling is necessary

As the TCI field is always present in the DCI associated to PDSCH scheduling, if the CORESET is configured with Is-TCI-Present=TRUE, it takes time for UE to decode the DCI, identify the Rx beams for reception based on TCI, and then switch Rx beams. When scheduling offset is less than the threshold, UE shall refer to the TCI state used for control channel QCL indication for the lowest CORESET ID in that slot. However, the number of TCI states may be larger than one for the lowest CORESET ID based on the agreements. Thus, there will be an ambiguity for UE when to refer the default TCI state. One straightforward way is to limit the number of TCI states configured for the CORESET with lowest ID to 1. Therefore, we have a proposal as follows:

Proposal 1: Limit the number of TCI states of the CORESET with the lowest ID to 1:

- K(n) TCI states are configured for each CORESET, n = 0, 1, ..., N-1 by RRC signaling, where n is the CORESET index

- For the CORESET n = 0, UE is expected to be configured with only one TCI state, i.e., K (0) = 1

- MAC CE timing design for beam indication

At the RAN1 # AH 1801 meeting [2], following agreements have been made to clarify MAC CE timing for downlink beam indication.

Agreement: Include in LS to RAN2 in response to R1-1800003: “These are candidate values that RAN1 is considering.” Downlink-related: - Time between the ACK transmission for the PDSCH carrying the MAC-CE activation message and the time that the UE is able to apply the QCL assumption carried by the MAC-CE message. Following is based on assumption of 120 kHz SCS for slot duration. RAN1 will down select a single value per SCS among the following candidate values which may be based on UE capability. - For PDSCH - Candidate values (slots): 8, 10, 20, 40, 80 (considering the minimum 10 ms periodicity for TRS in case of 80 slots) - For PDCCH: - Candidate values (slots): 8, 10, 20, 40, 80 - For SP-CSI-RS: - Candidate values (slots): 8, 10, 20, 40, 80 Uplink-related: - FFS

Similarly, in the UL, MAC-CE may be used for selecting the spatial QCL assumption for PUCCH transmission. For all these procedures involving MAC-CE signalling, there is ambiguity on application timing of the MAC-CE indication that should be solved. The timeline design applied for PDSCH/PDCCH beam indication that the time of application of QCL assumption is based on the ACK transmission can be a reference. However, as the beamforming for PUCCH based on the spatial relation indication is implemented in UE side, a shorter time duration is needed for PUCCH compared to downlink channels. Based on the discussion above, we have following proposals:

Proposal 2: Time between the ACK transmission for the PDSCH carrying the MAC-CE activation message and the time that the UE is able to apply the QCL assumption carried by the MAC-CE message. Following is based on assumption of 120 kHz SCS for slot duration. RAN1 will down select a single value per SCS among the following candidate values which may be based on UE capability.

- For PUCCH:

  - Candidate values (slots): 4, 8, 10, 20, 40

- Text proposal for CSI reporting configurations

At the RAN1 #90b meeting[3], folloing agreements were agreed:

Agreements: - For non-grouping based beam reporting, support the following reports parameters: - Maximal number of configured Tx beams for beam measurement: K equals 64 - Maximal number of configured Tx beams to be reported in one instance: N_max = 2, 4 where a subset of N (N<=N_max where N = 1, 2,3,4) beams can be selected by the gNB and indicated to the UE (FFS signaling mechanism) - Reporting differential L1-RSRP when multiple beams are reported in one reporting instance. Reference is the largest L1-RSRP in that reporting instance. FFS other reference for differential reporting. - FFS applicable reporting channels and number of beams, and associated reporting contents - FFS: the UE adjusts the L1-RSRP of multiple RS resources according to the power offset between them - Bit-width: 7bit for L1-RSRP ranging from -140dBm to -44dBm with 1dB stepping size (analogous with LTE) and 4bit for differential L1-RSRP FFS stepping size of differential quantization

Based on the highlighted part in the agreements, a subset of N beams can be selected by the gNB and indicated to the UE. However, this part has not been captured into the lastest specification [4]. Therefore, we propose to add the following setence in 3GPP TS 38.214 section 5.2.1.4 to capture the RAN 1 agreements.

3GPP TS 38.214 5.2.1.4 Reporting configurations if the UE is configured with the higher layer parameter group-based-beam-reporting set to 'OFF', the UE is not required to update measurements for more than 64 [CSI-RS and or SSB] resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different [CRI and SSBRI (SSB Resource Indicator)] and part of the nrofReportedRS (higher layer configured) different [CRI and SSBRI (SSB Resource Indicator)] can be indicated to the UE by [signalling] for each report setting. If the higher layer parameter nrofReportedRS is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size.

- Summary

In this contribution, we provide further views on beam management for NR. Following is our proposals.

Proposal 1: Limit the TCI state number of the CORESET with the lowest ID to be 1:

- K(n) TCI states are configured for each CORESET, n = 0, 1, ..., N-1 by RRC signaling

- For the CORESET n = 0, UE is expected to be configured with only one TCI state, i.e., K (0) = 1

Proposal 2: Time between the ACK transmission for the PDSCH carrying the MAC-CE activation message and the time that the UE is able to apply the QCL assumption carried by the MAC-CE message. Following is based on assumption of 120 kHz SCS for slot duration. RAN1 will down select a single value per SCS among the following candidate values which may be based on UE capability.

- For PUCCH:

  - Candidate values (slots): 4, 8, 10, 20, 40

References

[1]	3GPP, “Draft Report of 3GPP TSG RAN WG1 #91”, Nov. 2017

[2]	3GPP, “Draft Report of 3GPP TSG RAN WG1 AH #1801”, Jan. 2018

[3]	3GPP, “Draft Report of 3GPP TSG RAN WG1 #90b”, Oct. 2017

[4]	3GPP TS 38.214 V15.0.1 (2018-02)

Appendix

Change request for 3GPP TS 38.214 V15.0.1 (2018-02)

5.2.1.4	Reporting configurations

…unraleted part….

When the UE is configured with higher layer parameter ResourceSetConfig a CSI-RS resource set and when the higher layer parameter CSI-RS-ResourceRep is set to 'OFF', the UE shall determine a CRI from the supported set of CRI values as defined in Subclause [TBD] of [5, TS 38.212] When the higher layer parameter CSI-RS-ResourceRep is set to 'ON', UE is not expected to receive reporting configuration except “No report”.

…unraleted part….

- if the UE is configured with the higher layer parameter group-based-beam-reporting set to 'OFF', the UE is not required to update measurements for more than 64 [CSI-RS and or SSB] resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different [CRI and SSBRI (SSB Resource Indicator)] and part of the nrofReportedRS (higher layer configured) different [CRI and SSBRI (SSB Resource Indicator)] can be indicated to the UE by [signalling] for each report setting. If the higher layer parameter nrofReportedRS is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size. If the higher layer parameter nrofReportedRS is configured to be larger than one, the UE shall use largest L1-RSRP and differential L1-RSRP based reporting, where the largest value of L1-RSRP uses a 7-bit value and the differential L1-RSRP uses a 4-bit value. The differential L1-RSRP values are computed with 2 dB step size with a reference to the largest L1-RSRP value which is part of the same L1-RSRP reporting instance.