3GPP TSG RAN WG1 Meeting #92							R1-1802483

Athens, Greece, February 26th – March 2nd, 2018

Source:	NTT DOCOMO, INC.

Title:	PUCCH structure in short-duration

Agenda Item:	7.1.3.2.1

Document for: 	Discussion and Decision

- Introduction

In this contribution, remaining issue for short-PUCCH is discussed.

- Base-sequence hopping

  - Slot-level vs. frequency-hop-level

In RAN1 #91, slot-level base-sequence hopping was agreed, and whether and how to support symbol-level base sequence hopping was FFS. As shown in figure 1, in LTE PUCCH, the base-sequence was hopped on each f frequency-hop. Even if one of the frequency-hops has the same base sequence with around cells, the other frequency hop can be different. In order to maintain a robustness of inter-cell interference with LTE PUCCH, we believe the base sequence should be hopped on each frequency-hop.

Fig. 1  Base-sequence hopping of LTE/NR PUCCH.

In RAN1 AH#1801, we proposed frequency-hop-level base-sequence hopping [1]. In the proposed base-sequence hopping, base-sequence is hopped by frequency hop; as shown in figure 2, the base-sequence index of 1st hop and 2nd hop is calculated according to the index of each frequency hop within the PUCCH on top of slot index. If frequency hopping is enabled, the base-sequence index of 1st hop and 2nd hop is used for 1st hop and 2nd hop, and if frequency hopping is disabled, the base-sequence index of 1st hop is used. An example of proposed frequency hop-level base-sequence hopping is shown in figure 3.

Proposal 1:

- For short-PUCCH and long-PUCCH, base-sequence is hopped by frequency hop.

  - The base-sequence index of 1st hop and 2nd hop is calculated according to the index of each frequency hop within the PUCCH on top of slot index.

    - If higher layer parameter PUCCH-frequency-hopping = enabled, the base-sequence index of 1st hop and 2nd hop is applied to each frequency hopping of the PUCCH in the slot.

    - If higher layer parameter PUCCH-frequency-hopping = disabled, the base-sequence index of 1st hop is applied to the whole PUCCH in the slot.

Fig. 2  Proposed frequency hop-level base-sequence hopping.

Fig. 3  Example of proposed frequency hop-level base-sequence hopping.

Therefore, our proposal is to change the text TS38.211 [2] section 6.3.2.2.1 as following:

#### 6.3.2.2.1 Group and sequence hopping The sequence group and the sequence number within the group depends on the higher-layer parameter PUCCH-GroupHopping: - if PUCCH-GroupHopping equals ‘neither’ where is given by [5, TS 38.213]. - if PUCCH-GroupHopping equals ‘enabled’ fgh=m=072mc82nsμ+nhop+mmod30 fss=nIDmod30 v=0 where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by [5, TS 38.213], and nhop=0, 1 is a frequency hopping index of each slot given by, - nhop = 0 if PUCCH-frequency-hopping = disabled - nhop = 0 if PUCCH-frequency-hopping = enabled, and if 1st frequency hopping - nhop = 1 if PUCCH-frequency-hopping = enabled, and if 2nd frequency hopping - if PUCCH-GroupHopping equals ‘disabled’ where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by [5, TS 38.213].

- Conclusion

In this contribution, we discussed remaining issue for short-PUCCH. Following proposals were made:

Proposal 1:

- For short-PUCCH and long-PUCCH, base-sequence is hopped by frequency hop.

  - The base-sequence index of 1st hop and 2nd hop is calculated according to the index of each frequency hop within the PUCCH on top of slot index.

    - If higher layer parameter PUCCH-frequency-hopping = enabled, the base-sequence index of 1st hop and 2nd hop is applied to each frequency hopping of the PUCCH in the slot.

    - If higher layer parameter PUCCH-frequency-hopping = disabled, the base-sequence index of 1st hop is applied to the whole PUCCH in the slot.

#### 6.3.2.2.1 Group and sequence hopping The sequence group and the sequence number within the group depends on the higher-layer parameter PUCCH-GroupHopping: - if PUCCH-GroupHopping equals ‘neither’ where is given by [5, TS 38.213]. - if PUCCH-GroupHopping equals ‘enabled’ fgh=m=072mc82nsμ+nhop+mmod30 fss=nIDmod30 v=0 where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by [5, TS 38.213], and nhop=0, 1 is a frequency hopping index of each slot given by, - nhop = 0 if PUCCH-frequency-hopping = disabled - nhop = 0 if PUCCH-frequency-hopping = enabled, and if 1st frequency hopping - nhop = 1 if PUCCH-frequency-hopping = enabled, and if 2nd frequency hopping - if PUCCH-GroupHopping equals ‘disabled’ where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by [5, TS 38.213].

References

- R1-1801269, ‘WF on base-sequence hopping for PUCCH (updated),’ NTT DOCOMO, INC.

- R1-1801291, ‘draftCR to 38.211 capturing the Jan18 ad-hoc meeting agreements,’ Ericsson.