3GPP TSG RAN WG1 Meeting #92			R1-1802606

Athens, Greece, February 26th – March 2nd, 2018

Agenda item:	7.4.4

Source: 	AT&T

Title: 	Link level simulation assumption for NOMA evaluations

Document for:	Discussion/Decision

# Introduction

In RAN #71, a new study item New Radio (NR) Access Technology was approved.  The technical specifications were agreed during the last RAN meeting. However, during the initial phase of NR SI, RAN1 studied on non-orthogonal multiple access (NOMA). Due to the necessity of completion of more urgent features, NOMA was delayed in NR. The main objectives of NOMA study are

Non-orthogonal multiple transmission scheme

- Transmitter side signal processing schemes for non-orthogonal multiple access [RAN1]:

  - Modulation and symbol level processing, including spreading, repetition, interleaving, new constellation mapping, etc.

  - Coded bit level processing including interleaving and/or scrambling, etc.

  - Symbol to resource element mapping, sparse or not, etc.

  - Demodulation reference signal. Other signal is not excluded.

- Receivers for non-orthogonal multiple access: [RAN1, RAN4]

  - MMSE receiver, successive/parallel interference cancellation (SIC/PIC) receiver, joint detection (JD) type receiver, combination of SIC and JD receiver, or other receivers

  - The study should consider performance, receiver complexity, etc.

- Procedures related to the non-orthogonal multiple access  [RAN1]

  - UL transmission detection

  - HARQ, including transmission scheme, feedback scheme, and combining scheme

  - Link adaptation MA signature allocation/selection

  - Synchronous and asynchronous operation

  - Adaptation between orthogonal and non-orthogonal multiple access

- Link and system level performance evaluation or analysis for non-orthogonal multiple access continued from performance metrics identified from Rel-14. The benchmark for comparison is OFDM contention based multiple access. Realistic modelling of Tx/Rx impairment including potential PAPR issue, channel estimation error, power control accuracy, collision, etc. should be considered. [RAN1]

  - Traffic model and Deployment scenarios of eMBB (small packet), URLLC and mMTC

  - Device power consumption

  - Coverage (link budget)

  - Latency and signalling overhead

  - BLER reliability, capacity and system load

  - Physical abstraction (link-to-system mapping model)

Note: targeting common solution for mMTC, URLLC and eMBB small packet.

In this contribution, we list the simulation assumptions for evaluating the potential gains of NOMA systems.

# 3. Link Level Simulation Assumptions

The tables below lists the baseline parameters to be used for link simulations for link throughput evaluations.  Other values may be used as well but they should be clearly described.

Table 1: Link level simulation parameters.

| Parameters | Value |
| --- | --- |
| Carrier Frequency | 2 and/or 4 GHz |
| Waveform (data part) | CP-ODM |
| Numerology (data part) | 15 KHz |
| Channel Coding | LDPC with 50 decoding iterations |
| Allocated bandwidth | 4 or 6 RB as baseline, 12 RB as optional |
| Target per UE spectral efficiency | [0.1-0.5] |
| Target BLER for one transmission | 10%, 0.1% |
| Number of UEs multiplexed in the same allocated bandwidth | 2, 4,8 |
| BS antenna configuration | 4 Rx as baseline 32 Rx as optional |
| UE antenna configuration | 1 Tx |
| Propagation channel & UE velocity | TDL-A 30ns and TDL-C 300ns in TR38.901, 3km/h |
| Max number of HARQ transmission | 1 |
| Channel estimation | Realistic channel estimation |
| MA signature allocation (for data) | Fixed/Random |
| DMRS allocation | Fixed |
| Timing/frequency offset | 0 |
| Receiver algorithm | MMSE-IRC as the baseline, advanced receivers |

# 3.  Conclusions

In this contribution, we listed baseline link level simulation assumptions for evaluating the performance of NOMA systems.