3GPP TSG RAN WG1 Meeting #92	R1-1802736

Athens, Greece, 26th – 30th February, 2018

Source:	Ericsson

Title:	Corrections on CW mapping and data scrambling

Agenda Item:	7.1.2.1.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss minor corrections related to CW mapping and data scrambling. The corrections and proposals are based on the latest version of TS 38.211.

# Issues related transport block to codeword mapping

In TS 38.212 there is currently no description on when DCI contains a disabled transport block. Furthermore, there is also no definition on how a disabled transport block is indicated.  Hence, we propose to add the same principles as in LTE. See the proposed changes in red below.

## Text proposal for 7.3.1.2.2 of TS38.212

- […]

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Subclause x.x of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Subclause x.x of [6, TS38.214]

For transport block 2:

-	Modulation and coding scheme – 5 bits as defined in Subclause x.x of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Subclause x.x of [6, TS38.214]

-	HARQ process number – 4 bits

-	Downlink assignment index – number of bits as defined in the following

-	4 bits if the higher layer parameter HARQ-ACK-codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	0 bits otherwise.

-	TPC command for scheduled PUCCH – 2 bits as defined in Subclause x.x of [5, TS38.213]

-	PUCCH resource indicator – 2 bits as defined in Subclause x.x of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Subclause x.x of [5, TS38.213]

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Subclause x.x of [6, TS38.214].

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-5.

-	CBG transmission information –0, 2, 4, 6, or 8 bits as defined in Subclause x.x of [6, TS38.214], determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for the PDSCH.

-	CBG flushing out information –0 or 1 bit as defined in Subclause x.x of [6, TS38.214], determined by higher layer parameter codeBlockGroupFlushIndicator.

-	DMRS sequence initialization – 1 bit if transform precoding is enabled

In case the higher layer parameter Number-MCS-HARQ-DL-DCI indicates that two codeword transmission is enabled and if both transport blocks are enabled, transport block 1 is mapped to codeword 0 and transport block 2 is mapped to codeword 1. In case one of the transport blocks is disabled as specified in section 5.1.3.2 of [TS38.214], the transport block to codeword mapping is specified according to Table X1

Table X1: Transport block to codeword mapping
(one transport block enabled).

| transport block 1 | transport block 2 | codeword 0 (enabled) | codeword 1 (disabled) |
| --- | --- | --- | --- |
| enabled | disabled | transport block 1 | - |
| disabled | enabled | transport block 2 | - |

## Text proposal for 5.1.3.2 of TS38.214

In case the higher layer parameter Number-MCS-HARQ-DL-DCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if  and if rvid=1 for the corresponding transport block, otherwise the transport block is enabled.

For the PDSCH assigned by a PDCCH with DCI format 1_0/1_1 with CRC scrambled by C-RNTI,

if the higher layer parameter MCS-Table-PDSCH is set to '256QAM' is configured and  0 ≤ IMCS ≤27, or the higher layer parameter MCS-Table-PDSCH is not set to '256QAM' configured and  0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

[…]

# Issues related to data scrambling

The nID in the scrambling seed should equal the configured value only for some RNTIs (the C-RNTI and other RNTIs unique to a UE like the CS-RNTI.

## Text proposal for DL (Section 7.3.1.1)

>>>>>>>>>>>> Start text proposal >>>>>>>>>>>>

Up to two codewords can be transmitted, . In case of single-codeword transmission, .

For each codeword , the UE shall assume the block of bits , where  is the number of bits in codeword  transmitted on the physical channel, are scrambled prior to modulation, resulting in a block of scrambled bits according to

where the scrambling sequence  is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

where

-	 equals the higher-layer parameter Data-scrambling-Identity if configured and the RNTI equals the C-RNTI or CS-RNTI.

-	 otherwise

and where  corresponds to the RNTI associated with the PDSCH transmission as described in clause 5.1 of [TS 38.214,6].

>>>>>>>>>>>> End text proposal >>>>>>>>>>>>

## Text proposal for UL (Section 6.3.1.1)

The nID in the scrambling seed should equal the configured value only when the UE identity is known to the network . Hence, it should not be used for Temporary C-RNTI as when this is used, the network does not know whether the UE has been configured a scrambling seed (e.g by using LTE in non-standalone access). Hence, we suggest the following amendment:

>>>>>>>>>>>> Start text proposal for Section 6.3.1.1  >>>>>>>>>>>>

- […]

where x and y are tags defined in [4, TS38.212] and where the scrambling sequence  is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

where

-	 equals the higher-layer parameter Data-scrambling-Identity if configured and the RNTI equals any of the following RNTIs: C-RNTI, SP-CSI-RNTI or CS-RNTI

-	 otherwise

and where  corresponds to the RNTI associated with the PUSCH transmission as described in clause 6.1 of [TS 38.214,6].

>>>>>>>>>>>> End text proposal >>>>>>>>>>>>