3GPP TSG-RAN WG1 Meeting #92	R1-1802829

Athens, Greece,

February 26th- March 2nd, 2018

Agenda item:	7.1.2.3.4

Source: 	Qualcomm Incorporated

Title: 	PT-RS Considerations

Document for:	Discussion/Decision

# Introduction

In this contribution, we present our proposals to maintain the PT-RS related sections in the Release 15 standards.

# On the PT-RS symbol mapping rule

In the previous meeting, the following updates on the text to determine the PT-RS symbol index was agreed [1].

Agreement: The text in 7.4.1.2.2 of 38.211 is changed as below. Note: the text in 6.4.1.2.2.1 of 38.211 should also be changed in the same manner. The set of time indices defined relative to the start of the PDSCH allocation is defined by - set and 2. if any symbol from to overlaps with a symbol used for DM-RS according to clause 7.4.1.1.2 - set - set to the number of the last DM-RS symbol in a sequence of time-contiguous DM-RS occasions - repeat from step 2 above as long as is inside the PDSCH allocation. 3. add to the set of time indices for PT-RS 4. increment by one 5. repeat from step 2 above as long as is inside the PDSCH allocation where . Agreement: Agree to the following text proposal for section 6.4.1.2.2.2 of TS38.211 - The set of time indices l defined relative to the start of the PUSCH allocation is defined by 1. set i=0 and lref=0 2. if any symbol from max(lref+(i-1)LPTRS+1, lref) to lref+iLPTRS overlaps with a symbol used for DM-RS according to clause 6.4.1.1.2 - set i=1 - set lref to the number of the last DM-RS symbol in a sequence of time-contiguous DM-RS occasions - repeat from step 2 above as long as lref+iLPTRS is inside the PUSCH allocation. 3. add lref+iLPTRS to the set of time indices for PT-RS 4. increment i by one 5. repeat from step 2 above as long as lref+iLPTRS is inside the PUSCH allocation where is given by the higher-layer parameter UL-PTRS-time-density-transform-precoding.

Regarding the PT-RS symbol index mapping, however, we observe that additional clarifications are still required, at least for the following scenarios.

## PT-RS puncturing

The following agreement has been made that PT-RS is punctured in the REs colliding with certain reference signals or PDCCH [1].

Agreement: - Agree to the following text proposal for Section 7.4.1.2.2 If present, the UE shall assume the PDSCH PT-RS being mapped to resource elements according to when all the following conditions are fulfilled - is within the OFDM symbols allocated for the PDSCH transmission - resource element is not used for DM-RS, CSI-RS, SS/PBCH or by a configured CORESET

If LPT-RS>1, we find that the current PT-RS symbol mapping rule does not work well in all cases when

- PT-RS tones are punctured in certain symbols, e.g. due to the collision with CSI-RS, SS/ PBCH, or PDCCH in a configured CORESET, or

- the data symbols associated with the PT-RS occupy non-contigious location in a slot, e.g. when the slot is punctured by mini-slots of other users in the middle, or the data assignment consists of separate mini-slots.

In both cases of (i) and (ii), based on the current PT-RS puncturing rule, the actual time density of the remaining PT-RS can be significantly smaller than the intended density indicated by the MCS. For instance, assume LPT-RS=4, and the frame structure as shown in Fig. 1, where the PDSCH overlaps with CSI-RS or another user’s mini-slots over the full scheduled bandwidth in symbol 4 and 5. In this case, applying the latest mapping rule, PT-RS is first mapped to symbol 5 and 9, and then punctured in symbol 5. Namely, PT-RS remains only in symbol 9 after puncturing. Therefore, we expect large errors in the phase tracking for the symbols in the middle, e.g. symbol 6 and 7; as their CPE estimation will be interpolated over a larger gap of  8 symbols.

Figure 1. Example where PT-RS is punctured due to collision with CSI-RS.

Observation 1: After puncturing PT-RS tones, the remaining time domain density may be lower than the intended value, which may cause degradation in decoding performance.

Proposal 1: The PT-RS symbol mapping rule should take account for the case where the data symbols are allocated in non-contiguous locations in a slot due to collision with other reference signals and mini-slots from other users.

Besides the fully overlapping scenario described in Figure 1, if most of the PDSCH RBs collides with CSI-RS or SSB, and a large number of PT-RS tones are punctured accordingly, the phase tracking based on the remaining few PT-RS tones will be sensitive to thermal noise. Therefore, it would be better to avoid mapping PT-RS on the data symbols with a small non-overlapping bandwidth or few remaining PT-RS tones after potentially puncturing.

Proposal 2: PT-RS should be mapped only to the symbols with at least X RBs of PDSCH/ PUSCH that do not overlap with other reference signals and other UE’s assignment causing PT-RS puncturing; or equivalently, Y remaining PT-RS tones after potential puncturing.

- For simplicity, set X=[NRB0] in the Table 6.2.3-2 in 38.214 below; or Y=[1].

| Scheduled bandwidth | Frequency density () |
| --- | --- |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

## Proposed solution

Finally, in this section, we provide a simple rule for PT-RS symbol mapping that accounts for all the aforementioned concerns below. It can be easily checked that the proposed rule provides the same mapping as the current rule when there is no PT-RS puncturing or if the data are assigned to contiguous symbols.

Proposal 3: Starting from the first symbol containing PD(U)SCH RBs, a symbol is mapped with a PT-RS if

- At least X of its PD(U)SCH RBs do not overlap with other reference signals and other UEs’ assignment causing PT-RS puncturing

  - X=[NRB0 ] in the Table 6.2.3-2 in 38.214

- All its previous LPT-RS-1 symbols, if exist, do not contain DMRS or PT-RS as reference for its phase tracking

  - LPT-RS is symbol-specific, and determined by the MCS of the symbol.

Further, we provide the following pseudo code to reflect the proposed change in the mapping rule

The set of time indices defined relative to the start of the PDSCH allocation is defined by - lref=0 - if symbol lref satisfies all the conditions below: - there are at least X RBs of PDSCH that do not overlap with other reference signals and other UEs’ assignment causing PT-RS puncturing; - all its previous LPTRS-1 symbol do not contain DM-RS or PT-RS Then add lref to the set of time indices for PT-RS; - lref=lref+1; - Repeart Step 2-3 till lref exceeds than lmax, where lmax is the index of the last symbol containing PDSCH RBs in the slot.

In addition, the related change should be done both in DL and UL.

# PT-RS time domain density for PUSCH with UCI only

Now we discuss the method to determine the PT-RS time domain density for PUSCH with UCI only. As the MCS of the UCI-only PUSCH case may vary from BPSK to higher modulations. It is natural to support variable PT-RS time domain density depending on the MCS (modulation order and/ or coding rate) rather than a fixed density, as in the PUSCH slots containing regular data.

Proposal 4: Support variable PT-RS time domain density for UCI-only PUSCH.

To support the variable PT-RS time domain density, we have considered the following design options:

- Determine the PT-RS time density based on the MCS of the UCI-only PUSCH, e.g. based on the Table 1, where the thresholds ptrs-ModOrder are configured by RRC signalling;

- Joinly decide the PT-RS density and PUSCH modulation order based on the code rate calculated at the UE. Details on how to calculate the code rate can be found in our proposal [2].

- Based on the code rate calculated at the UE, find the closest MCS in the corresponding PUSCH MCS table (Table 6.1.4.1-1 in 38.214 for DFT-sOFDM, and Table 5.1.3.1-2 in 38.214 for CP-OFDM) with its code rate higher than the calculated code rate; and then determine the PT-RS time density based on Table 6.2.3-1 in 38.214.

| Scheduled Modulation Order | Time density () |
| --- | --- |
| IMCS < ptrs-ModOrder1 | PT-RS is not present |
| ptrs- ModOrder1 IMCS < ptrs- ModOrder2 | 4 |
| ptrs- ModOrder2 IMCS < ModOrder3 | 2 |
| ModOrder3 IMCS < ModOrder4 | 1 |

Table 1. PT-RS time domain density as a function of the Modulation Order in Option 1.

Since the determination of the PT-RS density relates to UE specific PN property, it appears that Option 1 and 2 requires additional overhead for UE to report its recommendation of PT-RS density, and for gNB to configure the thresholds in the mapping table. Therefore, we prefer Option 3 which reuses the existing mechanism, and does not require extra overhead.

Proposal 5: Support Option 3 to determine PT-RS time domain density for UCI-only PUSCH.

# On the PTRS collision with CORESET

Based on the current 38.211, a PT-RS RE is punctured if it collides with a configured CORESET as:

If present, the UE shall assume the PDSCH PT-RS being mapped to resource elements according to when all the following conditions are fulfilled - is within the OFDM symbols allocated for the PDSCH transmission - resource element is not used for DM-RS, CSI-RS, SS/PBCH, or by a configured CORESET

When grant level rate matching is enabled in a configured CORESET, we find it sufficient to puncture the PT-RS only in the PDCCH that schedule the PDSCH transmission, rather than over the whole CORESET; as unnecessarily reducing the number of PT-RS tones may lead to performance degradation due to larger phase tracking errors. Therefore we have the following proposal.

Proposal 6: PT-RS is not punctured over the CORESET but rather than in the CORESET RBs carrying PDCCH that schedules PDSCH, if grant level rate matching is enabled.

Further, we propose the following text change in 38.211

resource element is not used for DM-RS, CSI-RS, SS/PBCH, or by a configured CORESET is not declared as not available for PDSCH due to a detected PDCCH in a CORESET that scheduled the PDSCH or a configured CORESET.

# On the sequence generation of PTRS for DFT-s-OFDM

In 38.211, when transform precoding is enabled, the sequence generation is defined as:

If transform precoding is enabled, the phase-tracking reference signal shall be generated according to . where the pseudo-random sequence is defined in clause 5.2.1 and is given by Table 6.4.1.2.1.2-1. The pseudo-random sequence generator shall be initialized with where is the lowest OFDM symbol number in the PUSCH allocation in slot that contains PT-RS according to clause 6.4.1.2.2.2 and is given by the higher-layer parameter nDMRS-CSH-Identity-Transform-precoding.

We notice that the RRC parameter nDMRS-CSH-Identity-Transform-precoding has been revmoved in the latest standards, and therefore propose to use nPUSCH-Identity-Transform-precoding instead.

Proposal 7: In 38.211, update the sequence generation of PT-RS for DFT-s-OFDM as follows:

where is the lowest OFDM symbol number in the PUSCH allocation in slot that contains PT-RS according to clause 6.4.1.2.2.2 and is given by the higher-layer parameter nDMRS-CSH-Identity-Transform-precoding nPUSCH-Identity-Transform-precoding.

# Conclusions

Observation 1: After puncturing PT-RS tones, the remaining time domain density may be lower than the intended value, which may cause degradation in decoding performance.

Proposal 1: The PT-RS symbol mapping rule should take account for the case where the data symbols are allocated in non-contiguous locations in a slot due to collision with other reference signals and mini-slots from other users.

Proposal 2: PT-RS should be mapped only to the symbols with at least X RBs of PDSCH/ PUSCH that do not overlap with other reference signals and other UE’s assignment causing PT-RS puncturing; or equivalently, Y remaining PT-RS tones after potential puncturing.

- For simplicity, set X=[NRB0] in the Table 6.2.3-2 in 38.214 below; or Y=[1].

| Scheduled bandwidth | Frequency density () |
| --- | --- |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

Proposal 3: Starting from the first symbol containing PD(U)SCH RBs, a symbol is mapped with a PT-RS if

- At least X of its PD(U)SCH RBs do not overlap with other reference signals and other UEs’ assignment causing PT-RS puncturing

  - X=[NRB0] in the Table 6.2.3-2 in 38.214

- All its previous LPT-RS-1 symbols, if exist, do not contain DMRS or PT-RS as reference for its phase tracking

  - LPT-RS is symbol-specific, and determined by the MCS of the symbol.

Further, we provide the following pseudo code to reflect the proposed change in the mapping rule

The set of time indices defined relative to the start of the PDSCH allocation is defined by - lref=0 - if symbol lref satisfies all the conditions below: - there are at least X RBs of PDSCH that do not overlap with other reference signals and other UEs’ assignment causing PT-RS puncturing; - all its previous LPTRS-1 symbol do not contain DM-RS or PT-RS Then add lref to the set of time indices for PT-RS; - lref=lref+1; - Repeart Step 2-3 till lref exceeds than lmax, where lmax is the index of the last symbol containing PDSCH RBs in the slot.

Proposal 4: Support variable PT-RS time domain density for UCI-only PUSCH.

Proposal 5: Support Option 3 to determine PT-RS time domain density for UCI-only PUSCH.

Proposal 6: PT-RS is not punctured over the CORESET but rather than in the CORESET RBs carrying PDCCH that schedules PDSCH, if grant level rate matching is enabled.

Further, we propose the following text change in 38.211

resource element is not used for DM-RS, CSI-RS, SS/PBCH, or by a configured CORESET is not declared as not available for PDSCH due to a detected PDCCH in a CORESET that scheduled the PDSCH or a configured CORESET.

Proposal 7: In 38.211, update the sequence generation of PT-RS for DFT-s-OFDM as follows:

where is the lowest OFDM symbol number in the PUSCH allocation in slot that contains PT-RS according to clause 6.4.1.2.2.2 and is given by the higher-layer parameter nDMRS-CSH-Identity-Transform-precoding nPUSCH-Identity-Transform-precoding.

# References

Chairman's Notes, 3GPP TSG RAN WG1 Meeting AH 1801, Vancouver, Canada, Jan. 22nd -26th, 2018.

R1-1802839, Remaining issues for multiplexing UCI on PUSCH, 3GPP TSG RAN WG1 Meeting #92, Athens, Greece, February 26th - March 2nd, 2018.