3GPP TSG-RAN WG1 Meeting #92	Tdoc R1-1802883

Athens, Greece, 26th Feb – 2nd March, 2018

Agenda Item:	6.2.7.3

Source:	Ericsson

Title:	Reliability for URLLC

Document for:	Discussion

# Introduction

In this paper we discuss the expression for total reliability of a sequence of transmissions.

# Discussion

We will here consider both HARQ-based (see Figure 1) and automatic repetitions (see Figure 2) and the related physical channels.

Figure 1. HARQ-based retransmission.

Figure 2. Automatic repetitions.

## Total reliability expressions

With a few exceptions, the discussion here assumes that the retransmissions are uncorrelated, which is reasonable to assume if e.g. retransmissions are done on a different frequency allocation. In the following we will write the success probabilities on the channel level according to Table 1.

Table 1: Success probabilities for calculating total reliability

| Probability | Description |
| --- | --- |
| p0 | SR |
| p1 | (S)PDCCH |
| p2 | PDSCH/PUSCH |
| p3 | (S)PUCCH NACK detection |
| p4 | (S)PUCCH DTX detection |

DL data, HARQ-based

In the DL we can describe the total reliability after N transmission as:

pt=n=1Np1p2,n1-p1p4+p1p31-p2,nn-1

Where p2,n is the probability of a data block correctly received after n transmissions are soft combined.

DL data, automatic repetitions

For the case of automatic repetitions, we can consider two cases, either with one DL control per DL repetition, or only one DL control. Based on the above expression we derive the following expressions:

Single-DCI repetition:
pt=p1(1-1-p2,1N)

for the assumption of independent repetitions. However, it is more reasonable to see the repetitions as parts of one continuous transmission, also taking soft combining into account, giving instead

pt=p1p2,N

Multi-DCI repetition: 
pt=1-(1-p1p2,1)N

for the assumption of independent repetitions. Again, if we don’t see the transmissions as independent, and taking into account that the DL control is not soft combined we can use the expression

pt=n=1Np1,np2,n

Where p1,n is the probability of correctly decoding n DL control transmissions.

UL data, SR-based

With SR-based UL scheduling the total reliability can be described as:

pt=(1-1-p0M)(1-(1-p1p2,n)N)

for a repetition of M times SR, and N times UL grant and data transmissions/retransmissions.

UL data, SPS-based

With SPS-based scheduling instead we remove the SR step (p0 = 1) and the total reliability can be described as:

pt=1-1-p2,11-p1p2,nN-1

Assuming perfect energy detection performance on the PUSCH resource.

UL data, SPS-based repetition

For the case of SPS-based automatic repetitions, p1 = 1, and we have the following expression:

pt=1-1-p2,1N

which only relies on the data channel performance. As for the case of DL repetition we can instead consider the repetitions as part of a longer transmission:

pt=p2,N

This relies on the fact that the BS can detect the start and end of a repetition sequence. This is possible for instance if a certain DMRS pattern is configured such that the presence of a DMRS in a symbol indicates a certain position in the repetition sequence.

## Expressions for studied cases

In a companion paper [2] we look at the possible combinations given the latency targets. We can now relate these solutions to expressions for total error rate. This is shown in Table 1 for the 1ms target, in Table 2 for the 10ms target for FDD, and in Table 3 for the 10ms target in TDD.

A few observations can be done on the solutions and their error expressions;

- subslot-PUCCH performance (p0, p3 or p4) does not appear in any equation.

- SPS-based UL is required for the 1ms target.

- HARQ repetition is only required for 1ms and slot.

- Automatic repetition is required for 1ms target.

- DL repetition is limited by PDCCH performance.

Table 2. Solutions for 1ms target, FDD.

|  | DL | UL SPS | UL SR |
| --- | --- | --- | --- |
| Case 1 | One-shot transmission, 2/3os sTTI: pt=p1p2,1 | One-shot transmission, 2/3os sTTI: pt=p2,1 | - |
| Case 2a (3os proc.) | One repetition (k=2), 2/3os sTTI: pt=n=12p1,np2,n | One repetition (k=2), 2/3os sTTI: pt=p2,2 | - |
| Case 2b (2os proc.) | Two repetitions (k=3), 2/3os sTTI: pt=n=13p1,np2,n | Two repetitions (k=3), 2/3os sTTI: pt=p2,3 | - |

Table 3. Solutions for 10ms target, FDD.

|  | DL | UL SPS | UL SR |
| --- | --- | --- | --- |
| Case 1 | One HARQ retransmission, 1ms TTI: pt=2p1p2,n(1-p1p4+p1p3(1-p2,n))n-1 | One HARQ retransmission, 1ms TTI: pt=1-(1-p2,1)(1-p1p2,2) | One-shot transmission, 1ms TTI: pt=p0p1p2,1 |
| Case 2 | 2 HARQ retransmissions, 7os sTTI: pt=3p1p2,n(1-p1p4+p1p3(1-p2,n))n-1 | 2 HARQ retransmissions, 7os sTTI: pt=1-(1-p2,1)(1-p1p2,2)(1-p1p2,3) | 1 HARQ retransmissions, 7os sTTI: pt=p0(1-1-p1p2,1(1-p1p2,2)) |

Table 4. Solutions for 10ms target, TDD.

|  | DL | UL SPS | UL SR |
| --- | --- | --- | --- |
| Case 1 | One-shot transmission, 1ms TTI: pt=p1p2,1 | One-shot transmission, 1ms TTI: pt=p2,1 | One-shot transmission, 7os sTTI: pt=p0p1p2,1 |
| Case 2 | 1 HARQ retransmissions, 7os sTTI: pt=2p1p2,n(1-p1p4+p1p3(1-p2,n))n-1 | 1 HARQ retransmissions, 7os sTTI: pt=1-(1-p2,1)(1-p1p2,2) | - |

Subslot-PUCCH is not required for reaching the studied targets

For 1ms target we require the use of SPS-based UL and automatic repetition in UL and DL.

DL repetition is limited by PDCCH performance.

# Conclusion

In Section 2 we made the following observations:

- Sub-slot PUCCH is not required for reaching the studied targets

For 1ms target we require the use of SPS-based UL and automatic repetition in UL and DL.

DL repetition is limited by PDCCH performance.

# References

RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #88bis, April 2017.

R1-1802882, Latency for URLLC.