3GPP TSG RAN WG1 Meeting#92	R1-1802979

Athens, Greece, 26nd Feb-2nd Mar 2018

Agenda Item:	7.7

Source:	Ericsson

Title:	IMT-2020 self-evaluation: UP latency in NR

Document for:	Information

# Introduction

The ITU target for user plane (UP) latency in IMT 2020 has been set to 1ms [1]. In this paper, we make an evaluation of the UP latency in NR with different configurations, and show that the target can be reached in both FDD and TDD. In the ITU target for reliability there is also a requirement on latency (1ms) during which the packet should be delivered with a certain probability. For this case, it is useful to evaluate how many retransmissions can be done within the latency limit.

We consider this as useful exercise already before NR WI is completed to check if latency is according to the requirements or if some changes are needed.

# Discussion

In the following we will analyse the worst-case UP latency after a first transmission and up to 3 retransmissions. We will follow the ITU definition [1] of UP latency as being defined from L2/L3 ingress to L2/L3 egress.

## Assumptions

Processing delay

For the UE, the processing delay is assumed to be according to processing delay capability 2, that is 3 symbols for 15 and 30kHz SCS. For 120kHz the capability has been assumed to be 9 symbols. In the gNB we have here assumed the same processing delay as in the UE, for simplicity. It is assumed that data can be delivered to higher layers after processing but before ACK feedback is transmitted.

Alignment delay

The alignment delay is the time required after being ready to transmit until transmission can start. We assume the worst-case latency meaning the alignment delay is assumed to the longest possible.

gNB timing

The minimum response timing in the gNB between SR and UL grant, and between DL HARQ and retransmission, is assumed to be n+1 TTI. For higher SCS and fewer symbols in the mini-slot, the TTI is shorter, and more TTIs should be used for processing. The used timing assumptions for gNB are given in Table 1.

UE timing

The minimum response timing in the UE between DL data and DL HARQ, and between UL grant and UL data, is assumed to be n+1 TTI. As for the gNB timing this value is higher for higher SCS and shorter mini-slot, see Table 1.

UL scheduling

For UL data, the scheduling can either be based on SR or SPS UL. In both cases it is assumed that the period is set to 1TTI.

Table 1. Timing assumptions.

| Timing | 15/30kHz SCS |  |  | 120kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| #TTI | 7os TTI | 4os TTI | 2os TTI | 7os TTI | 4os TTI | 2os TTI |
| gNB processing | 1 | 1 | 2 | 2 | 3 | 5 |
| UL timing | 1 | 1 | 2 | 2 | 3 | 5 |

TTI length and pattern

In this evaluation, we study slot lengths of 14 symbols and 7 symbols (mini-slot). Mini-slot length of 2 and 4 symbols are interesting scheduling options for low latency, but not studied here. For TDD an alternating DL-UL pattern has been assumed, to represent the most latency-optimized setup in a carrier.

Figure 1: illustration of latency components for DL and UL data.

## FDD

For the case of FDD the HARQ RTT is here n+3 TTI. The resulting UP latency for SCS of 30 and 120 kHz is shown in Table 1. As can be seen, the 1ms requirement can be reached for SCS 30kHz and up.

Table 2: FDD UP one-way latency for data transmission with HARQ-based retransmission.

| Latency (ms) | HARQ | 15kHz SCS |  |  | 30kHz SCS |  |  | 120kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7-os TTI | 4-os TTI | 2-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 1st transmission | 1.4 | 1.0 | 0.71 | 0.71 | 0.50 | 0.36 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 2.9 | 1.9 | 1.4 | 1.5 | 0.93 | 0.71 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 4.4 | 2.7 | 2.1 | 2.2 | 1.4 | 1.1 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 5.9 | 3.6 | 2.9 | 3.0 | 1.8 | 1.4 | 1.2 | 0.98 | 0.79 |
| UL data (SR) | 1st transmission | 2.5 | 1.6 | 1.4 | 1.3 | 0.82 | 0.68 | 0.54 | 0.46 | 0.38 |
|  | 1 retx | 4.4 | 2.7 | 2.1 | 2.2 | 1.4 | 1.1 | 0.91 | 0.73 | 0.59 |
|  | 2 retx | 5.9 | 3.6 | 2.9 | 3.0 | 1.8 | 1.4 | 1.2 | 0.98 | 0.79 |
|  | 3 retx | 7.4 | 4.4 | 3.6 | 3.7 | 2.2 | 1.8 | 1.5 | 1.2 | 0.98 |
| UL data (SPS) | 1st transmission | 1.4 | 1.0 | 0.71 | 0.71 | 0.50 | 0.36 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 2.9 | 1.9 | 1.4 | 1.5 | 0.93 | 0.71 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 4.4 | 2.7 | 2.1 | 2.2 | 1.4 | 1.1 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 5.9 | 3.6 | 2.9 | 3.0 | 1.8 | 1.4 | 1.2 | 0.98 | 0.79 |

NR FDD can fulfil the 1ms UP latency target with SCS above 15kHz.

HARQ retransmissions can be done within the latency target above 30kHz SCS.

## TDD

With TDD there are additional alignment delays caused by the sequence of subframes. Depending on when the data arrives in the transmit buffer the latency may be same or longer than the FDD latency. For a DL-UL pattern with HARQ RTT of n+4 TTI, the resulting latency is as indicated in Table 2. As can be seen in the table, the target can be reached with a SCS of 60kHz or higher for a 7-symbol mini-slot, and with 120kHz SCS with a 14-symbol slot. Also worth noting is that SPS gives significantly lower latency as compared to SR-based UL scheduling.

Table 3. TDD UP one-way latency for data transmission with alternating DL-UL pattern.

| Latency (ms) | HARQ | 15kHz SCS |  | 30kHz SCS |  | 120kHz SCS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7-os TTI | 4-os TTI | 7-os TTI | 4-os TTI | 7-os TTI | 4-os TTI |
| DL data | 1st transmission | 1.9 | 1.3 | 0.96 | 0.64 | 0.35 | 0.27 |
|  | 1 retx | 3.9 | 2.4 | 2.0 | 1.2 | 0.72 | 0.55 |
|  | 2 retx | 5.9 | 3.6 | 3.0 | 1.8 | 1.1 | 0.84 |
|  | 3 retx | 7.9 | 4.7 | 4.0 | 2.4 | 1.5 | 1.1 |
| UL data (SR) | 1st transmission | 3.5 | 2.2 | 1.8 | 1.1 | 0.67 | 0.53 |
|  | 1 retx | 5.9 | 3.6 | 3.0 | 1.8 | 1.1 | 0.84 |
|  | 2 retx | 7.9 | 4.7 | 4.0 | 2.4 | 1.5 | 1.1 |
|  | 3 retx | 9.9 | 5.9 | 5.0 | 2.9 | 1.8 | 1.4 |
| UL data (SPS) | 1st transmission | 1.9 | 1.3 | 0.96 | 0.64 | 0.35 | 0.27 |
|  | 1 retx | 3.9 | 2.4 | 2.0 | 1.2 | 0.72 | 0.55 |
|  | 2 retx | 5.9 | 3.6 | 3.0 | 1.8 | 1.1 | 0.84 |
|  | 3 retx | 7.9 | 4.7 | 4.0 | 2.4 | 1.5 | 1.1 |

NR TDD can fulfil the 1ms UP latency target with SCS above 30kHz.

SPS can reduce the UL latency significantly.

The observations above can be used as a starting point for discussion on CP latency in the self-evaluation study item.

# Conclusion

In section 2 we made the following observations:

Observation 1	NR FDD can fulfil the 1ms UP latency target with SCS above 15kHz.

Observation 2	HARQ retransmissions can be done within the latency target above 30kHz SCS.

Observation 3	NR TDD can fulfil the 1ms UP latency target with SCS above 30kHz.

Observation 4	SPS can reduce the UL latency significantly.

The observations above can be used as a starting point for discussion on CP latency in the self-evaluation study item.

# References

ITU-R M.[IMT-2020-TECH PERF REQ] (WP5D-TD-0300)