3GPP TSG RAN WG1 Meeting #92  			           R1-1803230

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.4.1

Source: 	LG Electronics

Title: 	Correction on Table-based TBS Determination Procedure

Document for:	Discussion and decision

- Introduction

In RAN1 AH 1801, the following agreements were reached [1].

Agreement: - Quantized number of REs allocated for PDSCH within a PRB is removed - In the formula-based TBS determination, the minimum TBS is aligned to 3840

In this contribution, we present the effective code rate distortion that 1) the effective code rate is higher than the maximum supportable code rate (0.95 in NR) and 2) the difference between the nominal code rate denoted by the MCS option Tables (R) and effective code rate is severe for the short information size regime in the table-based TBS determination. Before discussion, let us denote the maximum supportable code rate and the maximum nominal code rate as Rmax and Rmax, respectively.

For avoidance of the above effective code distortion, this contribution provides a minimal change based proposal for the table-based TBS determination problem, which can be incorporated into TS 38.214 for NR: Physical Layer Procedures for Data.

- Effective code rate distortion in the table-based TBS determination

From [1], the table-based TBS determination (when Ninfo≤3824) is as follows.

3) When Ninfo≤3824, TBS is determined as follows - quantized intermediate number of information bits Ninfo'=max⁡(24,2nNinfo2n), where n= max⁡(3,log2(Ninfo)-6) - use Table 5.1.3.2-2 find the closest TBS that is not less than Ninfo'

Fig. 1 presents the effective code rates obtained by the table-based TBS determination using maximum MCS indices 28 and 27 in Tables 1 and 2 in [1]. Here, the effective code rate is defined by TBS+16NRE×Qm×v. Also, the TBS values and resource allocations for the cases that the effective code rates are higher than the maximum supportable code rate Rmax= 0.95 are summarized in Tables 1.1-2.

Figure 1. Effective code rate using MCS indices 28 and 27 in Tables 1 and 2 in [1]

| TBS | Ninfo' | NRE×Qm×v | TBS | Ninfo' | NRE×Qm×v |
| --- | --- | --- | --- | --- | --- |
| 40 | 40 | 48 | 576 | 560,560,568 | 606,612,618 |
| 48 | 48,48 | 54,60 | 608 | 584,592,592, 600 | 636,642,648,654 |
| 56 | 56 | 64 | 640 | 616,616,624, 632,632 | 666,672,678, 684,690 |
| 64 | 64 | 72 | 672 | 648,648,656,664 | 702,708,714,720 |
| 72 | 72,72 | 78,84 | 704 | 680,688,688,696 | 738,744,750,756 |
| 80 | 80 | 90 | 736 | 712,720,720 | 774,780,786 |
| 88 | 88,88 | 96,102 | 768 | 744,744,752,760 | 804,810,816,822 |
| 96 | 96 | 108 | 808 | 776,776,784, 792,792 | 840,846,852, 858,864 |
| 104 | 104,104 | 114,120 | 848 | 816,816,824, 832,832 | 882,888,894, 900,906 |
| 112 | 112 | 126 | 888 | 856,864,872 | 930,936,948 |
| 120 | 120,120 | 132,138 | 928 | 896,904,912 | 972,984,990 |
| 128 | 128 | 144 | 984 | 936,944,944, 952,960,968 | 1014,1020,1026, 1032,1044,1050 |
| 136 | 136 | 150 | 1032 | 992,1008,1016 | 1080,1092,1098 |
| 144 | 144,144 | 156,162 | 1064 | 1040,1040 | 1128,1134 |
| 152 | 152 | 168 | 1128 | 1072,1072,1088, 1088,1104 | 1164,1170,1176, 1188,1200 |
| 160 | 160,160 | 174,180 | 1160 | 1136,1136 | 1230,1236 |
| 168 | 168 | 186 | 1224 | 1200 | 1302 |
| 176 | 176 | 192,198 | 1256 | 1232 | 1332 |
| 184 | 184 | 204 | 1288 | 1264 | 1368 |
| 192 | 192,192 | 210,216 | 1320 | 1296 | 1404 |
| 208 | 200,208 | 222,228 | 1416 | 1360,1360,1360, 1376,1376,1376 | 1470,1476,1482, 1488,1494,1500 |
| 224 | 216,216,224 | 234,240,246 | 1480 | 1424,1424,1440, 1440,1440 | 1548,1554,1560, 1566,1572 |
| 240 | 232,232,240 | 252,258,264 | 1544 | 1488,1488,1504, 1504 | 1608,1620,1632, 1638 |
| 256 | 248,248,256 | 270,276,282 | 1608 | 1552,1552,1568 | 1680,1692,1704 |
| 272 | 264,272,272 | 288,294,300 | 1672 | 1616,1616,1632, 1632,1632 | 1746,1752,1764, 1770,1776 |
| 288 | 280,288,288 | 306,312,318 | 1736 | 1680,1680,1680, 1696 | 1818,1824,1830, 1836 |
| 304 | 296,304,304 | 324,330,336 | 1800 | 1744,1744,1760 | 1890,1896,1908 |
| 320 | 312,320 | 342,348 | 1864 | 1808,1808,1824 | 1962,1968,1974 |
| 336 | 328,336 | 360,366 | 1928 | 1872,1872,1888, 1888 | 2028,2034,2040, 2046 |
| 352 | 344,344,352 | 372,378,384 | 2024 | 1936,1936,1952, 1952,1968,1968, 1968 | 2100,2106,2112, 2124,2130,2136, 2142 |
| 368 | 360,360,368 | 390,396,402 | 2088 | 2032,2032,2048 | 2196,2208,2214 |
| 384 | 376,376,384 | 408,414,420 | 2408 | 2304,2304,2304, 2304,2304,2336, 2336,2336 | 2490,2496,2502, 2508,2520,2538, 2544,2550 |
| 408 | 392,392,400,408 | 426,432,438,444 | 2976 | 2880,2880,2880 | 3120,3132,3144 |
| 432 | 416,416,424,432 | 450,456,462,468 | 3104 | 3008,3008,3008 | 3264,3270,3276 |
| 456 | 440, 448, 448 | 480, 486, 492 | 3240 | 3136,3136,3136, 3136 | 3390,3402,3408, 3420 |
| 480 | 464, 472, 472, 480 | 504, 510, 516, 522 | 3368 | 3264,3264,3264, 3264 | 3528,3534,3540, 3552 |
| 504 | 488, 488, 496, 504 | 528, 534, 540, 546 | 3496 | 3392,3392,3392, 3392 | 3666,3672,3690, 3696 |
| 528 | 512, 520, 520 | 558, 564, 570 | 3624 | 3520,3520,3520, 3520 | 3810,3816,3822, 3828 |
| 552 | 536, 544, 544 | 582, 588, 594 | 3752 | 3648,3648,3648 | 3942,3948,3960 |

Table. 1-1. TBS and resource allocation combination that effective code rates are higher than 0.95 for MCS index 28 in Table 1

| TBS | Ninfo' | NRE×Qm×v | TBS | Ninfo' | NRE×Qm×v |
| --- | --- | --- | --- | --- | --- |
| 56 | 56 | 64 | 608 | 584,592,592,600 | 632,640,648,656 |
| 64 | 64 | 72 | 640 | 616,624,632 | 672,680,688 |
| 72 | 72 | 80 | 672 | 648,656,664 | 704,712,720 |
| 80 | 80 | 88 | 704 | 680,688,696 | 736,744,752 |
| 88 | 88 | 96 | 736 | 712,720 | 776,784 |
| 96 | 96,96 | 104,112 | 768 | 744,752,760 | 808,816,824 |
| 104 | 104 | 120 | 808 | 776,784,792,792 | 840,848,856, 864 |
| 112 | 112 | 128 | 848 | 816,824, 832 | 888,896,904 |
| 120 | 120 | 136 | 888 | 856,864,872 | 928,936,944 |
| 128 | 128 | 144 | 928 | 896,896,904,912 | 968,976,984,992 |
| 136 | 136 | 152 | 984 | 936,944,952, 960,968 | 1016,1024,1032, 1040,1048 |
| 144 | 144 | 160 | 1032 | 992,992,1000,1008 | 1072,1080,1088,1096 |
| 152 | 152 | 168 | 1064 | 1040,1040 | 1128,1136 |
| 160 | 160 | 176 | 1128 | 1072,1072,1088, 1088,1088,1104 | 1160,1168,1176, 1184,1192,1200 |
| 168 | 168 | 184 | 1160 | 1136 | 1232 |
| 176 | 176 | 192 | 1192 | 1168 | 1264 |
| 184 | 184 | 200 | 1288 | 1264 | 1368 |
| 192 | 192,192 | 208,216 | 1320 | 1296 | 1400 |
| 208 | 200,208 | 224,232 | 1352 | 1328 | 1440 |
| 224 | 216,224 | 240,248 | 1416 | 1360,1360,1376, 1376,1392 | 1472,1480,1488, 1496,1504 |
| 240 | 232,240 | 256,264 | 1480 | 1424,1440,1440 | 1552,1560,1568 |
| 256 | 248,256 | 272,280 | 1544 | 1488,1488,1488, 1504,1504 | 1608,1616,1624, 1632,1640 |
| 272 | 264,272 | 288,296 | 1608 | 1552,1568,1568 | 1680,1696,1704 |
| 288 | 280,288 | 304,312 | 1672 | 1616,1616,1632, 1632 | 1752,1760,1768, 1776 |
| 304 | 296,296,304 | 320,328,336 | 1736 | 1680,1696 | 1824,1840 |
| 320 | 312,320 | 344,352 | 1800 | 1744,1744,1760 | 1888,1896,1904 |
| 336 | 328,336 | 360,368 | 1864 | 1808,1808,1824 | 1960,1968,1976 |
| 352 | 344,352 | 376,384 | 1928 | 1872,1872,1888 | 2024,2032,2040 |
| 368 | 360,368 | 392,400 | 2024 | 1936,1952,1952, 1968,1968,1984 | 2096,2112,2120, 2128,2136,2144 |
| 384 | 376,384 | 408,416 | 2088 | 2032,2032 | 2200,2208 |
| 408 | 392,392,400 | 424,432, 440 | 2408 | 2304,2304,2336 ,2336 | 2496,2520,2528, 2544 |
| 432 | 416,424 | 456,464 | 2976 | 2880,2880,2880, 2880 | 3120,3128,3136, 3144 |
| 456 | 440,448,456 | 480,488,496 | 3104 | 3008,3008,3008 | 3256,3264,3280 |
| 480 | 464,472,480 | 504,512,520 | 3240 | 3136,3136,3136, 3136,3168 | 3392,3400,3408, 3416,3424 |
| 504 | 488,496,504 | 528, 536,544 | 3368 | 3264,3264,3264, 3264 | 3528,3536,3552, 3560 |
| 528 | 512,520 | 560,568 | 3496 | 3392,3392,3392 | 3672,3680,3696 |
| 552 | 536,544 | 584,592 | 3624 | 3520,3520 | 3808,3816 |
| 576 | 560,568 | 608,616 | 3752 | 3648,3648,3648 | 3944,3952,3960 |

Table. 1-2. TBS and resource allocation combination that effective code rates are higher than 0.95 for MCS index 27 in Table 2

From the above results, we can remark the following observations.

Observation 1: For MCS index 28 in Table 1, the effective code rates are higher than the maximum supportable code rate 0.95 for almost TBS values (79 ones among 93 values) except for 24, 32, 1352, 2152, 2216, 2280, 2472, 2536, 2600, 2664, 2728, 2792, 2856 and 3824.

Observation 2: For MCS index 27 in Table 2, the effective code rates are higher than the maximum supportable code rate 0.95 for almost TBS values (76 ones among 93 values) except for 24, 32, 40, 48, 1224, 1256, 2152, 2216, 2280, 2472, 2536, 2600, 2664, 2728, 2792, 2856 and 3824.

Observation 3: For maximum MCS indices (27 and 28) in Tables 1 and 2, the gap between the nominal code rate and the effective code rate is large especially in the short information size regime.

We analyze why the above effective code rate distortion happens and summarize the following reasons.

- For the given Ninfo, the TBS value is overestimated due to the TBS determination rule that “less than Ninfo'” prohibits the closest TBS value selection in some cases and does not consider CRC bit length so that the TBS value is aligned far from Ninfo in some cases and then the effective code rate higher than the maximum supportable code rate occurs.

- Proposed Solutions: Adjustment of TBS determination rule

As mentioned earlier, the current TBS determination rule leads to the effective code rate distortions especially in the short information size. Hence, in order to solve the effective code rate distortion, we have the following proposals.

Proposal 1: In Step 3), replace “use Table 5.1.3.2-2 find the closest TBS that is less than Ninfo'” with “use Table 5.1.3.2-2 find the TBS less than Ninfo-16 that is closest to Ninfo'”.

As shown in Fig. 2, even though Proposal 1 can solve the effective code rate higher than the maximum supportable code rate, the effective code rate distortion still exists.

Figure 2. Nominal and effective code rates comparison of the conventional approach in [1] and Proposal 1 using MCS indices 6, 20 and 27 in Table 2 for Ninfo≤3824

Proposal 2: In Step 3), replace “use Table 5.1.3.2-2 find the closest TBS that is less than Ninfo'” with “use Table 5.1.3.2-2 find the TBS less than Ninfo×RmaxRmax-16 that is closest to Ninfo'”.

Here, Rmax=0.9258 and Rmax=0.95 in NR. The condition of “the TBS less than Ninfo×RmaxRmax-16” can obviously prevent the effective code rate from being higher than the maximum supportable code rate, as shown in Figs. 3 and 4. Also, Proposal 2 can significantly improve the effective code rate distortion.

Figure 3. Nominal and effective code rates comparison of the conventional approach in [1] and Proposal 2 using MCS indices 6, 20 and 27 in Table 2 for Ninfo≤3824

Figure 4. Maximum difference between the nominal and effective code rates for the conventional approach in [1] and Proposals 1 and 2 according to TBS values using MCS indices 28 and 27 in Tables 1 and 2 for Ninfo≤3824

- Conclusion

We made the following observations and proposals in this contribution:

Observation 1: For MCS index 28 in Table 1, the effective code rates are higher than the maximum supportable code rate 0.95 for almost TBS values (79 ones among 93 values) except for 24, 32, 1352, 2152, 2216, 2280, 2472, 2536, 2600, 2664, 2728, 2792, 2856 and 3824.

Observation 2: For MCS index 27 in Table 2, the effective code rates are higher than the maximum supportable code rate 0.95 for almost TBS values (76 ones among 93 values) except for 24, 32, 40, 48, 1224, 1256, 2152, 2216, 2280, 2472, 2536, 2600, 2664, 2728, 2792, 2856 and 3824.

Observation 3: For maximum MCS indices (27 and 28) in Tables 1 and 2, the gap between the nominal code rate and the effective code rate is large especially in the short information size regime.

Proposal 1: In Step 3), replace “use Table 5.1.3.2-2 find the closest TBS that is less than Ninfo'” with “use Table 5.1.3.2-2 find the TBS less than Ninfo-16 that is closest to Ninfo'”.

Proposal 2: In Step 3), replace “use Table 5.1.3.2-2 find the closest TBS that is less than Ninfo'” with “use Table 5.1.3.2-2 find the TBS less than Ninfo×RmaxRmax-16 that is closest to Ninfo'”.

- Text proposal for TS 38.214

Anyone of Proposals can solve the effective code rate distortion. However, we prefer to the Proposal 2 in the perspective of the most minimal effective code rate distortion and the text proposal for step 3) in TS. 38.214 V15.0.0, Section 5.1.3.2 is as follows. The modified text is marked in red.

#### 5.1.3.2	Transport block size determination

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the TBS less than Ninfo×0.950.9258-16 that is  closest to.

- Reference

3GPP TS 38.214 V15.0.0 (2018.02), NR; Physical layer procedures for data (Release 15).