3GPP TSG RAN WG1#92 Meeting	R1-1803263

Athens, Greece, February 26th – March 2nd, 2018

Source:	Sharp

Title:	Calibration results of Indoor Hotspot – eMBB for ITU self-evaluation

Agenda Item:	7.7

Document for:	Discussion

# Introduction

This contribution is a revision of R1-1802668. At the RAN#77, the following work plan and three steps for self-evaluation were agreed [1], and step 1 calibration work has started.

- Step 1: From Sep 2017 to Dec 2017, discussions in RAN ITU-R Ad-Hoc - Calibration for self-evaluation - Prepare and finalize initial description template information that is to be submitted to ITU-R WP 5D#29. - Step 2: From early 2018 to Sep 2018, targeting “update & self eval” submission in Sep 2018 - Performance evaluation against eMBB, mMTC and URLLC requirements and test environments for NR and LTE features. - Update description template and prepare compliance template according to self evaluation results. - Provide description template, compliance template, and self evaluation results based on Rel-15 in Sep 2018. - Step 3: From Sep 2018 to June 2019, targeting “Final” submission in June 2019 - Performance evaluation update by taking into account Rel-16 updates in addition to Rel-15 - Update description template and compliance template to take into account Rel-16 updates in addition to Rel-15 - Provide description template, compliance template, and self evaluation results based on Rel-15 and Rel-16 in June 2019.

At the RAN#78 meeting, it was agreed that 3GPP will start step 2 activity for self-evaluation study. In the self-evaluation study, step 1 calibration is also important to align the understanding of implementation of channel model with companies. In this contribution, we present our calibration results of Indoor-Hotspot – eMBB scenario in step 1.

# Calibration results

In step 1, the following metrics are evaluated.

- Coupling loss

- Geometry

Figure 1 – 3 summarize our calibration results in Indoor-Hotspot – eMBB scenario. The evaluation assumptions are listed in Annex.

| (a) Coupling loss (InH_A channel model) | (b) Coupling loss (InH_B channel model) |
| --- | --- |
| (c) Geometry (InH_A channel model) | (d) Geometry (InH_B channel model) |
| Figure 1. Calibration results in configuration A (4 GHz) |  |

| (a) Coupling loss (InH_A channel model) | (b) Geometry (InH_B channel model) |
| --- | --- |
| Figure 2. Calibration results in configuration B (30 GHz) |  |

| (a) Coupling loss (InH_A channel model) | (b) Geometry (InH_B channel model) |
| --- | --- |
| Figure 3. Calibration results in configuration C (70 GHz) |  |

# Conclusion

In this contribution, we provided the step1 calibration results of Indoor-Hotspot – eMBB for ITU self-evaluation

# References

RP-172101, Huawei, Ericsson, Telecom Italia, “WF on Work plan of Self Evaluation SI”, September 2017.

RT-172532, Huawei, Ericsson, “Work plan for self evaluation towards 3GPP Update Submission to ITU-R”, Telecom Italia, December 2017.

ITU-R, M. [IMT-2020.EVAL], “Guidelines for evaluation of radio interface technologies for IMT-2020”, October, 2017.

TR 36.873, “Study on 3D channel model for LTE”, v. 12.7.0, January, 2018.

# Annex

- Table 1 and Table 2 show evaluation assumptions in single sector case and 3-sector case, respectively.

- Table 1. Evaluation assumptions in single sector case

|  | - Configuration A | - Configuration B | - Configuration C |
| --- | --- | --- | --- |
| - Cell layout | - Indoor hotspot sites layout [3] | - Indoor hotspot sites layout [3] | - Indoor hotspot sites layout [3] |
| - Carrier frequency | - 4 GHz | - 30 GHz | - 70 GHz |
| - Simulation bandwidth | - 10 MHz | - 40 MHz | - 40 MHz |
| - Transmit power per TRP | - 21 dBm | - 20 dBm | - 18 dBm |
| - Inter-site distance | - 20 m | - 20 m | - 20 m |
| - BS antenna height | - 3 m | - 3 m | - 3 m |
| - Number of antenna elements per TRP | - 32 Tx/Rx - (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 45, −45 (degree) | - 64 Tx/Rx - (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 45, −45 (degree) | - 256 Tx/Rx - (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 45, −45 (degree) |
| - Number of TXRUs per TRP | - 32 TXRUs - (Mp, Np, P, Mg, Ng) = (4, 4, 2, 1, 1) | - 8 TXRUs - (Mp, Np, P, Mg, Ng) = (2, 2, 2, 1, 1) | - 8 TXRUs - (Mp, Np, P, Mg, Ng) = (2, 2, 2, 1, 1) |
| - BS antenna gain | - 5 dBi | - 5 dBi | - 5 dBi |
| - BS antenna radiation pattern | - Table 8-7 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) | - Table 8-7 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) | - Table 8-7 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) |
| - BS mechanical downtilt | - 180° in GCS | - 180° in GCS | - 180° in GCS |
| - BS electrical downtilt | - 90° in LCS | - Zenith angle of analog beam set - (see “beam set at BS side”) | - Zenith angle of analog beam set - (see “beam set at BS side”) |
| - BS array boresight | - x ↓ - (note: before mechanical tilting) | - x ↓ - (note: before mechanical tilting) | - x ↓ - (note: before mechanical tilting) |
| - UE antenna height | - 1.5 m | - 1.5 m | - 1.5 m |
| - Number of antenna elements per UE | - 4 Tx/Rx - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) - (dH, dV) = (0.5 λ, N/A) - Polarization: 0, 90 (degree) | - 32 Tx/Rx - (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2) - (dH, dV) = (0.5 λ, 0.5 λ) - (dg,H, dg,V) = (0 λ, N/A) - Polarization: 0, 90 (degree) - Panel angle: Ω0,1 = Ω0,0 + 180° | - 32 Tx/Rx - (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2) - (dH, dV) = (0.5 λ, 0.5 λ) - (dg,H, dg,V) = (0 λ, N/A) - Polarization: 0, 90 (degree) - Panel angle: Ω0,1 = Ω0,0 + 180° |
| - Number of TXRUs per UE | - 4 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 2, 2, 1, 1) | - 4 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 2) | - 4 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 2) |
| - UE antenna gain | - 0 dBi | - 5 dBi | - 5 dBi |
| - UE antenna radiation pattern | - Omni-directional | - Table 8-8 in [3] - θ3dB = 90 (degree), SLAv = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) | - Table 8-8 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) |
| - UE array orientation | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° |
| - Polarized antenna model | - Model 2 [4] | - Model 2 [4] | - Model 2 [4] |
| - Device deployment | - 100 % indoor - Randomly and uniformly distributed over the area | - 100 % indoor - Randomly and uniformly distributed over the area | - 100 % indoor - Randomly and uniformly distributed over the area |
| - UE mobility model | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| - UE speed of interest | - 3 km/h | - 3 km/h | - 3 km/h |
| - Inter-site interference modeling | - Explicitly modeled | - Explicitly modeled | - Explicitly modeled |
| - BS noise figure | - 5 dB | - 7 dB | - 7 dB |
| - UE noise figure | - 7 dB | - 10 dB | - 10 dB |
| - Channel model | - InH_A, InH_B | - InH_A & InH_B - (InH_A & InH_B are the same) | - InH_A & InH_B - (InH_A & InH_B are the same) |
| - Traffic model | - Full buffer | - Full buffer | - Full buffer |
| - Thermal noise level | - −174 dBm/Hz | - −174 dBm/Hz | - −174 dBm/Hz |
| - UE density | - 10 UEs per TRP | - 10 UEs per TRP | - 10 UEs per TRP |
| - Min. distance between TRP and UE | - d2D_min = 0 m | - d2D_min = 0 m | - d2D_min = 0 m |
| - UE attachment | - RSRP based from port 0 [4] | - RSRP based from port 0 [4] with the best analog beam pair | - RSRP based from port 0 [4] with the best analog beam pair |
| - Handover margin | - 0 dB | - 0 dB | - 0 dB |
| - Beam set at TRP side |  | - 8 2D DFT analog beams in LCS - Azimuth angle φi = [−3π/8, −π/8, π/8, 3π/8] - Zenith angle: θj = [π/4, 3π/4] | - 32 2D DFT analog beams in LCS - Azimuth angle φi = [−7π/16, −5π/16, −3π/16, −π/16, π/16, 3π/16, 5π/16, 7π/16] - Zenith angle: θj = [π/8, 3π/8, 5π/8, 7π/8] |
| - Beam set at UE side |  | - 8 2D DFT analog beam in LCS - Azimuth angle φi = [−3π/8, −π/8, π/8, 3π/8] - Zenith angle: θj = [π/4, 3π/4] | - 8 2D DFT analog beam in LCS - Azimuth angle φi = [−3π/8, −π/8, π/8, 3π/8] - Zenith angle: θj = [π/4, 3π/4] |
| - Analog beam selection for serving TRP |  | - Select the best beam among analog beams based on the criteria of maximizing RSRP after beamforming | - Select the best beam among analog beams based on the criteria of maximizing RSRP after beamforming |
| - Panel selection at UE side |  | - Select a panel of the criteria of maximizing RSRP | - Select a panel of the criteria of maximizing RSRP |
| - Analog beam selection for interfering TRP |  | - Select a beam randomly among analog beam for non-serving TRP | - Select a beam randomly among analog beam for non-serving TRP |

- Table 2. Evaluation assumptions in 3-sector case

|  | - Configuration A | - Configuration B | - Configuration C |
| --- | --- | --- | --- |
| - Cell layout | - Indoor hotspot sites layout [3] | - Indoor hotspot sites layout [3] | - Indoor hotspot sites layout [3] |
| - Carrier frequency | - 4 GHz | - 30 GHz | - 70 GHz |
| - Simulation bandwidth | - 10 MHz | - 40 MHz | - 40 MHz |
| - Transmit power per TRP | - 21 dBm | - 20 dBm | - 18 dBm |
| - Inter-site distance | - 20 m | - 20 m | - 20 m |
| - BS antenna height | - 3 m | - 3 m | - 3 m |
| - Number of antenna elements per TRP | - 32 Tx/Rx - (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 45, −45 (degree) | - 64 Tx/Rx - (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 45, −45 (degree) | - 256 Tx/Rx - (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1) - (dH, dV) = (0.5 λ, 0.5 λ) - Polarization: 45, −45 (degree) |
| - Number of TXRUs per TRP | - 32 TXRUs - (Mp, Np, P, Mg, Ng) = (4, 4, 2, 1, 1) | - 8 TXRUs - (Mp, Np, P, Mg, Ng) = (2, 2, 2, 1, 1) | - 8 TXRUs - (Mp, Np, P, Mg, Ng) = (2, 2, 2, 1, 1) |
| - BS antenna gain | - 5 dBi | - 5 dBi | - 5 dBi |
| - BS antenna radiation pattern | - Table 8-7 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) | - Table 8-7 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) | - Table 8-7 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) |
| - BS mechanical downtilt | - 110° in GCS | - 110° in GCS | - 110° in GCS |
| - BS electrical downtilt | - 90° in LCS | - Zenith angle of analog beam set - (see “beam set at BS side”) | - Zenith angle of analog beam set - (see “beam set at BS side”) |
| - BS array boresight | - 30°, 150°, 270° - → x | - 30°, 150°, 270° - → x | - 30°, 150°, 270° - → x |
| - UE antenna height | - 1.5 m | - 1.5 m | - 1.5 m |
| - Number of antenna elements per UE | - 4 Tx/Rx - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) - (dH, dV) = (0.5 λ, N/A) - Polarization: 0, 90 (degree) | - 32 Tx/Rx - (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2) - (dH, dV) = (0.5 λ, 0.5 λ) - (dg,H, dg,V) = (0 λ, N/A) - Polarization: 0, 90 (degree) - Panel angle: Ω0,1 = Ω0,0 + 180° | - 32 Tx/Rx - (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2) - (dH, dV) = (0.5 λ, 0.5 λ) - (dg,H, dg,V) = (0 λ, N/A) - Polarization: 0, 90 (degree) - Panel angle: Ω0,1 = Ω0,0 + 180° |
| - Number of TXRUs per UE | - 4 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 2, 2, 1, 2) | - 4 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 2) | - 4 TXRUs - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 2) |
| - UE antenna gain | - 0 dBi | - 5 dBi | - 5 dBi |
| - UE antenna radiation pattern | - Omni-directional | - Table 8-8 in [3] - θ3dB = 90 (degree), SLAv = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) | - Table 8-8 in [3] - θ3dB = 90 (degree), SLAV = 25 (dB) - φ3dB = 90 (degree), Am = 25 (dB) |
| - UE array orientation | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° | - Bearing angle: uniform distribution with Ω0,0 = [0°, 360°] - Mechanical angle: Θmg, ng = 90° |
| - Polarized antenna model | - Model 2 [4] | - Model 2 [4] | - Model 2 [4] |
| - Device deployment | - 100 % indoor - Randomly and uniformly distributed over the area | - 100 % indoor - Randomly and uniformly distributed over the area | - 100 % indoor - Randomly and uniformly distributed over the area |
| - UE mobility model | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction | - Fixed and identical speed ¦v¦ of all UEs, randomly and uniformly distributed direction |
| - UE speed of interest | - 3 km/h | - 3 km/h | - 3 km/h |
| - Inter-site interference modeling | - Explicitly modeled | - Explicitly modeled | - Explicitly modeled |
| - BS noise figure | - 5 dB | - 7 dB | - 7 dB |
| - UE noise figure | - 7 dB | - 10 dB | - 10 dB |
| - Channel model | - InH_A, InH_B | - InH_A & InH_B - (InH_A & InH_B are the same) | - InH_A & InH_B - (InH_A & InH_B are the same) |
| - Traffic model | - Full buffer | - Full buffer | - Full buffer |
| - Thermal noise level | - −174 dBm/Hz | - −174 dBm/Hz | - −174 dBm/Hz |
| - UE density | - 10 UEs per TRP | - 10 UEs per TRP | - 10 UEs per TRP |
| - Min. distance between TRP and UE | - d2D_min = 0 m | - d2D_min = 0 m | - d2D_min = 0 m |
| - UE attachment | - RSRP based from port 0 [4] | - RSRP based from port 0 [4] with the best analog beam pair | - RSRP based from port 0 [4] with the best analog beam pair |
| - Handover margin | - 0 dB | - 0 dB | - 0 dB |
| - Beam set at TRP side |  | - 8 2D DFT analog beams in LCS - Azimuth angle φi = [−3π/8, −π/8, π/8, 3π/8] - Zenith angle: θj = [π/4, 3π/4] | - 32 2D DFT analog beams in LCS - Azimuth angle φi = [−7π/16, −5π/16, −3π/16, −π/16, π/16, 3π/16, 5π/16, 7π/16] - Zenith angle: θj = [π/8, 3π/8, 5π/8, 7π/8] |
| - Beam set at UE side |  | - 8 2D DFT analog beam in LCS - Azimuth angle φi = [−3π/8, −π/8, π/8, 3π/8] - Zenith angle: θj = [π/4, 3π/4] | - 8 2D DFT analog beam in LCS - Azimuth angle φi = [−3π/8, −π/8, π/8, 3π/8] - Zenith angle: θj = [π/4, 3π/4] |
| - Analog beam selection for serving TRP |  | - Select the best beam among analog beams based on the criteria of maximizing RSRP after beamforming | - Select the best beam among analog beams based on the criteria of maximizing RSRP after beamforming |
| - Panel selection at UE side |  | - Select a panel based on the criteria of maximizing RSRP | - Select a panel based on the criteria of maximizing RSRP |
| - Analog beam selection for interfering TRP |  | - Select a beam randomly among analog beam for non-serving TRP | - Select a beam randomly among analog beam for non-serving TRP |