3GPP TSG-RAN WG1 Meeting #92	R1-1803264

Athens, Greece, February 26th– March 2nd, 2018

Source:	Intel Corporation

Title:	Remaining details of PDCCH search spaces and CORESETs

Agenda item:		7.1.3.1.2

Document for:	Discussion and Decision

# Introduction

In RAN1 AH 1802, we have reached the following working assumption regarding search space design.

Working assumption: - At least for case 1-1 and case 1-2, all UE supports channel estimation capability for 48 CCEs for a given slot per scheduled cell - FFS: cross-carrier scheduling - FFS: wideband RS - FFS: overbooking and/or nested structure - FFS: exceptional case of CCE counting - FFS: for case 2

This contribution discusses details on the channel estimation complexity on PDCCH decoding.

This contribution is updated from R1-1802407 with updated analysis and evaluation results in Section 2 that considers highest AL of AL16 for a search space set configured to a UE to further illustrate the benefits from a nested search space design.

# Discussion

For the working assumption on the channel estimation capability for 48CCEs, it needs to be confirmed for the sake of reasonable UE burden. We need to discuss the remaining issues raised in last meeting.

Overbooking and/or nested structure

For this discussion, we provided some analysis on the relationship between blocking probabity and channel estimation capability in Section 3. As can be observed from Section 3, with an EPDCCH-type SS design, the gNB may not be able to utilize the full capability of the UE in processing the specified maximum number of BD candidates within a slot due to the constraint on maximum number of CCEs becoming the bottleneck. However nested structure provides the comparable blocking probabilty while keeping the channel estimation constraint and the number of blind decodings.

Wideband RS

Discussion may be related to how to count the number of CCEs for channel estimation capabiltiy depending on the wideband RS is configured or not. If we define CCE limitation differently when wideband DMRS is configured, we may also need to have different definition for the case of different REG bundle size of 3 or 2 or 6, which makes specifications complicated. In order to avoid any complex rule, we define the channel estimation capabiltiy using the actual number of CCE regardless of precoding assumption for simple specification and simple implementation.

Cross-carrier scheduling

It is straighforward that the number of BDs increases if cross-carrier scheduling is concerned and thus the number of CCEs for channel estimation should be also increased as well to make sense of the increased blind decoding capability. Simple way is to increase the number of CCEs for channel estimation proportional to the increase of the blind decoding capability.

PDCCH monitoring Case 2

Similarly, the number of CCEs for channel estimation for PDCCH monitoring Case 2 can be increased in proportion to the increase in the number of blind decodings.

Proposal 1

  - The channel estimation capability is defined using the actual number of CCE regardless of precoding assumption.

  - Number of CCEs for channel estimation can be increased in proportion to the increase in the number of blind decodings.

# Analysis on nested structure

In current 38.213 [1], we have the following hashing function for defining the PDCCH search space.

For a control resource set , the CCEs corresponding to PDCCH candidate of the search space for a serving cell corresponding to carrier indicator field value are given by where for any common search space,; for a UE-specific search space,, , , , and ; ; is the carrier indicator field value if the UE is configured with a carrier indicator field for the serving cell on which PDCCH is monitored; otherwise, including for any common search space, ; is the number of CCEs, numbered from 0 to , in control resource set ; , where is the number of PDCCH candidates the UE is configured to monitor for aggregation level for a serving cell corresponding to ; for any common search space, ; for a UE-specific search space, is the maximum of for all corresponding DCI formats over all configured values for a CCE aggregation level in control resource set ; the RNTI value used for is defined in [5, TS 38.212] and in [6, TS 38.214]

For the clear understanding of the channel estimation complexity, we have evaluated the number of CCEs that require channel estimation. Table 1 compares the number of CCEs for channel estimation between current E-PDCCH based scheme and nested search space design that was also discussed previously. It is assumed that aggregation level distributions are 30%, 30%, 20%, 10%, and 10% for aggregation levels 1, 2, 4, 8, and 16 respectively. The numbers of PDCCH candidates are 6, 6, 2, 2, and 2 for aggregation levels 1, 2, 4, 8, and 16 respectively. Total number of CCEs inside a control resource set is 80 and 112 in the evaluation. In addition, we also added one more scheme, which uses nested approach with additional pseudo PDCCH candidates. Here it is assumed that UE only monitors 2 PDCCHs of aggregation level 16 but the PDCCHs of lower aggregation levels can be determined from the CCEs of 2 actual PDCCH candidates and one pseudo PDCCH candidates with AL 16. Figure 2 illustrates the nested search space approach with and without pseudo PDCCH candidates. The pseudo PDCCH candidates can be configured by higher layer separately from the PDCCH candidates of the higher aggregation level.

Under that assumption, the average number of CCEs of the E-PDCCH based scheme are much higher than Nested design and Nested design with pseudo PDCCH candidates as shown in Table 1. If E-PDCCH based scheme is used, average number of CCEs for channel estimation is larger than 48 which is the maximum number of CCEs that UE can supports, i.e., CCE overbooking happens. For resolving the overbooking issue, we may need to define a dropping rule and also any dropping rule will decrease the number of blind decodings and increase the blocking probability. The maximum number of CCEs are almost two times of the Nested design. There is no case that number of CCEs for channel estimation is larger than 48 for nested approach with and without pseudo candidates.

Figure 1 is showing the blocking probability comparison between search space designs. If we just use the nested search space without pseudo PDCCH candidates, then the number of CCEs for the PDCCHs of the non-largest aggregation levels are limited by the number of PDCCH candidates of highest aggregation level. So the blocking probability is a bit higher than that of E-PDCCH based scheme. However, if we introduce the pseudo PDCCH candidate concept, the number of CCEs for the PDCCHs of the non-largest aggregation levels can be increased a bit and the blocking probability is reduced back to the similar level with the E-PDCCH based scheme. However, the number of CCEs for channel estimation is still much less that E-PDCCH based scheme. Here, the blocking probability of the E-PDCCH based scheme is based on that channel estimation is performed for all CCEs for all PDCCH candidates. As discussed above, if any dropping rule is defined for avoiding the overbooking issue, the blocking probability can be increased again and it will be higher than the case of nested search space with pseudo PDCCH candidates.

Considering significant reduction of channel estimation complexity and no blocking probability issue, it is proposed to modify the hashing function in order to have nested property potentially with the concept of pseudo PDCCH candidates. The hashing function equation of the current specification can be the baseline but we can modify it for nested approach as proposed in the text proposal at the end of the contribution.

Table 1 Comparison of UE processing burden for PDCCH channel estimation

| Total number of CCEs | Average number of CCEs that needs channel estimation |  |  |
| --- | --- | --- | --- |
|  | E-PDCCH type [1] | Nested design | Nested design w/ pseudo PDCCH candidates |
| 80 | 53.52 | 32 | 42.32 |
| 112 | 58.70 | 32 | 42.38 |

| Total number of CCEs | Maximum number of CCEs that needs channel estimation |  |  |
| --- | --- | --- | --- |
|  | E-PDCCH type [1] | Nested design | Nested design w/ pseudo PDCCH candidates |
| 80 | 64 | 32 | 48 |
| 112 | 70 | 32 | 48 |

Figure 1: Blocking probability of the nested approaches: different options

- Without pseudo CCEs

- With pseudo CCEs

Figure 2. Nested search space structure without and with pseudo CCEs

Proposal 2

  - Modify the hashing function in order to have nested property potentially with the concept of pseudo PDCCH.

  - Adopt the text proposal in the Appendix.

# Some further corrections to Subclause 10.1 of TS38.213

Following is quoted from Section 10.1 of TS38.213:

If a UE is configured for downlink bandwidth part (BWP) operation, as described in Subclause 12, the above configurations for the common search spaces apply for the initial active DL BWP. The UE can be additionally configured a control resource set for Type0-PDCCH common search space, Type0A-PDCCH common search space, Type1-PDCCH common search space, or Type2-PDCCH common search space for each configured DL BWP on the primary cell, other than the initial active DL BWP, as described in Subclause 12. Table 10.1-1: CCE aggregation levels and number of candidates per CCE aggregation level for Type0/Type0A/Type2-PDCCH common search space CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; X. For each DL BWP configured to a UE in a serving cell, a UE can be provided by higher layer signalling with control resource sets where . For control resource set , where a UE-specific search space, a Type2-PDCCH common search space, or a Type3-PDCCH common search space is mapped, the higher layer signalling provides: - a control resource set index by higher layer parameter CORESET-ID; - a DM-RS scrambling sequence initialization value by higher layer parameter PDCCH-DMRS-Scrambling-ID; - a number of consecutive symbols provided by higher layer parameter CORESET-time-duration; - a set of resource blocks provided by higher layer parameter CORESET-freq-dom; - a CCE-to-REG mapping provided by higher layer parameter CORESET-CCE-to-REG-mapping-type; - a REG bundle size, in case of interleaved CCE-to-REG mapping, provided by higher layer parameter CORESET-REG-bundle-size; - a cyclic shift for the REG bundle interleaver [4, 38.211] by higher layer parameter CORESET-shift-index; - an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by higher layer parameter TCI-StatesPDCCH, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception; - an indication for a presence or absence of a transmission configuration indication (TCI) field for DCI format 1_0 or DCI format 1_1 transmitted by a PDCCH in control resource set , by higher layer parameter TCI-PresentInDCI.

While the first highlighted part above Table 10.1-1 follows from RAN1 agreements, the subsequent part seems to imply that Type0-PDCCH  CSS, Type0A-PDCCH CSS, and Type1-PDCCH CSS may not be mapped to other DL BWPs. Further the second part also appears to imply that at least Type2-PDCCH CSS may be mapped to any serving cell. However, Types 0, 0A, 1, and 2 PDCCH common search spaces may only be mapped to DL BWPs of the PCell or PSCell following the RAN1 decisions from RAN1 #91:

Agreements: - C-SS in each DL BWP of the PCell/PScell - On C-SS, Yp ,kp= 0. - In Rel.15, - For scheduling RMSI, OSI, Paging, UE monitors common search space in the PCell only - In addition, for random access and fall back, UE monitors common search space in the PCell and PSCell only - Working assumption: The UE is not expected to be configured without C-SS on the PCell (PSCell) in the active DL BWP - NOTE: RAN1 does not expect additional impact on the UE behavior due to not having PRACH resource in the BWP - Working assumption: In Rel.15, - A UE is expected to monitor C-SS (if configured) in the activated BWP - Full functionalities of C-SS (scheduling RMSI, OSI, Paging, random access, etc) are supported by the C-SS configured by UE-specific RRC signaling. - All RRC parameters defined for UE-SS are also defined for C-SS that is configured by UE-specific RRC signaling. Agreements: - C-SS (at least for SFI/PI if configured) in a Scell: - On C-SS, Yp ,kp= 0. - Working assumption: All RRC parameters defined for UE-SS are also defined for C-SS that is configured by UE-specific RRC signaling.

To resolve this conflict, it is proposed that the following modification is adopted.

=== Start of text proposal for 38.213 Subclause 10.1 ===

…

If a UE is configured for downlink bandwidth part (BWP) operation, as described in Subclause 12, the above configurations for the common search spaces apply for the initial active DL BWP. The UE can be additionally configured a control resource set for Type0-PDCCH common search space, Type0A-PDCCH common search space, Type1-PDCCH common search space, or Type2-PDCCH common search space for each configured DL BWP on the primary cell, other than the initial active DL BWP, as described in Subclause 12.

Table 10.1-1: CCE aggregation levels and number of candidates per CCE aggregation level for Type0/Type0A/Type2-PDCCH common search space

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | X |

For each DL BWP configured to a UE, and for Type0/Type0A/Type1/Type2-PDCCH common search space in the primary cell, or for Type3-PDCCH common search space or UE-specific search space in a serving cell, a UE can be provided by higher layer signalling with  control resource sets where . For control resource set , , the higher layer signalling provides:

- a control resource set index by higher layer parameter CORESET-ID;

- a DM-RS scrambling sequence initialization value by higher layer parameter PDCCH-DMRS-Scrambling-ID;

- a number of consecutive symbols provided by higher layer parameter CORESET-time-duration;

- a set of resource blocks provided by higher layer parameter CORESET-freq-dom;

- a CCE-to-REG mapping provided by higher layer parameter CORESET-CCE-to-REG-mapping-type;

- a REG bundle size, in case of interleaved CCE-to-REG mapping, provided by higher layer parameter CORESET-REG-bundle-size;

- a cyclic shift for the REG bundle interleaver [4, 38.211] by higher layer parameter CORESET-shift-index;

- an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by higher layer parameter TCI-StatesPDCCH, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception;

- an indication for a presence or absence of a transmission configuration indication (TCI) field for DCI format 1_0 or DCI format 1_1 transmitted by a PDCCH in control resource set , by higher layer parameter TCI-PresentInDCI.

…

=== end of text proposal for 38.213 Subclause 10.1 ===

Quoting another part from Subclause 10.1 of 38.213:

For each DL BWP of a serving cell where a UE is configured to monitor PDCCH in a search space other than Type0-PDCCH common search space, the UE is configured the following by higher layer parameter search-space-config: - an association between a search space set index , , where , and a control resource set index ;

In the above, the highlighted part appears to preclude configuration of Type0-PDCCH CSS in a DL BWP other than the initial DL BWP. Referring again to the RAN1 decisions quoted above, the highlighted part should be removed.

Accordingly, the following text proposal is recommended to be adopted:

=== Start of text proposal for 38.213 Subclause 10.1 ===

…

For each DL BWP of a serving cell where a UE is configured to monitor PDCCH in a search space, the UE is configured the following by higher layer parameter search-space-config:

- an association between a search space set index , , where , and a control resource set index ;

…

=== end of text proposal for 38.213 Subclause 10.1 ===

Proposal 3

  - Adopt the text proposals in Section 4 of this document.

# Conclusion

In this contribution, we provided our views on open issues regarding DL control search space and the following proposals are made:

Proposal 1

  - The channel estimation capability is defined using the actual number of CCE regardless of precoding assumption.

  - Number of CCEs for channel estimation can be increased in proportion to the increase in the number of blind decodings.

Proposal 2

  - Modify the hashing function in order to have nested property potentially with the concept of pseudo PDCCH.

  - Adopt the text proposal in the Appendix.

Proposal 3

  - Adopt the text proposals in Section 4 of this document.

# References

- TS38.213 V f.0.0

# Appendix

# Text proposal for 38.213:

If L=Lmax, where Lmax is the largest aggregation level configured for the UE, for a control resource set , the CCEs corresponding to PDCCH candidate  of the search space for a serving cell corresponding to carrier indicator field value  are given by

CCEs that are used for PDCCH candidates and pseudo PDCCH candidates of the highest aggregation level Lmax form an intermediate control resource set p’.

If L<Lmax, for a control resource set p’, the CCEs corresponding to PDCCH candidate  of the search space for a serving cell corresponding to carrier indicator field value  are given by

LYp,kp +mnCI∙(Lmax∙Mp,nCILmax,pseudo)L∙Mp,nCI(L)+nCImod(Lmax∙Mp,nCILmax,pseudo)L+i

where

for any common search space,;

for a UE-specific search space,, , , , and ;

;

is the carrier indicator field value if the UE is configured with a carrier indicator field for the serving cell on which PDCCH is monitored; otherwise, including for any common search space, ;

is the number of CCEs, numbered from 0 to , in control resource set ;

, where  is the number of PDCCH candidates the UE is configured to monitor for aggregation level  for a serving cell corresponding to ;

Mp,nCILmax,pseudo is the sum of the number of PDCCH candidates the UE is configured to monitor for aggregation level Lmax and the number of pseudo PDCCH candidates;

for any common search space, ;

for a UE-specific search space, is the maximum of  for all corresponding DCI formats over all configured  values for a CCE aggregation level  in control resource set ;

the RNTI value used for  is defined in [5, TS 38.212] and in [6, TS 38.214].

A UE configured to monitor PDCCH candidates in a serving cell with a DCI format size with carrier indicator field and CRC scrambled by C-RNTI, where the PDCCH candidates may have one or more possible values of carrier indicator field for the DCI format size, shall assume that an PDCCH candidate with the DCI format size may be transmitted in the serving cell in any PDCCH UE specific search space corresponding to any of the possible values of carrier indicator field for the DCI format size.