3GPP TSG RAN WG1 Meeting #92			R1-1803291

Athens, Greece, February 26th – March 2nd, 2018

Source:				ZTE, Sanechips, CMCC

Title:						WF on issues for SR/SPS periodicity configuration

Agenda item:		7.1.3.3.4

Document For: Discussion and Decision

# Introduction

Periodicity of the cell-specific RRC configuration of the semi-static DL/UL assignment was agreed. With the X ms + Y ms total periodicity, 2 ms and 2.5 ms configuration is supported. In the earlier stage of NR deployment, those cell-specific RRC configurations would be common case in lower frequency band. It can be seen that those can be categorized as two kinds of frame structure periodicity, i.e. 2^n slots based frame structure periodicity and n*5 slots based frame structure periodicity.

Periodic signal/channel (periodic CSI-RS, SRS, CSI reporting, SR, SPS etc.) should support more values to fit the 1st frame structure periodicity (i.e. introduce more 2^n slots value for periodicity). There are some agreements made to address this issue.

In MIMO session of last RAN1 AdHoc meeting, it is agree that CSI report will support the proposed additional periodicity. Agreement: Support additional periodicities of {4, 8, 16} slots for periodic on PUCCH and semi-persistent CSI reporting on PUCCH and PUSCH. Include the above agreement as part of LS to RAN2 Agreement: - Introduce additional periodicities of {4,8,16,32,64} slots and the corresponding slot offsets to at least the following periodic/semi-persistent RS: - CSI-RS (includes ZP-CSI-RS and NZP-CSI-RS) - SRS - FFS on whether and how to align to additional periodicities as agreed in the frame structure agenda - “The two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms” - FFS on CSI-IM periodicity of 1 slot It is also agreed updated accordingly for PDCCH monitoring periodicity. Agreements: - For a search space configuration, monitoring periodicity of slot(s) is updated as follows: - For all SCS, {1, 2, 4, 5, 8, 10, 16, 20} slots - For INT-RNTI, a subset of {1,2,4} slots is applied FFS: the case when concatenated semi-static DL/UL assignments is configured

However, UL SPS and SR periodicities are not fixed due to limited of time. To further complete the supporting of cell-specific UL/DL assignment with scheduling & HARQ aspects, we propose to complete the periodicities setting.

# Proposed change

- Support following updated periodicities of resources for SR configuration

Table9.2.4: UE-specific SR periodicity and offset configuration

| UE-specific SR periodicity and offset configuration (in symbols) |  |  |  |
| --- | --- | --- | --- |
|  | CP |  |  |
| 0 | Normal |  | 2, 7, n*14, where n={1, 2, 4, 5, 8,10, 16,20, 32, 40, 64, 80} |
| 1 | Normal |  | 2, 7, n*14, where n={1, 2, 4,5, 8,10,16, 20, 32, 40, 64, 80, 160} |
| 2 | Normal |  | 2, 7, n*14, where n={1, 2, 4,5,8, 10, 16,20, 32, 40, 64, 80, 160, 320} |
| 2 | Extended |  | 2, 6, n*12, where n={1, 2, 4,5,8, 10, 16,20, 32, 40, 64, 80, 160, 320} |
| 3 | Normal |  | 2, 7, n*14, where n={1, 2, 4,5, 8,10, 16,20, 32, 40, 64, 80, 160, 320, 640} |

- Support following updated periodicities of resources for UL transmission without UL grant

Table 6.1.2.3-1: Allowed periodicities P for uplink transmission without grant

|  | CP | Possible values of periodicities P [symbols] |
| --- | --- | --- |
| 0 | Normal | 2, 7, n*14, where n={1, 2,4, 5, 8,10, 16,20, 32, 40, 64, 80, 128, 160, 256,320, 512,640} |
| 1 | Normal | 2, 7, n*14, where n={1, 2, 4,5,8, 10, 16,20, 32, 40, 64, 80, 128, 160, 256, 320,512, 640, 1024,1280} |
| 2 | Normal | 2, 7, n*14, where n={1, 2, 4, 5,8,10,16,20, 32,40, 64,80, 128, 160, 256, 320, 512, 640,1024 1280, 2048,2560} |
| 2 | Extended | 2, 6, n*12, where n={1, 2, 4, 5,8, 10,16, 20,32, 40, 64,80, 128, 160, 256, 320, 512, 640, 1024,1280,2048, 2560} |
| 3 | Normal | 2, 7, n*14, where n={1, 2, 4, 5, 8, 10,16, 20,32,40, 64,80, 160, 256, 320, 512, 640, 1024, 1280, 2048,2560, 4096,5120} |

# References

- 3GPP TSG RAN WG1 Meeting #91, Chairman’s Note.

- 3GPP TSG RAN WG1 201801 AdHoc Meeting, Chairman’s Note.