3GPP TSG RAN WG1 Meeting #92	R1-1803293

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.2.3.1

Source:	Huawei, HiSilicon

Title:	Feature lead summary 1 of the remaining issues for RS multiplexing

Document for:	Discussion and decision

Introduction

In this contribution, the views of RS multiplexing are summarized from companies’ Tdocs. Some suggestions are also provided.

Remaining issues for RS multiplexing

- Issue 1: multiplexing between DMRS and SSB

Agreement: When SS block and PDSCH are scheduled in the same symbols, DMRS and SS block can be in a same symbol. Above applies at least for the case where DMRS and SS block are not overlapping in the frequency domain Above applies at least for the case where SS block and DMRS are spatially QCL-ed Above applies at least for the case where the same subcarrier spacing is used for SS block and DMRS

Capture the agreement, one more issue is whether the subcarrier spacing for SSB and DMRS can be different:

Alt.1 Support the same subcarrier spacing for SSB and DMRS except the case of 240kHz.

Alt.2 For PDSCH not carrying RMSI, the same subcarrier spacing is used for PDSCH and the SSB/PBCH block within a CC. Otherwise, for PDSCH carrying RMSI, the subcarrier spacing used for PDSCH and the SSB/PBCH block within a CC may be the same or different.

Alt.3 Support the same or different subcarrier spacing for SS/PBCH block and PDSCH DMRS within a CC. For the case of 240 kHz SCS, the same subcarrier spacing is precluded.

| Company | Views |
| --- | --- |
| Qualcomm | Alt.2 |
| Ericsson | Alt.3 |
| MTK | Prefer Alt. 1 to make thing simpler |
| LGE | Alt.1 |
| Samsung | Alt.3 |

Proposal-1: need some offline or online discussion for down-selection on the following three alternatives in this meeting:

- Alt.1 Support the same subcarrier spacing for SSB and DMRS except the case of 240kHz.

- Alt.2 For PDSCH not carrying RMSI, the same subcarrier spacing is used for PDSCH and the SSB/PBCH block within a CC. Otherwise, for PDSCH carrying RMSI, the subcarrier spacing used for PDSCH and the SSB/PBCH block within a CC may be the same or different.

- Alt.3 Support the same or different subcarrier spacing for SS/PBCH block and PDSCH DMRS within a CC. For the case of 240 kHz SCS, the same subcarrier spacing is precluded.

- Issue 2: Multiplexing between CSI-RS and SSB

Agreement A CSI-RS resource can be configured on RBs outside PBCH RBs in the symbols containing SS block from UE perspective. …… Above applies for the cases: CSI-RS only used for beam management

Alt.1 CSI-RS and SSB in the same symbol can be applied for the other cases, i.e., CSI-RS used for CSI acquisition (Captured in current 38.214 v 15.0.1)

Alt.2 CSI-RS and SSB in the same symbol can be applied for the only case: CSI-RS used for beam management (the same as previous agreement)

| Company | Views |
| --- | --- |
| Ericsson | Alt.1 In our view, there is no need to make a distinction in specs between CSI-RS for beam management and CSI-RS for CSI acquisition. In our understanding there is only one place currently in 38.214 where there is a distinction is made, and that is Section 5.1.6.1.2. It should be clarified that the multiplexing rule between CSI-RS and SS/PBCH block is not restricted to “CSI-RS for L1-RSRP computation,” as the title of Section 5.1.6.1.2 implies. |
| Huawei, HiSilicon | Alt.1 |
| LGE | Alt.2, using CSI-RS-ResourceRep for distinguish the CSI-RS for beam management, and also make the parameter CSI-RS-ResourceRep as optional in RAN2. Furthermore, If the higher layer parameter TRS-INFO is configured in a CSI-RS resource set, UE does not expect to be configured with the higher layer parameter CSI-RS-ResourceRep |
| Samsung | Alt.2, using NrofPorts to distinguish the CSI-RS for beam management |
| DCM | If FDM between CSI-RS and SSB/PBCH is allowed irrespective of the use cases, e.g., beam management, CSI acquisition and tracking, we don’t need to change text in the specification. Otherwise, following TP should be agreed for Sec. 5.1.6.1.2 in TS38.214 |
| OPPO | Alt.2, using CSI-RS-ResourceRep for distinguish the CSI-RS for beam management |
| Qualcomm | Alt. 2 (Agreement was clear and it was discussed a lot). Configuring CSI-RS-ResourceRep plus the fact that the associated reportconfig should be either ‘RSRP’ or ‘No report’ could be used for distinguishing the CSI-RS for beam management from CSIRS for CSI, and also make the parameter CSI-RS-ResourceRep as optional in RAN2. |
| MTK | Alt. 2. We should respect the earlier agreement. One comment to use ResourceRep. When it is ON, the Tx beam is the same across resources in a set. This is for P-3 procedure, and the UE ma change Rx beam to measure. For P-2 procedure, the Tx beam may be changed among the resources. If the UE is to change the beam to measure CSI-RS, how can UE receive SSB simultaneously? Also, if we consider ResouceRep to distinguish acquisition and BM CSI-RS, I wonder it is further to confine it is only for P-3 BM purpose. Then we kind of prefer to use NrofPorts, same as Samsung’s view. But overall, we doubt this feature is useful. |

Proposal-2: based on the discussion, it can be some further discussed as following alternatives:

- Alt.1 CSI-RS and SSB in the same symbol can be applied for the other cases, i.e., CSI-RS used for CSI acquisition. I.e., following text proposal in Section 5.2.2.3.1 of 38.214

------------------------------Text proposal----------

The UE may be configured the CSI-RS in same OFDM symbols as SS/PBCH block, but not in the same PRBs.

---------------------------------------------------------

- Alt.2 CSI-RS and SSB in the same symbol can be applied for the only case: CSI-RS used for beam management. The detailed text proposal should be further discussed due to the diverse views on how to distinguish the CSI-RS for beam management.

- Alt.3 CSI-RS and SSB in the same symbol can be applied for the cases, i.e., CSI-RS with NrofPorts =1 or 2. And no any changes in 38.214.

- Issue 3: There is an FFS part for spatial QCL assumption for CSI-RS and CORSET, whether only spatial QCLed CSI-RS and CORESET can be multiplexed in a symbol

Agreement: A CSI-RS resource can be configured on RBs outside CORESET in CORESET symbols from UE perspective. Above applies at least for the case where PDCCH and CSI-RS are spatially QCL-ed, and FFS for multi-panel UEs FFS: Use case when the above is applicable (ex: CSI reporting for wideband and partial band)

Alt.1. For CSI-RS and CORESET multiplexed in a symbol, support QCLed case

Alt.2. For CSI-RS and CORESET multiplexed in a symbol, support both QCLed and non-QCLed cases (in current spec 38.214)

| Companies | Views |
| --- | --- |
| Huawei, HiSilicon | Alt.2, Can be QCLed and non-QCLed depend on UE capability |
| OPPO | Alt.1, only for spatially QCLed case |
| Qualcomm | Alt. 1 (multi-panel UEs are deprioritized, we can discuss these aspects after stabilizing current spec). |
| MTK | Alt. 1 |
| Ericsson | Alt.1. Suggest the following text for 38.214: If QCL-TypeD is applicable, and the UE is configured with CSI-RS in the same OFDM symbols(s) as a CORESET, the UE may assume that the CSI-RS and the PDCCH DMRS transmitted in the CORESET are quasi co-located with QCL-TypeD. |
| LGE | Alt.1 (Ericsson’s proposal with re-wording as above: “may assume” “should only expect”. See the memo as well) |
| Samsung | Alt.1 |
| vivo | Alt.1, at least QCLed case should be supported, while non-QCLed case could be UE capability. |

Proposal-3:  Support Alt.1 and further discuss the multiplexing between CSI-RS and CORESET for non-QCLed cases in multiple TRPs/panels/beams session. Text proposal for Section 5.2.2.3.1 of 38.214 as follows:

------------------------------Text proposal----------

If spatial quasi co-location is applicable, the UE may be configured the CSI-RS in same OFDM symbols as CORESET when those are spatially quasi co-located, but not in the same PRBs, where PRBs associated with CSI-RS are outside of the PRBs configured for CORESET.

---------------------------------------------------------

- Issue 4: Collision between SRS and PRACH

PRACH and SRS resources are configured separately, it is possible that PRACH resources will collide with SRS transmission especially the PRACH and SRS transmission are from different UEs

| Companies | Views |
| --- | --- |
| ZTE | Whenever SRS transmission instance overlaps with region of the PRACH preamble format, the UE shall not transmit SRS. |
| CMCC | Whenever SRS transmission instance overlaps with the PRACH region or exceeds the range of uplink system bandwidth configured in the serving cell, the UE shall not transmit SRS |
| Qualcomm | In CMCC proposal why is it “uplink system BW”, why is this possible to happen? SRS is supposed to be transmitted inside an active BWP only. |
| MTK | For same UE, SRS is not transmitted when the configuration of PRACH and SRS collide. But I doubt that the gNB can’t manage the collision? |
| Samsung | Same question with Qualcomm |
| vivo | This issue should be handled for same UE case. Not sure for different UE case. |

- Issue 5: For multiplexing CSI-RS and DMRS on the same OFDM symbol

The following rules can be considered,

- Alt1: The UE is not expected to be configured to transmit CSI-RS and DMRS on the same OFDM symbols.

- Alt2: The UE shall assume DMRS and CSI-RS are transmitted on the symbols only when they are spatially QCLed (where applicable)

- Alt3: If DMRS and CSI-RS are multiplexed on the same symbols, UE would receive the symbols with the spatial reception parameter based on the TCI state of DMRS.

| Companies | Views |
| --- | --- |
| vivo | Alt.2 CSI-RS and DMRS can be multiplexed only for spatially QCLed cases |
| Qualcomm | Alt. 1 is the preference due to simplicity and the fact that CSIRS is already so flexible and has so many available positions to be in (aka all symbols in a slot). But, Alt.2 could also be possible given that the spec has addressed a few fundamental issues that currently exist: - CSI-RS and DMRS can be multiplexed only for spatially QCLed cases. Specifically, PDSCH/DMRS should not be FDMed with CSIRS on the same symbol when "Repetition" of CSIRS is ON, because the UE is expected to change beam. If "Repetition" is OFF and it happens that the QCL type D of CSIRS is the same as that of PDSCH/DMRS, then DMRS can be FDMed on the same symbol, otherwise it should not. - A related issue is that CSIRS should not be transmitted on the same CDM groups with the potential co-scheduled DMRS ports, because the UE does not know what to assume that is transmitted in that CDM group: Is it potential co-scheduled DMRS ports, is it CSIRS, or is it both superimposed? To clarify UE behavior we propose: Text Proposal 4 (Section 5.1.6.2, 38.214): A UE shall assume that the CDM groups indicated in the configured index from Table 7.3.1.2.2-1-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data or CSI-RS transmission, where “1”, “2” and “3” for the number of DMRS CDM group(s) in Table 7.3.1.2.2-1-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively. If issue 2 is not clarified, UE behavior is not clear, and we prefer to FDM CSIRS and DMRS on PRB level on the same symbol, but not inside the PRB. If either issue 1 is not clarified, then only TDMing DMRS and CSIRS make sense. |
| MTK | First, we support QC’s view on issue 2 Second, we prefer Alt. 2. For example, the TRS is assigned to UE A, B to Z, and it TRS beam is the same for them. But each UE may have its own PDSCH beam (narrower maybe). It is still feasible that, TRS is FDMed at the slot of transmitting PDSCH to UE A to Z and these UEs use their own beam to receive both PDSCH and TRS |
| Huawei, HiSilicon | Support Alt.1 to simplify the design of receiver |
| Samsung | Alt. 1 |
| Ericsson | What about sub-6 GHz operation? Is there a need to preclude FDM multiplexing of DMRS and CSI-RS? |

Further discuss this issue during the meeting.

- Issue 6: CSI-RS and PDSCH

- Introduce restriction for PDSCH REs mapping on OFDM symbols used to transmit NZP CSI-RS resource with antenna ports which are non quasi co-located with DM-RS antenna ports used to transmit corresponding PDSCH signals

Companies views:

| Companies | Views |
| --- | --- |
| Intel | Support this proposal (R1-1802399) |
| Qualcomm | Similar answer as above and in general supportive of the motivation of the proposal. A few details, PDSCH/DMRS should not be FDMed with CSIRS on the same symbol when "Repetition" of CSIRS is ON, because the UE is expected to change beam. If "Repetition" is OFF and it happens that the QCL type D of CSIRS is the same as that of PDSCH/DMRS, then data can be FDMed, otherwise it should not. |
| MTK | Agree with Alex. And we intend to add something more.When TRS_info is enabled, and when QCL type D of TRS is the same as that of PDSCH, as such TRS and PDSCH can be FDMed |
| Ericsson | Agree, see a sentence to capture this provided by Ericsson in the TRS summary (this applies when QCL type D is applicable) |
| vivo | This issue could be discussed with issue 5 together. |

Further discuss this issue during the meeting.

- Issue 7: CSI-RS and SIB1

In LTE, UE shall assume that CSI reference signals are not transmitted in subframes where transmission of a CSI-RS would collide with SystemInformationBlockType1 messages. However, it is not clear how to handle the collided issues between SIB1 and CSI-RS in NR.

ZTE, Sanechips: some clarifications of handling overlapping between SIB1 and CSI-RS should be captured in the current spec. e.g. ‘the UE shall assume that CSI reference signals are not transmitted in slots where transmission of a CSI-RS would collide with SIB1 messages’ or ‘the UE is not expected to receive CSI-RS and SIB1 message in the overlapping RBs’

| Companies | Views |
| --- | --- |
| ZTE | The UE is not expected to receive CSI-RS and SIB1 message in the overlapping RBs. |
| Qualcomm | Agree with ZTE. |
| MTK | Need to discuss this more. The CSI-RS here includes both acquisition and beam management purpose? And the UE can use same beam to use both SIB1 and CSI-RS? |
| Ericsson | Agree with ZTE |

Further discuss this issue during the meeting.

- Issue 8: For multiplexing ZP CSI-RS and NZP CSI-RS

- The UE may assume NZP CSI-RS REs are transmitted when overlapped with ZP CSI-RS REs.

- The UE performs interference measurement on CSI-IM REs regardless whether they are overlapping with ZP CSI-RS REs or not.

Proposed by vivo in [R1-1801523]

| Companies | Views |
| --- | --- |
| Qualcomm | Agree with both vivo’s proposals. |

Further discuss this issue during the meeting.

- Issue 9: Extend the use case of CSI-RS and SSB in the same symbol to same or different numerologies of CSI-RS and SSB

Proposed by Samsung in [R1-1801966]

Further discuss this issue during the meeting.

- Issue 10: CDM group without data for DMRS

A UE shall assume that the CDM groups indicated in the configured index from Table 7.3.1.2.2-1-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data or CSI-RS transmission

Proposed by QC in [R1-1802826]

Further discuss this issue during the meeting.

- Issue 11: Multiplexing between DMRS and CSI-RS dependent on UE capability

Mandatory: A UE only supports receiving DM-RS and CSI-RS on different OFDM symbols per CC.

Optional:

  - A UE additionally supports receiving CSI-RS and DM-RS on same OFDM symbols but in different PRBs per CC.

  - A UE additionally supports receiving CSI-RS and DM-RS on the same PRB and OFDM symbol with non-colliding resource elements per CC.

Proposed by QC in [R1-1802826]

Further discuss this issue during the meeting.

- Issue 12: Introduce restriction for simultaneous reception of “Reception Types” combinations according to TS 38.202 only for the scenario when QCL wrt to spatial Rx parameters is ensured by the NW

Proposed by Intel in [R1-1802399]

Further discuss this issue during the meeting.

Possible conclusions in this meeting:

Proposal-1: need some offline or online discussion for down-selection on the following three alternatives in this meeting:

- Alt.1 Support the same subcarrier spacing for SSB and DMRS except the case of 240kHz.

- Alt.2 For PDSCH not carrying RMSI, the same subcarrier spacing is used for PDSCH and the SSB/PBCH block within a CC. Otherwise, for PDSCH carrying RMSI, the subcarrier spacing used for PDSCH and the SSB/PBCH block within a CC may be the same or different.

- Alt.3 Support the same or different subcarrier spacing for SS/PBCH block and PDSCH DMRS within a CC. For the case of 240 kHz SCS, the same subcarrier spacing is precluded.

Proposal-2: based on the discussion, it can be some further discussed as following alternatives:

- Alt.1 CSI-RS and SSB in the same symbol can be applied for the other cases, i.e., CSI-RS used for CSI acquisition. I.e., following text proposal in Section 5.2.2.3.1 of 38.214

------------------------------Text proposal----------

The UE may be configured the CSI-RS in same OFDM symbols as SS/PBCH block, but not in the same PRBs.

---------------------------------------------------------

- Alt.2 CSI-RS and SSB in the same symbol can be applied for the only case: CSI-RS used for beam management. The detailed text proposal should be further discussed due to the diverse views on how to distinguish the CSI-RS for beam management.

- Alt.3 CSI-RS and SSB in the same symbol can be applied for the cases, i.e., CSI-RS with NrofPorts =1 or 2. And no any changes in 38.214.

Proposal-3:  Support Alt.1 and further discuss the multiplexing between CSI-RS and CORESET for non-QCLed cases in multiple TRPs/panels/beams session. Text proposal for Section 5.2.2.3.1 of 38.214 as follows:

------------------------------Text proposal----------

If spatial quasi co-location is applicable, the UE may be configured the CSI-RS in same OFDM symbols as CORESET when those are spatially quasi co-located, but not in the same PRBs, where PRBs associated with CSI-RS are outside of the PRBs configured for CORESET.

---------------------------------------------------------

- Agreements in previous meetings

In AH1801 meeting:

Agreement:

The following text proposal is agreed for Section 5.2.2.3.1 of 38.214

The UE may be configured the CSI-RS in same OFDM symbols as SS/PBCH block, but not in the same PRBs.

The UE may be configured the CSI-RS in same OFDM symbols as SS/PBCH block, but not in the same PRBs, if the higher layer parameter NrofPorts is configured as 1 or 2.

Agreement:

The following text proposal is agreed for Section 5.1.6.1.2 of 38.214:

The UE may be configured to use the same OFDM symbols for the CSI-RS and SSB/PBCH when those are spatially quasi co-located and resource elements PRBs associated with CSI-RS are the outside of PRBs configured for SSB/PBCH.

Agreement:

The following text proposal is agreed for Section 5.2.2.3.1 of 38.214:

The UE may be configured the CSI-RS in same OFDM symbols as CORESET, but not in the same PRBs.

The following text proposal is agreed for Section 5.1.6.1.2 of 38.214:

The UE may be configured to use the same OFDM symbols for the CSI-RS and CORESET when those are spatially quasi co-located and PRBs associated with CSI-RS are the outside of PRBs configured for CORESET.

Agreement:

- For multiplexing between CSI-RS and CORESET, follow the current description in TS38.214 and remove the corresponding parts from TS38.211, which means that the bandwidths of CSI-RS and CORESET/SSB shall be configured exclusively (i.e. no assumptions on CSI-RS RE puncturing at UE side).

Text proposal: in Section 7.4.1.5.3 of TS38.211

7.4.1.5.3		Mapping to physical resources

For each CSI-RS component configured, the UE shall assume the sequence  being mapped to physical resources according to

Resource elements overlapping with a configured CORESET or declared as 'reserved' according to clause 4.4.3 shall be counted in the mapping process but not assumed to be used for transmission of CSI-RS.

Agreement:

Accept the following modification of Section 7.4.1.5.3 in 38.211:

The UE is not expected to receive a CSI-RS and DMRS on the same resource elements. in resource elements overlapping with configured DM-RS

In RAN1#91 meeting:

Agreement:

Proposal 2: Only support TDM between SRS and PUSCH/UL DMRS/UL PTRS/Long PUCCH in Rel-15 from UE perspective.

Agreement:

Symbol location for CSI-RS:

{0,1,2,3,4,5,6,7,8,9,10,11,12,13}, where 2 is supported only when DL-DMRS-typeA-pos equals 3

- UE is not expected to receive CSI-RS and DMRS on overlapping REs

Agreement:

UE does not expect any DMRS RE to collide with SSB REs on the 4 symbols occupied by SSB

Agreement

- A CSI-RS resource can be configured on RBs outside PBCH RBs in the symbols containing SS block from UE perspective.

- Above applies for the case where SS block and CSI-RS are spatially QCL-ed

- Note: CSI-RS BW discussion should be taken into account. If beam management is agreed, the requirement on minimum BW for CSI acquisition and beam management may be different.

- Above applies at least for the case where the same subcarrier spacing is used for SS block and CSI-RS

- Above applies for the cases: CSI-RS only used for beam management

In RAN1#90bis meeting:

Agreement: on page 7 in R1-1718998:

The PT-RS according to the mapping pattern is not transmitted in OFDM symbols that contains PDSCH/PUSCH DMRS

The PT-RS according to the mapping pattern is not transmitted in RE that overlaps with a configured CORESET

Agreement:

When SS block and PDSCH are scheduled in the same symbols, DMRS and SS block can be in a same symbol.

Above applies at least for the case where DMRS and SS block are not overlapping in the frequency domain

Above applies at least for the case where SS block and DMRS are spatially QCL-ed

Above applies at least for the case where the same subcarrier spacing is used for SS block and DMRS

Agreement:

Multiplexing schemes for TRS with DMRS/PDSCH/PDCCH/SS block follow the multiplexing schemes for CSI-RS with DMRS/PDSCH/PDCCH/SS block.

Agreement:

A CSI-RS resource can be configured on RBs outside CORESET in CORESET symbols from UE perspective.

Above applies at least for the case where PDCCH and CSI-RS are spatially QCL-ed, and FFS for multi-panel UEs

FFS: Use case when the above is applicable (ex: CSI reporting for wideband and partial band)

Note: CSI-RS BW discussion should be taken into account

Above at least applies for non-slot based cases

Above feature is supported for slot-based transmissions as well

Agreement:

A CSI-RS resource can be configured on RBs outside PBCH RBs in the symbols containing SS block from UE perspective.

Above applies at least for the case where SS block and CSI-RS are spatially QCL-ed, FFS for multi-panel UEs.

FFS: If non-QCLed, study UE’s behavior

Note: CSI-RS BW discussion should be taken into account. If beam management is agreed, the requirement on minimum BW for CSI acquisition and beam management may be different.

Above applies at least for the case where the same subcarrier spacing is used for SS block and CSI-RS

Down select following alternatives:

Alt.1 Above applies for the cases: CSI-RS used for beam management and CSI acquisition

Alt.2 Above applies for the cases: CSI-RS only used for CSI acquisition

Alt.3 Above applies for the cases: CSI-RS only used for beam management

Agreement:

In the case of collision of SRS and short PUCCH carrying only CSI report/beam failure recover request, support the prioritization rules in the table below:

The channel listed in the entries below are prioritized

|  | Aperiodic SRS | Semi-persistent SRS | periodic SRS |
| --- | --- | --- | --- |
| sPUCCH with aperiodic CSI report only | No rule** | sPUCCH | sPUCCH |
| sPUCCH with semi persistent CSI report only | SRS | sPUCCH | sPUCCH |
| sPUCCH with periodic CSI report only | SRS | sPUCCH | sPUCCH |
| sPUCCH with beam failure recover request* | sPUCCH | sPUCCH | sPUCCH |

In case SRS is dropped, dropping can be partial in time domain, i.e., only those OFDM symbols that collide with short PUCCH

*If short PUCCH is supported for beam failure recovery request and collision between short PUCCH with beam failure recovery request and aperiodic/semi persistent/periodic SRS occurs, prioritize short PUCCH

** UE can assume that this collision will not occur

In RAN1 NR Ad-Hoc#3 meeting:

Agreement:

- Down-select among the following two options

  - Option 1-1: From a UE perspective, CSI-RS is not multiplexed on SS block OFDM symbol(s)

  - Option 1-2: From a UE perspective, CSI-RS can be multiplexed on SS block symbol(s)

    - FFS on the conditions when multiplexing is allowed

- Down-select among the following two options

  - From a UE perspective, for REs that is in the same OFDM symbol of configured CORESET but outside of the CORESET,

    - Option 2-1: NZP CSI-RS can be multiplexed

      - FFS: Whether or not a UE needs to be aware of the CORESET of another UE, e.g., through configuration of a ZP-CSI-RS

      - FFS: Whether or not PBCH is affected for common CORESET configuration, e.g., through a configuration of ZP-CSI-RS

    - Option 2-2: NZP CSI-RS is not multiplexed

- Down-select among the following two options

  - Option 3-1: Within a BWP, a UE is not expected to be configured with NZP-CSI-RS in OFDM symbols for which it is configured to receive DMRS

    - E.g., from a UE perspective in the slot with scheduled PDSCH, CSI-RS can be transmitted on the potential additional DMRS OFDM symbol(s), when the additional DMRS does not exist in the OFDM symbol(s).

    - Note: In Option 3-1, CSI-RS is not multiplexed on the potential front-loaded DMRS OFDM symbol(s)

  - Option 3-2: From a UE perspective, CSI-RS can be multiplexed on all any potential DMRS OFDM symbol locations(s)

  - Note: Additional restriction from cell-/UE-group perspective will be further discussed based on the down-selection above

In RAN1#90 meeting:

Conclusion:

Regarding CSI-RS and TRS multiplexing/sharing, discuss after the TRS design is mature (target later this week)

Agreements:

- For UE’s perspective,

  - For REs that is configured as a CORESET,

    - NZP CSI-RS is not multiplexed.

  - For REs that is in the same OFDM symbol of configured CORESET but outside of the CORESET,

    - FFS: Whether NZP CSI-RS can be multiplexed or not

Agreements:

- NR supports FDM between DMRS and PDSCH for CP-OFDM at least for some cases

  - FFS the conditions for this feature

- NR supports FDM between DMRS and PUSCH for CP-OFDM at least for some cases

  - FFS the conditions for this feature

- NR supports signaling for PDSCH rate matching in DMRS symbols

  - FFS details

Agreements:

- When one or more of PT-RS RE(s) is overlapped with CSI-RS

  - The one or more overlapping PT-RS RE(s) is punctured

Agreements:

- For collision avoidance between short PUCCH and SRS, from a UE perspective, NR supports at least the following two options on a given carrier

  - Collision is defined whenever SRS and PUCCH are transmitted in the same symbol, regardless of whether there are overlapped REs or not

  - Option 1-1: symbol level TDM

  - (Working assumption) Option 2: Prioritize SRS or short PUCCH transmission, i.e., drop SRS or short PUCCH in case of collision

    - FFS whether to have one prioritization rule, or configurable prioritization

    - Examples of prioritization rules

      - Example 1

        - Always prioritize PUCCH over SRS

      - Example 2

        - If PUCCH contains ACK/NACK, prioritize PUCCH

        - Otherwise prioritize SRS

  - FFS the case of FDM SRS and short PUCCH

In RAN1 NR Ad-Hoc#2 meeting:

Agreements:

- In NR, for a given BWP, support the case where CSI-RS for CSI acquisition and beam management is always transmitted with the same numerology as the PDSCH of the UE if PDSCH is present

  - FFS the case when PDSCH is not present

Agreements:

- Down-select among the following two options in the next meeting

  - Option 1-1: From a UE perspective, CSI-RS is not multiplexed on SS block OFDM symbol(s)

  - Option 1-2: From a UE perspective, CSI-RS can be multiplexed on SS block symbol(s)

Agreements:

- Down-select among the following two options in the next meeting:

  - Option 2-1: From a UE perspective, CSI-RS is not multiplexed on PDCCH OFDM symbol(s) for a slot

  - Option 2-2: From a UE perspective, CSI-RS can be multiplexed on PDCCH OFDM symbol(s) for a slot

    - Note: PDCCH decoding behavior at UE side will not be changed by Option 2-2.

  - Note: up to each company to define PDCCH OFDM symbols based on the max possible number of symbols or the configured PDCCH OFDM symbols for the slot

Agreements:

- Study further aspects related to DMRS and data multiplexing in DL and UL considering 14 and 7 symbol slots/mini-slots, 1 vs. 2 front loaded DM-RS symbols, additional DM-RS, etc.

- Study further aspects related to possibly power boosting DM-RS (performance, complexity, spec impact)

Agreements:

- Study further how to handle DM-RS and SS block collision (if any)

  - E.g., changing DM-RS symbol(s) position in time domain, no PDSCH transmission on the collided PRBs, dropping the DM-RS symbol in collision, etc.

Agreements:

- Study further how to handle PT-RS collision with CSI-RS

- References

RAN1-89 Chairman Notes.

RAN1 NR-AH2 Chairman Notes.

RAN1-90 Chairman Notes.

RAN1 NR-AH3 Chairman Notes.

RAN1-90b Chairman Notes.

RAN1-91 Chairman Notes.

R1-1801456	Remaining issues on RS multiplexing	Huawei, HiSilicon

R1-1801523	Remaining issues on multiplexing of different types of RSs	vivo

R1-1801584	Remaining details on RS multiplexing	ZTE, Sanechips

R1-1801966	Issues on RS multiplexing	 Samsung

R1-1802036	Discussion on priority rules to handle the collision between SRS and PRACH	CMCC

R1-1802098	Remaining issues on RS multiplexing	Guangdong OPPO Mobile Telecom

R1-1802198	Text proposals on RS multiplexing	LG Electronics

R1-1802399	On multiplexing of PDSCH	Intel Corporation

R1-1802474	Remaining details on RS multiplexing	NTT DOCOMO, INC.

R1-1802752	Remaining issues related to RS multiplexing	Ericsson

R1-1802826	Maintenance for Multiplexing of RS	Qualcomm Incorporated