3GPP TSG RAN WG1 Meeting #92    	            R1-1803299

Athens, Greece, February 26th – March 2nd, 2018

Source:	NTT DOCOMO, INC.

Title:	Offline summary for AI 7.1.3.1.2 Search space

Agenda Item:	7.1.3.1.2

Document for: 	Discussion and Decision

1. Number of PDCCH blind decodes per slot for Case 1-2 and Case 2

Still diverged views especially for Case 2. However, given the Rel.15 timeline, having additional UE capability for PDCCH blind decodes in Rel.15 would not be desirable. Therefore, the suggestion here is following:

Proposal 1: Confirm the value for Case 1-2. X=0 and Y=0 for Case 2. No consensus on additional Case 2’.

| Max no. of PDCCH BDs per slot | SCS |  |  |  |
| --- | --- | --- | --- | --- |
|  | 15kHz | 30kHz | 60kHz | 120kHz |
| Case 1-1 | 44 | 36 | 22 | 20 |
| Case 1-2 | [44] |  |  | - |
| Case 2 | [44+X] | [36+Y] | [22+Y] | [20] |

2. Number of PDCCH blind decodes per slot for Type0/Type0A/Type2-PDCCH common search space

Comment: {4, 2, 1} is the majority view, but AL16 is available only if the CORESET has equal to or more than 16 CCEs. Therefore, reasonable way is {4, 2, 1} if the CORESET has equal to or more than 96 REGs, otherwise {4, 2}.

Proposal: {4, 2, 1} if the CORESET has equal to or more than 96 REGs, otherwise {4, 2}.

Corresponding TP for TS38.213 Section 10.1 is following.

=== Start ===

Table 10.1-1: CCE aggregation levels and number of candidates per CCE aggregation level for Type0/Type0A/Type2-PDCCH common search space

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | 0 for less than 96 REGs, 1 for otherwise |

=== End ===

3. Number of CCEs for channel estimation for Case 1-1, 1-2, and Case 2

Following WA was made at RAN1-AH1801:

Working assumption: - At least for case 1-1 and case 1-2, all UE supports channel estimation capability for 48 CCEs for a given slot per scheduled cell - FFS: cross-carrier scheduling - FFS: wideband RS - FFS: overbooking and/or nested structure - FFS: exceptional case of CCE counting - FFS: for case 2

Many companies investigate blind decoding aspects assuming the WA is confirmed. On the other hand, there is a proposal to introduce a UE capability signaling to report PDCCH channel estimation capability. Need some more discussions on this, together with the necessity of nested search space design and dropping rules for PDCCH candidate(s).

Proposal: Discuss further offline. Following options are considered.

Option 1: Confirm the WA.

Option 2: Introduce a UE capability signaling reporting the channel estimation capability for {48, 74} CCEs for a given slot per scheduled cell.

4. Whether/how to drop PDCCH candidate(s) when the number(s) of candidate(s)/CCE(s) exceeds

Some companies propose to introduce PDCCH candidate(s) dropping rules, while some companies show concern on this behavior. If dropping rules are not specified at all, gNB shall make sure that “overshoot” of the number of PDCCH candidate(s) and CCE(s) to monitor are capable values.

Proposal: Discuss further offline. Following options are considered.

Option 1: Specify PDCCH candidate dropping rules. Working assumption: PDCCH candidates are dropped in the following order: (1) USS before CSS, (2) higher search space index before lower search space index for a given SS type, (2) Lower-AL before higher-AL for a given search space ID and SS type.

Option 2: gNB shall ensure the maximum value of PDCCH candidate(s) and CCEs for channel estimation across all search space sets for a given slot per carrier does not exceed the maximum allowable values.

5. Whether/how to support nested hash function

Many companies propose nested hash function, but the proposals are not yet converged. One company shows a concern on nested hash function. Proponents need to resolve the concern.

Proposal: Discuss further offline. Recommend to look at each proposal at offline session.

Option 1: Select one of the proposed nested hash function.

Option 2: No consensus.

6. Rules for selecting survived PDCCH candidate(s) when dropping is applied

ZTE proposes to select n candidates out of N candidates with floor(N/n). If no any other proposals, we can take this.

Proposal: If n candidates are monitored out of N candidates as a result of dropping (N-n) candidates, floor(N/n)-th candidates are monitored.

7. Clarification for kp

Two approaches are identified: kp is updated per slot (by ZTE, Ericsson), or is updated per monitoring occasion (by CATT, Samsung, Qualcomm). Unless complicated, randomizing per monitoring occasion is better than per slot. Here, it is suggested to take the randomization across monitoring occasions, where the update is carried out per minimum monitoring occasion across search space configurations (i.e., take the proposal in R1-1801974).

Proposal: Clarify following “for UE-specific search space, kp is the minimum kp,s for all corresponding DCI formats over all configured search space sets s”.

Text proposal for TS38.213 Section 10.1:

=== Start ===

for a UE-specific search space, is the minimum of for all corresponding DCI formats over all configured search space sets s.

=== End ===

8. Details on UE capability for PDCCH in CA

The UE capability signaling needs to be concluded. Here it is proposed to define a capability signaling which is integer value from {4, 5, …, 16}, where detailed number of blind decodes for CA should be discussed later. Not that this capability signalling is same as Rel.13 eCA.

Proposal: The UE capability signaling for PDCCH BDs in CA is integer value from {5, …, 16}.

9. Alignment between PDCCH monitoring periodicity/offset and semi-static D/U assignment

This is proposed by CMCC. How/whether we should realize PDCCH monitoring periodicity fitting with the concatenated semi-static D/U assignments? My impression is that we may not need to optimize PDCCH monitoring periodicity for some periodicities like 3ms, 7ms, etc, as long as not sure these configurations are really important. This change, if made, needs to be reflected for all periodic transmissions/receptions.

figure.2 example of PDCCH monitoring for concatenated semi-static DL/UL assignment (R1-1802040).

Proposal: Current PDCCH monitoring periodicity/offset is not influenced by semi-static D/U assignment; alignment between PDCCH monitoring periodicity/offset and semi-static D/U assignment is up to gNB implementation.

10: UE can detect AL8 when AL16 is transmitted in particular conditions

This is identified by Huawei and Panasonic. Various solutions can be considered, and different solutions may need to be discussed in different sessions (e.g., DL control channel, channel coding, rate-matching). Need to coordinate which session will discuss this.

Proposal: Discuss further offline.

11. CORESET 0/Search space 0 for SCell

Comment: Vivo raises this issue. Need to fix whether CORESET 0/Search space 0 is available for SCell. One way is that the CORESET 0 in SCell is not the same as CORESET 0 in PCell; CORESET 0 in SCell is one of the nominal CORESETs.

Proposal: Discuss further offline.

12. CORESET 0/Search space 0 after RRC connection setup

Comment: Vivo raises this issue. Current description of the spec is that Type0-CSS is based on PBCH in the initial-access procedure. So I assume this cannot be changed autonomously by the UE.

Proposal: Discuss further offline.

13. Rules for selecting PDCCH candidate(s) for DCI format 2_0 and 2_1

Comment: First {1 or 2} candidate(s) is simple and reasonable as proposed by MediaTek and CATT.

Proposal: DCI format 2_0 and 2_1 are the first {1 or 2} candidate(s) in the search space configuration.

References:

| R1-1801335 | Remaining issues on search space | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1801531 | Remaining details on PDCCH search space | vivo |
| R1-1801622 | Search space design and related issues | ZTE, Sanechips |
| R1-1801646 | Remaining details on search space | MediaTek Inc. |
| R1-1801728 | Remaining issues on NR PDCCH search space | CATT |
| R1-1801974 | Corrections on Search Space Design | Samsung |
| R1-1802040 | Discussion on remaining issues for search space | CMCC |
| R1-1802115 | Remaining issues on Search Space | Guangdong OPPO Mobile Telecom |
| R1-1802207 | Remaining issues on search space | LG Electronics |
| R1-1802290 | Remaining Issues Son PDCCH Search Space | Apple Inc. |
| R1-1802407 | Remaining details of PDCCH search spaces and CORESETs | Intel Corporation |
| R1-1802480 | Search space | NTT DOCOMO, INC. |
| R1-1802505 | Channel estimation complexity and search space | Panasonic |
| R1-1802537 | On NR operation under PDCCH channel estimation and BD limits | Nokia, Nokia Shanghai Bell |
| R1-1802538 | Remaining details on search space | Nokia, Nokia Shanghai Bell |
| R1-1802622 | On remaining issues of search space | InterDigital, Inc. |
| R1-1802834 | Remaining issues on control resource set and search space | Qualcomm Incorporated |
| R1-1802903 | Remaining issues of search space | Ericsson |

Agreements:

RAN1#90

Agreement: - The CORESET used to schedule the PDSCH containing the RMSI can be configured to contain also UE-specific PDCCH(s) Working assumptions: - For slot-based scheduling, the first DMRS position either on 3rd symbol or 4th symbol is configured by [PBCH]. - Maximum time duration of a CORESET is 2 symbols if the first DMRS position of a PDSCH with slot-based scheduling is on 3rd symbol, and is 3 symbols otherwise - This replaces the past working assumption linking DMRS position to bandwidth X Agreements: - Supported aggregation levels for NR-PDCCH are at least 1, 2, 4, 8 - FFS 16 and 32 aggregation levels and also other numbers Agreements: - A PDCCH search space at an aggregation level in a CORESET is defined by a set of PDCCH candidates - For the search space at the highest aggregation level in the CORESET, the CCEs corresponding to a PDCCH candidate are derived as following - The first CCE index of a PDCCH candidate is identified by using at least some of the followings - (1) UE-ID, (2) candidate number, (3) total number of CCEs for the PDCCH candidate, (4) total number of CCEs in the CORESET, and (5) randomization factor - The other CCE indexes of the PDCCH candidate are consecutive from the first CCE index - Searching space design for the lower aggregation level can be discussed separately Working assumptions: - In the case when only CORESET(s) for slot-based scheduling is configured for UE, the maximum number of PDCCH blind decodes per slot per carrier is X - The value of X does not exceed 44 - FFS the exact value of X - FFS for multiple active BWP, multiple TRP, multiple carriers, multi beams - FFS for non-slot based scheduling - FFS numerology specific X

RAN1 #NR-AH3

Working assumption: - Re-use NR DL RA Type 0 basis in units of 6 RBs, where no restriction on the maximum number of segments for a given CORESET. Agreements: - At least two DCI sizes are defined. - One DCI size, which is at least for the purpose of fallback. - FFS: for other purposes. - One DCI size depending on configuration - FFS: whether both DL and UL have the same size or different. - FFS: for group-common DCI/PDCCH - Note: the UE is not necessarily required to monitor two DCI sizes at the same monitoring occasion Agreement: - In a given CORESET - Alt 1: different DCI formats - Alt 2: different search spaces can have different monitoring periodicities. - FFS which one

RAN1 #90bis

Agreements: - Confirm the WA ‘Re-use NR DL RA Type 0 basis in units of 6 RBs, where no restriction on the maximum number of segments for a given CORESET’ with the following clarifications: - This is for the case when the CORESET is configured by at least UE-specific RRC signalling. - FFS the RB indexing for resource allocation especially considering interaction with DL BWP - Details of resource allocation should take into account the interaction with DL BWP – FFS details Agreements: - For slot-based scheduling; - Confirm the following working assumption with updates: - The first DMRS position either on symbol #2 or symbol #3 is configured by PBCH - Maximum time duration of a CORESET is 2 symbols if the first DMRS position of a PDSCH with slot-based scheduling is on symbol #2, and is 3 symbols otherwise. - The starting OFDM symbol of a CORESET can be symbol #0, #1, or #2, in a slot. - However, the ending OFDM symbol of a CORESET is not later than symbol #2 in a slot. Agreements: - In a given CORESET, two types of search spaces (e.g., UE-common search space and UE-specific search space) can have different periodicities for a UE to monitor - FFS details of the corresponding search spaces Agreements PDCCH candidates having different DCI payload sizes count as separate blind decodes PDCCH candidates comprised by different sets of CCE(s) count as separate blind decodes. PDCCH candidates in different CORESETs count as separate blind decodes. PDCCH candidates having the same DCI payload size and comprised by the same set of CCE(s) in the same CORESET count as one blind decodes. Agreements: - For non-CA and for PDCCH monitoring periodicity of 14 or more symbols, the maximum number of PDCCH blind decodes per slot is: - Working assumption: 44 for SCS = 15kHz. - Working assumption: less than 44 at least for SCS = 60kHz and 120kHz. - For the given SCS, all UEs support the maximum number of PDCCH blind decodes per slot. - Companies are encouraged to complete the following table. - Aiming to finalize this at RAN1#91. No. of PDCCH BDs per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Periodicity of 14 or more symbols; 44; [22-44]; [11-44]; [6-44]. Periodicity of less than 14 symbols; [44-86]; [22-86]; [11-44]; [6-44]. Agreements: - Companies are encouraged to provide the views on the following aspects: - Whether to specify upper limit of channel estimations a UE can perform for PDCCH; - If yes, how channel estimation is defined (e.g., per CCE or per REG bundle, whether common counting principle between narrowband RS and wideband RS), and; - What is the exact value of the upper limit of channel estimation a UE can perform for PDCCH. No. of CCEs that the UE can perform channel estimation per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Periodicity of 14 or more symbols; [48-74]. Periodicity of less than 14 symbols. Agreements: - For CA with up to N CCs, maximum number of PDCCH blind decodes per slot for a UE depends on the number of configured CCs. - All UEs supporting CA with the same set of CCs supports the same maximum number of PDCCH blind decodes. - No explicit UE capability signaling to inform the maximum number of PDCCH blind decodes is reported. - For CA with more than N CCs, maximum number of PDCCH blind decodes for a UE depends on the explicit UE capability. - All UEs supporting CA with the same set of CCs supports at least the same number of PDCCH blind decodes. - FFS: the value of N (no more than 8). Agreements: - For each SCS, whether or not separate UE capabilities for PDCCH monitoring periodicities are needed is concluded at RAN1#91. SCS. 15kHz; 30kHz; 60kHz; 120kHz. Need of separate UE capabilities for (i) PDCCH monitoring of 14 or more symbols and (ii) PDCCH monitoring of less than 14 symbols; Y/N; Y/N; Y/N; Y/N. Agreements: - Take the same hash function of LTE EPDCCH as the hash functuion for NR-PDCCH - Further refinements can be further considered till next meeting if necessary Agreements: - One set of the following parameters determines a set of search spaces - A set of aggregation levels - The number of PDCCH candidates for each aggregation level - PDCCH monitoring occasion for the set of search spaces Agreements: - At least for cases other than initial access, to identify a set of search spaces, following parameters are configured by UE-specific RRC signaling: - The number of PDCCH candidates for each aggregation level of {1, 2, 4, 8, [16]} - One value from {0, 1, 2, 3, 4, 5, 6, 8} - PDCCH monitoring occasion for the set of search spaces - One value of from {1-slot, 2-slot, [5-slot], [10-slot], [20-slot]} (at least 5 values) - One or more value(s) from 1st symbol, 2nd symbol, …, 14th symbol within a monitored slot - Each set of search spaces associates with a CORESET configuration by RRC signaling Agreements: - For PDCCH monitoring occasion of 1-slot, 2-slot, [5-slot], [10-slot], and [20-slot], - Slot-level offset value for PDCCH monitoring occasion is also supported. - For N-slot monitoring occasion, the offset is one from [0, N-1]. - Note: symbol-level bit-map of monitoring occasion within a slot agreed at RAN1#90bis is still available. Agreements: - For the DMRS of NR-PDCCH in a CORESET, - The QCL configuration/indication is on a per CORESET basis (Alt.1). Agreements: - The value(s) of TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, and/or TPC-SRS-RNTI, are provided by RRC signaling. - The association between at least each of the following RNTIs and a DCI format is specified in the specification. - C-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI. FFS: other RNTI(s). - The value of C-RNTI is obtained as part of random access procedure. - The association between a DCI format and a type of search space (UE-common search space and UE-specific search space) is specified in the specification. - UE-common search space contains a DCI format of C-RNTI, RNTI(s) for SPS/grant-free, TPC-PUSCH-RNTI, TPC-PUCCH,RNTI, TPC-SRS-RNTI, and INT-RNTI. - UE-specific search space contains a DCI format of C-RNTI and RNTI(s) for SPS/grant-free. Agreements: - By PBCH, a UE obtains at least one CORESET configuration at least for PDCCH scheduling RMSI associated with a given SS block. - The set of aggregation levels and candidates per aggregation level for PDCCH scheduling RMSI is specified in the specification. - FFS the indication of the support of aggregation level 16 in the cell - FFS: Set of search spaces for OSI, random access, and paging. - By RMSI, the UE can be configured with at least one CORESET configuration at least for PDCCH for random access. - If not configured by RMSI, the CORESET configuration(s) for random access is/are the one(s) configured by PBCH. - FFS: whether the CORESET configuration can be configured outside of the initial active DL BWP. - By UE-specific RRC signalling, the UE can be configured with one or more CORESET configuration(s) at least for PDCCH scheduling UE-specific data. - Each CORESET configuration is associated with one or more sets of search spaces. - Note: each set of search spaces is associated with one CORESET configuration.

RAN1 #91

Agreements: - RRC parameter “CORESET-start-symb” is deleted from the RRC parameter list. Agreements: - For NR PDCCH associated with the CORESET(s) configured by PBCH, AL=16 is supported. - For NR PDCCH associated with the CORESET(s) configured by RMSI or UE-specific RRC signaling, AL=16 is supported - Note: additional complexity, if any, for NR PDCCH channel estimation is to be discussed separately - Discuss further offline whether or not AL=16 is associated with wideband RS only Agreements: - CORESET configured by RMSI is confined within the initial active DL BWP Agreements: - For a CORESET configured by UE-specific RRC signaling, DL BWP-specific RB indexing + RB-offset are used to configure frequency-domain resource. - The length of the bit-map is Floor((N_RB – (ceil(BWP_start/6)*6-BWP_start))/6) - CORESET starting RB is ceil(BWP_start/6)*6 - For a CORESET configured by PBCH/RMSI, RB indexing is for the initial DL BWP. Agreements: - C-SS in each DL BWP of the PCell/PScell - On C-SS, Yp ,kp= 0. - In Rel.15, - For scheduling RMSI, OSI, Paging, UE monitors common search space in the PCell only - In addition, for random access and fall back, UE monitors common search space in the PCell and PSCell only - Working assumption: The UE is not expected to be configured without C-SS on the PCell (PSCell) in the active DL BWP - NOTE: RAN1 does not expect additional impact on the UE behavior due to not having PRACH resource in the BWP - Working assumption: In Rel.15, - A UE is expected to monitor C-SS (if configured) in the activated BWP - Full functionalities of C-SS (scheduling RMSI, OSI, Paging, random access, etc) are supported by the C-SS configured by UE-specific RRC signaling. - All RRC parameters defined for UE-SS are also defined for C-SS that is configured by UE-specific RRC signaling. Agreements: - C-SS (at least for SFI/PI if configured) in a Scell: - On C-SS, Yp ,kp= 0. - Working assumption: All RRC parameters defined for UE-SS are also defined for C-SS that is configured by UE-specific RRC signaling. Agreements: - Introduce a linkage between search space set and CORESET via an index to the CORESET configuration - CORESET is removed from the search space configuration - In Rel-15, the max no. of CORESETs configurable for a BWP in a cell for a UE is [3] - In Rel-15, the max no. of search space sets configurable for a BWP in a cell for a UE is [10] Agreements: - Scrambling for PDCCH (after channel coding) is supported. - No additional RRC parameter is necessary. - Re-use the ID for DMRS initialization. Conclusion: - It is clarified that Mp,maxL is the maximum of “configured” number of PDCCH candidates for the given aggregation level L across all serving cells scheduled by the search space Agreements: - UE is not expected to receive PDSCH type A in the same slot if the PDCCH monitoring is after the first two or three symbols of a slot - Note: PUSCH mapping is up to MIMO decision Agreements: - For information, the following cases are clarified: - Case 1: PDCCH monitoring periodicity of 14 or more symbols - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot - Case 2: PDCCH monitoring periodicity of less than 14 symbols - Note: this includes the PDCCH monitoring of up to three OFDM symbols at the beginning of a slot - The numbers in bracket in the following table can be further adjusted but not to be increased - X<=16, Y<=8 - FFS whether or not to have case 2’, where the values of X and/or Y can be smaller than case 2 Max no. of PDCCH BDs per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Case 1-1; 44; 36; 22; 20. Case 1-2; [44]; -. Case 2; [44+X]; [36+Y]; [22+Y]; [20]. Working assumption: - For PDCCH monitoring for receiving RMSI, the number of PDCCH candidates are following: - 4 candidates for AL = 4 - 2 candidates for AL = 8 - DCI size for RMSI scheduling and DCI size for OSI scheduling are the same - FFS: Paging and fallback Conclusion: - RAN1 common understanding is that the PDCCH channel estimation complexity is not negligible at least in some cases. - FFS: Possible solutions to resolve the channel estimation complexity issue together with the impact on PDCCH blocking probability - Opt.1: Define the limits of “the number of CCEs for PDCCH channel estimation which refers to the union of the sets of CCEs for PDCCH candidates” - Note: the overlapped CCEs associated with different CORESETs are counted separately. - FFS: CCEs for the same precoder-granularity are counted as one channel estimation - FFS: whether/how to handle the variation on the actual number of CCEs for PDCCH channel estimation and BDs over time - Application of overbooking is considered - Strive for not having specific UE capability to report the maximum number of CCEs for PDCCH channel estimation. - Study the solutions considering the cases 1-1, 1-2, 2, and 2’. - Opt.2: Modify the hashing function - Opt.3: Increase the size of the precoder granularity

RAN1 #NR-AH1801

Agreements: - The maximum number of CORESETs per BWP per cell is 3 - For the 3rd CORESET, i.e., for p=2, Ap=39839. - Confirm 10 as the maximum number of search space sets per BWP per cell. Agreements: - CORESET ID of the CORESET configured by PBCH is 0. - Search space ID of the search space configured by PBCH is 0. Agreements: - A UE can be configured with a search space configuration by UE-specific RRC signaling which includes following: - CORESET ID (range: 0-11, to indicate which CORESET the search space is mapped to) - The search space can be associated with any CORESET configuration - When the CORSET ID is UE-specifically configured to be 0, it is mapped to the one configured by PBCH - Search space ID (range: 0-39) - When the search space ID is UE-specifically configured to be 0, it is mapped to the one configured by PBCH Agreements: - DCI format 0_1 and 1_1 are monitored only in USS. - DCI format 0_0 and 1_0 are monitored in CSS. - DCI format 0_0 and 1_0 can be monitored in USS. - They have the same DCI payload size. - One of the following is configured by RRC signaling for the USS: - Monitoring DCI format 0_1 and 1_1 only - Monitoring DCI format 0_0 and 1_0 only Agreements: - NR supports a DCI format having the same size as the DCI format 1_0 to be used for scheduling RMSI/OSI, for Paging, and for random access. Agreements: - For each search space configuration configured by UE-specific RRC signaling, the UE is informed whether the search space configuration is CSS or USS, together with the following information, as part of the search space configuration: - Which DCI format(s) to monitor - For a CSS, - DCI format 0_0 and DCI format 1_0 - In which case, the UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), SP-CSI-RNTI (if configured), RA-RNTI, TC-RNTI, P-RNTI, SI-RNTI - DCI format 2_0 - In which case, the UE monitors the DCI format with CRC scrambled by SFI-RNTI, and the SFI-related parameters SFI-PDCCH is provided as part of the search space configuration - FFS: how to select one or two decoding candidates if the configured PDCCH candidates are larger than the value - DCI format 2_1 - In which case, the UE monitors the DCI format with CRC scrambled by INT-RNTI, and the PI-related parameters Preemp-DL is provided as part of the search space configuration - DCI format 2_2 - In which case, the UE monitors the DCI format with CRC scrambled by TPC-PUSCH-RNTI or TPC-PUCCH-RNTI - DCI format 2_3 - In which case, the UE monitors the DCI format with CRC scrambled by TPC-SRS-RNTI - Monitoring of multiple DCI formats can be configured for one CSS - For USS, - A UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), TC-RNTI (if a certain condition is met), and SP-CSI-RNTI (if configured) - Further discussion offline the association of the RNTIs with DCI formats - Monitoring of multiple DCI formats can be configured for one USS Agreements: - For a search space configuration, monitoring periodicity of slot(s) is updated as follows: - For all SCS, {1, 2, 4, 5, 8, 10, 16, 20} slots - For INT-RNTI, a subset of {1,2,4} slots is applied - FFS: the case when concatenated semi-static DL/UL assignments is configured Working assumption: - At least for case 1-1 and case 1-2, all UE supports channel estimation capability for 48 CCEs for a given slot per scheduled cell - FFS: cross-carrier scheduling - FFS: wideband RS - FFS: overbooking and/or nested structure - FFS: exceptional case of CCE counting - FFS: for case 2 Agreements - For the following previous agreement, N=4 Agreements: - For CA with up to N CCs, maximum number of PDCCH blind decodes per slot for a UE depends on the number of configured CCs. - All UEs supporting CA with the same set of CCs supports the same maximum number of PDCCH blind decodes. - No explicit UE capability signaling to inform the maximum number of PDCCH blind decodes is reported. - For CA with more than N CCs, maximum number of PDCCH blind decodes for a UE depends on the explicit UE capability. - All UEs supporting CA with the same set of CCs supports at least the same number of PDCCH blind decodes. - FFS: the value of N (no more than 8).