3GPP TSG RAN WG1 Meeting #92	R1-180xxxx

Athens, Greece, February 26th – March 2nd, 2018

Source:	Ericsson

Title:	Offline Discussion on CQI and MCS table(s) for URLLC

Agenda Item:	7.2.1

Document for:	Discussion and Decision

# Introduction

In RAN1 #90bis, the following agreements on CQI report for URLLC have been made.

Agreement:

- N separate CQI table(s) are supported for URLLC

  - Downselect the value of N between 1 or 2

- Two target BLER are supported for URLLC

  - Note: RRC signalling is used by gNB to select one of the two target BLER

  - Note: The configuration of target BLER or CQI table is part of CSI report setting

This contribution provides a summary on the issues on CQI/MCS tables for URLLC. Solutions submitted by companies are listed, with possible agreements suggested.

# Discussion

## BLER Targets

The table below summarizes companies’ view on the two BLER targets, as expressed in their contributions.

| Company | Company View |
| --- | --- |
| Ericsson | 10-3 and 10-5 |
| Huawei | 10-3 and 10-5 |
| Samsung | 10-3 and 10-5 |
| Nokia | 10-2 and 10-4 |
| ZTE | 10-3 and 10-5 |
| Intel | 10-1 and 10-4 |
| Qualcomm | Prefer to define BLER targets as residual BLER |
| LG | --- |
| MediaTek | 10-3 and 10-5 |
| AT&T | --- |
| Interdigital | --- |

BLER targets are used in CQI reporting. The BLER targets are not necessarily the link performance target of actual transmission.

Outcome of offline discussion:

- Proposal 1. 	The two BLER targets that are configurable for URLLC are to be selected from the following options:

- Option A. (10-1, 10-4)

- Option B. (10-1, 10-5)

- Option C. (10-3, 10-5)

- Option D. (10-2, 10-4)

- NOTE: BLER target discussion does not affect other parts in CQI report definition.

- NOTE: Unless configured/defined otherwise, existing BLER target = 10-1 of eMBB is available for UE to report CQI, and the UE needs to be able to report CQI for 3 BLER targets for Option C and Option D.

## CQI Tables

### Number of CQI tables

The table below summarizes companies’ view on the number of CQI table to define for URLLC, as expressed in their contributions.

| Company | Company View |
| --- | --- |
| Ericsson | Two |
| Huawei | Two |
| Samsung | Two |
| Nokia | One (16 entry table in Proposal 2) |
| ZTE | One |
| Intel | Two (new table for lower BLER target, and legacy table for higher BLER target) |
| Qualcomm | One |
| LG | Two |
| MediaTek | Two |
| AT&T | Two (reuse eMBB CQI tables) |
| Interdigital | Two |

Outcome of offline discussion:

Companies cannot converge on how many CQI tables to define. Continue discussion till next meeting.

### Highest modulation order supported in the CQI and MCS tables

The table below summarizes companies’ view on the highest modulation order supported in the CQI and MCS tables for URLLC, as expressed in their contributions.

| Company | Company View |
| --- | --- |
| Ericsson | 64QAM |
| Huawei | 64QAM |
| Samsung | 16QAM for one CQI table, and 64QAM for the second CQI table |
| Nokia | 16QAM |
| ZTE | 16QAM (OK with 64-QAM) |
| Intel | 64QAM |
| Qualcomm | 64QAM |
| LG | --- |
| MediaTek | 64QAM |
| AT&T | 64QAM and 256QAM (reuse eMBB CQI tables) |
| Interdigital | --- |

Outcome of offline discussion:

Majority view is to support highest modulation order of 16QAM and/or 64QAM. Only one company support 256QAM for URLLC.

- Proposal 2. 	For new CQI table and MCS table constructed specifically for URLLC, 256QAM is not included.

### What should be the spectral efficiency range of CQI table?

Outcome of offline discussion:

For a new CQI table other than existing eMBB CQI tables, the following applies.

- Proposal 3. 	Lowest spectral efficiency in any/all CQI table is not lower than 30/1024 * 2 (QPSK)

- Proposal 4: 	Highest spectral efficiency in any/all CQI table is not greater than a value, where the value is selected from the following:

- 666/1024 * 6

- 772/1024 * 6

- 873/1024 * 6

- 948/1024 * 6

## MCS Tables

### What should be the spectral efficiency range of MCS table?

Outcome of offline discussion:

For a new MCS table other than existing MCS table defined for eMBB, the following applies.

- Proposal 5. 	The MCS table keeps at least all the higher N-2 spectral efficiency levels in the CQI table, where the CQI table and the MCS table correspond to the same BLER target, and N is the number of entries in the CQI table.

- Proposal 6. 	Lowest spectral efficiency in any/all MCS table is not lower than 30/1024 * 2.

- Proposal 7. 	Highest spectral efficiency in any/all MCS table is not greater than a value, where the value is selected from the following:

- 666/1024 * 6

- 772/1024 * 6

- 873/1024 * 6

- 948/1024 * 6

# Conclusions

In this contribution, the following are proposed according to offline discussion:

- Proposal 1. 	The two BLER targets that are configurable for URLLC are to be selected from the following options:

- Option A. (10-1, 10-4)

- Option B. (10-1, 10-5)

- Option C. (10-3, 10-5)

- Option D. (10-2, 10-4)

- NOTE: BLER target discussion does not affect other parts in CQI report definition.

- NOTE: Unless configured/defined otherwise, existing BLER target = 10-1 of eMBB is available for UE to report CQI, and the UE needs to be able to report CQI for 3 BLER targets for Option C and Option D.

- Proposal 2. 	For new CQI table and MCS table constructed specifically for URLLC, 256QAM is not included.

For a new CQI table other than existing eMBB CQI tables, the following applies.

- Proposal 3. 	Lowest spectral efficiency in any/all CQI table is not lower than 30/1024 * 2 (QPSK)

- Proposal 4: 	Highest spectral efficiency in any/all CQI table is not greater than a value, where the value is selected from the following:

- 666/1024 * 6

- 772/1024 * 6

- 873/1024 * 6

- 948/1024 * 6

For a new MCS table other than existing MCS table defined for eMBB, the following applies.

- Proposal 5. 	The MCS table keeps at least all the higher N-2 spectral efficiency levels in the CQI table, where the CQI table and the MCS table correspond to the same BLER target, and N is the number of entries in the CQI table.

- Proposal 6. 	Lowest spectral efficiency in any/all MCS table is not lower than 30/1024 * 2.

- Proposal 7. 	Highest spectral efficiency in any/all MCS table is not greater than a value, where the value is selected from the following:

- 666/1024 * 6

- 772/1024 * 6

- 873/1024 * 6

- 948/1024 * 6

# References

R1-1801563	CQI and MCS tables for URLLC	Ericsson

R1-1801631	Considerations on CQI /MCS table(s) and related aspects for URLLC	ZTE, Sanechips

R1-1801748	CQI design for NR URLLC	CATT

R1-1802546	Remaining details of CQI and MCS for URLLC	Nokia, Nokia Shanghai Bell

R1-1801354	MCS/CQI design for URLLC transmission	Huawei, HiSilicon

R1-1801676	MCS and CQI Tables design for URLLC	MediaTek Inc.

R1-1801999	Design of CQI tables for URLLC	Samsung

R1-1802225	Discussion on CQI and MCS table for URLLC	LG Electronics

R1-1802421	CQI and MCS tables for NR URLLC	Intel Corporation

R1-1802575	CQI and MCS tables for URLLC	InterDigital, Inc.

R1-1802603	CQI/MCS selection for URLLC	AT&T

R1-1802851	Considerations for CQI and MCS for URLLC	Qualcomm Incorporated

# Appendix. Relevant 38.214 V15.0.0 Specification Texts

## Appendix A. Existing 64-QAM CQI table for CP-OFDM

#### 5.2.2.1	Channel quality indicator (CQI)

- …

Based on an unrestricted observation interval in time unless specified otherwise in this Subclause, [and an unrestricted observation interval in frequency-TBD], the UE shall derive for each CQI value reported in uplink slot n the highest CQI index which satisfies the following condition:

-	A single PDSCH transport block with a combination of modulation scheme and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding:

-	0.1, if the higher layer parameter CQI-table configures Table 5.2.2.1-2, or Table 5.2.2.1-3, or

-	a higher layer configured BLER-target, if the higher layer parameter CQI-table configures Table 5.2.2.1-4.

- …

Table 5.2.2.1-2: 4-bit CQI Table

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 120 | 0.2344 |
| 3 | QPSK | 193 | 0.3770 |
| 4 | QPSK | 308 | 0.6016 |
| 5 | QPSK | 449 | 0.8770 |
| 6 | QPSK | 602 | 1.1758 |
| 7 | 16QAM | 378 | 1.4766 |
| 8 | 16QAM | 490 | 1.9141 |
| 9 | 16QAM | 616 | 2.4063 |
| 10 | 64QAM | 466 | 2.7305 |
| 11 | 64QAM | 567 | 3.3223 |
| 12 | 64QAM | 666 | 3.9023 |
| 13 | 64QAM | 772 | 4.5234 |
| 14 | 64QAM | 873 | 5.1152 |
| 15 | 64QAM | 948 | 5.5547 |

## Appendix B. Existing 64-QAM MCS tables for CP-OFDM

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate x [1024] R | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |