3GPP TSG RAN WG1 Meeting #92								R1-1803387

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1

Source: 	Ericsson

Title:	List of RAN1 agreements with ASN.1 impact

Document for:	Discussion and Decision

# Introduction

This contributions contain a list of the agreements relevant to ASN.1.  The document is sorted based on the RAN1 agenda.

# List of Agreements

## Initial access and mobility

Agreements:

- Agree on the following text proposal for 38.215

  - Note: this (E-UTRA RS-SINR) needs a NEW RRC parameter for a NR UE. It is understood that this parameter is intended for SA case

>>>>>>>>>>>> Start text proposal >>>>>>>>>>>>

2	References

<omitted>

[13]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

<omitted>

5.1.X E-UTRA RS-SINR

| Definition | E-UTRA reference signal-signal to noise and interference ratio (E-UTRA RS-SINR), is defined as the linear average over the power contribution (in [W]) of the resource elements carrying cell-specific reference signals divided by the linear average of the noise and interference power contribution (in [W]) over the resource elements carrying cell-specific reference signals within the same frequency bandwidth. For E-UTRA RS-SINR determination, the E-UTRA cell-specific reference signals R0 according TS 36.211 [13] shall be used. The reference point for the E-UTRA RS-SINR shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding E-UTRA RS-SINR of any of the individual diversity branches. If higher-layer signalling indicates certain subframes for performing E-UTRA RS-SINR measurements, then E-UTRA RS-SINR is measured in the indicated subframes. |
| --- | --- |
| Applicable for | RRC_CONNECTED inter-RAT |

>>>>>>>>>>>> End text proposal >>>>>>>>>>>>

Agreements:

- resourceElementMappingPattern of CSI-RS-Resource-Mobility IE is identical to resourceMapping of the NZP-CSI-RS-Resource IE.

  - Note: it was already agreed earlier that the value range for resourceElementMappingPattern corresponds to single port only

R1-1803352

Agreements:

|  | Value Range |
| --- | --- |
| ssb-Index | 0, .., 63 |
| maxNrofCSI-RS-CellsRRM | 96 in case associated-SSB is configured 64 in case associated-SSB is not configured |
| MaxNrofPCIsPerSMTC | 64 |

Agreements:

- For reception of cell defining SSB from the target PCell, frequency offset is not required. NR only supports cell defining SSBs of PCell that are always on the synchronization signal raster.

- For reception of SSB for the target SCell, frequency offset signaling will be required for SSB not on the synchronization signal raster.

- FFS the case of PScell

- Agree to update the value range for starting PRB index of CSI-RS configuration for L3 mobility as follows:

| Measurement-BW | Allowed measurement BW for CSI-RS | BW size: {24, 48, 96, 192, 264} PRBs in CSI-RS numerology Starting PRB index: {0, 1, …, [251][2169](=274*8-24+1)]} PRBs with respect to PRB0 in CSI-RS numerology | A common set of values are assigned across all the resources configured per cell |
| --- | --- | --- | --- |

## MIMO

Agreement (New RRC parameter):

Agree on the value ranges and include the following table as part of the RRC parameter update.

| RRC parameter | Value range | Description |
| --- | --- | --- |
| ZP-CSI-RS-ResourceConfigType | periodic, semi-persistent or aperiodic | Time domain behavior of ZP-CSI-RS resource configuration |
| SP-ZP-CSI-RS-ResourceList | Up to 16 ZP CSI-RS resource set IDs per BWP | Contains list of SP ZP-CSI-RS resource set IDs Each resource set can have up to 16 ZP CSI-RS resources |

- The maximum number of ZP CSI-RS resources per CC is 32

- The maximum number of ZP CSI-RS resources per BWP is 16

- The maximum number of ZP CSI-RS resources per ZP CSI-RS resource set is 16

Agreement (RRC parameter update):

At most J=2 multi-CSI PUCCH resource can be configured to the UE per UL BWP

Agreement (RRC parameter update)

In group based beam reporting, NR supports the following aspects:

- The reporting format of differential L1-RSRP for non-group based beam reporting is reused for group based beam reporting.

- The RRC parameter nrofBeamsToReport can be removed (TS38.331). Corresponding RAN1 parameter name is number-of-beams-reporting.

Agreement:

UE expects a dedicated SearchSpace configuration that is one-to-one mapped to CORESET-BFR (RRC parameter CORESET-BFR remains)

Agreement (RRC parameter update):

Value range for the higher layer parameter BeamFailureInstanceMaxCount is {1,2,3,4,5,6,8,10}

Agreement (RRC parameter update):

Value range of beam failure recovery timer is {10, 20, 40, 60, 80, 100, 150, 200} in units of msec

- Additional values can be added by RAN2 (e.g. 500msec)

Agreement (RRC parameter update):

Value range of maximum number of CF BFRQ transmissions is {3, 4, 5, 6, 7, 8, 10, 20, 50}

- Additional values can be added by RAN2 (e.g. 100, 200)

Agreement:

RRC parameter value: maxNrofCandidateBeams is 64

- Note: the total number of distinct resources for candidate beam RS and beam management RS is also constrained by 64

Agreement (RRC parameters update):

Send as part of LS to RAN2 to inform that the parameters DL-DMRS-add-pos/UL-DMRS-add-pos, DL-DMRS-config-type/UL-DMRS-config-type, DL-DMRS-max-len/UL-DMRS-max-len can be configured separately for PDSCH/PUSCH Type A and PDSCH/PUSCH Type B.

Agreement (RRC parameter update):

SRS resource set configuration is supported per BWP. SRS is always transmitted in an active BWP within an active CC.

Agreement (New RRC parameter):

- The slot offset with respect to DCI that triggers an SRS resource set is higher layer configured in a per SRS resource set

  - FFS: Details on the support for aperiodic SRS for 1T4R

## Scheduling/HARQ aspects

Agreements:

- Remove variable slotFormatIndicator from PDCCH-Config from 38.331

- subcarrierSpacing and subcarrierSpacing2 currently in SlotFormatCombination should be moved to SlotFormatCombinationsPerCell

- Recommend RAN2 to change the variable name maxNrofAggregatedCellsPerCellGroup to maxNrofSlotFormatCombinationsPerCell.

  - Use a value of 16 for the above parameter

- For parameter maxNrofSlotFormatCombinationsPerSet, use a value of 4096

- For parameter maxNrofSlotFormatsPerCombination, use a value of 256

- For parameter maxSFI-DCI-PayloadSize, use a value of 128.

- For cell-specific DL/UL assignment, which has tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationCommon2 as shown below,

  - Clarify that when tdd-UL-DL-ConfigurationCommon2 is configured, the cell specific DL/UL pattern is a concatenation of the pattern specified in tdd-UL-DL-ConfigurationCommon and the pattern specified in tdd-UL-DL-ConfigurationCommon2, which is tdd-UL-DL-ConfigurationCommon + tdd-UL-DL-ConfigurationCommon2.

-- A cell-specific TDD UL/DL configuration.

tdd-UL-DL-ConfigurationCommon		TDD-UL-DL-ConfigCommon											OPTIONAL, -- Cond TDD

-- A second cell-specific TDD UL/DL configuration.

-- FFS_CHECK: What does the UE do with two? Which one applies? A union of both? If so, how?

tdd-UL-DL-ConfigurationCommon2		TDD-UL-DL-ConfigCommon											OPTIONAL, -- Cond TDD

Agreements:

- Restrict the combined periodicity for cell-specific DL/UL assignment to such that 20ms is a multiple of the combined periodicity

Agreements:

- 3-bit ARI for DCI 1_0 and DCI 1_1

  - At least 8 (up to 32) PUCCH resources can be configured in a resource set with

    - CCE-index-based implicit mapping is additionally used when >8 resources are configured.

    - Note: Increasing RRC value range from 8 to 32

  - 8 PUCCH resources are configured in a resource set with .

    - No implicit mapping

  - Note: Changing RAN1#91 agreement.

Agreements:

- Capture interpretation of ‘VRB-to-PRB mapping’ bit in 38.212 (0=non-interleaved, 1=interleaved).

- Non-interleaved mapping is used as default in 38.211 when no ‘VRB-to-PRB mapping’ bit is present in the DCI.

- The default bundling size is 2.

Agreements:

- For DL TBLBRM calculation, by default:

  - the reference value for “maximum number of layers for one TB” is ref = 2, and the reference value for “maximum modulation order” is Qm,ref = 6. The “Maximum number of PRBs across all configured BWPs of a carrier” corresponds to that of the initial BWP

- By default:

  - LBRM is disabled for PUSCH transmissions.

Agreements:

  - Clarify that:

| General parameter |  |  |
| --- | --- | --- |
| RAN2 parameter name | RAN1 feature | Guidance |
|  | SRS carrier switching | Per Cell |
|  | SUO | Per UE |

Note: RAN1 guidance “Per UE” means that the related RRC configuration is not BWP-specific or cell-specific.

| PDCCH-Config |  |  |
| --- | --- | --- |
| RAN2 parameter name | RAN1 parameter name | Guidance |
| slotFormatIndicator | SFI-SS in SFI-PDCCH | Same as search space configuration |
|  | Other parameters in SFI-PDCCH | Per Cell |

Note: RAN1 guidance on CORESET & search-space-config, “Per Cell (but association with each configured BWP is needed)” in last LS (R1-1801281) means:

  - CORESETs/search spaces from 0 to the maximum numbers (11 for CORESETs & 39 for search spaces) are configured per cell

  - However, within each BWP, a UE is further configured with the association of search spaces (up to 10 per BWP per previous RAN1 agreements) with a configured CORESET (up to 3 in total per BWP per previous RAN1 agreements).

| PDSCH-Config |  |  |
| --- | --- | --- |
| RAN2 parameter name | RAN1 parameter name | Guidance |
| codeBlockGroupTransmission | CBG-DL | Per Cell |
| maxCodeBlockGroupsPerTransportBlock | CBGs-per-TB-DL | Per Cell |
| codeBlockGroupFlushIndicator | CBG-DL-FI | Per Cell |

| PUSCH-Config |  |  |
| --- | --- | --- |
| RAN2 parameter name | RAN1 parameter name | Guidance |
| codeBlockGroupTransmission | CBG-UL | Per Cell |
| maxCodeBlockGroupsPerTransportBlock | CBGs-per-TB-UL | Per Cell |

## Channel coding

No relevant agreement

## NR-LTE co-existence

No relevant agreement

## UL power control

Working Assumption (RRC parameter update):

| Delta-preamble-msg3 | Power offset between msg3 and RACH preamble transmission, i.e. similar as in LTE | INTEGER (-1..6) with 1dB step size |
| --- | --- | --- |
| p0-pusch | P0 value for PUSCH with grant (except msg3) | INTEGER (-16..15) with 1dB step size |
| p0-pucch | P0 value for PUCCH | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f0 | deltaF values configurable for PUCCH format 0 | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f1 | deltaF values configurable for PUCCH format 1 | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f2 | deltaF values configurable for PUCCH format 2 | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f3 | deltaF values configurable for PUCCH format 3 | INTEGER (-16..15) with 1dB step size |
| deltaF-pucch-f4 | deltaF values configurable for PUCCH format 4 | INTEGER (-16..15) with 1dB step size |

Agreement (New RRC parameter):

For group-common TPC command in DCI format 2-2, when a UE is not configured with 2 closed loops, NR uses the same approach as LTE (i.e. TPC_index is configured by higher layers and used by the UE to determine the location of the TPC command in the DCI)

- New RRC parameter needs to be introduced