3GPP TSG RAN WG1 Meeting #92 	R1-1803389

Athens, Greece, February 26th – March 2nd, 2018

Agenda Item:	7.1.3.2.2

Source:	Huawei, HiSilicon

Title:	Summary for Al 7.1.3.2.2. PUCCH structure in long-duration

Document for:	Discussion and decision

# Introduction

As the first version of NR NSA specification is published, to resolve remaining issues and facilitate the completion of standardization for NR PUCCH structure in long duration, this summary serves two purposes based on the review of the submitted contributions for RAN1 92 meeting

- Identify necessary corrections and missing agreements in spec

- Resolve remaining issues essential to Rel-15 features

# Summary of necessary corrections and missing agreements

After reviewing the submitted Tdoc, some important corrections/missing arguments could be summarized as follows

- Correction on UCI repetition procedure

- Correction on PUCCH multiplexing

- SR only transmission on PUCCH format 1

## Key correction 1: Correction on UCI repetition procedure

In 9.2.6 of TS 38.213, the description of slot determination procedure for UCI repetition is not aligned with the agreement reached in RAN1#91

Agreement:

- When a UE receives a grant to transmit the long PUCCH over K slots where K is configured by higher layers, with duration of N symbol in each slot indicated by the DCI grant, with transmission starting in slot M, the UE is expected to do the following

  - If the UE receives the semi-static UL/DL configuration, the UE is expected to transmit the long PUCCH on the slot(s), starting from slot M, where the number of consecutive UL (FFS:  and/or Unknown) symbols >= N starting at the starting symbol indicated by PUCCH resource allocation. This continues until the UE has transmitted K slot(s) of long PUCCH.

  - If the UE does not receive any semi-static UL/DL configuration, the UE is expected to transmit long PUCCH over K consecutive slots, starting from slot M, starting at the starting symbol in each slot indicated by PUCCH resource allocation.

In the agreement, the consecutive UL symbols is equal to or larger than N. But in the 9.2.6 of TS38.213, the consecutive UL symbols is only equal to N. Also some grammatical mistakes need to be corrected.

Based on the supporting companies of each alternative, the following proposal could be considered

Proposal 2-1: Change the description of slot determination procedure for UCI repetition in 9.2.6 of TS 38.213 to align with the above agreement in RAN1#91.

Suggested TP is contained in [R1-1802689]

## Key correction 2: Correction on PUCCH multiplexing

In 3GPP RAN1 #88-bis meeting, multiplexing of different PUCCH formats were discussed and the following was agreed:

Agreements:

- Two NR-PUCCHs can be transmitted from one UE on the same slot in TDM manner.

  - The two NR-PUCCHs can be short-PUCCH.

  - The two NR-PUCCHs can be long-PUCCH + short-PUCCH.

  - FFS whether or not to have the two NR-PUCCHs as long-PUCCH + long-PUCCH

  - FFS: other multiplexing scheme(s) between the two NR-PUCCHs

  - FFS the case of more than 2 NR-PUCCHs in one slot from a UE (if more than 2, only short-PUCCHs)

In RAN1#90bis, the periodicities of SR for one UE were agreed as 2 symbols and 7 symbols within a slot. According to the above agreements, one UE can transmit one or more PUCCH(s) with the same and different PUCCH formats on different symbols in one slot. However as one slot has 14 symbols, the UE can only transmit at most two long PUCCHs in different symbols in one slot. To clarify the UE behavior on PUCCH multiplexing in time, it is therefore necessary to add UE restriction in the description of PUCCH multiplexing in time

Proposal 2-2:  In clause 9.2.3 of TS 38.213, add UE behaviour restrictions in the description of PUCCH multiplexing in time.

Suggested TP is contained in [R1-1802694]

## Key correction 3: Correction on SR only transmission on PUCCH format 1

Since SR only transmission on PUCCH format 1 has been supported, in 6.3.2.4.1 of TS 38.211, the complex-valued symbol  should be defined for this cases:

Proposal 2-3:  Similar as in LTE’s design, d(0) for NACK and (NACK, NACK),  can be used for SR only transmission.

Suggested TP is contained in [R1-1801340]

< Unchanged parts are omitted >

6.3.2.4.1	Sequence modulation

For SR only transmission on PUCCH format 1, information is carried by the presence/absence of transmission of PUCCH from the UE. In the remainder of this clause, d0=22（1+j）shall be assumed for SR only transmission on PUCCH format 1.

The block of bits  shall be modulated as described in clause 5.1 using BPSK if  and QPSK if , resulting in a complex-valued symbol . 
The complex-valued symbol  shall be multiplied with a sequence  according to

< Unchanged parts are omitted >

# Summary of key issues

After reviewing the submitted Tdoc, some remaining key issues are identified and summarized as follows

- Multiplexing other uplink physical signals or channels with long PUCCH repetition transmission

- Intra-slot frequency hopping pattern

- Determination of slots for multi-slot long PUCCH transmission

- Group re-ordering of new CGS

- Frequency-hop level group hopping

- Mapping direction in Frequency Domain for PUCCH format 2 and 3

- Multiple SRs overlapped with PF2/3/4

- Partial overlap of different PUCCH

- Pre-DFT-OCC for PUCCH format 4

- Reference payload size of CSI part 2

## Key issue 1: Multiplexing other uplink physical signals or channels with long PUCCH repetition transmission

When other uplink physical signals or channels collide with long PUCCH with repetitions in multiple-slot, how to treat the other uplink physical signals or channels is an open issue. The companies who are interested in this issue are shown their preference below from the submitted proposals.

- Drop other uplink physical signals or channels

  - vivo, Samsung

- If the start symbol of the PUSCH transmission is earlier than the very first symbol of the multi-slot PUCCH transmission, the PUCCH transmission in the corresponding slot is dropped and starts from the next available slot. Otherwise, the PUSCH transmission is dropped.

  - LG, Intel

In addition, the priority of the PUSCH need to be considered for example for URLLC and GF transmission. In such case, the PUCCH may be dropped or postponed.

Proposal 3-1:

When other uplink physical signals or channels collide with long PUCCH repetition transmission, consider the following options

- Drop other uplink physical signals or channels

- Drop or postpone PUCCH

## Key issue 2: Intra-slot frequency hopping pattern

In TS 38.213, the description of intra-slot hopping pattern is missing in Section 9.2.6. Using different frequency hopping pattern in different slot can improve the frequency diversity. The companies who are interested in this issue are shown below with their preference from the submitted proposals.

- The hopping patterns are different for different slots to improve the frequency diversity

  - AT&T (configure different hopping offset in different slot)

  - Huawei (Intra-slot hopping pattern can be switched between slots)

This issue needs to be discussed and concluded for the spec. The following options could be considered

Proposed agreement

For intra-slot hopping pattern for long PUCCH over multi-slots, down-select from the following options

- Option 1: The intra-slot hopping pattern is the same for each slot

  - DCM, Samsung, QC, LGE, Intel

- Option 2: The intra-slot hopping pattern can be different for each slot

  - AT&T, HW

Discussion

- Option 1: Simple, no spec change and no new RRC, may have less frequency diversity gain

- Option 2 has two proposals

  - Configured multiple sets of PRBs (>2) for multiple slots

    - New RRC configuration, better frequency diversity

  - Switching two sets of PRB from PUCCH resource allocation as hopping patterns for each slot

    - No RRC impact, limited frequency diversity gain

## Key issue 3: Determination of slots for multi-slot long PUCCH transmission

According to the agreements at RAN1#AH_1801, PUCCH over multi-slots can be transmitted by the UE on unknown symbol(s) indicated by semi-static DL/UL assignment configuration. However, the unknown symbol(s) can also be used as the DL/UL switching gap or configured to transmit SS/PBCH block. So when the UE determines slots for transmitting long PUCCH on multiple slots, the slots should be determined by considering one or two unknown symbols(s) as the DL/UL switching gap and excluding semi-static unknown symbols configured to transmit SS/PBCH block [WILUS Inc, R1-1802935]. For that, the following proposal could be considered

Proposal 3-3:

- When the UE determines slots for transmitting long PUCCH on multiple slots, the slots should be determined by considering one or two flexible symbols(s) as the DL/UL switching gap and excluding semi-static unknown symbols configured to transmit SS/PBCH block.

Purpose:

- To clarify that certain flexible symbols shall not be used for PUCCH transmission

- Two cases are identified

    - Flexible symbols configured for SS/PBCH block

    - Flexible symbols used as DL/UL switching gap

Conclusion

- TS38.213, Section 11.1 should capture that the UE does not transmit PUCCH in symbols indicated by semi-static configuration for SS/PBCH reception.

- UE only start to transmit PUCCH from the X-th flexible symbol after the last downlink symbol.

  - X-th symbol is decided by RAN4

## Key issue 4: Group re-ordering of new CGS

In NR, base sequences with different lengths are divided into 30 groups. In order to reduce inter-cell interference, the cross-correlation of sequences with different lengths from different groups should be minimized. However, after introducing new sets of CGS in NR to reduce the PAPR, re-ordering (grouping) is not done to achieve the abovementioned purpose. The consequences would be that even though PAPR within each cell is reduced as compared with LTE, the inter-cell interference could still be high due to the lack of necessary re-ordering (grouping). Such inter-cell interference could be seen in some scenarios

- PF0/1 UCI and PF1 DMRS collide with DMRS of PF3

- PF3 DMRS collide with PF3 DMRS with different lengths (due to different number of PRB allocated)

- DMRS of PUSCH collide with DMRS of PUSCH with different length

The proposals to re-ordering of CGS among groups are proposed by [Huawei, R1-1803308] and [ZTE, R1 -1801626], which could be summarized as follows

Proposal 3-4:

- Group re-ordering of length-24, length-18 and length-12 CGS should be supported.

  - FFS:  detail of re-ordering

Discussion:

- Reduce the inter-group cross-correlation among CGS sequences with different lengths (otherwise the inter-cell interference could be large)

- No new CGS sequences introduced but simply re-assign CGS sequences in different groups

Conclusion

- Continue discussion and proponents (HW and ZTE) will provide single solution within this week

## Key issue 5: Frequency-hop level group hopping

In LTE, the base-sequence was hopped on each frequency-hop (slot boundary). One of the frequency-hops has the same base sequence, while the base sequence in the other frequency hop can be different. To maintain the similar robustness of inter-cell interference as LTE PUCCH, frequency-hop level base sequence hopping could be considered for NR

- Support

  - NTT DCM, Panasonic, ZTE.

Based on the supporting companies of each alternative, the following proposal could be considered

Proposal 3-5:

- Frequency-hop level base sequence hopping is supported.

## Key issue 6: Mapping direction in Frequency Domain for PUCCH format 2 and 3

The resources in frequency domain for a PUCCH resource are given by the starting PRB index for each hop in case of frequency hopping and the maximum number of allocated PRBs which is the same for both hops. Moreover, as per agreement below the number of utilized PRBs depends on the payload size and the max configured code rate and can be less than the number of configured PRBs. In case a PUCCH hop is close to the upper edge of UL BWP, mapping upward in frequency domain can result in inefficient resource utilization since it would be complicated to efficiently use the configured and unused resources for other purposes.

For that, the following proposal can be considered as proposed in [Ericsson, R1-1803234] and further updated during the session,

Proposal 3-6:

- If Frequency hopping is enabled,

  - For the hop with a PRB start index closer to the upper edge of the UL BWP, the mapping of the PUCCH REs in FD is done in decreasing order of the sub-carrier indexing.

  - For the other hop, the mapping of the PUCCH REs in FD is done in increasing order of the sub-carrier indexing.

- For PUCCH transmissions using format 2 or 3 with frequency hopping enabled

  - For the hop with a PRB start index ((i.e. startPRB in the PUCCH resource configuration) closer to the upper edge of the UL BWP, the mapping of the PUCCH REs in FD starts from the PRB with startPRB+(N-W) index where

    - W is the number of PRBs determined by UE for the UCI transmission on PUCCH and N is the number of configured PRB

  - For the other hop, the mapping of the PUCCH REs in FD starts from the PRB with startPRB index.

## Key issue 7: Multiple SRs overlapped with PF2/3/4

In last meeting, a conclusion on multiple SRs overlapped with PF2/3/4 was agreed:

Agreements:

- If a UCI transmission on a PUCCH form a UE using format 2 or 3 or 4 overlaps in time with K PUCCH resources, each configured for a SR, X bits are used to represent a SR being indicated by the UE and appended to the end of HARQ-ACK followed by CSI.

  - Note: X is used to indicate both the presence or absence of SR and which of the K configured SR is embedded.

    - FFS: X=ceil(log2(K+1))

- FFS PUCCH formats 0 & 1

Based on the agreement, value of X need to be FFS, and there are two options:

- X = ceil(log2(K+1)) where K is the number of PUCCH resource configured for SR overlap with PF2/3/4

Support: Lenovo, Intel, Samsung, sharp, HW

- X = ceil(log2(M+1)) where M is the number of configured SR resource

Support: Ericsson.

As up to 8 SR resource could be configured, so total X=4 bits needs to be used all the time to indicate which SR is transmitted (or none of them are transmitted), while K could vary depend on the number of SR resource actually colliding with PF2/3/4, and therefore, fewer bits could be needed overall.  Given that, the following proposal could be considered

Proposal 3-7:

- X = ceil(log2(K+1)), where K is the number of PUCCH resource configured for SR overlap with PF2/3/4

## Key issue 8: Partial overlap of different PUCCH

Unlike in LTE, the start and duration of PUCCH in NR is very flexible and dynamic, it is very likely that different PUCCH could collide with each other fully or partially even with the efforts of gNB on their scheduling to avoid that. For example, semi-static configured SR transmission may collide with very dynamically scheduled PUCCH to carry HARQ-ACK. To solve such collisions, some rules need to be defined and followed by both gNB and UE to reduce the misunderstanding on what is actually transmitted in the end.

This issue was discussed extensively at last meeting (RAN 1 Ad-Hoc 1801), however, due to the complication of the issue from various combination/aspects, no clear conclusion can be drawn. In this meeting, through review of the submitted contributions, the following observations could be summarized

- For SR and HARQ-ACK partial overlap

Table 1. Partial Overlap of SR and HARQ-ACK

|  | AN with format 0 | AN with format 1 | AN with format 2/3/4 |
| --- | --- | --- | --- |
| SR with format 0 | Drop SR at least overlapped symbols: Lenovo (or AN), CATT (SR after), Intel (drop all SR when k>1), Nokia (logical OR over all K SR processes) Drop SR or AN depends on priority Panasonic Multiplexing on the AN resource CATT (SR before), Ericsson, Oppo, Intel (k=1), LGE, Panasonic Change CS of overlapping symbols on AN resource ZTE | Drop SR at least overlapped symbols: Lenovo (or AN), CATT, Ericsson (all SR is dropped), Intel (drop all SRs) Drop SR or AN depends on priority Panasonic, ZTE(drop AN hop containing overlapping symbol if SR prioritize) Multiplexing: Panasonic Bundle HARQ-ACK Oppo Change CS of overlapping symbols on AN resource ZTE | Multiplexing (jointly coding) on AN resource: Lenovo (SR before), CATT, Ericsson, Oppo, LGE, Nokia (AN PF2 and SR PF0), Panasonic, ZTE, Intel Puncture short into long: Oppo Drop SR at least overlapped symbols: Lenovo (SR after) Drop AN: Oppo (SR before) Drop SR or AN depends on priority Panasonic |
| SR with format 1 | Drop SR at least overlapped symbols: Lenovo (or AN), CATT (SR before), Intel (drop all SR when k>1) Multiplexing on the AN resource Ericsson, Oppo (SR after), Intel (k=1), LGE Multiplexing on the SR resource CATT(SR after), Drop AN: Oppo (SR before) Change CS of overlapping symbols on SR resource ZTE | Drop SR at least overlapped symbols: Lenovo (or AN),CATT (SR before), Ericsson (not fully contained), Intel (drop all SRs) Drop SR or AN depends on priority Panasonic, ZTE(drop SR hop containing overlapping symbol if SR prioritize) Multiplexing on the AN resource Erisson ( SR is fully contained), Panasonic Multiplexing on the SR resource CATT(SR after), Channel selection (similar to LTE): Oppo, LGE |  |

- For HARQ-ACK/SR and CSI partial overlap

Table 2. HARQ-ACK/SR and CSI partial overlap w/o scheduling a new resource

|  | AN/SR with format 0/1 | AN/SR with format 2 | AN/SR with format 3/4 |
| --- | --- | --- | --- |
| CSI with format 2 | Jointly Coding on CSI resource: Lenovo (SR only and before), Intel (for SR only case append 1 bit at the beginning of CSI bits to indicate presence/absence of SR) Drop SR at least overlapped symbols: Lenovo (SR only and after), | Jointly Coding on CSI resource: Lenovo (SR only and before), Drop SR at least overlapped symbols: Lenovo (SR only and after), | Jointly Coding on CSI resource: Lenovo (SR only and before), Drop SR at least overlapped symbols: Lenovo (SR only and after), |
| CSI with format 3/4 | Jointly Coding on CSI resource: Lenovo (SR only and before), Intel (for SR only case append 1 bit at the beginning of CSI bits to indicate presence/absence of SR) Drop SR at least overlapped symbols: Lenovo (SR only and after), | Jointly Coding on CSI resource: Lenovo (SR only and before), Drop SR at least overlapped symbols: Lenovo (SR only and after), | Jointly Coding on CSI resource: Lenovo (SR only and before), Nokia (Part 1 only) Drop SR at least overlapped symbols: Lenovo (SR only and after), Puncturing short into long Nokia |

Table 3. HARQ-ACK/SR and CSI partial overlap w/ scheduling a new resource

|  | AN/SR & CSI with format 2 | AN/SR & CSI with format 3/4 |
| --- | --- | --- |
| CSI with format 2 | Jointly Coding on a resource with configured max code rate: CATT, Ericsson |  |
| CSI with format 3/4 | Jointly Coding CSI Part 1on a resource with configured max code rate and drop CSI part 2: CATT, Ericsson | Jointly Coding CSI part 1on a resource with configured max code rate and separately coding CSI part 2: CATT, Ericsson |

Other signals/channel collisions including SRS and PUCCH. In general, a couple of options have been discussed which can be considered

- Drop one PUCCH channel/signal completely or partially

- Multiplexing different PUCCH (or rate matching PUCCH)

- Puncturing one PUCCH and transmit the other in overlap period

To determine which channel is dropped or punctured, some priorities are also proposed

- Timeline priority (e.g., whoever scheduled to be transmitted first will be kept)

- UCI priority (e.g., HARQ-ARQ > SR > CSI)

All these aspects could be further discussed during the meeting in order to move forward.

## Key correction 9: Pre-DFT-OCC for PUCCH format 4

In [QC, R1-1802838], it was observed that for PUCCH format 4, the /2 Pi BPSK with pre-DFT-OCC index 0 and 2 will lead to higher PAPR, therefore it is proposed to replace them with two new OCC as follows

-> [+1, e-jπ6, e-jπ3, +1, e-jπ6, e-jπ3, +1, e-jπ6, e-jπ3, +1, e-jπ6, e-jπ3]

-> [+1, e-jπ6, e-jπ3, -1, -e-jπ6, -e-jπ3, +1, e-jπ6, e-jπ3, -1, -e-jπ6, -e-jπ3]

Companies are encouraged to express their view on this proposal.

Discussion

- Benefits:

  - Bring reduced PAPR at least for Pi/2 BPSK

  - Maintain the IFDM structure in frequency domain

- Comments

  - Whether we need PAPR reduction in PF4 for 4 users

  - Whether this will be used only for Bi/2 BPSK

Conclusion

- Continue discussion within this week and come back later

## Key correction 10: Reference payload size of CSI part 2

Since rank of CQI is determined by UE, payload size of CSI part 2 is unknown to gNB. At least for the following two cases:

Case 1: Dynamic HARQ-ACK and P-CSI on dynamical configured HARQ-ACK resource

Case 2: HARQ-ACK for SPS PDSCH and P-CSI on semi-statically configured P-CSI resource

When gNB configures PUCCH resource for UE, gNB cannot calculate the PUCCH resource caused by the unknown payload size. So a reference payload size of CSI part 2 is needed.

Proposal 3-10:

- PUCCH resource (set) selection and determination of PRB number for UCI transmission on PUCCH format 3/4 is done based on a reference payload size of CSI part 2.

  - FFS: how to determine reference payload size of CSI part 2.

Opt 1: When calculating the resource for CSI part 2 on PUCCH or PUSCH, rank 1 is assumed as the reference payload size of CSI part 2.

Opt 2: The reference numbers of CSI part 2 bits for PUCCH resource (set) and RB determination are set differently based on the expected HARQ-ACK payload bits as follows:

- Possible minimum number of CSI part 2 bits in case of HARQ-ACK/SR and CSI on PUCCH format 3 and 4 configured for HARQ-ACK

- Possible maximum number of CSI part 2 bits in case of HARQ-ACK/SR and CSI on PUCCH format 3 and 4 configured for CSI reporting

Opt 3: Reference payload size of CSI part 2 is determined by the RRC configured parameter, e.g. assumed RI.

# Contribution List

7.1.3.2.2 PUCCH structure in long-duration

| R1-1801340 | Remaining issues on PUCCH structure in long-duration | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1801536 | Remaining issues on PUCCH with long-duration | vivo |
| R1-1801626 | Rmaining issues for Long PUCCH | ZTE, Sanechips |
| R1-1801661 | Remaining issues of long-PUCCH | MediaTek Inc. |
| R1-1801732 | Remaining issues on long PUCCH formats | CATT |
| R1-1801979 | Corrections on PUCCH for more than 2 Symbols | Samsung |
| R1-1802024 | Remaining open items on long PUCCH | Nokia, Nokia Shanghai Bell |
| R1-1802106 | Remaining issues on long PUCCH | Guangdong OPPO Mobile Telecom |
| R1-1802211 | Remaining issues on long-duration PUCCH | LG Electronics |
| R1-1802411 | PUCCH and PUSCH collision handling | Intel Corporation |
| R1-1802484 | PUCCH structure in long-duration | NTT DOCOMO, INC. |
| R1-1802511 | Discussion on PUCCH base sequence hopping | Panasonic Corporation |
| R1-1802838 | Remaining issues for long PUCCH | Qualcomm Incorporated |
| R1-1802907 | On PUCCH Remaining Issues | Ericsson |
| R1-1802935 | Remaining issues on long PUCCH over multiple slots | WILUS Inc. |

7.3.2.5  Other

| R1-1801784 | PUCCH resource allocation for non-slot scheduling | Huawei, HiSilicon |
| --- | --- | --- |
| R1-1801788 | Discussion on partially overlapped PUCCH and PUSCH | Huawei, HiSilicon |
| R1-1802249 | Collision between PRACH transmission and SR transmission | ASUSTEK COMPUTER (SHANGHAI) |
| R1-1802689 | Correction on UCI repetition procedure in 38.213 | Huawei, HiSilicon |
| R1-1802690 | Handling of PUCCH transmsision with partial overlap | Huawei, HiSilicon |
| R1-1802691 | Group re-ordering of new CGS | Huawei, HiSilicon |
| R1-1802692 | Transmission of PUCCH and PUSCH with partial overlap | Huawei, HiSilicon |
| R1-1802693 | Aperiodic CSI feedback on PUCCH in NR | Huawei, HiSilicon |
| R1-1802694 | Correction on PUCCH multiplexing in 38.213 | Huawei, HiSilicon |
| R1-1802695 | Correction on DMRS for PUCCH in 38.211 | Huawei, HiSilicon |
| R1-1802696 | Intra-slot hopping pattern for long PUCCH over multiple slots | Huawei, HiSilicon |
| R1-1802713 | TP on UCI multiplexing between UL and SUL | Huawei, HiSilicon |
| R1-1802727 | Discussion and text proposal for HARQ-ACK on PUCCH format 0/1 | Huawei, HiSilicon |

# Appendix: Past meeting agreement

Agreements:

- Msymb is to be replaced by NSFPUCCH,sMsymb in  and  in clause of 6.3.2.6.4  in TS 38.211

Agreements:

- Revise bqi to bi and cqi to ci. for UCI scrambling for PUCCH format 3 & 4 in clause of 6.3.2.6.1 of TS 38.211.

Agreements:

- To introduce a linkage in the spec between CS index of DMRS and index of OCC for pre-DFT-OCC (PUCCH PF4) as following

- 0,6,3,9 (CS index of DMRS ) ->  0,1,2,3 (index for OCC) for SF = 4

- 0,6 (CS index of DMRS ) -> 0,1  (index of OCC)  for SF =2

Agreements:

- Revise “length of the orthogonal cover code” to “Spreading factor of PUCCH format 4” in clause of 9.2.1 of TS38.213 for multiplexing capacity of PUCCH format 4.

Agreements:

- For PUCCH format 1, time domain OCC index obtained from PUCCH resource is used as OCC index for both UCI and DMRS within each hop when frequency hopping is enabled or within the duration of PUCCH when frequency hopping is disabled.

Agreements:

- The ‘unknown’ symbols in semi-static DL/UL assignment can be used for long PUCCH transmission over multiple slots when a UE receives a grant to transmit the long PUCCH

Agreements:

- Add to clause of 6.3.2.1, 38.211 to clarify the hopping boundary in time when frequency hopping is enabled for PUCCH format 1, 3 and 4, the number of symbols in the first hop is given by floor(PUCCH length in symbols/2).

Agreements:

For inter-slot hopping pattern for long PUCCH over multiple slots, the following options can be down-selected

- A first PRB for PUCCH transmission is provided by higher layer parameter PUCCH-starting-PRB and a second PRB for PUCCH transmission is provided by higher layer parameter PUCCH-2nd-hop-PRB (already agreed)

  - After long PUCCH over multiple slot starts, the first PRB is used for PUCCH transmssion in the even slot, the second PRB is used for PUCCH transmssion in the odd slot.

    - The slot index is counted after long PUCCH starts regardless if it is actually used for long PUCCH transmssion over multiple slots