TSG-RAN WG1 #92	R1-1803393

Athens, Greece, February 26 – March 2, 2018

Source:	Ericsson

Title:	Outcome of offline session on RACH and DCI

Agenda Item:	7.1.3.1.4

Document for:	Discussion and Decision

# Random-access response

RAN1#91 agreements

Agreements:

- UE adjusts its power setting for Msg. 3 using the transmit power control command in Msg2 and the transmit power of the latest PRACH preamble

- The size of UL grant in RAR for Msg3 is left to control channel session (Scheduling/HARQ agenda item)

Questions:

- Is 48 bits, leaving 48-16-12=20 bits enough for the NR grant in msg 2?

- What fields should be included? What are their respective size?

Offline proposal:

- Increase total number of bits from 48 to 56. Send LS to RAN2.

- Working assumption: Bit fields according to the table below.

- Need to check the entries in the time domain allocation table to fulfill ITU requirements.

|  | LTE | NR | Comment |
| --- | --- | --- | --- |
| Reserved | 1 | 3 |  |
| Timing advance | 11 | 12 | Agreed in 38.321 |
| Hopping flag | 1 | 1 | Need to ensure FH is mandatory for the UE, at least for msg3. |
| Frequency domain RA | 10 | 12 | Resource allocation type 1. Reuse LTE wording on interpretation (but with numbers adjusted accordingly) If FH, the first 1-2 bits are used for hopping information. |
| Time domain RA | 1 | 4 | Point to entries in the time allocation table. Check with resource allocation session regarding the number of bits. |
| Truncated MCS | 4 | 4 | Same MCS table as for “normal” transmission without 256QAM, only lowest part used. |
| TPC command | 3 | 3 | -6 dB to 8 dB with a 2 dB step size. Check with power control session. |
| CSI request | 1 | 1 | Only for contention-free RA in Rel-15. A CSI configuration to be used in case the bit is set needs to be defined/configured. |
| TC-RNTI | 16 | 16 |  |
| Total | 48 | 56 |  |

Discussion:

Increase the number of bits to 28. Inform RAN2.

Need to define interpretation of FH bit(s).

Need to check the entries in the time domain allocation table to fulfill ITU requirements.

# DCI contents of msg 2

DCI format 1-0 with RA-RNTI is used to schedule msg 2.

Offline proposal: working assumption according to the table below

| Field | Bits (from latest 38.212, may be updated during RAN1#92) | Comment |
| --- | --- | --- |
| Identifier for DCI formats | 1 | Reserved |
| Frequency domain resource assignment |  |  |
| Time domain resource assignment | ? | Use default time allocation table |
| VRB-to-PRB mapping | 1 |  |
| Modulation and coding scheme | 5 | Use MCS table without 256QAM (UE capabilities not yet known) |
| New data indicator | 1 | Reserved |
| Redundancy version | 2 | Reserved |
| HARQ process number | 4 | Reserved |
| Downlink assignment index | 2 | Reserved |
| TPC command for scheduled PUCCH | 2 | Reserved |
| PUCCH resource indicator | 2 | Reserved |
| PDSCH-to-HARQ_feedback timing indicator | 3 | Reserved |

# DCI content for msg 3 retransmission

Msg 3 retransmission: use DCI format 0-0 with TC-RNTI

Offline proposal: working assumption according to the table below

| Field | Bits (from latest 38.212, may be updated during RAN1#92) | Comment |
| --- | --- | --- |
| Identifier for DCI formats |  | Indicate UL |
| Frequency domain resource assignment |  |  |
| Time domain resource assignment |  |  |
| Frequency hopping flag |  |  |
| Modulation and coding scheme |  | Use UE-capability-independent MCS table |
| New data indicator |  | Reserved |
| RV |  |  |
| HARQ proc |  | Reserved. HARQ process 0 is always used. |
| TPC for PUSCH |  |  |
| UL/SUL |  | Reserved (retransmission on same carrier as initial transmission) |

# DCI content for msg 4

Msg4 using DCI format 1-0 with TC-RNTI

Offline proposal: working assumption according to the table below

| Field | Bits (from latest 38.212, may be updated during RAN1#92) | Comment |
| --- | --- | --- |
| Identifier for DCI formats | 1 | Indicate DL |
| Frequency domain resource assignment |  |  |
| Time domain resource assignment | ? |  |
| VRB-to-PRB mapping | 1 |  |
| Modulation and coding scheme | 5 | Use UE-capability-independent MCS table Assume LBRM in the UE? Check! |
| New data indicator | 1 |  |
| Redundancy version | 2 |  |
| HARQ process number | 4 |  |
| Downlink assignment index | 2 | Reserved from functionality point-of-view (ongoing discussion in PUCCH session whether to use these bits for other purposes or not) |
| TPC command for scheduled PUCCH | 2 |  |
| PUCCH resource indicator | 2 |  |
| PDSCH-to-HARQ_feedback timing indicator | 3 | Used but potentially different interpretation |