TSG-RAN WG1 #92	R1-1803395

Athens, Greece, February 26 – March 2, 2018

Source:	Ericsson

Title:	TP for 38.212 from agenda point 7.1.3.3.1

Agenda Item:	7.1.3.3.1

Document for:	Discussion and Decision

# VRB-to-PRB interleaver

Agreements:

- Capture interpretation of ‘VRB-to-PRB mapping’ bit in 38.212 (0=non-interleaved, 1=interleaved).

- Non-interleaved mapping is used as default in 38.211 when no ‘VRB-to-PRB mapping’ bit is present in the DCI.

- The default bundling size is 2.

--- Begin text proposal (for UL) ---

##### 7.3.1.1.2	Format 0_1

##### […]

-	VRB-to-PRB mapping – 0 or 1 bit

-	0 bit if only resource allocation type 0 is configured or if PUSCH-tp=Enabled;

-	1 bit according to Table 7.3.1.1.2-33 otherwise, only applicable to resource allocation type 1, as defined in Subclause 6.3.1.7 of [4, TS38.211].

[…]

Table 7.3.1.1.2-33: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

--- End text proposal ---

--- Begin text proposal (for DL) ---

##### 7.3.1.2.1	Format 1_0

##### […]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

##### 7.3.1.2.2	Format 1_1

-	VRB-to-PRB mapping – 0 or 1 bit

-	0 bit if only resource allocation type 0 is configured;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Subclause xxx of [4, TS38.211].

[…]

Table 7.3.1.2.2-5: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

--- End text proposal ---