TSG-RAN WG1 #92	R1-1803396

Athens, Greece, February 26 – March 2, 2018

Source:	Ericsson

Title:	TP for 38.214 from agenda point 7.1.3.3.1

Agenda Item:	7.1.3.3.1

Document for:	Discussion and Decision

# RBG sizes

Agreements:

- Adopt the following RBG sizes.

| BWP size (RBs) | RBG size |  |
| --- | --- | --- |
|  | Config 1 | Config 2 |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

----- Text proposal ----

##### 5.1.2.2.1	Downlink resource allocation type 0

In downlink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive physical resource blocks defined by higher layer parameter rbg-Size configured for PDSCH and the size of the carrier bandwidth part as defined in Table 5.1.2.2.1-1.

Table 5.1.2.2.1-1: Nominal RBG size P

| Carrier Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

--- End text proposal ---

# RNTIs

Agreements:

- Update 38.214 such that the same MCS table and TB size determination mechanism for data channels as defined for the C-RNTI is used for TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI

  - Note: there could be limitation(s) for TBS/MCS determination, e.g., modulation order limit for certain cases, etc.

----- Text proposal ----

#### 5.1.3.1	Modulation order and target code rate determination

For the PDSCH assigned by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI,,if the higher layer parameter MCS-Table-PDSCH is not set to '256QAM',

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

end

#### 5.1.3.2	Transport block size determination

For the PDSCH assigned by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI, if the higher layer parameter MCS-Table-PDSCH is set to '256QAM' and  0 ≤ IMCS ≤27, or the higher layer parameter MCS-Table-PDSCH is not set to '256QAM' and  0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below:

#### 6.1.4.1	Modulation order and target code rate determination

For the PUSCH is assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI,

If transform precoding is disabled and MCS-Table-PUSCH is not set to '256QAM',

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif transform precoding is disabled and MCS-Table-PUSCH is set to '256QAM',

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

#### 6.1.4.2	Transport block size determination

For the PUSCH is assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI,

if

--- End text proposal ---